3GPP TSG-RAN WG1 #90bis	R1-171xxxx

Prague, Czech Republic, October 9 – 13, 2017

Source:	Intel Corporation

Title:	Evaluations for Polar DL Channel interleaver

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

# Introduction

We report performance evaluations with channel interleaver and no channel interleaver for the Polar DL.

# Evaluation assumptions and results

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Waveform | OFDMA |
| Numerology | 15 kHz |
| Payload (not including CRC) | 32, 60 bits |
| FEC type and Modulation | Polar with CRC size =24, QPSK |
| Tx-Rx antenna configuration | 2x2 |
| Transmit diversity scheme | 1-port per REGB precoder cycling |
| Channel estimation | 1/4 DM-RS density, practical channel estimation (MMSE) |
| Channel model | TDL-C 300ns |
| Number of REGs per CCE | 6 |
| Aggregation levels | 1, 8 |
| REG bundle size | 2 REGs, 6 REGs |
| CORESET configuration | 1 symbol, 48 PRBs (i.e. PRB0,PRB1…PRB47) |
| CCE-to-REG mapping | Frequency first |
| Interleaving for CCE-to-REG mapping | For evaluation only, Sub-block interleaver operating on REG bundles |

We make the following observations based on the results in Figures 1 and 2, and make a recommendation.

Observation 1: For 32 bits/AL1/ 2 REGs per REGB, performance gains of DL channel interleaver over no channel interleaver are present.

Observation 2: For 32 bits/AL1/ 6 REGs per REGB, no significant performance difference between no DL channel interleaver and DL channel interleaver is observed.

Observation 3: For AL8, no significant performance difference between no DL channel interleaver and DL channel interleaver is observed.

Observation 4: For 60 bits, no significant performance difference between no DL channel interleaver and DL channel interleaver is observed.

Observation 5: For AL8, no significant performance difference between no DL channel interleaver and DL channel interleaver is observed.

Proposal : Given the performance benefits of DL channel interleaver over no DL channel interleaver are not significanct except for small block sizes and AL1 and smaller REG bundle size, it is recommended to NOT adopt channel interleaver for DL Polar code.

Figure 1. Performance of DCI =32 bits, with and without channel interelaver for two different REGB sizes (2 and 6).

Figure 2. Performance of DCI =60 bits, with and without channel interelaver for two different REGB sizes (2 and 6).

Conclusion