3GPP TSG RAN WG1 Meeting #92  	R1-1801992

# Athenes, Greece, Feburary 26th – March 2nd, 2018

Agenda item:	7.1.4.2

Source:	Samsung

Title: 				Remaining Issues on NR Polar Coding

Document for:		Discussion and Decision

1. Introduction

In this contribution, we address some remaining issues on NR polar coding.

First, UE-specific scrambling sequences for DCI is reviewed. According to the agreements, two different UE-specific scrambling schemes are applied to PDCCH transmission in user-specific search space. Two scrambling schemes independently performed may be cancelled out when they are combined, so we check the performance of the current agreement.

Second, we reviewed the zero-padding for short DCI. It was agreed that zero-padding is applied to short DCI smaller than 12 bit to utilize polar coding and CRC coding. Zero padding usually means putting zeros after the payload, and it is also considered in the current technical specification. If the decoder takes full advantage of the information about zero-padding bits, the error-correcting performance varies depending on the locations of the zero-padding bits. We evaluate the performance of short DCI depending on the locations of zero-padding bits.

2. Scrambling Sequence for DCI

In [1]-[3], we had agreements on how to generate PDCCH scrambling sequences.

Agreement (RAN1 #90bis): - RNTI is masked onto the last NRNTI CRC bits on the PDCCH, where NRNTI is the number of bits of the RNTI Agreement (RAN1 #91): - Confirm the working assumption that there is no additional UE-specific scrambling motivated by channel coding. Agreements (RAN1 NR-AH4, Jan. 2018): - Adopt following text proposal for PDCCH scrambling (TS38.211 Section 7.3.2.3) The scrambling sequence generator shall be initialized with cinit=nRNTI⋅216+nID mod 231 Where - nID∈0,1,…,216-1 equals the higher-layer parameter Control-scrambling-Identity if configured and RNTI is equal to C-RNTI, nID=NIDcell otherwise, and - nRNTI is C-RNTI for PDCCH in USS, while is 0 for PDCCH in CSS. Companies can check whether the above agreements are consistent with previous coding discussion/agreement or not.

According to the agreements, two UE-specific scrambling schemes are considered for DCI. First, UE-specific RNTI is mapped onto the last 16 CRC parity bits before polar encoding. After polar encoding and rate-matching, a Gold sequence generated by linear feedback shift register (LFSR) with nRNTI initialization is exploited to scramble codewords in user-specific search space. One may be concerned that two independent scrambling schemes can be cancelled out when they are combined. In order to confirm the current agreement, in this section, we evaluate cross-correlation of scrambling sequences, which is a general metric for evaluating scrambling sequences.

Consider a polar code of size N=2n and the number of codeword bits after rate-matching is denoted by M. As in the convention, we write u, x, and F⊗n to denote an encoder input vector, a rate-matching output vector, and a polar code generator matrix, respectively. In addition, we define a function R:F2N→F2M to describe the NR polar code rate-matching procedure [4], where F2 is the binary field. In PDCCH transmission, a codeword vector after rate-matching and scrambling is given by

x=Ru+uRNTIF⊗n+sGold=RuF⊗n+RuRNTIF⊗n+sGold,

where uRNTI∈F2N is a vector obtained by masking RNTI onto the last 16 CRC parity bit locations in a zero vector and sGold∈F2M is a Gold sequence generated by LFSR initialized. In the equation, RuF⊗n is a desired codeword vector, and RuRNTIF⊗n can be seen as an effective scrambling sequence generated by CRC-masking and polar coding. Thus, the final scrambling sequence is RuRNTIF⊗n+sGold according to the current agreement.

If the mother polar code size and the rate-matching scheme are decided, the locations of the last 16 CRC parity bits are independent of the number of information bits K. Even though a distributed-CRC (D-CRC) scheme is adopted for DCI, the last 16 CRC parity bits are not distributed and interleaved. In addition, these CRC parity bits are always allocated at the most reliable locations in the polar code input vector u. Thus, the locations of the last 16 CRC parity bits only depend on the mother code size and the rate-matching scheme (puncturing, shortening, and repetition). Table 1 shows the indices of these last 16 CRC parity bits in a vector u according to the codeword length M and the rate-matching scheme. We only need to consider these 6 cases to calculate RuRNTIF⊗n.

Table 1  Indices of the Last 16 CRC Parity Bits in Polar Code Input Vector

| Codeword length M | Mother code size N | Rate-matching | Indices of the last 16 CRC parity bits (index begins from 1) |
| --- | --- | --- | --- |
| 108 | 128 | Puncturing | 113:128 |
|  |  | Shortening | 93:108 |
| 216 | 256 | Puncturing | 241:256 |
|  |  | Shortening | 201:216 |
| 432 | 512 | Puncturing | 497:512 |
| 864 | 512 | Repetition | 497:512 |

To check whether two independent scrambling techniques conflict or not, we calculate cross-correlation for all possible pairs of two different scrambling sequences. The number of all possible pairs of two different RNTIs is about 2.1475×109. Given two different scrambling sequences si and sj, we calculate the cross-correction based on the ±1 convention, and it is given by k=0M-1sik×sjk. Two scrambling schemes are compared: Gold sequence only and CRC-masking plus Gold sequence. Only the results with NID=0 are shown in this paper, but we confirm by numerous experiments that similar results are obtained even though we set different NID.

In Fig. 1 to Fig. 6, the cross-correlation distributions are depicted. We calculate mean and variance of each cross-correlation distribution, and the results are also described in each corresponding figure. As shown in the figures, the combined technique in the current agreement achieves a similar level of randomization to the Gold sequence only case.

Figure 1  Cross-correlation of two different scrambling sequences (M=108, puncturing)

Figure 2  Cross-correlation of two different scrambling sequences (M=108, shortening)

Figure 3  Cross-correlation of two different scrambling sequences (M=216, puncturing)

Figure 4  Cross-correlation of two different scrambling sequences (M=216, shortening)

Figure 5  Cross-correlation of two different scrambling sequences (M=432, puncturing)

Figure 6  Cross-correlation of two different scrambling sequences (M=864, puncturing)

Observation 1: The current agreement on PDCCH scrambling achieves a similar level of cross-correlation to Gold sequence only case.

Proposal 1: Confirm the current agreement on PDCCH scrambling on user-specific search space.

3. Zero-Padding for Short DCI

It was agreed in [2] that zero-padding is applied to DCI whose payload size is smaller than 12.

Agreements: - In Rel-15, on coding of any PDCCH (including GC-PDCCH) - The coding has at least 12 bits before CRC - If the payload size is 11 bits or less, zero-padding to 12 bits

Zero-padding generally means that inserting zeros in the end of the payload, and it is already considered in [4] that 12-K zero bits are put after the payload. However, there was no explicit agreement on the location of the zero-padding bits. Thus, we simply review the zero-padding scheme for DCI and evaluate the performance depending on the location of the padding bits.

If the decoder treats the zero-padding bits as information bits and discards them after decoding, the position of the zero-padding bits is irrelevant to the performance. On the other hand, if zero-padding bits are treated as frozen bits in the decoder, then the decoder achieves additional coding gain. Since the reliability of each split channel is different in polar coding, the location of practical DCI payload should be carefully chosen. We consider two conventions: zero- appending (zero-padding in the end of payload) and zero-prepending (zero-padding in the front of payload). Since the bits with lower indices are usually less reliable in polar encoding, one expects that zero-padding bits, which is to be treated as frozen bits at the decoder, should be placed ahead of payload bits. However, the D-CRC scheme is exploited to DCI, and payload bits and zero-padding bits are interleaved together. The zero-padding bits are interleaved and distributed by D-CRC interleaving, and thus, we have to check the performance of each zero-padding scheme by simulations. Table 2 shows the assumption of performance evaluation to find good locations of the zero-padding bits.

Table 2  Evaluation Assumption for Zero-Padding Locations

| Modulation and Channels | QPSK and AWGN channels |
| --- | --- |
| Information bits (K) | 3:12 |
| Rate-matching output bits (M) | 108, 216, 432, 864 |
| Zero-padding | 1) Zero-appending (zero-padding in the end of the payload) 2) Zero-prepending (zero-padding in the front of the payload) |

Fig. 7 shows the performance of short DCI transmission depending on the location of zero-padding bits. In most cases of short UCI, the performance of zero-appending is better that that of zero-prepending. Especially when M=108, the performance gap is upto 0.3dB.

Observation 2: The performance of short DCI with zero-appending is generally better than that with zero-prepending.

Proposal 2: As in the current TS38.212, 12-K zero bits are appended in the end of short DCI of length K<12.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 7  Performance comparison of short DCI depending on the locations of zero-padding bits

4. Summary

Observation 1: The current agreement on PDCCH scrambling achieves a similar level of cross-correlation to Gold sequence only case.

Proposal 1: Confirm the current agreement on PDCCH scrambling on user-specific search space.

Observation 2: The performance of short DCI with zero-appending is generally better than that with zero-prepending.

Proposal 2: As in the current TS38.212, 12-K zero bits are appended in the end of short DCI of length K<12.

References

- Chairman’s notes in 3GPP TSG RAN WG1 #90bis, October 2017.

- Chairman’s notes in 3GPP TSG RAN WG1 #91, November 2017.

- Chairman’s notes in 3GPP TSG RAN WG1 NR Ad-hoc, January 2018.

- 3GPP TS 38.212 V15.0.1 (2018-02), 3GPP TSG RAN NR multiplexing and channel coding (release 15).