3GPP TSG RAN WG1 Meeting #92bis		R1-1804672

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.5.2

Source:	AT&T

Title:	Channel Model consideration for NR V2X

Document for:	Discussion/Approval

# Introduction

NR V2X study item has been approved by plenary, and performance evaluation on V2X for above 6Ghz is the first step of the study. In this paper, we give our opinion on the channel model for sidelink above 6Ghz.

# Channel model for V2X side-link at above 6Ghz

LTE V2X used a sidelink channel model which is based on Winner+ B1 model (Manhattan layout and Hexagonal layout corresponding to urban and freeway scenario separately) V2X sidelink, NR V2X study needs to find a suitable channel model for above 6Ghz. In NR channel model study, the channel model has been extended to above 6Ghz, therefore, we propose to update NR channel model, TR38.900, for V2X sidelink study.

Proposal: Update UMi-Street Canyon channel model in TR38.900 for V2X sidelink simulation.

Table.1 Channel Model for V2X sidelink

| Parameter | Urban case | Freeway case |
| --- | --- | --- |
| Pathloss model | TR38.900 UMi-Street Canyon | LOS in TR38.900 UMi-Stree Canyon |
| Shadowing distribution | Follow 38.900 UMi | Follow 38.900 UMi LOS |
| Penetration Loss | 0dB (assuming the Vehicle antenna is integrated in the car body) | 0dB(assuming the Vehicle antenna is integrated in the car body) |
| Spatial consistency correlation distance | 10 m | 25 m |
| Blockage modeling | Model-A without self-block region. | No blockage |
| Fast fading | Follow TR38.900 UMi-SC |  |

# Conclusion

In this contribution, we give our view on the channel model for side-link performance evaluation in above 6Ghz. In particular, we propose to use the UMi-Street Canyon channel model with some small modifications defined in TR38.900 for vehicle to vehicle side-link performance evaluation.

# Reference

[1] 	RP-170837 New SI proposal: Study on evaluation methodology of new V2X use cases for LTE and NR, LGE