3GPP TSG RAN WG1 Meeting #92bis																R1-1804705

Sanya, China, April 16 - 20, 2018

Source: 	Intel Corporation

Title:	Remaining details of RMSI

Agenda item:	7.1.1.2.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss remaining details on RMSI PDSCH transmission and CORESET for RMSI PDCCH with the focus on finalizing the CORESET configuration signaling in NR PBCH.

# Corrections to RMSI CORESET signaling

The current 38.213 contains potential issue regarding the UE procedure for monitoring Type0-PDCCH common search space.

The start OFDM symbol indexing of pattern 2 and pattern 3 is not correctly described. This is actually a mistake in the working assumption made in RAN1 #91. The specification correctly captured the working assumption. Unfortunately, the working assumption contained errors.

In case of SSB and RMSI SCS of {240kHz, 120kHz}, it would not be possible to support 2 OFDM symbol duration CORESET configuration. Therefore, entries need to be removed.

Proposal 1:

- Agree to following text proposal

Text proposal begin

Unchanged text omitted

Table 13-10: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {240, 120} kHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 8 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 48 | 2 | 8 |
| 4 | 2 | 24 | 1 | -41 if condition A -42 if condition B |
| 5 | 2 | 24 | 1 | 25 |
| 6 | 2 | 48 | 1 | -41 if condition A -42 if condition B |
| 7 | 2 | 48 | 1 | 25 |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Unchanged text omitted

Text proposal end

# Discussion on Actually Transmitted SSB Indication in RMSI

In FR2, actually transmitted SSBs are indicated in RMSI by means of 8 bit length group bitmap and (i.e., groupPresence parameter) and 8 bit length bitmap in group indication (i.e. inOneGroup parameter). However, in cases when SSB presence pattern cannot be indicated, e.g., due to TDD DL/UL configuration, the following alternatives are possible:

Alt.1:	Leave RMSI signaling as it is and avoid using incompatible SSB presence patterns in RMSI and TDD DL/UL configurations simultaneously;

Alt.2:	Use full 64 bit length bitmap instead of the two 8 bit length bitmaps;

Alt.3:	The semi-static DL/UL configuration in RMSI is used to disable SSBs colliding with UL transmissions.

Proposal 2:

- Down-select between following alternatives on actually transmitted SSB indication in RMSI:

  - Alt.1:	Leave RMSI signaling as it is and avoid using incompatible SSB presence patterns in RMSI and TDD DL/UL configurations simultaneously;

  - Alt.2:	Use full 64 bit length bitmap instead of the two 8 bit length bitmaps;

  - Alt.3:	The semi-static DL/UL configuration in RMSI is used to disable SSBs colliding with UL transmissions.

# Discussion on new Type0-PDCCH search space configurations

In RAN4 #86, the following was agreed for band n41 [1]:

Agreement: For Band 41 SCS - Specification is unchanged at this time. A LS can be sent to RAN1 to change the design to accommodate the RAN4 decision - RAN4 also agreed that no further default SCS for SSB is added in future meetings for Rel-15 bands. Step size - For sync raster step size, a conservative approach of <3> for band n41 and <16>for band <n79> is chosen, giving some guard band for SSB at band edges.

The RAN4 agreement above means that SSB SCS 15 kHz is also possible in band n41 in addition to SSB SCS 30 kHz [2]. The minimal channel bandwidth defined for band n41 is 10 MHz. In order not to define a new configuration tables for RMSI CORESET when {min CH BW, SSB SCS} = {10 MHz, 15 kHz} it’s proposed to reuse the existing tables for {min CH BW, SSB SCS} = {5 MHz, 15 kHz}. For that purpose the raster definition for n41 should be reconsidered and changed as in the re-farming bands where 900 kHz SS raster with SSB position shift (TBD from 70 kHz to 100 kHz) is used. This is because the 900 kHz SS raster is the largest used in 5 MHz channel bandwidth. This approach requires zero specification changes in RAN1 and most desirable because RAN1 specifications are about to be completed. At the same time, there are still ongoing discussions on potential changes for RAN4 specifications which may not require RAN1 specification changes [3]. Therefore, there is no hurt if RAN1 send an LS to RAN4 asking to change the SS raster definition for band n41.

Proposal 3:

- No new Type0-PDCCH search space configurations are defined in RAN1. Reuse the existing configuration tables for {min CH BW, SSB SCS} = {5 MHz, 15 kHz}.

- Send LS to RAN4 asking to reconsider the SS raster definition for band n41 making it the same as for re-farming bands, i.e., 900 kHz SS raster with SSB frequency shift (TBD from 70 kHz to 100 kHz).

# Conclusions

In this contribution, we discussed the remaining details of RMSI design. Our proposals are summarized as below:

Proposal 1:

- Agree on proposed text changes shown in section 2 of contribution (R1-1804705)

Proposal 2:

- Down-select between following alternatives on actually transmitted SSB indication in RMSI:

  - Alt.1:	Leave RMSI signaling as it is and avoid using incompatible SSB presence patterns in RMSI and TDD DL/UL configurations simultaneously;

  - Alt.2:	Use full 64 bit length bitmap instead of the two 8 bit length bitmaps;

  - Alt.3:	The semi-static DL/UL configuration in RMSI is used to disable SSBs colliding with UL transmissions.

Proposal 3:

- No new Type0-PDCCH search space configurations are defined in RAN1. Reuse the existing configuration tables for {min CH BW, SSB SCS} = {5 MHz, 15 kHz}.

- Send LS to RAN4 asking to reconsider the SS raster definition for band n41 making it the same as for re-farming bands, i.e., 900 kHz SS raster with SSB frequency shift (TBD from 70 kHz to 100 kHz).

# Reference

- RAN4#86 Meeting report, 3GPP RAN4 #86bis, Melbourne, Australia, 16 - 20 April 2018.

- R4-1802403, “Implementation of endorced CRs to 38.101-1,” 3GPP RAN4 #86, Athens, Greece, 26 February – 2 March 2018.

- RP-180155, “Consideration on NR-LTE co-existence,” KDDI Corporation, 3GPP RAN #79, Chennai, India, March 19 - 22, 2018.