3GPP TSG-RAN WG1 Meeting#92	R1-1805231

Athens, Greece, Feb 26th – March 2nd, 2018

Source:	Ericsson

Title:	IMT-2020 self evaluation: mMTC non-full buffer connection density for LTE-MTC and NB-IoT

Agenda Item:	7.8.2

Document for:	Discussion

# Introduction

3GPP has as part of the New Study Item on Self Evaluation towards IMT-2020 Submission [1] agreed to “Provide self evaluation results against technical performance requirements for mMTC as per defined in Report ITU-R M.[IMT-2020. TECH PERF REQ]  [RAN1, RAN2], including Connection density”.

A submission time plan has also been agreed [2] where the 3GPP meetings for submission of description and compliance templates according to report IMT-2020.SUBMISSION [3] are set. The Compliance template for technical performance contains the minimum technical performance requirement item Connection density. To fulfil this requirement 3GPP must show that it’s candidate Radio Interface Technologies (RITs) submitted to ITU-R supports a connection density of 1 000 000 devices per km2. The evaluation is to be performed in accordance to test environment Urban Macro-mMTC as described in report IMT-2020.EVAL [4].

In this contribution, we present initial results on Connection Density for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, and for NB-IoT. The LTE BL/CE mode operation is hereafter referred to as LTE-M operation. It is shown that LTE-M and NB-IoT meets the IMT-2020 requirement and should be part of the 3GPP submission to ITU-R.

The contribution also introduces the connection density requirement definition including the by IMT-2020 allowed system simulation procedures and some of the most relevant simulation assumptions defining the test environment Urban Macro-mMTC.

# Introduction to the Connection density requirement

## Requirement definition

The connection density requirement requires a RIT to provide a certain Quality of Service (QoS) to 1 000 000 devices per km2 at a grade of service (GoS) of 99 percent. Service is considered provided when a message latency of less than 10 seconds is supported for a user attempting to send an uplink data packet of 32 bytes defined at layer 2. Besides the supported connection density, it is encouraged to report the connection efficiency which is defined as the connection density normalized by the required system bandwidth.

The connection density requirement requires 99% grade of service where acceptable quality of service is defined by a message latency of 10 seconds or less.

## System simulation procedure

Report IMT-2020.EVAL [4] outlines two system simulator procedures for evaluating connection density. The first is a non-full buffer system level simulation that requires a state of the art system simulator to perform the evaluations. In this paper, we have followed this setup. The second approach is for a full buffer system simulation that allows input based on a more rudimentary system simulator combined with post processing supported by link level simulations. Both approaches have their merits and we provide input for the full buffer methodology in [6].

Table 1 Non-full buffer system level simulation procedure [4].

## Test environment

Report IMT-2020.EVAL specifies the test environment to be used in the evaluations according to Table 2. The simulations presented in this contribution where to a large extent following this set of assumptions. Further detailed assumptions are outlined in section 3.

Table 2: Urban Macro-mMTC test environment definition [4].

| Parameters | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss Note: Applies only to Channel model B. | 20% high loss, 80% low loss Note: Applies only to Channel model B. |
| Inter-site distance | 500 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | Up to 64 Tx/Rx |
| Number of UE antenna elements | Up to 2 Tx Up to 2 Rx | Up to 2 Tx Up to 2 Rx |
| Device deployment | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. |
| Simulation bandwidth | Up to 10 MHz | Up to 50 MHz |
| UE antenna height | 1.5 m | 1.5 m |

# Simulation configuration

## Simulation assumptions

Table 3 presents the herein investigated LTE-M and NB-IoT system level configurations, that overrides or comes in addition to those presented in Table 2. Worth to mention is that:

- All four combinations of system configurations (Conf. A/B) and channel models (UMA A/B) are investigated.

- Overhead from sync and broadcast channels were not modelled which corresponds to the modelling of NB-IoT non-anchors, and LTE-M narrowbands (NB) not overlapping with the center of the LTE carrier.

- (N)PRACH was configured to fully overlap with the simulated resource blocks.

Table 3: Simulation configuration.

| Parameter | LTE-M | NB-IoT |
| --- | --- | --- |
| More of operation | Inband | Inband |
| DL carrier configuration | NB not carrying SSS, PSS, PBCH or PDSCH containing SIB-BR transmission. | Non-anchor carrier not carrying NSSS, NPSS, NPBCH or NPDSCH containing SIB1-NB transmission. |
| UL carrier configuration | PRACH configured on 1 subframe in each radio frame corresponding to 10% overhead. | NPRACH configured on 7% of all UL resources. |
| System bandwidth | 1 NB (6 PRB) | 1 PRB |
| Total transmit power per TRxP | 29 dBm/PRB | 29 dBm/PRB |
| Number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] |
| Number of UE antenna elements | 1Tx/Rx 0° polarization [5] | 1Tx/Rx 0° polarization [5] |
| Number of TXRU per UE | 1TXRU [5] | 1TXRU [5] |
| UE antenna element gain | 0 dBi [5] | 0 dBi [5] |
| UE antenna element pattern | Omni-directional [5] | Omni-directional [5] |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 [5] | 3 [5] |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 90° in LCS | 90° in LCS |
| Handover margin (dB) | 2 dB (handover modelled explicitly) | 2 dB (handover modelled explicitly) |
| TRxP boresight | 30 / 150 / 270 degrees [5] | 30 / 150 / 270 degrees [5] |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] |
| Wrapping around method | Radio distance based wrapping | Radio distance based wrapping |
| Minimum distance of TRxP and UE | 10 m [5] | 10 m [5] |
| Polarized antenna model | Model-2 in TR36.873 [5] | Model-2 in TR36.873 [5] |
| System layout | 7 sites, 3 sectors per site | 7 sites, 3 sectors per site |
| Scheduling strategy | Round robin | Round robin |
| Channel estimation | Ideal | Ideal |
| (N)PRACH model | Access attempt failed in case of collision | Access attempt failed in case of collision |
| Packet drop timer | 20 sec | 20 sec |
| RRC Connection setup procedure | CIoT UP Optimization (RRC Resume) | CIoT UP Optimization (RRC Resume) |
| RLC Mode | AM | AM |

## Channel model

The coupling gain for the different configurations and channel models was recorded by the simulator and are shown in Figure 1 below. These include antenna gains, Urban Macro A and Urban Macro B SCM channel models and outdoor to indoor losses. The large cell size of Configuration B leads to a higher coupling loss than what is seen for Configuration A. Channel model UMA A contains aggressive assumptions for the indoor to outdoor loss which is also visible in the below figure.

Figure 1: Coupling gain distribution for test environment Urban Macro mMTC based on the settings of Table 1.

# Connection density evaluation

The achieved latencies for LTE-M and NB-IoT are presented in Figure 2. The results are reported per cell and per resource block (RB). To translate the x-axis to arrival intensity per km2 the Configuration A results should be scaled by a factor 1/0.072 and the Configuration B results by a factor 1/0.866. To get the LTE-M results per narrowband a scaling factor of 6 should be applied.

LTE-M offers a superior latency until the system reaches the point of congestion. This thanks to its support of higher order modulation and bandwidths up to 6 PRBs. NB-IoT offers a higher connection density partly thanks to its robust performance under low SINR conditions and partly due to a more efficient channel allocation strategy. These observations are supported by Figure 3 in the Annex: It is shown that LTE-M has a higher DL resource utilization than NB-IoT which is believed to be partly due to the use of higher bandwidth to facilitate low latency and partly due to use of more repetitions than NB-IoT requires. Figure 4 and Figure 5 indicate that the higher resource utilization leads to higher interference levels, which eventually limits the performance of the systems. It is expected that there is room to optimize the power control settings and the scheduling strategy to improve the SINR characteristics and the overall performance. This especially for LTE-M.

Configuration B, UMA B is offering the largest supported connection density per cell. This thanks to its large cell size and relatively gentle channel profile. For Configuration B, UMA A the relatively high coupling loss is on the other hand limiting the supported connection density despite the large cell size.

In absolute numbers the highest connection density is supported for Configuration A. This since the cell area is 12 times smaller for the 500 m ISD, compared to the 1732 m ISD in Configuration B. This supports a densification of the cell grid which supports higher access loads.

It should be noted that the reported latency does not include the time to synchronize and read the master information block. This latency will be added later and will increase the shown latencies and naturally reduce the presented system capacities.

Figure 2: LTE-M and NB-IoT service latency at the 99th percentile.

In Table 4 the connection density is reported based on the results presented in Figure 2. Given the assumed cell size of 0.07 km2 in Configuration A and 0.86 km2 in Configuration B and the traffic model 1 message/2 hours/device the arrival intensity can be translated into number of supported devices per km2.

Table 4 also presents the needed bandwidth to support the connection density target. In addition to the reported bandwidth a NB is needed for LTE-M to carry synchronization and master/system information signalling. For NB-IoT an additional anchor PRB is needed to carry synchronization and master/system information signalling. For completeness the connection efficiency and spectral efficiency is presented.

Table 4 LTE-M and NB-IoT performance metrics.

|  | Connection density @ 99 percent GoS [devices/NB or PRB] | Bandwidth to support 1 000 000 devices per km2 [NB or PRB] | Connection efficiency [devices/Hz] | Spectral efficiency [bits/s/Hz] |
| --- | --- | --- | --- | --- |
| LTE-M: UMA A, Conf A | 4.220.000 devices/NB | 1 NB | 3.91 devices/Hz | 0.14 bits/s/Hz |
| LTE-M: UMA B, Conf A | 4.220.000 devices/NB | 1 NB | 3.91 devices/Hz | 0.14 bits/s/Hz |
| LTE-M: UMA A, Conf B | 309.000 devices /NB | 4 NB | 0.29 devices/Hz | 0.01 bits/s/Hz |
| LTE-M: UMA B, Conf B | 466.000 devices /NB | 3 NB | 0.43 devices/Hz | 0.02 bits/s/Hz |
| NB-IoT: UMA A, Conf A | 778.000 devices /1 PRB | 2 PRBs | 4.3 devices/Hz | 0.15 bits/s/Hz |
| NB-IoT: UMA B, Conf A | 733.000 devices/1 PRB | 2 PRBs | 4.1 devices/Hz | 0.15 bits/s/Hz |
| NB-IoT: UMA A, Conf B | 51.000 devices/1 PRB | 20 PRBs | 0.28 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA B, Conf B | 108.000 devices/1 PRB | 10 PRBs | 0.6 devices/Hz | 0.02 bits/s/Hz |

# Conclusions

In this contribution, we have discussed the IMT-2020 connection density requirement and provided preliminary input to the 3GPP Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that LTE-M and NB-IoT have the potential to fulfil the IMT-2020 connection density requirement.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-172098, “3GPP submission towards IMT-2020”, ITU-R Ad-Hoc Contact Person

[PRELIMINARY] DRAFT NEW REPORT ITU-R M. [IMT-2020.SUBMISSION], 27th meeting of WP5D, ITU-R WP5D

[PRELIMINARY] DRAFT NEW REPORT ITU-R M. [IMT-2020.EVAL], 27th meeting of WP5D, ITU-R WP5D

RP-172728, LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”, RAN#78, Huawei

R1-1805232, “IMT-2020 self evaluation: mMTC full buffer connection density”, RAN1#92b, Ericsson

# Annex: Simulation results

Figure 3 Average DL resource utilization.

Figure 4 1st percentile DL SINR.

Figure 5 1st percentile UL SINR.