3GPP TSG RAN WG1 Meeting #93           	R1-1806104
Busan, Korea, May 21st – 25th, 2018

Agenda item:		7.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation methodology and assumptions for NR-U evaluations

Document for:		Discussion and Decision

# 1	Introduction

The study item on NR-based access to unlicensed spectrum that was approved at RAN#75 [1] includes the following objectives:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

…

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 37GHz, 60GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

The simulation methodology for NR-U operation was discussed during RAN1#92bis with following agreements:

Agreement:

- For sub7 indoor simulation evaluation:

  - Scenario: Option 2 (3+3) with indoor mixed office model

    - Target to reach 10%-15% serving links below -72dBm

    - Further layout parameter fine tuning may be needed. An example procedure for fine tuning is the following sequence.

      - Currently a-b-a=15-20-15

      - If not reaching target, try a-b-a=15-30-15 and a-b-a=20-40-20

      - If not reaching target, apply a scaling factor to the layout with a-b-a=20-40-20

  - Other parameters: Default is NR parameters in 38.901 and 38.802 with the exception of the following

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline, 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23 dBm (total across all TX antennas) |
| UE/STA Tx Power | 18 dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Email discussion on further layout parameter fine tuning until May 3, 2018 (Jing, Qualcomm)

Agreement:

- For sub7 outdoor simulation evaluation:

  - Select one of the following for the Outdoor sub-7 GHz scenario

    - Alt 1: Each operator randomly drop [1 or 2] micro-layer TRPs within each macro cell with minimum dibstance between gNBs as in NR

      - Use NR dense Urban option 1 (gNB dropped at the center of the hot-spot)

      - Independent dropping between two operators

      - Use the NR current [57.9] meters intra-operator minimum distance

      - Use [10] meters as the inter-operator minimum distance

      - UE randomly dropped within [28.9] meters within the serving cell

    - Alt 2: Drop [1 or 2 or 3] hot spots as in NR urban option 1

      - Within each hot-spot, randomly drop one gNB from each operator within a circle of radius [10] meters centered at the center of the hot-spot

        - The minimum inter-gNB distance is [10] meters

      - Within each hot-spot, drop UE within [28.9] meters from the hot-spot center

  - Parameters: Use the indoor sub7 table as baseline, with further fine tunes possible

Agreement:

- For calibration for sub-7 GHz indoor and outdoor scenarios, companies should submit for the baseline scenario:

  - Cdf of received signal power from serving cell

  - Optional: Cdf of received signal power from each of the all non-serving cells (including the cells from the other operator)

Further agreement was made based [92b-NR-05] Email discussion on NR-U indoor sub7 calibration:

Proposal: Adopt layout as in Figure 1 with a=20 meters, b=40 meters, c=20 meters, and d=40 meters for indoor sub7GHz NR-U evaluation.

Figure 1. Indoor sub7 simulation office layout

In this contribution, we summarize the simulation assumptions to perform the agreed co-existence analyses. Generic assumptions related to evaluation methodology are discussed in Section 2. Simulation parameters for below 7 GHz outdoor and indoor cases are listed in Sections 3 and the APPENDIX, respectively.

# 2	Evaluation methodology

To prove that NR-based operation in unlicensed spectrum does not impact deployed Wi-Fi services more than an additional Wi-Fi network on the same carrier would do, we propose to primarily focus on NR-U-WiFi co-existence evaluations below 7GHz.

Proposal 1: Prioritize NR-U-WiFi co-existence evaluations below 7 GHz.

The evaluation methodology for 5 GHz bands shall assume the same principles as in LAA coexistence evaluations in [2]. First the performance is evaluated with two operators’ WiFi networks co-deployed in the specified simulation environments and operating on the same unlicensed channel(s). Then the WiFi access points and stations of one of the WiFi networks are replaced (at the identical locations) with NR-U gNBs and UEs and performance re-evaluated. The performance of the first WiFi network should not be degraded. On the other hand, on bands without legacy current WiFi presence (e.g. 6 GHz), this kind of definition with one-way fairness would easily lead into a situation where NR-U is more fair towards WiFi than the other way around. In other words, the impact of WiFi on NR-U performance should also be considered, such that both WiFi and NR-U are equally fair towards each other.

Proposal 2: For the 6 GHz band, in addition to the impact that NR-U has on WiFi, the evaluations shall also consider the impact of WiFi on NR-U to ensure fairness both ways.

Though carrier aggregation with DL-only NR-U SCell is among the agreed deployment scenarios in RAN1#92, our view is that support of DL and UL transmission on unlicensed spectrum should be introduced in the specifications at the same time and not in a phased approached as done in LTE LAA. Therefore, we think that evaluations of co-existence between DL+UL Wi-Fi and DL + UL NR-U should be prioritized.

Proposal 3: Co-existence evaluations between DL+UL Wi-Fi and DL+UL NR-U are the baseline.

The evaluations should also target at assessing the coexistence within NR-U (i.e. between two NR-U operators’ networks), as well as NR-U-LAA coexistence.

Proposal 4: NR-U / NR-U and NR-U / LTE LAA should also be evaluated though with lower priority.

One remaining open point related to assumption for Indoor case is the minimum distance between a small cell and a UE, and between two UEs. We propose to set both to 3 meters

Proposal 5: In the evaluations, the minimum distance between a small cell and a UE, and between two UEs is three meters.

To achieve faster convergence on evaluation methodology and simulation assumptions, we propose to reuse the general evaluations assumptions specified in [2] for LTE LAA coexistence evaluations as much as possible. Deviations for the simulations assumptions in [2] should be well justified.

Proposal 6: For coexistence evaluations below 7GHz, reuse general evaluations assumptions specified for LTE (e)LAA coexistence evaluations.

Though simulations assumptions and parameters are only listed for unlicensed cells, they are generally applicable to study co-existence of different NR-U deployment scenarios (e.g. CA, SA and DC), and as noted in the agreements from RAN1#92 (“A single set of evaluations may be applicable to multiple scenarios”).

Regarding the evaluations for mmWave bands, we see that the basic system design (e.g. waveform, subcarrier spacing, etc.) should be discussed first before agreeing on any specific simulation assumptions.

# 3	Outdoor below 7 GHz

For outdoor below 7 GHz case we propose to generally follow Alt 1 defined in RAN1#92bis, with following detailed parameters.

Proposal 7: For sub-7 GH outdoor case, adopt Alt 1 as the simulation scenario.

|  | Unlicensed cell |  |
| --- | --- | --- |
| Layout for nodes | Each operator independently drops [3] hot spots within each macro cell The geNB is dropped at the center of the hot spot - 6 or 9 hot spots per operator per macro cell is optional) |  |
| System bandwidth per carrier | 20 MHz |  |
| Carrier frequency | 6.0 GHz |  |
| Subcarrier spacing | 30 kHz, optional: 60 kHz, 15 kHz |  |
| Number of carriers | 1, optional: 4 |  |
| Total BS TX power | 24 dBm across aggregated carriers (Optional: 30 dBm) |  |
| Max Total UE TX power | 18 dBm across aggregated carriers |  |
| Distance-dependent path loss and Shadowing | [referring to Table 7.4.1-1 in 38.901] Small cell-to-Small cell: UMi-Street canyon (hUE =10m) Small cell-to-UE: UMi-Street canyon UE-to-UE: UMi-Street canyon (hBS =1.5m), penetration loss between UEs follows Table A.2.1-12 in 38.802 [3D distance between an gNB and a UE is applied. Working assumption is that 3D distance is also used for break point distance and LOS probability] [Working assumption is that 3D distance is used for shadowing correlation distance] |  |
| Outdoor-to-Indoor penetration | According to the description in chapter 7.4.3.1 in 38.901 |  |
| Fast fading | UMi-Street canyon [referring to Section 7.5 in 38.901] |  |
| BS antenna panel and antenna positioning | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |  |
| BS antenna height | 10 m |  |
| BS antenna gain | 0dBi |  |
| BS antenna pattern | Omni-directional is baseline; directional antenna is not precluded |  |
| UE antenna panel and antenna positioning | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |  |
| UE antenna height |  | 1.5m |
| UE antenna gain |  | 0 dBi |
| UE antenna pattern |  | Omni-directional |
| BS noise figure |  | 5dB |
| UE noise figure |  | 9dB |
| UE receiver |  | MMSE-IRC as baseline |
| UE speed |  | 3km/h |
| Number of UEs |  | 5 per gNB per 20MHz |
| UE dropping per network |  | Users are randomly and uniformly dropped within [28.9, 21.2, 16] meters from the corresponding serving cell for [3,6,9] hotspots per operator, respectively |
| Minimum distance (2D distance) |  | Intra-operator small cell-small cell: [57.9m, 42.4m, 32m] for [3,6,9] hotspots per operator, respectively |
|  |  | Inter-operator small cell-small cell: 10 m |
|  |  | Small cell-UE (both intra and inter-operator): 10 m |
|  |  | UE-UE: 3m |
|  |  | Macro – small cell: 105m |
|  |  | Macro – UE: 35m |
| Traffic model |  | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by NR-U. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100%. Statistics are independently reported in each direction - No associated control plane traffic is modelled - For DL+UL coexistence evaluations the voice activity of the VoIP users is 50% for both DL and UL. - For DL+UL coexistence evaluations for each VoIP user, On and Off periods of length X (e.g., X = 5) second alternates with each other in such a way that both DL and UL are not active at the same time. Independent traffic generation on the DL and UL for both NR-U, Wi-Fi and LAA for FTP traffic model Each UE has the same UL/DL traffic arrival rate ratio Overall offered load is the same for both the coexisting networks DL/UL traffic ratio for DL+UL coexistence evaluations: - 50% DL traffic and 50% UL traffic - Optional: 80% DL traffic and 20% UL traffic |

# 4	Conclusions

In this section we summarize the proposals made in this contribution.

Proposal 1: Prioritize NR-U-WiFi co-existence evaluations below 7 GHz.

Proposal 2: For the 6 GHz band, in addition to the impact that NR-U has on WiFi, the evaluations shall also consider the impact of WiFi on NR-U to ensure fairness both ways.

Proposal 3: Co-existence evaluations between DL+UL Wi-Fi and DL+UL NR-U are the baseline.

Proposal 4: NR-U / NR-U and NR-U / LTE LAA should also be evaluated though with lower priority.

Proposal 5: In the evaluations, the minimum distance between a small cell and a UE, and between two UEs is three meters.

Proposal 6: For coexistence evaluations below 7GHz, reuse general evaluations assumptions specified for LTE (e)LAA coexistence evaluations.

Proposal 7: For sub-7 GH outdoor case, adopt Alt 1 as the simulation scenario.

# References

- R1-1803272, “New SID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- 3GPP TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”, V13.0.0 (2015-06)

- 3GPP TR 38.802, “Study on New Radio Access Technology – Physical Layer Aspects”, V14.2.0 (2017-09)

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V14.3.0 (2017-12)

# APPENDIX

The following table contains the main simulation assumptions for indoor scenario below 7 GHz. The additional assumptions as compared to what already agreed in RAN1#92bis and the following email discussions are highlighted.

|  | Unlicensed cell |  |
| --- | --- | --- |
| Layout for nodes | Two operators deploy 3 small cells each in the single-floor building as illustrated in the figure below (Option 2 in the agreements from RAN1#92). |  |
| System bandwidth per carrier | 20 MHz, 80 Mhz optional |  |
| Carrier frequency | 5.0 GHz |  |
| Subcarrier spacing | To be reported together simulation results |  |
| Number of carriers | 1 |  |
| Total BS TX power | 23 dBm across aggregated carriers |  |
| Max Total UE TX power | 18 dBm across aggregated carriers |  |
| Distance-dependent path loss and Shadowing | [referring to Table 7.4.1-1 in 38.901] Small cell-to-Small cell: 5GCM indoor office (hUE = 3m) Small cell-to-UE: 5GCM indoor office Indoor UE-to-Indoor UE: 5GCM Indoor-office (hBS =1.5m). [Use indoor mixed office LOS probability] [3D distance between an gNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance] [Working assumption is that 3D distance is used for shadowing correlation distance] |  |
| Fast fading | 5GCM indoor office [referring to Section 7.5 in 38.901] |  |
| BS antenna panel and antenna positioning | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |  |
| BS antenna height | 3 m |  |
| BS antenna gain | 0 dBi |  |
| BS antenna pattern | Omni-directional is baseline (Option 3 in Section A.2.1 of 38.802) |  |
| UE antenna panel and antenna positioning | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |  |
| UE antenna height |  | 1.5m |
| UE antenna gain |  | 0 dBi |
| UE antenna pattern |  | Omni-directional |
| BS noise figure |  | 5 dB |
| UE noise figure |  | 9 dB |
| UE receiver |  | MMSE-IRC as baseline |
| UE speed |  | 3 km/h |
| Number of UEs |  | 5 per gNB per 20MHz |
| UE dropping per network |  | Users are randomly and uniformly dropped within the building area. UEs experiencing less than -82 dBm wideband RSRPs are re-dropped |
| Minimum distance (2D distance) |  | Small cell-UE, UE-UE: 3m |
| Traffic model |  | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by NR-U. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100%. Statistics are independently reported in each direction - No associated control plane traffic is modelled - For DL+UL coexistence evaluations the voice activity of the VoIP users is 50% for both DL and UL. - For DL+UL coexistence evaluations for each VoIP user, On and Off periods of length X (e.g., X = 5) second alternates with each other in such a way that both DL and UL are not active at the same time. Independent traffic generation on the DL and UL for both NR-U, Wi-Fi and LAA for FTP traffic model Each UE has the same UL/DL traffic arrival rate ratio Overall offered load is the same for both the coexisting networks DL/UL traffic ratio for DL+UL coexistence evaluations: - 50% DL traffic and 50% UL traffic - Optional: 80% DL traffic and 20% UL traffic |
| UE bandwidth |  | 20 MHz |