- 3GPP TSG RAN WG1 Meeting #93                                                                            R1-1806155

Busan, Korea, May 21st – 25th, 2018

Source:	ZTE, CATR

Title:	Discussion on the channel model for sub-urban

Agenda Item:	7.3.1

Document for:     Discussion and Decision

# Introduction

In RAN1#92bis, the structure of channel model for NTN has been agreed. However, the detailed parameter is still not available for both large and small scale. In this contribution, both large and small scale characteristics of NTN channel is provided for the sub-urban case.

# Large scale channel model

Based on the agreed path loss model has been agreed in RAN1#92bis, the corresponding clutter loss (NLoS) and SF (both LoS and NLoS) have been derived according to RT-based simulation listed in Table 1.

Table 1 Large scale parameters for suburban

| Elevation | Clutter loss (NLoS) [dB] |  | SF [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NLoS |  | LoS |  |
|  | Satellite | HAPS | Satellite | HAPS | Satellite | HAPS |
| 10 | 17.44 | 16.29 | 9.69 | 8.98 | 0.37 | 0.4 |
| 20 | 18.34 | 17.69 | 9.88 | 9.68 | 0.18 | 0.24 |
| 30 | 20.84 | 21.38 | 9.90 | 10.31 | 0.15 | 0.21 |
| 40 | 21.85 | 20.97 | 10.68 | 9.99 | 0.13 | 0.18 |
| 50 | 23.55 | 23.09 | 10.21 | 9.78 | 0.12 | 0.15 |
| 60 | 22.26 | 22.66 | 10.26 | 10.28 | 0.1 | 0.14 |
| 70 | 20.39 | 21.01 | 11.29 | 11.74 | 0.12 | 0.13 |
| 80 | 20.20 | 19.14 | 13.64 | 13.03 | 0.18 | 0.17 |

According to these results, only slight difference can be found between satellite and HAPS. It’s reasonable since for the propagation with far-away BS, only few paths can be observed which are only determined by the local scatter around the UE side. For simplifying the channel model, either one of them can be considered for NTN system.

Proposal 1: Capture the large scale parameters listed in Table 1 into TR38.811 for sub-urban.

# Small scale channel model

In this section, the small scale channel model is established based on the channel realization from RT-based simulation. For each UE, the multiple-paths within dynamic range (30 dB) are selected for characterizing the statistic properties of channel. The detailed results are listed in Table 2. According to this table, only few clusters can be observed in the channel due to the limited scatters around UE side only. Moreover, since the BS is far-away from UE, the angular difference among the rays from BS side is negligible as well as the tabulated spread, e.g., ZoD/AoD.

Furthermore, since the multiple paths are mainly originated from the over-the-roof diffraction and reflection from nearby building as well as ground, in most of cases, the difference of propagation distance is within 50m, and the corresponding delay spread is also much smaller than terrestrial scenario.

Table 2 Parameters of fast fading for satellite

| Scenarios |  | LEO 600km LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -8.54 | -8.83 | -8.98 | -8.97 | -9.02 | -8.95 | -8.44 | -8.39 | - |
|  | lgDS | 1.34 | 1.30 | 1.08 | 1.13 | 1.58 | 1.74 | 0.77 | 0.89 | - |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -3.91 | -4.31 | -4.40 | -4.23 | -4.08 | -3.95 | -3.81 | -3.50 | - |
|  | lgASD | 2.26 | 2.43 | 2.40 | 2.24 | 2.08 | 1.99 | 1.90 | 1.75 | - |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -1.02 | -1.42 | -1.55 | -1.52 | -1.43 | -1.32 | -0.79 | -0.58 | - |
|  | lgASA | 2.55 | 2.67 | 2.40 | 2.52 | 2.60 | 2.68 | 1.60 | 1.48 | - |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | -4.04 | -4.40 | -4.52 | -4.60 | -4.66 | -4.69 | -4.30 | -4.39 | - |
|  | lgZSD | 0.85 | 1.14 | 0.79 | 1.21 | 1.48 | 1.74 | 0.76 | 0.95 | - |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 1.19 | 0.70 | 0.33 | 0.12 | -0.05 | 0.03 | 0.66 | 0.36 | - |
|  | lgZSA | 1.34 | 1.32 | 0.67 | 1.11 | 1.36 | 1.65 | 0.87 | 1.06 | - |
| Shadow fading (SF) [dB] | SF | 0.37 | 0.18 | 0.15 | 0.13 | 0.12 | 0.1 | 0.12 | 0.18 |  |
| K-factor (K) [dB] | K | 13 | 25 | 32 | 35 | 37 | 34 | 19 | 21 | - |
|  | K | 8 | 14 | 23 | 22 | 20 | 20 | 8 | 14 | - |
| Delay scaling parameter r |  | 2.20 | 3.36 | 3.50 | 2.81 | 2.39 | 2.73 | 2.07 | 2.04 | - |
| Number of clusters |  | 6 | 5 | 4 | 4 | 4 | 3 | 4 | 4 | - |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | - |
| Scenarios |  | LEO 600km NLOS |  |  |  |  |  |  |  |  |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -8.50 | -9.21 | -9.58 | -9.31 | -9.43 | -9.38 | -9.85 | -10.31 | - |
|  | lgDS | 1.96 | 1.99 | 2.00 | 1.64 | 1.59 | 1.54 | 1.91 | 2.10 | - |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -4.31 | -4.55 | -4.75 | -5.00 | -5.17 | -4.90 | -4.12 | -3.50 | - |
|  | lgASD | 2.51 | 1.99 | 2.32 | 2.05 | 2.08 | 1.92 | 2.21 | 2.25 | - |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 0.23 | 0.09 | -0.37 | -0.42 | -0.58 | -0.58 | -0.64 | -0.71 | - |
|  | lgASA | 2.37 | 1.71 | 2.03 | 2.00 | 1.98 | 1.97 | 1.69 | 1.38 | - |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | -4.64 | -4.52 | -4.71 | -4.86 | -4.98 | -5.06 | -4.35 | -3.80 | - |
|  | lgZSD | 2.27 | 1.71 | 2.15 | 1.74 | 1.69 | 1.65 | 1.93 | 2.19 | - |
| ZOAspread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.98 | -0.46 | -0.64 | -0.27 | -0.49 | -0.46 | -0.22 | -0.45 | - |
|  | lgZSA | 2.51 | 1.80 | 1.96 | 1.72 | 1.83 | 1.68 | 1.30 | 1.19 | - |
| Shadow fading (SF) [dB] | SF | 9.69 | 9.88 | 9.90 | 10.68 | 10.21 | 10.26 | 11.29 | 13.64 |  |
| Delay scaling parameter r |  | 2.28 | 2.33 | 2.43 | 2.26 | 2.71 | 2.10 | 2.19 | 2.06 | - |
| Number of clusters |  | 6 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | - |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |  |

Proposal 2: Capture the parameters of fast fading listed in Table 2 into TR38.811 for sub-urban.

# Link level channel model

In order to conduct the link level simulation for evaluating the required enhancement of NTN system, the corresponding channel model, i.e., TDL/CDL, is generated based on the obtained system model in section 3. The detailed results are listed from Table 3 to Table 6.

Table 3 CDL model for LoS

| Elevation Angle[°] | Cluster # | Cluster PAS | Normalized Delay | Power in [dB] | AOA in [°] | AOD in [°] | ZOA in [°] | ZOD in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.00 | 1.00 | Specular(LOS path) | 0.00 | -0.12 | 0.00 | -180.00 | 80 | 100 |
|  | 1.00 | Laplacian | 0.00 | -16.01 | 0.00 | -180.00 | 80 | 100 |
|  | 2.00 | Laplacian | 13.56 | -33.40 | 17.23 | 179.81 | 161.13 | 99.99 |
|  | 3.00 | Laplacian | 19.18 | -30.55 | 208.86 | 179.83 | 161.72 | 99.99 |
|  | 4.00 | Laplacian | 20.35 | -28.69 | 132.16 | 180.16 | 47.88 | 100.01 |
|  | 5.00 | Laplacian | 61.50 | -51.63 | 214.82 | 180.27 | 106.53 | 99.99 |
|  | 6.00 | Laplacian | 82.41 | -58.91 | 314.61 | 179.71 | 72.01 | 100.02 |
| 20.00 | 1.00 | Specular(LOS path) | 0.00 | -0.08 | 0.00 | -180.00 | 70 | 110 |
|  | 1.00 | Laplacian | 0.00 | -25.95 | 0.00 | -180.00 | 70 | 110 |
|  | 2.00 | Laplacian | 3.12 | -21.80 | 307.50 | 179.99 | 37.35 | 110 |
|  | 3.00 | Laplacian | 5.29 | -25.56 | 70.74 | 180.00 | 121.31 | 110 |
|  | 4.00 | Laplacian | 9.38 | -23.52 | 162.43 | 179.99 | 87.56 | 110 |
|  | 5.00 | Laplacian | 23.40 | -30.60 | 90.74 | 179.98 | 142.85 | 110 |
| 30.00 | 1.00 | Specular(LOS path) | 0.00 | -0.09 | 0.00 | -180.00 | 60 | 120 |
|  | 1.00 | Laplacian | 0.00 | -20.33 | 0.00 | -180.00 | 60 | 120 |
|  | 2.00 | Laplacian | 4.44 | -20.31 | 302.20 | 180.00 | 65.9 | 120 |
|  | 3.00 | Laplacian | 21.12 | -27.61 | 72.24 | 179.99 | 48.3 | 120 |
|  | 4.00 | Laplacian | 22.65 | -40.22 | 19.21 | 179.98 | 88.28 | 120 |
| 40.00 | 1.00 | Specular(LOS path) | 0.00 | -0.07 | 0.00 | -180.00 | 50 | 130 |
|  | 1.00 | Laplacian | 0.00 | -29.43 | 0.00 | -180.00 | 50 | 130 |
|  | 2.00 | Laplacian | 0.85 | -22.68 | 344.15 | 180.00 | 36.9 | 130 |
|  | 3.00 | Laplacian | 2.42 | -20.59 | 223.76 | 180.01 | 55.68 | 130 |
|  | 4.00 | Laplacian | 51.16 | -34.42 | 314.50 | 180.01 | 5.82 | 130 |
| 50.00 | 1.00 | Specular(LOS path) | 0.00 | -0.06 | 0.00 | -180.00 | 40 | 140 |
|  | 1.00 | Laplacian | 0.00 | -27.61 | 0.00 | -180.00 | 40 | 140 |
|  | 2.00 | Laplacian | 2.45 | -20.83 | 204.46 | 180.00 | 105.26 | 140 |
|  | 3.00 | Laplacian | 3.02 | -27.18 | 326.25 | 180.01 | 23.98 | 140 |
|  | 4.00 | Laplacian | 24.10 | -27.92 | 128.24 | 180.01 | 65.75 | 140 |
| 60.00 | 1.00 | Specular(LOS path) | 0.00 | -0.06 | 0.00 | -180.00 | 30 | 150 |
|  | 1.00 | Laplacian | 0.00 | -23.33 | 0.00 | -180.00 | 30 | 150 |
|  | 2.00 | Laplacian | 5.13 | -21.78 | 188.72 | 180.00 | 12.75 | 150 |
|  | 3.00 | Laplacian | 21.09 | -27.29 | 143.81 | 180.00 | 135.87 | 150 |
| 70.00 | 1.00 | Specular(LOS path) | 0.00 | -0.10 | 0.00 | -180.00 | 20 | 160 |
|  | 1.00 | Laplacian | 0.00 | -17.94 | 0.00 | -180.00 | 20 | 160 |
|  | 2.00 | Laplacian | 4.39 | -23.97 | 355.04 | 179.98 | 102.87 | 160 |
|  | 3.00 | Laplacian | 15.02 | -31.08 | 20.17 | 180.03 | 61.01 | 160 |
|  | 4.00 | Laplacian | 17.66 | -26.18 | 12.20 | 179.98 | 119.34 | 160 |
| 80.00 | 1.00 | Specular(LOS path) | 0.00 | -0.08 | 0.00 | -180.00 | 10 | 170 |
|  | 1.00 | Laplacian | 0.00 | -26.98 | 0.00 | -180.00 | 10 | 170 |
|  | 2.00 | Laplacian | 4.54 | -21.21 | 353.43 | 179.99 | 176.58 | 170 |
|  | 3.00 | Laplacian | 9.08 | -20.75 | 1.50 | 179.99 | 21.21 | 170 |
|  | 4.00 | Laplacian | 13.70 | -30.54 | 346.33 | 179.97 | 6.37 | 170 |

Table 4 CDL model for NLoS

| Elevation Angle | Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.00 | 1.00 | 0.00 | 0.00 | 1.45 | 180.00 | 52.14 | 100 |
|  | 2.00 | 0.40 | -2.40 | 240.37 | 180.09 | 57.29 | 99.99 |
|  | 3.00 | 1.12 | -0.32 | 311.71 | 180.03 | 121.4 | 100 |
|  | 4.00 | 1.88 | -3.86 | 91.98 | 179.89 | 43.46 | 100.01 |
|  | 5.00 | 2.94 | -9.12 | 54.81 | 180.17 | 164.87 | 100.03 |
|  | 6.00 | 4.33 | -9.75 | 312.54 | 180.18 | 176.46 | 99.97 |
| 20.00 | 1.00 | 0.00 | 0.00 | 55.46 | 180.00 | 17.25 | 110 |
|  | 2.00 | 0.95 | -7.82 | 125.53 | 179.97 | 80 | 109.99 |
|  | 3.00 | 2.77 | -11.84 | 145.00 | 179.96 | 39.49 | 109.99 |
|  | 4.00 | 6.67 | -17.89 | 294.15 | 180.05 | 144.8 | 110.02 |
|  | 5.00 | 13.95 | -37.32 | 64.94 | 179.93 | 96.85 | 110.04 |
| 30.00 | 1.00 | 0.00 | -0.77 | 293.47 | 180.02 | 11.86 | 120 |
|  | 2.00 | 0.53 | 0.00 | 40.20 | 180.00 | 61.14 | 120 |
|  | 3.00 | 1.05 | -5.71 | 47.38 | 180.04 | 135.42 | 120.01 |
|  | 4.00 | 2.45 | -3.83 | 117.76 | 180.04 | 83.24 | 119.99 |
|  | 5.00 | 3.47 | -8.97 | 290.95 | 179.95 | 66.95 | 120.02 |
| 40.00 | 1.00 | 0.00 | 0.00 | 31.68 | 180.00 | 133.97 | 130 |
|  | 2.00 | 0.84 | -6.00 | 281.38 | 180.01 | 147.82 | 130 |
|  | 3.00 | 0.94 | -6.04 | 282.65 | 180.01 | 132.83 | 130 |
|  | 4.00 | 1.05 | -4.76 | 219.71 | 179.99 | 28.78 | 130 |
|  | 5.00 | 7.42 | -15.22 | 63.82 | 180.01 | 76.58 | 130.01 |
| 50.00 | 1.00 | 0.00 | 0.00 | 158.76 | 180.00 | 40.76 | 140 |
|  | 2.00 | 0.31 | -3.98 | 163.52 | 180.00 | 147.45 | 140 |
|  | 3.00 | 4.18 | -17.39 | 298.49 | 179.99 | 57.26 | 139.99 |
|  | 4.00 | 4.69 | -12.39 | 261.81 | 179.99 | 83.08 | 140 |
| 60.00 | 1.00 | 0.00 | 0.00 | 341.59 | 180.00 | 8.73 | 150 |
|  | 2.00 | 0.07 | -3.53 | 32.34 | 179.99 | 39.66 | 150 |
|  | 3.00 | 0.94 | -2.57 | 53.95 | 179.99 | 68.47 | 150 |
|  | 4.00 | 5.12 | -11.14 | 157.54 | 180.01 | 3.08 | 150 |
| 70.00 | 1.00 | 0.00 | -11.55 | 359.48 | 180.00 | 21.4 | 160 |
|  | 2.00 | 0.08 | 0.00 | 360.00 | 180.00 | 20 | 160 |
|  | 3.00 | 0.14 | -3.61 | 359.71 | 180.00 | 20.45 | 160 |
|  | 4.00 | 2.97 | -6.10 | 0.38 | 180.00 | 20.74 | 160 |
| 80.00 | 1.00 | 0.00 | -3.84 | 359.62 | 180.00 | 9.68 | 170 |
|  | 2.00 | 1.76 | 0.00 | 360.00 | 180.00 | 10 | 170 |
|  | 3.00 | 2.08 | -3.78 | 0.37 | 180.00 | 9.68 | 170 |
|  | 4.00 | 3.95 | -8.40 | 359.45 | 180.00 | 10.69 | 170 |

Table 5 TDL model for LoS

| Elevation Angle[°] | Cluster # | Cluster PAS | Normalized Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- | --- | --- |
| 10.00 | 1.00 | Specular(LOS path) | 0.00 | -0.12 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -16.01 | Rayleigh |
|  | 2.00 | Laplacian | 13.56 | -33.40 | Rayleigh |
|  | 3.00 | Laplacian | 19.18 | -30.55 | Rayleigh |
|  | 4.00 | Laplacian | 20.35 | -28.69 | Rayleigh |
|  | 5.00 | Laplacian | 61.50 | -51.63 | Rayleigh |
|  | 6.00 | Laplacian | 82.41 | -58.91 | Rayleigh |
| 20.00 | 1.00 | Specular(LOS path) | 0.00 | -0.08 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -25.95 | Rayleigh |
|  | 2.00 | Laplacian | 3.12 | -21.80 | Rayleigh |
|  | 3.00 | Laplacian | 5.29 | -25.56 | Rayleigh |
|  | 4.00 | Laplacian | 9.38 | -23.52 | Rayleigh |
|  | 5.00 | Laplacian | 23.40 | -30.60 | Rayleigh |
| 30.00 | 1.00 | Specular(LOS path) | 0.00 | -0.09 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -20.33 | Rayleigh |
|  | 2.00 | Laplacian | 4.44 | -20.31 | Rayleigh |
|  | 3.00 | Laplacian | 21.12 | -27.61 | Rayleigh |
|  | 4.00 | Laplacian | 22.65 | -40.22 | Rayleigh |
| 40.00 | 1.00 | Specular(LOS path) | 0.00 | -0.07 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -29.43 | Rayleigh |
|  | 2.00 | Laplacian | 0.85 | -22.68 | Rayleigh |
|  | 3.00 | Laplacian | 2.42 | -20.59 | Rayleigh |
|  | 4.00 | Laplacian | 51.16 | -34.42 | Rayleigh |
| 50.00 | 1.00 | Specular(LOS path) | 0.00 | -0.06 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -27.61 | Rayleigh |
|  | 2.00 | Laplacian | 2.45 | -20.83 | Rayleigh |
|  | 3.00 | Laplacian | 3.02 | -27.18 | Rayleigh |
|  | 4.00 | Laplacian | 24.10 | -27.92 | Rayleigh |
| 60.00 | 1.00 | Specular(LOS path) | 0.00 | -0.06 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -23.33 | Rayleigh |
|  | 2.00 | Laplacian | 5.13 | -21.78 | Rayleigh |
|  | 3.00 | Laplacian | 21.09 | -27.29 | Rayleigh |
| 70.00 | 1.00 | Specular(LOS path) | 0.00 | -0.10 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -17.94 | Rayleigh |
|  | 2.00 | Laplacian | 4.39 | -23.97 | Rayleigh |
|  | 3.00 | Laplacian | 15.02 | -31.08 | Rayleigh |
|  | 4.00 | Laplacian | 17.66 | -26.18 | Rayleigh |
| 80.00 | 1.00 | Specular(LOS path) | 0.00 | -0.08 | LOS path |
|  | 1.00 | Laplacian | 0.00 | -26.98 | Rayleigh |
|  | 2.00 | Laplacian | 4.54 | -21.21 | Rayleigh |
|  | 3.00 | Laplacian | 9.08 | -20.75 | Rayleigh |
|  | 4.00 | Laplacian | 13.70 | -30.54 | Rayleigh |

Table 6 TDL model for NLoS

| Elevation Angle | Cluster # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- | --- |
| 10.00 | 1.00 | 0.00 | 0.00 | Rayleigh |
|  | 2.00 | 0.40 | -2.40 | Rayleigh |
|  | 3.00 | 1.12 | -0.32 | Rayleigh |
|  | 4.00 | 1.88 | -3.86 | Rayleigh |
|  | 5.00 | 2.94 | -9.12 | Rayleigh |
|  | 6.00 | 4.33 | -9.75 | Rayleigh |
| 20.00 | 1.00 | 0.00 | 0.00 | Rayleigh |
|  | 2.00 | 0.95 | -7.82 | Rayleigh |
|  | 3.00 | 2.77 | -11.84 | Rayleigh |
|  | 4.00 | 6.67 | -17.89 | Rayleigh |
|  | 5.00 | 13.95 | -37.32 | Rayleigh |
| 30.00 | 1.00 | 0.00 | -0.77 | Rayleigh |
|  | 2.00 | 0.53 | 0.00 | Rayleigh |
|  | 3.00 | 1.05 | -5.71 | Rayleigh |
|  | 4.00 | 2.45 | -3.83 | Rayleigh |
|  | 5.00 | 3.47 | -8.97 | Rayleigh |
| 40.00 | 1.00 | 0.00 | 0.00 | Rayleigh |
|  | 2.00 | 0.84 | -6.00 | Rayleigh |
|  | 3.00 | 0.94 | -6.04 | Rayleigh |
|  | 4.00 | 1.05 | -4.76 | Rayleigh |
|  | 5.00 | 7.42 | -15.22 | Rayleigh |
| 50.00 | 1.00 | 0.00 | 0.00 | Rayleigh |
|  | 2.00 | 0.31 | -3.98 | Rayleigh |
|  | 3.00 | 4.18 | -17.39 | Rayleigh |
|  | 4.00 | 4.69 | -12.39 | Rayleigh |
| 60.00 | 1.00 | 0.00 | 0.00 | Rayleigh |
|  | 2.00 | 0.07 | -3.53 | Rayleigh |
|  | 3.00 | 0.94 | -2.57 | Rayleigh |
|  | 4.00 | 5.12 | -11.14 | Rayleigh |
| 70.00 | 1.00 | 0.00 | -11.55 | Rayleigh |
|  | 2.00 | 0.08 | 0.00 | Rayleigh |
|  | 3.00 | 0.14 | -3.61 | Rayleigh |
|  | 4.00 | 2.97 | -6.10 | Rayleigh |
| 80.00 | 1.00 | 0.00 | -3.84 | Rayleigh |
|  | 2.00 | 1.76 | 0.00 | Rayleigh |
|  | 3.00 | 2.08 | -3.78 | Rayleigh |
|  | 4.00 | 3.95 | -8.40 | Rayleigh |

Proposal 3: Capture the parameters of fast fading listed from Table 3 to Table 6 into TR38.811 for sub-urban.

# Conclusion

In this contribution, the channel model for suburban is established based on simulated results with following proposals:

Proposal 1: Capture the large scale parameters listed in Table 1 into TR38.811 for sub-urban.

Proposal 2: Capture the parameters of fast fading listed in Table 2 into TR38.811 for sub-urban.

Proposal 3: Capture the parameters of fast fading listed from Table 3 to Table 6 into TR38.811 for sub-urban.

# Reference

R1-1804235 Discussion on the large scale channel model for NTN, ZTE