3GPP TSG RAN WG1 Meeting #93                           R1-1806204

Busan, Korea, May 21st – May 25th, 2018

Source: 	General Motors

Title:  	V2X NR Evaluation Methodology – Vehicle UE Antenna Radiation Patterns

Agenda Item:	7.5.2

Document for:	Discussion and decision

# Introduction

In our last contribution [1] we have shown a more realistic radiation patterns of vehicle antennas located in several locations. As these patterns differ rather significantly from an isotropic ideal antenna, it was agreed in Sanya meeting that we should try to describe the radiation pattern in 4 locations with compliance to [2] radiation tables. This contribution outlines the 3D radiation patterns as a linear combination of the vertical & horizontal planes in accordance to the tables given in [2].

# Radiation patterns of Rooftop & Bumpers Vehicle Antennas

In this section, we tried to find the analytical fit for the radiation pattern simulated in [1] using HFSS. As agreed the chosen locations of vehicle antennas are: Front bumper, Front rooftop, Rear rooftop and Rear bumper.

## Simulation Setup

For each antenna location, we analyzed the simulated 3D radiation pattern using Matlab and used curve fitting tools to find the best fit. The fitting was done on the horizontal & vertical planes (2D fitting) separately and then was combined linearly to get the 3D pattern. We chose a sum of sine fitting functions of different lengths to best describe the horizontal & vertical planes. Each table outlines the vertical cut with a sum of sines terms and sometimes with a combination of φ=0 and 180 slices, the horizontal cut with a sum of sines terms as well.

We used the following coordinates: φ indicate the azimuth angle and θ the zenith

Figure 1: Coordinate system

## Front Bumper

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 9.7132; 0.9808; -0.5981. 2; 52.687; 3.5189; 1.7337. 3; 1.9398; 7.6978; -0.567. 4; 1.493; 31.729; 2.9118. 5; 1.2458; 133.66; 2.0582. 6; 49.54; 3.6133; -1.4585. Av180dB"θ,ϕ=180°=k=16aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 96.124; 1.5876; 2.2991. 2; 103.76; 1.5571; -0.7673. 3; 5.1303; 4.1706; -0.7641. 4; 1.8347; 8.1647; 1.6036. 5; 1.3695; 10.225; 1.7803. 6; 1.4223; 105.75; -1.2654. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=90°,ϕ=k=17aksin(bkϕ+ck), ϕ∈[0°,360°] k; ak; bk; ck. 1; 14.853; 0.8704; 1.9834. 2; 34.526; 1.5913; -3.4096. 3; 0; 0; 0. 4; 22.568; 1.8851; 5.1064. 5; 341.79; 7.6418; -3.5488. 6; 0; 0; 0. 7; 343.83; 7.6442; -0.4147. |
| 3D radiation power pattern (dB) | AdB"θ,ϕ=AvdB"θ,ϕ=0°, ϕ∈[0°,180°) Av180dB"θ,ϕ=180°, ϕ∈[180°,360°)+AhdB"θ=90°,ϕ |
| Maximum directional gain of an antenna element GE,max | 15dBi |

Figure 2: Front bumper 3D radiation pattern simulated and fitted

Figure 3: Front bumper azimuth radiation pattern at θ=90 degrees

Figure 4: Front bumper elevation radiation pattern at φ=0,180 degrees

## Front Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=13aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 192.93; 0.9427; -0.4122. 2; 1.7273; 3.9284; -1.6847. 3; 180.17; 0.9609; 2.7332. Av180dB"θ,ϕ=180°=k=13aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 192.93; 0.9427; -0.4122. 2; 1.7273; 3.9284; -1.6847. 3; 180.17; 0.9609; 2.7332. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=90°,ϕ=k=18aksin(bkϕ+ck), ϕ∈[0°,360°] k; ak; bk; ck. 1; 11.185; 0.2051; 4.1487. 2; 7.528; 0.5807; -0.1995. 3; 1.3186; 2.6255; -3.584. 4; 0.9018; 3.3371; -2.7043. 5; -0.1354; 4.7353; 1.408. 6; 0.1872; 8.143; 1.3413. 7; -0.339; 4.7214; -1.0956. 8; 0.1436; 15.035; -1.5848. |
| 3D radiation power pattern (dB) | AdB"θ,ϕ=AvdB"θ,ϕ=0°, ϕ∈[0°,180°) Av180dB"θ,ϕ=180°, ϕ∈[180°,360°)+AhdB"θ=90°,ϕ |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Figure 5: Front rooftop 3D radiation pattern simulated and fitted

Figure 6: Front rooftop azimuth radiation pattern at θ=90 degrees

Figure 7: Front rooftop elevation radiation pattern at φ=0 degrees

## Rear Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=13aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 93.163; 1.0139; -0.3791. 2; 80.811; 1.0781; 2.763. 3; 1.2623; 3.926; -1.7473. Av180dB"θ,ϕ=180°=k=13aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 3673.7; 1.1581; 2.7532. 2; 3684.8; 1.1565; -0.3885. 3; 2.0611; 4.0737; -1.5889. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=90°,ϕ=k=18aksin(bkϕ+ck), ϕ∈[0°,360°] k; ak; bk; ck. 1; 12.585; 0.0495; 5.3295. 2; 3.2424; 0.3245; 1.0107. 3; 1.3083; 2.2681; -2.4179. 4; 1.1816; 5.0733; -1.808. 5; 0.8098; 6.8816; -1.2234. 6; 0.7859; 3.6043; -3.5476. 7; 0.4764; 15.903; -1.2973. 8; 0.4331; 17.887; -1.2497. |
| 3D radiation power pattern (dB) | AdB"θ,ϕ=AvdB"θ,ϕ=0°, ϕ∈[0°,180°) Av180dB"θ,ϕ=180°, ϕ∈[180°,360°)+AhdB"θ=90°,ϕ |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Figure 8: Rear rooftop 3D radiation pattern simulated and fitted

Figure 9: Rear rooftop azimuth radiation pattern at θ=90 degrees

Figure 10: Rear rooftop elevation radiation pattern at φ=0,180 degrees

## Rear Bumper

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-90°,90°] k; ak; bk; ck. 1; 34.137; 0.7566; 1.9548. 2; 8.4361; 4.7741; -2.1219. 3; 47.354; 0.7249; -0.8207. 4; 3.8461; 8.5329; 0.0521. 5; 9.0891; 5.2761; 1.0035. 6; 3.0768; 9.9553; -2.0133. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=90°,ϕ=k=14aksin(bkϕ+ck), ϕ∈[0°,360°] k; ak; bk; ck. 1; 53.959; 0.1497; 3.7638. 2; 66.001; 0.0684; 0.4298. 3; 3.5172; 1.3133; 0.6461. 4; 1.8232; 3.2785; 0.7957. |
| 3D radiation power pattern (dB) | AdB"θ,ϕ=AvdB"θ,ϕ=0°+AhdB"θ=90°,ϕ |
| Maximum directional gain of an antenna element GE,max | 20dBi |

Figure 11: Rear bumper 3D radiation pattern simulated and fitted

Figure 12: Rear bumper azimuth radiation pattern at θ=90 degrees

Figure 13: Rear bumper elevation radiation pattern at φ=0 degrees

# References

[1] R1-1805259, “Suggestion for the incorporation of realistic vehicle antenna patterns”. Sanya, China, April 16th – 20th 2018.

[2] 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”.

# Summary

Observation 1:

The fitting of a 3D radiation pattern can’t fully describe the simulated radiation pattern.

Observation 2:

The fitted 3D radiation is only an approximation for a general radiation pattern simulated on a generic vehicle. We assume it will change for different vehicle of same type but hope not significantly.

Observation 3:

A better fit (or even no fit at all) will probably yield better results in the performance evaluation.

Proposal 1:

Explore 3D fitting algorithms instead of a simple linear combination of 2D planes.

Proposal 2:

Add real world antenna patterns into the simulation methodology. FFS