3GPP TSG RAN WG1 Meeting #93	R1- 1806314

Busan, Korea, 21st  – 25th May 2018

# Source:              CATT

Title:                  Frame Structure for NR Unlicensed Operations

Agenda Item:     7.6.2

Document for:   Discussion and Decision

Introduction

In RAN1#92bis, the frame structure of NR access in unlicensed spectrum were discussed with the following agreements,

Agreement:

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- For sub-7 GHz, NR-U study the SCSs, 15/30/60KHz

  - Study performance difference between different SCS

  - Study if changes to UL design are needed to meet the PSD and OCB requirements

  - Study if an SS block design/RMSI/OSI with 60KHz SCS is needed

  - Impact on MIB and SIB1 content

  - Need for use of ECP for 60KHz

  - RACH design with 60KHz SCS in addition to options currently part of NR

  - Other considerations are not precluded.

  - Impact on support of different BWs with different SCS

- Study supporting more than one switching points within a TxOP

  - FFS the LBT requirement for each DL/UL data/control burst in the TxOP

In this contribution, we mainly discuss the frame structure of  NR system operation in unlicensed bands.

Frame Structure Aspects of NR System Operation in Unlicensed Spectrum

NR system is designed with flexible frame structure and the consideration of operation in unlicensed spectrum in the beginning.   NR frame structure includes the slot and mini-slot structures, different numerology of subcarrier spacing with power 2, flexible starting time of PDSCH/PUSCH transmission in mini-slot with 1 symbol granularity, support of slot aggregation, flexible HARQ timing, dynamic DL/UL configuration within a slot and dynamic bandwidth adaptation through BWP configuration and support of beamforming in the frame structure design.

The slot is the basic unit of NR frame structure with 14 symbols in length.  Similar to LTE, each radio frame contains 10 subframes in NR.   Each subframe contains 2µ number of slots with µ defined at Table 4.3.2-1 of [2] for different numerologies.   Each SCS would support the range of operating BW in PRBs defined in [3].

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | [78] | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | [38] | 51 | 65 | 79 | 107 | 135 |

Different numerologies could also be configured for different type of service with different QoS requirements, such as latency requirements, for the access in the unlicensed spectrum.   It was agreed in RAN1#92bis to study SCS 15/30/60 kHz.   For large SCS, the symbol duration is shorter.  The granularity in time and the length of a slot would be relatively smaller for large SCS to have small delay in the access of unlicensed spectrum.

It was agreed in RAN1#92bis that both Type-A and Type-B mapping with consideration of additional starting position.   The slot boundary starts at the radio frame boundary.  The length of mini-slot varies from 1 to 14 symbols.    The flexibility starting time of PDSCH/PUSCH transmission in mini-slot with 1 symbol granularity enables the immediate NR DL/UL access in unlicensed spectrum when the channel is sensed to be clear.    The support of slot aggregation would extend the NR transmission interval up to the allowed maximum channel occupancy time (MCOT).   This would improve the efficiency of NR access in unlicensed spectrum.   In Rel-15, only 2-symbol, 4-symbol, and 7-symbol mini-slots are supported.   In order to have efficient operation in unlicensed spectrum, the flexible mini-slot structure from 1-14 symbols with starting time granularity of 1 symbol should be supported in Rel-16.

Proposal 1: Mini-slot structure should be used as the starting point for the NR frame structure operation in the unlicensed band.

Proposal 2: The flexible mini-slot structure from 1-14 symbols with starting time granularity of 1 symbol should be supported in Rel-16 for efficient operation in unlicensed spectrum.

The dynamic DL/UL configuration allows dynamic selection of gNB DL transmission or UE UL access in unlicensed spectrum based on DL/UL traffic arrival when the channel is sensed to be clear.   The dynamic DL/UL configuration is explicitly indicated by the SFI in the group common PDCCH.   The DL/UL configuration could also be indicated implicitly through the DL assignment or UL grant.  UE does not need to know the resource unlicensed spectrum would be used for the DL or UL in advance.   UE would assume that the resource is used for DL or UL transmission based on the reception of DL assignment or UL grant.    UE would receive DL PDSCH transmission right after the reception of DL assignment.   For UL transmission, the UE will transmit PUSCH at the scheduled time after receiving the UL grant.  UE might not be able to start PUSCH transmission at the scheduled transmission in unlicensed spectrum.   Thus, implicit DL/UL configuration through DL/UL resource allocation needs to be further studied for the NR access in unlicensed spectrum.

Proposal 3: Implicit DL/UL configuration through DL/UL resource allocation needs to be further studied for the NR access in unlicensed spectrum.

Conclusion

In this contribution, we discussed the frame structure aspects of NR-based standalone operation in unlicensed bands.  We have the following proposals,

- Proposal 1: Mini-slot structure should be used as the starting point for the NR frame structure operation in the unlicensed band.

- Proposal 2: The flexible mini-slot structure from 1-14 symbols with starting time granularity of 1 symbol should be supported in Rel-16 for efficient operation in unlicensed spectrum.

- Proposal 3: Implicit DL/UL configuration through DL/UL resource allocation needs to be further studied for the NR access in unlicensed spectrum.

- References

RP-170828, “New SID on NR-based Access to Unlicensed Spectrum”, Dubrovnik, Croatia, March 6 - 9, 2017.

TS38.211v15.1.0, “Physical Channels and Modulation”

TS38.101v15.1.0, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

R1-1803678, “NR frame structure and scheduling on unlicensed bands”, Huawei, HiSilicon

R1-1803778, “Frame Structure for NR Unlicensed Operations”, 	CATT

R1-1803855, “Frame structure for NR-U operation	vivo

R1-1803947, “Flexible starting transmission point for NR-U”, Fujitsu

R1-1803949, “Discussion on frame structure and scheduling for NR-U”, ZTE, Sanechips

R1-1803973, “Frame structure for NR-U”, OPPO

R1-1804063, “On frame structure design for NR-U operation”, MediaTek Inc.

R1-1804275, “Frame structure for NR-U operation”, Nokia, Nokia Shanghai Bell

R1-1804403, “Frame structure for NR-U”, 	Samsung

R1-1804580, “Frame structure for NR unlicensed operation”, LG Electronics

R1-1804592, “Frame structure and numerology for NR-U”, ITL

R1-1804598, “Considerations on frame structure for NR unlicensed operations”, Sony

R1-1804750, “On the flexibility of NR to operate in unlicensed spectrum”, 	Intel Corporation

R1-1804829, “Frame structure for NR-U operation”, Qualcomm Incorporated

R1-1804868, “On Frame Structure for NR-U Operation”, 	InterDigital, Inc.

R1-1805009, “On Frame Structure for NR in Unlicensed Spectrum”, Ericsson

R1-1805253, “Frame structure design in NR-U”, CATR

- .