3GPP TSG RAN WG1 Meeting #93		                                         R1-1806371

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.2.1

Source: 	CMCC

Title: 	CQI and MCS design for URLLC

Document for:	Discussion and Decision

- Introduction

In the RAN#78, the scope for URLLC work for Rel-15 is endorsed and the following is included: Specify, CQI table and MCS table design targeting high reliability. In RAN1#92bis meeting, the following agreements have been made:

Agreements:

- The two BLER targets that are configurable for URLLC for CSI reporting are:

  - Option B. (10-1, 10-5)

  - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors.

Agreements:

- Highest spectral efficiency for CQI based on 10-5 BLER target for URLLC is no more than 772/1024*6

- Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6

  - It doesn’t necessarily mean that the CQI table introduced for eMBB cannot be directly reused for URLLC – it’s still a separate discussion

- Note that

  - Whether or not to have two tables or a single table covering both BLER targets is a separate issue

R1-1805642	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Agreements:

- In total, there are two CQI tables for URLLC CQI reporting

  - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC

    - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned

  - The new table will have entries corresponding to BLER target 10-5

- For CSI reporting, the CQI field is 4-bit.

Agreements:

- For BLER 10-5,

- Companies are encouraged to perform simulations for the new CQI table for URLLC, including

  - The lowest SE entry

    - E.g., 30~50/1024*2

  - Note that the highest SE entry of no more than 772/1024*6 is already agreed

  - Consider using approximately equally spaced SNR values

    - Other options are not precluded

    - Whether or not some existing CQI entries for BLER 10-1 can be reused

      - Consider existing CQI entries when applicable

  - In total 15 CQI entries (+1 OOR entry)

- In performing the simulations, consider

  - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns)

    - Other options are not precluded

  - Payload of 32 bytes

    - Other payload sizes can also be considered, up to each company

  - SNR at 5% geometry for the lowest SE entry

    - Other options are not precluded

  - For other simulation assumptions, refer to agreements from RAN1#92

- Similar considerations are also applicable to the MCS table evaluations

In this contribution, we will further discuss the design of CQI and MCS table for URLLC.

- CQI table

In RAN1 #92bis meeting, it was agreed that two CQI tables are used for URLLC CSI reporting corresponding to two BLER targets i.e. 10-1 and 10-5 respectively. The CQI table with 10-1 BLER target is the same as the existing 64QAM CQI table without any change, however, when it comes to 10-5 BLER target, it is expected to focus on much lower modulation order and code rate with maximum reuse of existing CQI entities for BLER 10-1. Therefore, we suppose that the CQI table for 10-5 BLER target is modified from 64QAM CQI table (Table 1) for BLER target 10-1. Moreover, some entities with high spectral efficiency should be removed from the table to allow lower spectral efficiency entities to be added to satisfy 10-5 BLER.

In last RAN1 meeting, it was agreed that the lowest SE entry should be in range of 30~50/1024*2. The minimum SE entry needs to take two factors into account: 1) As we discussed in R1-1804105 [3], in some scenarios, occasions for retransmissions or repetitions are unavailable due to the latency restrictions, hence it is reasonable to suppose that the minimum SE entry has to support 10-5 target BLER within one-shot transmission at 5% CDF point; 2) The SNR spacing should be approximately equal with that between the existing adjacent entries. In Figure.1, we observe from downlink SINR CDF curve that 5% CDF point is found within range of -2.39dB~-2.14dB in typical UMa scenario which is assumed for URLLC operation. Figure.2 shows the BLER performance of low SE entries over TDL-C channel with the assumptions in table 3, we can see that the minimum SE point of 0.1523 in legacy table requires SNR around 1dB at the receiver which is higher than 5% CDF point (-2.39dB~-2.14dB) in Figure.1. Therefore, taking the two factors which are mentioned above into consideration, we propose that two low SE entries {code rate = 30/1024} and {code rate = 50/1024} should be added to the CQI table for 10-5 BLER target and the corresponding SNR-SE curve can be found in Figure. 3.

Since that two new SE entries are added into the CQI table, we can obviously derive that the highest SE entry should be 772/1024*6 from a table size perspective. Therefore, table 2 should be adopted as CQI table for 10-5 BLER target.

Proposal 1: Table 2 is used for URLLC CQI table with 10-5 target BLER. The Minimum SE point in the new CQI table is 30/1024*2.

Figure 1: Downlink geometry for URLLC urban macro scenario

Table 1: Legacy CQI Table (DL 64QAM): Table 5.2.2.1-2: TS 38.214

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 120 | 0.2344 |
| 3 | QPSK | 193 | 0.3770 |
| 4 | QPSK | 308 | 0.6016 |
| 5 | QPSK | 449 | 0.8770 |
| 6 | QPSK | 602 | 1.1758 |
| 7 | 16QAM | 378 | 1.4766 |
| 8 | 16QAM | 490 | 1.9141 |
| 9 | 16QAM | 616 | 2.4063 |
| 10 | 64QAM | 466 | 2.7305 |
| 11 | 64QAM | 567 | 3.3223 |
| 12 | 64QAM | 666 | 3.9023 |
| 13 | 64QAM | 772 | 4.5234 |
| 14 | 64QAM | 873 | 5.1152 |
| 15 | 64QAM | 948 | 5.5547 |

Table 2: Proposed CQI Table for 10-5 BLER target

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | 16QAM | 449 | 0.8770 |
| 8 | 16QAM | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 64QAM | 490 | 1.9141 |
| 11 | 64QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

Figure 2: Performance of CQI entries with low spectral efficiency

Figure 3: Spectral efficiency for the proposed CQI table entries

Table 3: Link level simulation assumptions

| Parameters | Value |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| Modulations | QPSK |
| Code rate | {30,40,50,78,120}/1024 |
| Channel coding | LDPC |
| Transmission scheme | 1-port precoder fixed |
| Allocation in time | Slot transmission 7os (5os for data,2os for DMRS) |
| TB size | 256bits |
| Channel estimation | LMMSE |
| Channel model | TDL-C (delay spread: 300ns) |
| UE speed | 3 km/h |
| Number of BS antennas | 1Tx |
| Number of UE antennas | 2Rx |

- MCS table

Similar with the construction methodology for legacy MCS table, the URLLC MCS table for 10-5 BLER target should consist of the CQI entries and the interpolation between adjacent CQI entries. Specifically, in order to achieve 10-5 reliability for URLLC with one-shot transmission, we propose that the MCS table should include the same lowest SE entry as the proposed CQI table, hence we can derive the MCS index 0~5 highlighted in table 5. And MCS entries with code rate of larger than 666/1024 highlighted in table 4 are precluded. This is because that 2/3 code rate is used as the thresholds to select LDPC BG1 or BG2 for transport block encoding and BG2 provides better performance than BG1 towards code rate lower than 2/3 with lower implementation complexity. If maximum code rate is limited no larger than 666/1024, it will reduce URLLC coding/decoding processing time and complexity. Moreover, the rest of MCS entries up to code rate of 666/1024 and the last 3 entries corresponding to 3 different modulation orders can reuse the current DL 64 QAM MCS table for eMBB.

Table 4: Table 5.1.3.1-1: MCS index table 1 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate x [1024] R | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 5: Proposed new MCS table for PDSCH and PUSCH with CP-OFDM for URLLC

| MCS Index | Modulation Order | Target code Rate x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 340 | 1.3281 |
| 17 | 4 | 378 | 1.4766 |
| 18 | 4 | 434 | 1.6953 |
| 19 | 4 | 490 | 1.9141 |
| 20 | 4 | 553 | 2.1602 |
| 21 | 4 | 616 | 2.4063 |
| 22 | 4 | 658 | 2.5703 |
| 23 | 6 | 438 | 2.5664 |
| 24 | 6 | 466 | 2.7305 |
| 25 | 6 | 517 | 3.0293 |
| 26 | 6 | 567 | 3.3223 |
| 27 | 6 | 616 | 3.6094 |
| 28 | 6 | 666 | 3.9023 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

For UL CP-OFDM, the proposed MCS table for PDSCH can be reused. However, for DFT-s-OFDM, pi/2 BPSK can be applied to decrease PAPR and improve coverage performance. Therefore, pi/2 BPSK entries should be considered for the 6 lowest entries highlighted in Table 6.

Table 6: Proposed new MCS table for PUSCH with DFT-s-OFDM for URLLC

| MCS Index | Modulation Order | Target code Rate x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | q | 240/q | 0.2344 |
| 7 | q | 314/q | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 340 | 1.3281 |
| 17 | 4 | 378 | 1.4766 |
| 18 | 4 | 434 | 1.6953 |
| 19 | 4 | 490 | 1.9141 |
| 20 | 4 | 553 | 2.1602 |
| 21 | 4 | 616 | 2.4063 |
| 22 | 4 | 658 | 2.5703 |
| 23 | 6 | 466 | 2.7305 |
| 24 | 6 | 517 | 3.0293 |
| 25 | 6 | 567 | 3.3223 |
| 26 | 6 | 616 | 3.6094 |
| 27 | 6 | 666 | 3.9023 |
| 28 | 1 | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Proposal 2: Table 5 is used for MCS table for PDSCH and PUSCH with CP-OFDM for URLLC. The Minimum SE point in the new MCS table is 30/1024*2.

Proposal 3: Table 6 is used for MCS table for PUSCH with DFT-s-OFDM for URLLC. The Minimum SE point in the new MCS table is 60/1024.

- Conclusion

In this contribution, we discussed the design of CQI and MCS table for URLLC and the following proposals are made:

Proposal 1: Table 2 is used for URLLC CQI table with 10-5 target BLER. The Minimum SE point in the new CQI table is 30/1024*2.

Proposal 2: Table 5 is used for MCS table for PDSCH and PUSCH with CP-OFDM for URLLC. The Minimum SE point in the new MCS table is 30/1024*2.

Proposal 3: Table 6 is used for MCS table for PUSCH with DFT-s-OFDM for URLLC. The Minimum SE point in the new MCS table is 60/1024.

Table 2: Proposed CQI Table for 10-5 BLER target

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | 16QAM | 449 | 0.8770 |
| 8 | 16QAM | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 64QAM | 490 | 1.9141 |
| 11 | 64QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

Table 5: Proposed new MCS table for PDSCH and PUSCH with CP-OFDM for URLLC

| MCS Index | Modulation Order | Target code Rate x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 340 | 1.3281 |
| 17 | 4 | 378 | 1.4766 |
| 18 | 4 | 434 | 1.6953 |
| 19 | 4 | 490 | 1.9141 |
| 20 | 4 | 553 | 2.1602 |
| 21 | 4 | 616 | 2.4063 |
| 22 | 4 | 658 | 2.5703 |
| 23 | 6 | 438 | 2.5664 |
| 24 | 6 | 466 | 2.7305 |
| 25 | 6 | 517 | 3.0293 |
| 26 | 6 | 567 | 3.3223 |
| 27 | 6 | 616 | 3.6094 |
| 28 | 6 | 666 | 3.9023 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

| Table 6: Proposed new MCS table for PUSCH with DFT-s-OFDM for URLLCMCS Index | Modulation Order | Target code Rate x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | q | 240/q | 0.2344 |
| 7 | q | 314/q | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 340 | 1.3281 |
| 17 | 4 | 378 | 1.4766 |
| 18 | 4 | 434 | 1.6953 |
| 19 | 4 | 490 | 1.9141 |
| 20 | 4 | 553 | 2.1602 |
| 21 | 4 | 616 | 2.4063 |
| 22 | 4 | 658 | 2.5703 |
| 23 | 6 | 466 | 2.7305 |
| 24 | 6 | 517 | 3.0293 |
| 25 | 6 | 567 | 3.3223 |
| 26 | 6 | 616 | 3.6094 |
| 27 | 6 | 666 | 3.9023 |
| 28 | 1 | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

- References

- RP-172817, NR High-Reliability URLLC scope for RAN1/RAN2, Ericsson. RAN #78, September, 2017.

- Chairman’s notes RAN1 #92bis, April, 2018.

- R1-1804105 Discussion on BLER requirement for URLLC, CMCC, RAN1 #92bis, April, 2018.