3GPP TSG RAN WG1 Meeting #93                                              R1-1806797

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.6.4.1

Source: 	MediaTek Inc.

Title: 	Channel access to NR-based unlicensed spectrum

Document for:	Discussion and decision

# Introduction

The study item of NR-based access to unlicensed spectrum was approved in RAN #75 meeting. Besides NR based license assisted access, standalone (SA) operation of unlicensed spectrum is also in the scope of this study item. In this contribution, we take a high level survey of current NR features in licensed spectrum that may have impact on unlicensed operation, with a focus on channel access mechanism since it is crucial to the efficient utilization of the unlicensed spectrum. In addition, we also share our views on wideband operation for NR-based unlicensed spectrum. This is a revision of R1-1800154.

Agreement: (RAN1 #92bis) - Study possible enhancements for HARQ operation - Study changes needed for Configured Grant support in NR-U - Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer multiple of 20MHz - At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz. - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz. - Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms - Techniques to cope with directional antennas/transmissions - Receiver assisted LBT : RTS/CTS type mechanism - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed - Techniques to enhance spatial reuse - Preamble detection - Enhancements to baseline LBT mechanisms above 7 GHz - Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz. - Note: Other aspects are not precluded from being included

# Intra-RAT Frequency Reuse

Modern cellular network in licensed spectrum uses reuse-1 deployment and adaptive rate control to mitigate co-channel inter-cell interference. Unlicensed access can adopt similar principle within the same Radio Access Technology (RAT), or at least in the same RAT deployed by the same operator. Uncoordinated LBT operation among different nodes of the same RAT in the same network results in unnecessary blocking of one another and thus lower efficiency.

Facilitating tighter frequency reuse in unlicensed spectrum is of particular interest to the deployment in the 3.5 GHz Citizens Broadband Radio Service (CBRS) band, in which tiered access rights are granted to the fee-paying Priority Access Licensees (PAL) as well as General Authorized Access (GAA) users. The former has higher priority over the latter and may deploy a network with tighter frequency reuse. In the absence of the PAL and other GAA user, a GAA user may opportunistically operate its network with tighter reuse.

# Omni-directional Carrier Sensing

LBT is a spectrum sharing mechanism that works across different RATs. However, it suffers from the hidden node and exposed node problems. The well-known issues are due to the fact that coverages of the nodes in a network are different. The problem becomes more severe and complicated when the coverage of sensing and the coverage of transmitting are different. This may occur when an omnidirectional antenna pattern is used for carrier sensing while a directional antenna (or array) pattern is used for transmission, leading to higher chance of the node’s being exposed.

Figure 1 (a) shows that a device A senses the carrier using omnidirectional antenna pattern and overhears the transmission from device C to D. Device A subsequently refrains from transmitting to its targeted receiving device B, which is an unnecessary move since transmission from A to B using highly directional beams Figure 1 (b) does not interfere with the transmission from C to D.

Figure 1: (a) Omnidirectional carrier sensing, (b) two concurrent directional communication links

If the direction of the communication is known, directional carrier sensing may help in certain situations. However, this leads to a different kind of problem, as illustrated in [1] (Figure 1). Effects of the directivity of carrier sensing and transmission should therefore be studied thoroughly and necessary measures be taken to maximize the system’s performance.

# Channel Access based on Message Exchange

If the nodes in the network are of the same RAT and can therefore exchange short control messages (which are themselves subject to LBT) before the long data transaction, the problems of hidden and exposed nodes can be better managed. The Request To Send (RTS) and Clear To Send (CTS) protocol in WiFi is a simple example. RTS/CTS protocol is not currently adopted in LAA/eLAA. However, it may be worthy of reconsideration in NR unlicensed access since the hidden node and exposed node problems become more severe in directional link as mentioned above.

Figure 2 shows an example of RTS/CTS resolving exposed node problem when device A is attempting to transmit to device B. In the figure, solid line denotes transmit beam while dashed line denotes receive beam. Before A transmits its data to B, it sends a short RTS carrying the identities of the source (A) and destination (B) along with the expected duration of the data transmission. The RTS is heard by both its intended target B as well as a third device C. Upon receiving the RTS of its source node A, device B transmits a CTS, which is heard by A but not by C. After receiving the CTS from B, A understands that the medium is available and has been reserved by the RTS/CTS exchange for a certain duration. It then starts transmitting to B. Since device C never hears a CTS within an expected short interval, device C realizes its being an exposed node and may transmit to D. Two concurrent directional links can thus be established without interfering each other.

Figure 2: RTS/CTS in directional links

The benefits of such message-exchange based spectrum sharing technique depend on the propagation environment and traffic condition and should be carefully evaluated in different NR unlicensed scenarios. Also message exchange among nodes to improve frequency reuse factor as discussed in Section 1 should be studied.

# Wideband Operation

Both carrier aggregation (CA) and bandwidth part (BWP) adaptation mechanisms are supported in NR for wideband operation. In NR-U, we think both mechanisms should be supported as well since they offer different benefits. With CA, the access channels are not necessarily to be contiguous to each other. But it takes longer time to activate and deactivate a carrier component. BWP is adopted to NR in order to provide more dynamic and efficient control over wideband operation. When UE is in a low activity stage such as idle or inactive modes, UE can stay on a narrowband BWP so that it can save its power significantly. Power saving is very important to UEs. Hence, we think BWP should be adopted to NR-U. Based on the previous agreement, when the absence of Wi-Fi cannot be guaranteed, LBT is operated on a 20MHz bandwidth basis. Consequently, BWP should be configured as integer multiples of 20MHz. Details of BWP configuration for NR-U should be further studied.

# Simulation on Directional-LBT

To have a closer look at the directional-LBT, we use the system level simulation to evaluate the difference between omni-transmission/LBT and directional-transmission/LBT. Analog beamforming is used to generate the directional pattern for LBT/transmission/reception. According to the agreements in RAN1 #92bis meeting, the topology shown in Figure 3 is agreed as the final topology for evaluations on NR-U indoor scenario. Here we set (a, b, c, d) as (15, 20, 20, 40) in this evaluation.

There are 3 different schemes in this directional-LBT evaluation. The first scheme is tagged as “w/ Beams,” indicating that all transmissions/receptions/LBT are done with the selected analog beam at all devices. The second scheme is tagged as “w/o Beams,” representing the case that all transmissions/receptions/LBT are done with omni-directional pattern. The third scheme is tagged as “Omni-sensing”. In the third scheme, the LBT is done with omni-directional pattern, and all transmissions/receptions are executed with the selected analog beam. Since analog beamforming is used to generate the directional pattern, each TXRU is connected to 4 antenna elements for both BS side and UE side. The training of the analog beam is done at the start of the simulation, and the selected beam pair is then applied for the whole simulation because the UEs are assumed to be with low mobility. The detailed evaluation assumptions can be found in Table 1 in the appendix.

Figure 3: Indoor office layout proposed in e-mail discussion [92b-NR-05]

In this evaluation, we collect the UL/DL UPT and the statistics of the “channel busy” event in the CCA state. When both the BS and the UE have packets in the buffer to transmit, the device enters CCA state and starts to sense the channel. If the device finds the channel occupied in a TTI, the counter of channel busy event is increased by 1. Figure 4 shows the collected evaluation results on both UL/DL UPT and channel busy event. All values shown in the figure is normalized by the value of the scheme “w/ Beams.”

For channel busy event, the number of detected channel busy events in “Omni-sensing” is the highest among the 3 simulated schemes. It is expected because all transmissions in this scheme are beamformed transmissions and omni-directional LBT, which can detect energy from larger angular coverage, is used in this scheme. However, it is surprising that the number of detected channel busy events in “w/ Beams” is higher than the one in “w/o Beams.” Directional transmissions/LBT captures more channel busy events in this sparsely deployed topology. It seems the beamforming gain pushes some of the interference over the energy detection threshold while the angular coverage of each beam is not small enough to filter most of the interference. For UL/DL UPT, though “w/ Beams” scheme detects more channel busy events, its UPT still slightly outperforms the UPT of the scheme “w/o Beams” due to higher SINR for the serving links, which is expected. The scheme “Omni-sensing” has the lowest UPT among the 3 schemes due to its conservative LBT behaviors. This is aligned with the observations from results of channel busy event.

Figure 4: Evaluation results on UL/DL UPT and channel busy event detected in CCA state. The ratios are defined as (Number of events detected/Value in each scheme) / (Number of events detected/Value in “w/ Beams”)

In conclusion, from the evaluation, it is observed that direction transmissions/receptions/LBT has the potential to improve system performance while careful designs are considered. Without comprehensive studies and careful designs, directional transmission/reception/LBT may harm the system performance instead. Further study on directional transmission/reception/LBT is needed for the NR-U system.

# Conclusion

In summary, we have the following observation and proposals:

Observation 1: The areas in LAA/eLAA that have not been fully addressed or can be further improved include at least

- The lack of support for intra-RAT tight frequency reuse

- The lack of support for LBT in directional transmission/reception

- The lack of support for interference mitigation based on message exchange among devices

Proposal 1: NR-based Access to Unlicensed Spectrum should resolve issues that are not fully addressed in LAA/eLAA as well as issues that arise due to newly introduced NR features.

Proposal 2: Both CA and BWP operations should be adopted to NR-U for wideband operation.

Proposal 3: NR-U should study at least the following aspects for BWP operation

- LBT mechanism

- BWP configuration for NR-U

Proposal 4: NR-U should study directional-LBT.

# References

- R1-1717911, “High level consideration on NR unlicensed band operation,” Huawei, HiSilicon

# Appendix

Table 1: Evaluation assumptions for the system level simulation of beamformed transmission/LBT

| Parameters | Values |
| --- | --- |
| Carrier frequency | 5 GHz |
| Bandwidth | 20 MHz |
| Channel model | Indoor -Office in TR38.901 |
| BS antenna configuration | (M, N, P) = (1, 4, 1) (dV,dH) = (0.5, 0.5) λ Θ = 180 (Attached to the ceiling, face down to the ground) 1 TXRU at BS side |
| UE antenna configuration | (M, N, P) = (1, 4, 1) (dV,dH) = (0.5, 0.5) λ 1 TXRU at UE side |
| BS antenna pattern | Omni-directional |
| UE antenna pattern | Omni-directional |
| Number of BS | Operator 1: 3 BSs, Operator 2: 3 BSs |
| Number of UE | 5 UEs for each BS |
| UE velocity | 3 km/hr |
| Traffic model | FTP model 3, traffic arrival rate ratio UL:DL = 1:4 |