3GPP TSG RAN WG1 Meeting #93							R1-1807148

Busan, Korea, May 21st – 25th, 2018

Source:	NTT DOCOMO, INC.

Title:	Simultaneous reception of DL/UL physical channels and reference signals

Agenda Item:	7.1.2.2.6

Document for: 	Discussion and Decision

- Introduction

In this contribution, simultaneous reception of DL/UL physical channels and reference signals is discussed.

- Simultaneous reception of SSB-RLM+PDSCH and SSB-BM+PDSCH

At the RAN4#86bis meeting, RAN4 discussed on the remaining details regarding scheduling availability during SSB-based RRM measurement and corresponding CR was agreed [1], [2].

At the RAN1#92bis meeting, RAN1 discussed on scheduling availability during SSB based RLM and SSB based beam management, and RAN1 made following conclusion [3].

For further discussion in RAN1#93 - RAN4 agreement exists for SSB-RRM + PDSCH multiplexing for non-CA and CA cases - Consider at least the following alternatives. At least for cases outside SMTC window (SSB-RLM+PDSCH, SSB-BM+PDSCH) including CA: - Alt 1: (multiplexing restriction) Extend the RAN4 agreement to the case of SSB+PDSCH multiplexing for both CA and non-CA without PDSCH associated with SI-RNTI, P-RNTI. This means that the UE is not expected to receive PDSCH on SSB symbols (for same numerology in FR2) - Alt 2: gNB will ensure at least spatial QCL - Alt 3: The UE is not expected to perform Rx beam refinement on SSB locations (Rx beam refinement may be associated with BM, RLM, BFD) Note: The above applies to the case with same numerology for SSB and PDSCH. The case with different numerologies for SSB and PDSCH is to be addressed.

At the RAN4#86bis meeting, RAN4 discussed the above RAN1 discussion, and RAN4 recognized this discussion should be concluded by RAN4 [1].

TS38.133 draft CR R4-1805963 Draft CR for scheduling availability during intra-freqeuency measurement 38.133 v15.1.0 Source: NTT DOCOMO Abstract: Discussion: Qualcomm: we will have the similar restriction of RLM. We did not reach the requirements for RLM. NTT DOCOMO: RAN1 MIMO session discussed something related to RLM. We should wait for RAN1 conclusion. Qualcomm: They are waiting for RAN4 conclusion. Decision: Endorsed

Hence, we propose to stop RAN1 discussion and wait for RAN4 conclusion.

Proposal 1:

- Send LS to RAN4 to ask for providing guidance about the simultaneous reception of SSB-RLM+PDSCH and SSB-BM+PDSCH.

  - Note: RAN4 recognized this discussion should be concluded by RAN4.

Regarding to the detail for the above discussion, please refer to our companion contributions [4], [5].

- Conclusion

In this contribution, simultaneous reception of DL/UL physical channels and reference signals, and following proposal was made:

Proposal 1:

- Send LS to RAN4 to ask for providing guidance about the simultaneous reception of SSB-RLM+PDSCH and SSB-BM+PDSCH.

  - Note: RAN4 recognized this discussion should be concluded by RAN4.

Regarding to the detail for the above discussion, please refer to our companion contributions [4], [5].

References

- 3GPP RAN4#86bis, Chairman’s note, April. 2018.

- 3GPP, R4-1805963, NTT DOCOMO, INC., “[draft] CR on scheduling availability during intra-frequency measurement,” Feb. 2018.

- 3GPP RAN1#92bis, Chairman’s note, April. 2018.

- 3GPP, R4-1806391, NTT DOCOMO, INC., “Scheduling availability during L1 measurement,” May. 2018.

- 3GPP, R4-1806394, NTT DOCOMO, INC., “Scheduling availability during RLM,” May. 2018.