3GPP TSG RAN WG1 Meeting #93	R1-1807192

Busan, Korea, April 21st – 25th 2018

Agenda item:		7.1.2.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PUSCH Beam Indication

Document for:		Discussion and Decision

# 1	Introduction

In RAN Plenary meeting #75, a WID on NR was agreed. The work item targets to develop and specify the functionalities for eMBB operation as well as support the URLLC type of operation. [1]

In the contribution we discuss about PUSCH beam indication.

# 2	Discussion

Before RAN1#92bis beam indication of PUSCH was not specified

- When the UE is scheduled PUSCH with DCI format 0_0

- When the UE is scheduled PUSCH with DCI format 0_1 without SRI and prior triggered SRS transmission

In RAN1#92bis a solution for 1) was agreed as follows [2]:

Agreement: - For PUSCH scheduled by DCI format 0_0, the UE shall use a default spatial relation corresponding to the spatial relation, if applicable, used by the PUCCH resource with the lowest ID configured in the active UL BWP - Above applies for a cell configured with PUCCH - Note: the UE is configured with a list of spatial relations in PUCCH-SpatialRelationInfo. MAC-CE indicates a single selected spatial relation from the list on a per-PUCCH resource basis if the list has more than one element.

In RAN1#92bis different alternatives were discussed for PUSCH scheduled by DCI format with the intention to support dynamic spatial relation indication for PUCCH [3]:

Offline Agreement: For PUSCH scheduled by DCI format 0_1, down-select to one of the following approaches for enabling dynamic spatial relation indication for PUSCH - Alt-1: For PUSCH scheduled by DCI format 0_1 and under the condition [A or B], the UE may assume a default spatial relation corresponding to the configured/activated spatial relation for a fixed PUCCH resource. The fixed PUCCH resource has ID [0 or 127]. Note: the spatial relation(s) for the fixed PUCCH resource are configured in PUCCH-SpatialRelationInfo. If the list has more than one entry, MAC-CE indicates a single selected spatial relation from the list. - Condition A: One SRS resource configured (no SRI in DCI) and SRS-SpatialRelationInfo NOT configured for that SRS resource - Supporting: SS, Sony, Ericsson, Nokia, NSB - Object: LG, QC, ZTE, Sanechips - Condition B: No priori SRS transmission triggered - Supporting: Nokia, NSB, Ericsson, DOCOMO, vivo, Sony - Object: LG, ZTE, Sanechips - Alt-2: Introduce MAC-CE spatial relation indication for an ap-SRS set - Supporting: LG, SS - Object: QC - Alt-3: No change to current specs, but SP-SRS is made a MANDATORY UE capability - Supporting: SS, ZTE, Sanechips, Ericsson - Object: Intel - Alt-4: No change to current specs (No support for dynamic spatial relation indication; only RRC indication supported) - Supporting: LG, OPPO, Qualcomm, Lenovo, Motorola, Huawei, HiSilicon, Fujitsu, Intel, DOCOMO, MediaTek - Object: Ericsson, SS, Nokia, NSB Update on Alt-3 after further offline discussion (from WF R1-1805640): - Support SP-SRS as mandatory with UE capability signalling - Minimum possible number on UE capability signalling of “Maximum number of semi-persistent SRS resources (configured to UE) per BWP” is 1 - Minimum possible number on UE capability signalling of “Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot” is 1. - Supporting: Samsung, ZTE, Sanechips, Ericsson, KDDI, LGE, Verizon Wireless, InterDigital, vivo, KT, NTT DOCOMO, SK Telecom, CHTTL - Object: Intel

It was then agreed that SP-SRS is mandatory for the UE. However, still case 2) still remains unspecified. PUSCH scheduled with DCI format 0_1 without need for SRS triggering is considered very attractive for the system operation by providing means for codebook based transmission where uplink channel state information can be determined e.g. from PUCCH and/or PUSCH DRMS.

Observation 1: PUSCH beam indication is not specified for the case when PUSCH is scheduled with 0_1 without SRI and SRS transmission triggering.

Observation 2: PUSCH scheduled with DCI format 0_1 without need for SRS resource triggering is attractive transmission scheme for the network because uplink CSI can be determined from PUCCH and/or PUSCH DMRS without need for configuring and activating SRS causing overhead.

The following options can be considered for the PUSCH beam indication when scheduled with 0_1 without SRI in DCI and without triggering configured SRS resource:

- PUSCH TX beam would be the same as used by the PUCCH resource with the lowest ID configured in the active UL BWP

- Pros

  - This would allow gNB to decouple DL and UL beam pair links if needed

  - MAC level dynamic beam switching is supported

- Cons

  - No L1 triggered dynamic beam indication

- UE determines PUSCH TX beam from the scheduling PDCCH beam

- Pros

  - This would be relatively simple

- Cons

  - In some cases MPE limitation may require different TX direction than RX direction at UE and thus it’s considered that following PDCCH may not be available in all cases

- Use the beam used for Msg3 in RACH. Problem that it’s get out-dated over time

- Cons

  - Problem is that Msg3 TX beam gets out-dated over time

To provide flexibility to decouple downlink and uplink beam pair links and to support dynamic beam switching our preference is to adopt the same mechanism as for the case when PUSCH is scheduled with DCI format 0_0, i.e. the UE shall uses a spatial relation corresponding to the spatial relation used by the PUCCH resource with the lowest ID configured in the active UL BWP.

Proposal 1: For PUSCH scheduled by DCI format 0_1 without SRI and with no triggering of SRS resource, the UE shall use a spatial relation corresponding to the spatial relation used by the PUCCH resource with the lowest ID configured in the active UL BWP.

# 3	Conclusions

This contribution discussed about PUSCH beam indication. Based on discussion the following proposals and observations were made:

Observation 1: PUSCH beam indication is not specified for the case when PUSCH is scheduled with 0_1 without SRI and SRS transmission triggering.

Observation 2: PUSCH scheduled with DCI format 0_1 without need for SRS resource triggering is attractive transmission scheme for the network because uplink CSI can be determined from PUCCH and/or PUSCH DMRS without need for configuring and activating SRS causing overhead.

Proposal 1: For PUSCH scheduled by DCI format 0_1 without SRI and with no triggering of SRS resource, the UE shall use a spatial relation corresponding to the spatial relation used by the PUCCH resource with the lowest ID configured in the active UL BWP.

# References

- RP-170847, “New WID on New Radio Access Technology”, NTT DoCoMo

- RAN1#92bis Chairman Notes

- R1-1805574, “Feature lead summary 2 on beam measurement and reporting”, Ericsson.