3GPP TSG RAN WG1 Meeting #93	R1-1807214

Busan, Republic of Korea, 21st - 25th May, 2018

Agenda item:		6.2.1.2.4

Source:		Nokia, Nokia Shanghai Bell, Ericsson

Title:	Enabling transmit diversity fallback for DL TM 8 for shorter TTI (36.212, 36.213) - TP#3

Document for:		Discussion and Decision

# 1	Introduction

TX diversity fallback for legacy LTE is done by using DCI Format 1A, which is not possible for shorter TTI.

In the current version of the 36.212, dynamic fallback to TX diversity is enabled for TM3 and TM4, by having state of the precoding information in the DCI to point to 2/4 layer transmission diversity. In this contribution we discuss on how to enable this also for TM8 without any additional DM-RS overhead (so support the same fallback flexibility than given for CRS-based TDD operation).

# 2	Enabling TX diversity fallback for TM8

For TM8, there is currently one unused state in the antenna port and layer indication that can be easily used for TX diversity fallback indication. So dynamic TX diversity fallback can also for TM8 be done without additional DCI overhead and therefore should be supported.

Proposed agreement: Support dynamic TX diversity fallback for TM8 / DCI Format 7-1E.

The changes shown in the TP in the next subsection take into account the TX diversity fallback through DCI Format 1A, but restricted to using transmit diversity only (port 0 only transmission not supported for simplicity, as this can be anyhow done through single layer port 7 or 8 already).

The related envisioned needed changes can be summarized as follows:

| Reason for change: |  | Support dynamic TX diversity fallback for sTTI DL TM8 by using the one reserved state in the precoding indication |
| --- | --- | --- |
| Summary of change: |  | Enable dynamic TX diversity fallback for sTTI DL TM8 |
| Consequences if not approved: |  | Dynamic TX diversity fallback for sTTI DL TM8 is not supported |
| Clauses affected: |  | 36.212 Sections 5.3.3.1.21 & 36.213 Section 7.1 |

# 3	TX diversity fallback for TM8 - TP /pseudo CR in 36.212 Sec.  5.3.3.1.21

< Unchanged parts are omitted >

##### 5.3.3.1.21	Format 7-1E

DCI format 7-1E is used for the scheduling of one PDSCH codeword with slot or subslot duration in one cell.

The following information is transmitted by means of the DCI format 7-1E:

-	The fields of format 7-1A

-	Precoding information – 2 bits as specified in Table 5.3.3.1.21-1.

Table 5.3.3.1.21-1: Content of precoding information field

| Bit field mapped to index | Message |
| --- | --- |
| 0 | 2 layers: Transmit diversity |
| 1 | 1 layer: antenna port 7 |
| 2 | 1 layer: antenna port 8 |
| 3 | 2 layers: antenna ports 7, 8 |

If the UE is configured to decode SPDCCH CRC scrambled by the C-RNTI and the number of information bits in format 7-1E mapped onto a given search space is less than that of format 7-0A/B depending on the configured uplink transmission mode for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 7-1E until the payload size equals that of format 7-0A/B, except when format 7-1E assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

< Unchanged parts are omitted >

# 4	TX diversity fallback for TM8 - TP /pseudo CR in 36.213 Sec. 7.1

## 7.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

Table 7.1-5: PDCCH and PDSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |  |
| --- | --- | --- | --- | --- |
| … | … | … | … |  |
| Mode 8 | DCI format 1A | Common and UE specific by C-RNTI | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see Subclause 7.1.1), otherwise Transmit diversity (see Subclause 7.1.2) |  |
|  | DCI format 2B | UE specific by C-RNTI | Dual layer transmission, port 7 and 8 (see Subclause 7.1.5A) or single-antenna port, port 7 or 8 (see Subclause 7.1.1) |  |
|  | DCI format 7-1E | UE specific by C-RNTI | Dual layer transmission, port 7 and 8 (see Subclause 7.1.5A) or single-antenna port, port 7 or 8 (see Subclause 7.1.1) or Transmit Diversity (see Subclause 7.1.2) |  |
| … | … | … | … |

< Unchanged parts are omitted >

Table 7.1-5C: SPDCCH and PDSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PDSCH corresponding to SPDCCH |
| --- | --- | --- | --- |
| … | … | … | … |
| Mode 8 | DCI format 7-1E | UE specific by C-RNTI | Dual layer transmission, port 7 and 8 (see Subclause 7.1.5A) or single-antenna port, port 7 or 8 (see Subclause 7.1.1) or Transmit Diversity (see Subclause 7.1.2) |
| … | … | … | … |

< Unchanged parts are omitted >