3GPP TSG-RAN WG1 Meeting #93	Tdoc R1-1807261

Busan, South Korea, May 21-25 2018

Agenda Item:	7.1.3.3.7

Source:	Ericsson

Title:	TP on Remaining issues for UL/DL Resource Allocation

Document for:	Discussion/Decision

# 1	Introduction

In this contribution, we provide text proposals for our company contribution [1].

# 2	Discussion

## 2.1	TP for xOverhead for broadcast TBS

- <begin TP>

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1for the corresponding transport block, otherwise the transport block is enabled.

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if the higher layer parameter mcs-Table is set to '256QAM' and 0 ≤ IMCS ≤27, or the higher layer parameter mcs-Table is not set to '256QAM' and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead is not configured (a value from 0, 6, 12, or 18), the xOverhead is set to 0. xOverhead is set to 0 for PDSCH scheduled by PDCCH with DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

- <end TP>

## 2.2	TP for TBS determination for Msg3 PUSCH

TBS determination for Msg3 PUSCH is missing from current specs. Proposed to add it as follows.

- TP for 38.214

- <begin TP>

#### 6.1.4.1	Modulation order and target code rate determination

For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI

Unchanged text omitted.

#### 6.1.4.2	Transport block size determination

For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

- <end TP>

# References

- R1-1807255	“On remaining issues for DL UL resource allocation”, Ericsson, TSG-RAN WG1 #93