3GPP TSG-RAN WG1 Meeting #93 	R1-1807375

May 21st  –25th 2018

Busan, Korea

Agenda item:	7.1.7

Source: 	Qualcomm Incorporated

Title: 	Signaling aspects for UE capabilities

Document for:	Discussion/Decision

# Introduction

Currently in the NR specifications, there are several references to how the system should operate based on “if the UE has reported its corresponding capability”. Although this may seem clear at a high level, whether UE has reported its capability or not does not seem to be the right terminology since UE capability enquiry may only happen at the time of NAS attach and is not tracked by UE or gNB.

It is proposed to replace instances in the specifications where the text uses “if the UE has reported its corresponding capability” (or related phrases), and use term “UE is capable of”. Moreover, default operation tied to cases where the capability is not known can be handled by defining a default for any related higher layer parameters which have not been configured. This would allow more well-defined operation.

In some cases it is beneficial to introduce a higher layer parameter for the purpose only of resolving ambiguity, but in other cases this already exists and only wording changes are necessary.

The included text proposals are based on the CRs submitted in [1-5].

# Scheduling and HARQ related text proposals

## PDCCH reception related capabilities

Proposal 1: The following text in 38.213 subclause 10 should be changed accordingly.

Text proposal for 38.213.

If a carrier aggregation capability for a UE is larger than 4, the UE includes a separate capability for a maximum number of PDCCH candidates the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When the UE is configured for carrier aggregation operation over more than 4 cells, the UE is not expected to be configured with a number of PDCCH candidates to monitor per slot that is larger than the maximum number.

## HARQ-ACK related capabilities

Proposal 2: The following text in 38.213 subclause 9.1.2.1 should be changed accordingly.

Text proposal for 38.213.

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

…

If the UE is not  capable of receiving more than one unicast PDSCH per slot and,

# EN-DC related capabilities

Proposal 4: The following text in 38.213 subclause 7.6 should be changed accordingly.

Text proposal for 38.213.

### 7.6.1	EN-DC

…

  - If the UE does is not capable of dynamic power sharing between EUTRA and NR, the UE is not expected to transmit in a slot on the SCG when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

…

- If the UE is not capable of dynamic power sharing between EUTRA and NR and

…

- If the UEis not capable of dynamic power sharing between EUTRA and NR, the UE is expected to be configured with reference TDD configuration for EUTRA (by higher layer parameter SUO-case1 in [13, TS 36.213]).

…

# MIMO related text proposals

## Uplink transmission related capabilities

Proposal 5: The following text in 38.214 subclause 6.1.1.1 should be changed accordingly.

Text proposal for 38.214

A UE capable of 'partialAndNonCoherent ' transmission shall not expect to be configured by codebookSubset with ‘fullAndPartialAndNonCoherent’.

A UE capable of 'Non-Coherent' transmission shall not expect to be configured by codebookSubset with ‘fullAndPartialAndNonCoherent’ or with ‘partialAndNonCoherent '.

## MCS and modulation-related capabilities

Proposal 6: The following text in 38.214 subclause 6.1.4.1 should be changed accordingly. Note this includes the introduction of higher layer parameter for q.

Text proposal for 38.214

#### 6.1.4.1	Modulation order and target code rate determination

…

-	for MCS index 0 and 1, q is configurable by higher layer parameter PUSCH-tp-Qmin depending on whether UE is capable of pi/2 BPSK modulation; if this has not been configured then q is assumed to be 2

## PTRS and related capabilities

Proposal 7: The following text in 38.214 subclause 6.2.3 should be changed accordingly.

Text proposal for 38.214

### 6.2.3	UE PT-RS transmission procedure

…

If a UE is capable of supporting [full-coherent UL transmission], the UE may expect the number of UL PT-RS ports to be configured as one.,

## SRS related capabilities

Proposal 8: The following text in 38.214 subclause 5.1.1.1 should be changed accordingly.

Text proposal for 38.214. Note some “…” have been used for brevity to focus on common text between paragraphs.

#### UE sounding procedure between component carriers

For a carrier of a serving cell with slot formats comprised of DL and UL symbols,

…

beyond the uplink carrier aggregation capability of the UE [SRS_capability [13, TS 38.306].

# Summary of RRC Impact

The following RRC parameters are introduced along with the above text proposals. These parameters are intended for inclusion into TS 38.331.

| Sub-feature Group | RAN1 Spec(s) | RAN2 Parent IE | Parameter Name | Description | Values (default) | UE/Cell Specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 38.214 | PUSCH-Config | PUSCH-tp-Qmin | Minimum modulation order configured for the UE (depending on if the UE is capable) | {1,2} (2) | UE specific | Introduced to clarify default capability, need not be BWP-specific. |

# References

- R1-1805793, “CRs to 38.211”, RAN1, RAN1 #92bis Sanya, China

- R1-1805794, “CRs to 38.212”, RAN1, RAN1 #92bis Sanya, China

- R1-1805795, “CRs to 38.213”, RAN1, RAN1 #92bis Sanya, China

- R1-1805796, “CRs to 38.214”, RAN1, RAN1 #92bis Sanya, China

- R1-1805797, “CRs to 38.215”, RAN1, RAN1 #92bis Sanya, China