3GPP TSG RAN WG1 Meeting #93 	R1-1807401

# Busan, Korea, May 21-25, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of V2X Phase 2 based on LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  |  | Date: |  | 2018-05-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inclusion of V2X Phase 2 based on LTE in 36.212. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support V2X Phase 2 based on LTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X Phase 2 based on LTE will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 5.4.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 36.211, 36.213 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

### 5.4.2	Sidelink shared channel

The processing of the sidelink shared channel follows the downlink shared channel according to section 5.3.2, with the following differences:

-	Data arrives to the coding unit in the form of a maximum of one transport block every transmission time interval (TTI)

-	In the step of code block concatenation, the sequence of coded bits corresponding to one transport block after code block concatenation is referred to as one codeword in section 9.3.1 of [2].

-	PUSCH interleaving is applied according to sections 5.2.2.7 and 5.2.2.8 without any control information in order to apply a time-first rather than frequency-first mapping, where . For SL-SCH configured by higher layers for V2X sidelink, is used if the Transmission Format field of SCI format 1 is present and set to 1, otherwise  .

### < Unchanged parts are omitted >

##### 5.4.3.1.2	SCI format 1

SCI format 1 is used for the scheduling of PSSCH.

The following information is transmitted by means of the SCI format 1:

-	Priority - 3 bits as defined in section 4.4.5.1 of [7].

-	Resource reservation – 4 bits as defined in section 14.2.1 of [3].

-	Frequency resource location of initial transmission and retransmission –  bits as defined in section 14.1.1.4C of [3].

-	Time gap between initial transmission and retransmission - 4 bits as defined in section 14.1.1.4C of [3].

-	Modulation and coding scheme – 5 bits as defined in section 14.2.1 of [3].

-	Retransmission index – 1 bit as defined in section 14.2.1 of [3].

-	Transmission format– 1 bit, where value 1 indicates rate-matching and value 0 indicates puncturing for handling of the last symbol of the subframe. This field is only present if the transport mechanism selected by higher layers indicates the support of 64-QAM.

-	Reserved information bits are added until the size of SCI format 1 is equal to 32 bits. The reserved bits are set to zero.

### < Unchanged parts are omitted >