3GPP TSG RAN WG1 Meeting #93	R1-1807490

Busan, Korea, May 21- 25, 2018

Agenda Item:	6.2.1.2.7

Source:	Huawei, HiSilicon

Title:	Corrections on HARQ-ACK reporting procedure for FS2 with sTTI

Document for:	Discussion and decision

Introduction

In this contribution, we will further discuss some remaining issues on FS2 for sTTI.

TDD HARQ-ACK reporting for FS2 sTTI

We have achieved following agreements considering the timing between UL grant and corresponding sPUSCH transmission:

Agreement: For slot sTTI in FS2 and for SSC 1,2,3,4,6,7,8, the following pre-defined table is used for the timing between UL grant and sPUSCH. In the following table describing k, sPUSCH is transmitted in slot n+k, where the corresponding UL grant is received in slot n. TDD UL/DL; slot number n. Configuration; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19. 0; 4; 4; 4, 5; 5,6; 4; 4; 4, 5; 5,6. 1; 4; 4; 4; 4; 4; 4; 4; 4. 2; 4; 4; 4; 4. 3; 6; 6; 6; 6; 6; 6. 4; 4; 4; 4; 4. 5; 4; 4. 6; 6; 6; 6; 6; 4; 4; 4; 4; 6; 6. For slot sTTI in FS2 and SSC 0,5,9,10, the following pre-defined table is used for the timing between UL grant and sPUSCH. In the following table describing k, sPUSCH is transmitted in slot n+k, where the corresponding UL grant is received in slot n. The red-colored font is for SSC 10. - TDD UL/DL; - slot number n. Configuration; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19. 0; 4,5; 5,6; 6,7,11; 4,5; 5,6; 6,7,11. 1; 5; 5; 5; 5; 5; 5; 5; 5; 5; 5. 2; 4; 4; 4; 4; 4; 4. 3; 7; 7; 7; 7; 7; 7; 7. 4; 5; 5; 5; 5; 5. 5; 4; 4; 4. 6; 4,5; 5,6; 6,7,11; 4; 4; 4,5; 4. For the timing between sPDSCH and HARQ-ACK, the following table is used for SSC 0-10. - Note: Different numbers in the table below can apply to different SSCs as in case of the same table in legacy. TDD UL/DL; slot number n. Conf.; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16; 17; 18; 19. 0; 4; 4; 4; 4; 4; 4. 1; 6,5; 5,4; 4; 6,5; 5,4; 4. 2; 8,7,12; 7,6,5,4; 8,7,12; 7,6,5,4. 3; 14,13,12; 12,11,10; 10,9; 9,8; 8,7; 7. 4; 16,15,14,13; 13,12,11,10; 10,9,8,7; 7,6,5. 5; 18,17,16,15,14,13,12, 22; 12,11,10,9,8,7,6,5,4. 6; 6; 6; 6; 6; 6; 4; 4; 4.

The timing of and for special subframe configuration 0, 5, 9 and 10 and TDD configuration 1/6 are discussed in this TP. Currently, the timing for UL grant to sPUSCH and the timing for sPDSCH to HARQ-ACK feedback result in conflicts when determining the timing for  and , considering following principles:

- The timing for and should be aligned with the timing for UL grant to this sPUSCH.

- The timing for and should be less than or equal to the minimum value in the set of timing from sPDSCH to HARQ-ACK for this sPUSCH.

- The value of  and of no more than one sPUSCH can be indicated by one DL grant.

To resolve  the conflicts, we propose to update the DAI timing as below:

Proposal 1:

For the timing between sPDSCH and HARQ-ACK, the following table is used for TDD configuration 0,2,3,4,5.

- Note: Different numbers in the table below can apply to different SSCs as in case of the same table in legacy.

| TDD UL/DL | slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conf. | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |
| 2 |  |  |  |  | 8,7,12, 11 | 7,6,5,4 |  |  |  |  |  |  |  |  | 8,7,12, 11 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14,13,12 | 12,11,10 | 10,9 | 9,8 | 8,7 | 7,6 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16,15,14,13 | 13,12,11,10 | 10,9,8,7 | 7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18,17,16,15,14,13,12, 22,21 | 12,11,10,9,8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: The number highlighted in red are used for SSC 3,4,8

For the timing between sPDSCH and HARQ-ACK, the following table is used for TDD configuration 1,6.

- Note: Different numbers in the table below can apply to different SSCs as in case of the same table in legacy.

| TDD UL/DL Conf. | Special subframe configuration | slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 1 | 1,2,3,4,6,7,8 |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |
| 1 | 0, 5, 9, 10 |  |  |  |  | 6, 5 | 5 | 5 | 5 |  |  |  |  |  |  | 6,5 | 5 | 5 | 5 |  |  |
| 6 | 1,2,3,4,6,7,8 |  |  |  |  | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |
| 6 | 0, 5, 9, 10 |  |  |  |  | 6, 5 |  | 6,5 |  | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

NOTE: The number highlighted in red are used for SSC 3,4,8

Proposal 2:

For FS2 configured with sTTI operation with special subframe configuration 0, 5, 9 and 10,  for slot n, the and are detected in SPDCCH in slot n-k, where k is defined in following table:

| - TDD UL/DL | - slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 1 |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  | 5 | 5 | 5 | 5 |  |  |
| 6 |  |  |  |  | 4 |  | 5 |  | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

Proposal 3: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

<Unchanged parts are omitted>

- 7.3.2	TDD HARQ-ACK reporting procedure

<Unchanged parts in this subclause are omitted>

- .3.2.1	TDD HARQ-ACK reporting procedure for same UL/DL configuration

Unless otherwise stated, the procedure in this subclause applies to non-BL/CE UEs.

For TDD, the UE shall upon detection of a PDSCH transmission or a PDCCH/EPDCCH/SPDCCH indicating downlink SPS release (defined in Subclause 9.2) within subframe/slot(s) , where , intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe/slot n.  is defined in

- Table 10.1.3.1-1C if the UE is configured with higher layer parameter shortTTI for slot-based PDSCH with special subframe configuration 1, 2, 6, 7,

- Table 10.1.3.1-1D if the UE is configured with higher layer parameter shortTTI for slot-based PDSCH with special subframe configuration 3, 4 or 8,

- Table 10.1.3.1-1E if the UE is configured with higher layer parameter shortTTI for slot-based PDSCH with special subframe configuration 0, 5, 9 or 10,

- Table 10.1.3.1-1B if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH is in the UE-specific search space for subframe-based PDSCH, and,

- Table 10.1.3.1-1 otherwise.

For TDD, when PUCCH format 3/4/5 is configured for transmission of HARQ-ACK, for special subframe configurations 0 and 5 with normal downlink CP or configurations 0 and 4 with extended downlink CP in a serving cell, shown in table 4.2-1 [3], the special subframe of the serving cell is excluded from the HARQ-ACK codebook size determination. In this case, if the serving cell is the primary cell, there is no PDCCH/EPDCCH indicating downlink SPS release in the special subframe.

<Unchanged parts are omitted>

Table 7.3-Y2: Uplink association index k' for TDD with special subframe configuration 0, 5, 9, 10 if configured with higher layer parameter shortTTI

| - TDD UL/DL | - slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 1 |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  | 5 | 5 | 5 | 5 |  |  |
| 2 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  | 4 | 4 |  |  |  |  |
| 3 |  |  |  |  | 7 | 7 | 7 | 7 | 7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 4 |  | 5 |  | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

<Unchanged parts are omitted>

- 10.1.3.1	TDD HARQ-ACK procedure for one configured serving cell

<Unchanged parts are omitted>

Table 10.1.3.1-1C: Downlink association set :  for TDD with special subframe configuration 1, 2, 6, 7 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |
| 1 |  |  |  |  | 6,5 | 5,4 | 4 |  |  |  |  |  |  |  | 6,5 | 5,4 | 4 |  |  |  |
| 2 |  |  |  |  | 8,7, 12 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

Table 10.1.3.1-1D: Downlink association set :  for TDD with special subframe configuration 3, 4 or 8 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |
| 1 |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |
| 2 |  |  |  |  | 8,7, 12,11 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12,11 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7,6 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22,21 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |

Table 10.1.3.1-1E: Downlink association set :  for TDD with special subframe configuration 0, 5, 9, 10 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |
| 1 |  |  |  |  | 6,5 | 5 | 5 | 5 |  |  |  |  |  |  | 6,5 | 5 | 5 | 5 |  |  |
| 2 |  |  |  |  | 8,7, 12 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6,5 |  | 6,5 |  | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------