3GPP TSG RAN WG1 Meeting #93	R1-1807526

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.5.1.1

Source: 	LG Electronics

Title:	Summary of further offline discussion on A.I. 6.2.5.1.1

Document for:	Discussion and Decision

Introduction

This contribution summarizes further offline discussion of remaining issues on A.I. 6.2.5.1.1 at RAN1 #93.

Summary of further offline discussion on A.I. 6.2.5.1.1

Possible Agreement When random selection is configured by upper layers, for cases when limited tx capability the UE cannot support transmission(s) over carrier(s): - For (a), (b), (c), the UE shall follow Option 1-1 - Otherwise, the UE shall follow Option 1-2

Agreement in RAN1 #92bis:

For cases when limited tx capability the UE cannot support transmission(s) over carrier(s):

- The UE shall follow Option 1-1 for (a), (b), (c)

- Otherwise, the UE shall follow Option 1-2

Option 1-1: When the UE performs the resource selection for a certain carrier, any subframe of that carrier shall be excluded from the reported candidate resource set if using that subframe exceeds its TX capability limitation under the given resource reservation in the other carriers.

Feature leader’s observation: Option 1-1 seems to apply irrespective of the resource selection method, but since the “reported” wording in option 1-1 is only used in the sensing based resource selection, some companies think that option 1-1 should be applied only when sensing is configured by upper layer.

Proposal:

- When random selection is configured by upper layers, total candidate single-subframe resources within a resource pool are considered as reported from physical layer.

- When random selection is configured by upper layers, for (a), (b), (c), option 1-1 is applied, otherwise, option 1-2 is applied.

Open issue: Whether the additional resource exclusion procedure is specified in MAC layer spec or physical layer spec. Proposals Alt. 1) MAC layer (i.e. specified in TS36.321) Supported by CATT, Nokia, NSB, OPPO, ZTE, Panasonic Alt. 2) Physical layer (i.e. specified in TS36.213) Supported by Ericsson

Feature leader recommends that this issue is discussed in online.

For further offline discussion Open issue: When a UE with limited TX capability performs resource selection for a certain carrier, there could be ambiguity about the duration for which the current reserved resources of the other carriers are valid. Alt. 1) a UE with limited TX capability performs resource selection on a given carrier without taking into account possible resource reselection on other carriers: proposed by LGE Alt. 2) Determining the duration for which the current resource reservation on other carriers is valid can be left to UE implementation: Proposed by Nokia, NSB Alt. 3) The duration for the current reservation for other carriers is valid CATT

Proposal: When a (re)selection is triggered in a carrier (e.g., CC#X), a UE assumes that the current reserved resources of the other carriers (e.g., CC#Y) are valid without taking into account possible resource reselection on the other carriers.

For further offline discussion Open issue: Whether PPPP/CBR is considered in carrier resource selection order or not. Positions: Alt. 2) considering PPPP/CBR in carrier resource selection is up to UE implementation Supported by Huawei, HiSilicon, LGE, ZTE, Qualcomm, OPPO Alt. 3) considering at least PPPP in carrier resource selection is specified. - FFS within this week: CBR Supported by Intel, Docomo, Samsung, Ericsson, Nokia, NSB, CATT, Panasonic

Feature leader recommends that this issue is discussed in online.