3GPP TSG RAN WG1 Meeting #93    	            R1-1807637

Busan, Korea, May 21st – 25th, 2018

Source:	NTT DOCOMO, INC.

Title:	Offline summary for AI 7.1.3.1.1 PDCCH structure for URLLC

Agenda Item:	7.1.3.1.1

Document for: 	Discussion and Decision

- Introduction

This is the offline summary for AI 7.1.3.1.1 PDCCH structure for URLLC

- Discussion

  - PDCCH repetition

(a), TDL-C,300ns, AL=8 (b), TDL-A,30ns, AL=8 Figure 3: PDCCH performance with AL=8 Proposal 1: PDCCH repetition among multiple monitoring occasions should be supported and configurable.

Offline proposal:

- To be discussed.

  - PDCCH and SSB overlap

Proposal 1: Rate matching of PDCCH, when SSB and PDCCH collide, is a capability of UE. Proposal 2: Support REG-level rate matching of PDCCH in URLLC, when SSB and PDCCH collide. Proposal 3: REG-reindexing is not supported for rate matching of PDCCH in Rel-15 URLLC. Proposal 4: Adopt TP in Appendix for 38.213.

Offline proposal:

- To be discussed.

- Summary of proposals over AI 7.1.3.1.1/7.1.3.1.2

PDCCH structure

- PDCCH and SSB overlap ( wait for the conclusion on SSB identification?) - R1-1806915 Panasonic - For connected in FR2 and after RMSI reception in FR1, PDCCH is not monitored - For FR2, the decision depends on the outcome of initial-access session - R1-1806615 LGE - For RS of WB RS CORESET vs SSB, RS in overlapping REG is not assumed to be present - R1-1807352 QCM - For Type0/0A/1/2-CSS, no SSB is assumed - For Type3-CSS and USS, overlapped PDCCH is not monitored Corrections: - R1-1805880 Huawei - Correction to 7.3.2.2 and 7.4.1.3.2 (use 331 parameter names in the spec) - R1-1806127 ZTE - Remove “common” to clarify the reference point k in 7.4.1.3.2 - Remove “DCI format 1_0” in 213 10.1 to clarify that TCI field is not in the DCI format 1_0 - R1-1806141 Nokia - PDCCH DMRS sequence generation is changed to the relative to PDCCH starting symbol - Align 6PRB grid for dedicated CORESET configuration with PRB0 of CORESET#0 for serving cells with initial active BWP, otherwise align 6PRB grid for dedicated CORESET configuration with Point A - R1-1807352 QCM - Correction to interleaving such that first within a CCE then across CCEs - Correction to max no. segments for WB RS CORESET to 3 - R1-1806288 CATT ( R2-1807566 addresses ASN.1 directly) - Correction to several inconsistency (shift index, scramble ID) b/w 331 and 211

PDCCH structure for URLLC

- PDCCH repetition - R1-1806362 CMCC - SSB/PDCCH collision - R1-1806408 Spreadtrum - REG-level rate-matching based on capability for URLLC

Search space

- PDCCH mapping rules across multiple monitoring occasions within a slot - R1-1805881 Huawei - For a given SS set, each monitoring occasion has equal number, and mapping rules across SS sets is per monitoring occasion - R1-1806056, vivo - First map within a monitoring occasion, then map across occasions with ascending order of symbols - R1-1806128, ZTE - Time-domain mapping occasions are interleaved - R1-1806289 CATT - Comb-manner across monitoring occasions - R1-1806142 Nokia, R1-1806616 LGE, R1-1806779 MTK, R1-1807016 InterDigital, R1-1807058 DCM - Same as Case 1-1/1-2 (monitoring occasions are not distinguished) - R1-1807247 Ericsson, R1-1806729 Samsung - SS-set ID only, but if whole SS-set cannot be mapped, map candidates at earlier monitoring occasion - Number of BDs/CCEs for CA - R1-1805881 Huawei - For up to 4 DL-CCs with self-scheduling, - The numbers per CC per slot are the same as those for non-CA case - For up to 4 DL-CCs with cross-carrier scheduling, - If scheduling CC SCS <= scheduled CC SCS, the numbers are determined by: - (No. of BDs/CCEs per slot with scheduling CC SCS) x (No. of CCs where the scheduling CC can schedule) - If scheduling CC SCS >= scheduled CC SCS, the numbers are determined by: - (No. of BDs/CCEs per slot with scheduling CC SCS) x 1 + (No. of BDs/CCEs per slot with scheduled CC SCS) / (scheduling CC SCS/scheduled CC SCS) x (No. of CCs where the scheduling CC can schedule - 1) - For more than 4 DL-CCs, - The value y is reported based on the reference numerology, e.g., 15kHz. - A UE reporting the value y shall support y x (No. of BDs/CCEs per slot with 15kHz). - R1-1806142, Nokia - For CA, max no. of BDs/CCEs per slot for a given numerology is determined by: - (no. of CCs with the given numerology) / (no. of CCs across all numerologies) x (y or X) x (max. no. of BDs/CCEs with the given numerology) - This can be shared across serving cells with the same numerology - This is the max no. of BDs/CCEs on a single serving cell for a UE capable of cross-carrier scheduling - This for higher SCS CC is the limits for cross-carrier scheduling - R1-1806779 MTK - For CA with mixed numerologies, max no. of BDs across CCs per 1ms is 44X or 44y, and max no. of BDs per CC per 1ms is 44 x (X or y) / X - R1-1807058 DCM - Across numerology, minimum of value y is reported - R1-1807353 QCM - For a UE capable of BD/CCE for CA with the configured no. of CCs; - No. of BDs/CCEs does not exceed the single-carrier limits - For a UE less capable of BD/CCE for CA with the configured no. of CCs; - No. of BDs/CCEs does not exceed the per CC limits - Per CC limit is (single-carrier limit) x y / (no. of configured CCs) - PDCCH mapping rules across serving cells for CA - R1-1806128 ZTE, R1-1806879 Panasonic, R1-1807058 DCM, R1-1806729 Samsung - PDCCH mapping is from lower-to-higher CC index and from lower-to-higher SS set index - R1-1806142 Nokia, (can also be acceptable to Samsung) - Total number of BDs/CCEs across CCs with the same numerology is split into CCs with the same ratio as configured candidates in the CC - R1-1807353 QCM (can also be acceptable to Samsung but least preferable) - PDCCH overbooking is not supported for SCell - CORESETs overlap - R1-1806615 LGE - Priority is defined across CORESETs with different TCI states on overlapping case - Simple rule is to prioritize CORESET with CSS or BFR-SS - R1-1807353 QCM - UE-specific CORESET is reduced when it collides with the CORESET where Type0-CSS is associated to - UE-specific CORESET is reduced when it collides with the CORESET where Type0A/1/2-CSS is associated to and if Type0A/1/2-CSS monitoring periodicity is larger than a threshold (e.g., 4 slots) - R1-1806728 Samsung - Any CORESET confined within the initial active DL BWP is considered as CORESET#0 - Dedicated CORESET can override CORESET#0 if the no. of configured BWPs is 4 - Search space design - R1-1806879 Panasonic, R1-1807353 QCM - Nested structure is supported - R1-1807353 QCM - Replace the higher-layer parameter periodicity/offset by “periodicity + offset bit-map” - Clarification on CORESET#0 and/or CSS configuration - R1-1806550 Intel - Confirm the WAs at RAN1#91 meeting - Delete “if configured” - PRACH is configured for each BWP - R1-1806666 ETRI - Collision b/w Type0A-CSS and SSB is handled by the same way as collision b/w Type0-CSS and SSB - R1-1806289 CATT - Remove CORESET restriction in PDCCH-ConfigCommon - R1-1806142 Nokia - Recommend RAN2 to support a configuration of searchSpaceSIB1 by pdcch-ConfigSIB1 or SearchSpace by choice in PDCCH-ConfigCommon - R1-1806729 Samsung - Adding periodicities {5ms, 10ms, 20ms, 40ms, 80ms, 160ms} and slot-level monitoring occasions for the RRC IE SearchSpace - R1-1807353 QCM - Remove additional CORESET in the PDCCH-ConfigCommon - R1-1806055 vivo - Type0/0A/1/2-CSS is configured by PDCCH-ConfigCommon for each DL BWP - R1-1806127 ZTE - CORESET for Type0A/2-CSS is same as that for Type0-CSS only when these CSSs are in the same BWP - R1-1807160 ETRI - Support NW-based and UE-based switching of CORESET#0 for unicast PDSCH - PDCCH monitoring during BFR ( to be discussed in BFR session?) - R1-1805880 Huawei - When Type3-CSS is associated with CORESET #0, q0 should include RS QCLed to CORESET #0 - R1-1806616 LGE - UE continues to monitor other CORESETs than BFR-CORESET during BFR procedure - Prioritize BFR-SS set over other SS sets during BFR procedure - If the BFR CORESET overlaps with other CORESET(s) with different QCL in time-domain, BFR CORESET is monitored. - R1-1806550 Intel - TCI state is not configured for the CORESET used for BFR - R1-1806729 Samsung - Monitor CORESET-BFR only if the UE is on figured with failureDetectionResources - Otherwise the UE monitors all the configured CORESETs including CORESET-BFR - CORESET-BFR has the highest priority than other CORESETs - R1-1807257 Ericsson - Remove the sentence “the UE does not expect to be provided another search space set for monitoring PDCCH in the control resource set provided by recoveryControlResourceSetId” - Remove the sentence “Unless the UE transmitted PRACH in response to Beam-failure-recovery-request-RACH-Resource, the UE is not expected to monitor PDCCH candidates in the control resource set provided by recoveryControlResourceSetId after the UE receives higher layer parameter ControlResourceSet or after the UE receives a MAC CE activation for a TCI state” - TCI state configuration for a CORESET #0 ( to be discussed in BM session?) - R1-1806550 Intel - Up to gNB whether to configure by signaling but the available TCI state configurations are only TCI states associated with a valid SSB index - R1-1806055 vivo - UE cannot change SSB autonomously, and this is done by TCI state configuration for CORESET #0 - Default TCI state for CORESET(s) before MAC CE activation is SSB/CSI-RS for RACH - R1-1806361 CMCC - TCI state can be configured by RRC and can be activated MAC CE for CORESET #0 - R1-1806778 MTK - TCI state can be configured by RRC - R1-1807246 Ericsson - TCI state can be configured by RRC and can be activated MAC CE for CORESET #0 - However, for Type0/0A/2-CSS, PDCCH is assumed to be QCLed with the selected SSB - PDCCH monitoring for DCI format 2_1 ( to be discussed in mux. Session?) - R1-1807353 QCM - For the CSS which a DCI format 2_1 is configured to be monitored on, the UE will only monitor the first one or two PDCCH candidates of the configured aggregation level for DCI format 2_1. - If a configured DCI format 2_1 is not received, PDCCH monitoring is performed till the next configured DCI format 2_1 monitoring occasion. Corrections: - R1-1805881 Huawei - Association between A0, A1, A2, and the CORESETs for each DL BWP - Change “monitored PDCCH candidates” to “blind decodes” - R1-1806056, vivo - TC-RNTI is monitored on Type3-CSS and USS - R1-1807247 Ericsson - TC-RNTI is not monitored on Type3-CSS and USS - R1-1806142 Nokia - Delete a text in 38.213 section 10.1, “If, for a search space set s, a CCE aggregation level L, and a control resource set p, any CCE index for a second PDCCH candidate m_s,n_CI overlaps with any CCE index for a first PDCCH candidate with index m_s,n_CI, where the PDCCH candidates are indexed in ascending order of a respective first CCE index, the UE is not expected to monitor the second PDCCH candidate with index m_s,n_CI”. - R1-1806550 Intel - Even if a UE supports CA with large number of CCs, if the UE is configured with CA with m CCs, the number of BDs/CCEs is limited by the value m. - R1-1806729 Samsung - PDCCH monitoring causing partially overlap within the same CORESET is not allowed. - R1-1806779 MTK - For cross-carrier scheduling, the search space configuration of the scheduled CC is the same as that of scheduling CC.

Search space for URLLC

- Number of BDs/CCEs for URLLC - R1-1806019 Ericsson - Numbers of BDs/CCEs for June drop URLLC is not per slot, but per half-slot - No change on the values - R1-1806655 Nokia - Numbers of BDs/CCEs for June drop URLLC is not per slot, but per half-slot - Number of BDs is {36, 22} for {15kHz, 30kHz} - Number of CCEs is {56, 48} for {15kHz, 30kHz} - R1-1807059 DCM - Numbers of BDs/CCEs for June drop URLLC per slot is following - Number of CCEs is {112, 112} for {15kHz, 30kHz}

- Agreements in the past RAN1 meetings

RAN1#88bis

Agreements - UE can be configured to “monitor DL control channel” in terms of slot or OFDM symbol with respect to the numerology of the DL control channel - Specification supports occasion of “DL control channel monitoring” per 1 symbol with respect to the numerology of the DL control channel - Note: This may not be applied to all type of the UEs and/or use-cases - FFS whether or not total number of blind decodings in a slot when a UE is configured with “DL control channel monitoring” per symbol can exceed the total number of blind decodings in a slot when a UE is configured with “DL control channel monitoring” per slot - Data channel (PDSCH, PUSCH) duration and starting position - Specification supports data channel having minimum duration of 1 OFDM symbol of the data and starting at any OFDM symbol to below-6GHz, in addition to above-6GHz - Note: This may not be applied to all type of UEs and/or use-cases - UE is not expected to blindly detect the presence of DMRS or PT-RS - FFS: Whether a 1 symbol data puncturing can be indicated by preemption indication - FFS: combinations of data duration and granularities of data position - Specification supports data having frequency-selective assignment with any data duration - FFS: relations between “DL control channel monitoring” occasions and data channel durations - Note: this is addition to the agreements at RAN1#86. - Note : 1-symbol case may be restricted depending on the BW. Working assumption: - One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH - FFS the bundling size - FFS: REG bundling is also for localized mapping in time and/or frequency-domain - Companies are encouraged to provide evaluation results for 10 MHz and 20 MHz for larger aggregation levels and 5 MHz and 10 MHz for smaller aggregation levels Agreements: - MU-MIMO is supported NR-PDCCH using at least non-orthogonal DMRS. - FFS: orthogonal DMRS for UE-specific NR-PDCCH Agreement:: NR-PDCCH can be mapped contiguously or non-contiguously in frequency with localized or distributed mapping of REGs to a CCE (in the physical domain) - Note: The number of contiguous REGs in the CCE needs further discussion. - Note: Localized/distributed mapping can be achieved without/with interleaving. Agreements (from offline): - A CCE may be mapped to REGs with interleaved or non-interleaved REG indices within a CORESET - Definition of a REG bundle: The UE may assume that the same precoder is used for the REGs in a REG bundle and that the REGs in a REG bundle are contiguous in frequency and/or time - REG bundling per CCE is supported for NR-PDCCH - FFS: Whether this applies to common search space - FFS: Whether all REGs have DMRS or not - FFS: Whether wideband precoding is supported and the definition of a REG bundle if it is supported - FFS: whether REG bundle size is different for mapping of NR-PDCCH with or without interleaved mapping of CCE to REGs - FFS on REG bundle size - FFS whether REG bundle size is configurable Working assumption: - A NR-CCE is defined as 6 REGs - Candidate bundle sizes for distributed REG-to-CCE mapping: 2 or 3 REGs if NR-CCE is defined as 6 REGs - FFS: impact of the NR-CCE definition on CORESET size, CCE aggregation levels, data resource allocation granularity, etc.

RAN1#89

Agreements: - CCE = 6 REGs (confirm Working Assumption) - One of following is configured for REG-to-CCE mapping for a 1-symbol CORESET: - Opt.1: No interleaving – 6 REGs for a given CCE are grouped to form a REG bundle and all REGs for a given CCE are consecutive - CCE(s) of one PDCCH is/are also consecutive - FFS: Whether the UE can assume the same precoder across multiple REG bundles - Opt.2: Interleaving – [2 or 3 or 6] REGs for a given CCE are grouped to form a REG bundle and REG bundles are interleaved in the CORESET - FFS: Whether the UE can assume the same precoder across multiple REG bundles - FFS: down selection among {2}, {3}, {2,3}, {2,6}, {3,6}, {2,3,6} - Note: UE can assume the same precoder within a REG bundle - For REG-to-CCE mapping for a CORESET with more than 1-symbol; - REG bundle is defined in time and frequency-domain - At least support following: - Time-first mapping where one of the following is configured - Support REG bundle in time-domain being equal to the CORESET semi-statically configured time duration - Opt.1: Non interleaving - 6 REGs for a given CCE are grouped to form a REG bundle and all REGs for a given CCE are time and frequency localized - FFS: Whether the UE can assume the same precoder across multiple REG bundles - Opt.2: Interleaving – [2 or 3 or 6] REGs for a given CCE are grouped to form a REG bundle and REG bundles are interleaved in the CORESET - FFS: Whether the UE can assume the same precoder across multiple REG bundles - FFS: time-domain precoder-cycling - Support REG bundle in time-domain being equal to 1 symbol, or; - Support following: - REG-to-CCE mapping is exactly same as the case where a CORESET with 1 symbol - A PDCCH candidate can be mapped across OFDM symbols Agreements: - Confirm working assumption: - One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH - FFS: DMRS RE overhead for the REG transmitting DMRS is 1/3 - FFS on DMRS pattern Agreements: - In time domain, a CORESET can be configured with one or a set of contiguous OFDM symbols - The configuration can indicate the starting OFDM symbol and time duration - A CORESET is configured with only one CCE-to-REG mapping Working assumptions: - For a time-duration of a CORESET: - Support 1-3 OFDM symbol as time duration for a CORESET on the NR carrier with less than or equal to X PRBs - Support 1-2 OFDM symbol as time duration for a CORESET on the NR carrier with wider than X PRBs - FFS: X values - FFS: Other time duration - FFS: Relationship of a first PDSCH DMRS symbol with one or more symbols of a CORESET for slot-based scheduling - FFS: restriction in the certain conditions

RAN1#NR-AH2

Agreements: For a 1-symbol CORESET with interleaving, - At least REG bundle size = 2 is supported - Working assumption: - REG bundle size = 6 is also supported - FFS whether configuration between 2 and 6 is explicit or implicit - Precoder granularity in frequency domain is equal to the REG bundle size in the frequency domain For a 2 or 3 symbol CORESET with interleaving, - At least REG bundle size = CORESET length is supported - Working assumption: - REG bundle size = 6 is also supported - FFS whether configuration between CORESET length and 6 is explicit or implicit - Precoder granularity in frequency domain is equal to the REG bundle size in the frequency domain (Note: REG bundle size = REGs in frequency domain x symbols in time domain) Agreement - DMRS is mapped on all REGs on all the OFDM symbols of a given PDCCH candidate - The DMRS density is the same on all REGs Agreements: - For a CORESET which is configured by UE-specific higher-layer signalling, at least following are configured. - Frequency-domain resources, which may or may not be contiguous - Each contiguous part of a CORESET is equal to or more than the size of REG-bundle in frequency - FFS: exact size and number of contiguous parts for a CORESET - Starting OFDM symbol - Time duration - REG bundle size if the configuration is explicit - Transmission type (i.e., interleaved or non-interleaved) - More parameters may be added if agreed - For a CORESET which is configured by UE-specific higher-layer signalling, at least following is configured. - Monitoring periodicity - FFS: it is a configuration per CORESET or per one or a set of PDCCH candidates - FFS: relation with DRX - FFS: default/fallback value Agreements: - For PDCCH blind decoding, at least for the non-initial access, at least the following can be configured: - Number of PDCCH candidates per CCE aggregation level, per DCI format size that the UE monitors - Set of aggregation levels - FFS explicit or implicit configuration - Set of DCI format sizes - FFS explicit or implicit configuration - FFS: per CORESET not used for initial access or search space - FFS: Signalling details - Note that the number of candidates can be zero - UE blind decoding capability is known by NW - FFS: How the capability is derived

RAN1#90

Agreements: - Working assumptions are confirmed with the following details. - For 1/2/3-symbol CORESET, REG bundle size of 6 is supported. - A REG bundle size is as part of CORESET configuration for a CORESET configured by UE-specific higher-layer signalling. - FFS: CORESET(s) configured by non UE-specific signaling. - FFS: UE assumes that precoder granularity in frequency domain is equal to the REG bundle size in the frequency domain - FFS: gNB can inform to the UE whether or not to assume the same precoder over multiple REG bundles. - Note: more than one CORESET(s) with the UE-specific higher-layer signaling can be configured for the same UE Agreements: - Interleaving operates on REG bundles - FFS: interleaving in the case if and when gNB informs to the UE to assume the same precoder over multiple REG bundles Agreements: - For interleaving CORESET, the interleaving pattern is derived by the CORESET configuration and is not dependent on other CORESET configuration. - Note: Following metrics can be considered - Good frequency distribution of REG bundles within the CORESET - Blocking probability for potential overlapped CORESET(s) - Inter-cell/inter-TRP interference randomization Agreements: - DMRS density for a CORESET is down-selected between 1/3 or 1/4. - FFS: need of additional DMRS density. - Sequence, density, and applicability of MU-MIMO is still under discussion Working assumption: - DM-RS density per REG is 1/4 at least for normal CP - FFS: orthogonal DMRS for MU-MIMO at RAN1 NR AH#3. - FFS: URLLC Agreement: - The CORESET used to schedule the PDSCH containing the RMSI can be configured to contain also UE-specific PDCCH(s) Working assumptions: - For slot-based scheduling, the first DMRS position either on 3rd symbol or 4th symbol is configured by [PBCH]. - Maximum time duration of a CORESET is 2 symbols if the first DMRS position of a PDSCH with slot-based scheduling is on 3rd symbol, and is 3 symbols otherwise - This replaces the past working assumption linking DMRS position to bandwidth X Agreements: - Supported aggregation levels for NR-PDCCH are at least 1, 2, 4, 8 - FFS 16 and 32 aggregation levels and also other numbers Agreements: - A PDCCH search space at an aggregation level in a CORESET is defined by a set of PDCCH candidates - For the search space at the highest aggregation level in the CORESET, the CCEs corresponding to a PDCCH candidate are derived as following - The first CCE index of a PDCCH candidate is identified by using at least some of the followings - (1) UE-ID, (2) candidate number, (3) total number of CCEs for the PDCCH candidate, (4) total number of CCEs in the CORESET, and (5) randomization factor - The other CCE indexes of the PDCCH candidate are consecutive from the first CCE index - Searching space design for the lower aggregation level can be discussed separately Working assumptions: - In the case when only CORESET(s) for slot-based scheduling is configured for UE, the maximum number of PDCCH blind decodes per slot per carrier is X - The value of X does not exceed 44 - FFS the exact value of X - FFS for multiple active BWP, multiple TRP, multiple carriers, multi beams - FFS for non-slot based scheduling - FFS numerology specific X

RAN1#NR-AH3

Agreements: - Confirm the following working assumption: - DM-RS density per REG is 1/4 at least for normal CP. - FFS: orthogonal DMRS for MU-MIMO at RAN1 NR AH#3. - FFS: URLLC - DMRS density per REG for extended CP is same as that for normal CP Working assumption - For a CORESET, precoder granularity in frequency domain is: - Configurable between i) equal to the REG bundle size in the frequency domain; or ii) equal to the number of contiguous RBs in the frequency domain within the CORESET - For ii), DMRS is mapped over all REGs within the CORESET. Agreement: DMRS positions for PDCCH - Working assumption: Equally-distributed within a REG Working assumption: - Re-use NR DL RA Type 0 basis in units of 6 RBs, where no restriction on the maximum number of segments for a given CORESET. Agreements: - At least two DCI sizes are defined. - One DCI size, which is at least for the purpose of fallback. - FFS: for other purposes. - One DCI size depending on configuration - FFS: whether both DL and UL have the same size or different. - FFS: for group-common DCI/PDCCH - Note: the UE is not necessarily required to monitor two DCI sizes at the same monitoring occasion Agreement: - In a given CORESET - Alt 1: different DCI formats - Alt 2: different search spaces can have different monitoring periodicities. - FFS which one

RAN1#90bis

Agreements: - Support for a UE, configurable ID for PDCCH DMRS at least for the initialization of DMRS sequence/scrambling. - FFS: whether this is used also for other purpose. - FFS details including the applicability of configurable ID Agreements: - NR support the following interleaver - The number of rows A configured from {2, 3, and 6}, and the number of columns (P/A), where P is the number of interleaving unit for the given CORESET, and write in the row and read in the column - No any further permutation is introduced - Cyclic shift of the interleaving unit is applied based on the configurable ID which is independent parameter from the configurable ID for DMRS Agreements: - Confirm the WA: DMRS is evenly distributed within a REG - DMRS REs are #1, #5, #9 Agreements: - DMRS sequence is a Gold sequence as in LTE - Note: in case if new Gold sequence is introduced for NR, this would be revisited - DMRS sequence for NR-PDCCH is obtained according to a reference point in frequency domain. - FFS the reference point Agreements: - Confirm the WA with the following clarifications (in red): - For each CORESET, precoder granularity in frequency domain is: - Configurable between i) equal to the REG bundle size in the frequency domain; or ii) equal to the number of contiguous RBs in the frequency domain within the CORESET - For ii), DMRS is mapped over all REGs within CORESET. - RAN1 assumes that CORESET for PDCCH scheduling RMSI can be configured with Option i) - In Option ii) UE may assume DMRS is present in all REGs within the set of contiguous RBs of the CORESET where and when at least one REG of a candidate is mapped. Agreements: - Confirm the WA ‘Re-use NR DL RA Type 0 basis in units of 6 RBs, where no restriction on the maximum number of segments for a given CORESET’ with the following clarifications: - This is for the case when the CORESET is configured by at least UE-specific RRC signalling. - FFS the RB indexing for resource allocation especially considering interaction with DL BWP - Details of resource allocation should take into account the interaction with DL BWP – FFS details Agreements: - For slot-based scheduling; - Confirm the following working assumption with updates: - The first DMRS position either on symbol #2 or symbol #3 is configured by PBCH - Maximum time duration of a CORESET is 2 symbols if the first DMRS position of a PDSCH with slot-based scheduling is on symbol #2, and is 3 symbols otherwise. - The starting OFDM symbol of a CORESET can be symbol #0, #1, or #2, in a slot. - However, the ending OFDM symbol of a CORESET is not later than symbol #2 in a slot. Agreements: - In a given CORESET, two types of search spaces (e.g., UE-common search space and UE-specific search space) can have different periodicities for a UE to monitor - FFS details of the corresponding search spaces Agreements PDCCH candidates having different DCI payload sizes count as separate blind decodes PDCCH candidates comprised by different sets of CCE(s) count as separate blind decodes. PDCCH candidates in different CORESETs count as separate blind decodes. PDCCH candidates having the same DCI payload size and comprised by the same set of CCE(s) in the same CORESET count as one blind decodes. Agreements: - For non-CA and for PDCCH monitoring periodicity of 14 or more symbols, the maximum number of PDCCH blind decodes per slot is: - Working assumption: 44 for SCS = 15kHz. - Working assumption: less than 44 at least for SCS = 60kHz and 120kHz. - For the given SCS, all UEs support the maximum number of PDCCH blind decodes per slot. - Companies are encouraged to complete the following table. - Aiming to finalize this at RAN1#91. No. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Periodicity of 14 or more symbols; 44; [22-44]; [11-44]; [6-44]. Periodicity of less than 14 symbols; [44-86]; [22-86]; [11-44]; [6-44]. Agreements: - Companies are encouraged to provide the views on the following aspects: - Whether to specify upper limit of channel estimations a UE can perform for PDCCH; - If yes, how channel estimation is defined (e.g., per CCE or per REG bundle, whether common counting principle between narrowband RS and wideband RS), and; - What is the exact value of the upper limit of channel estimation a UE can perform for PDCCH. No. of CCEs that the UE can perform channel estimation per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Periodicity of 14 or more symbols; [48-74]. Periodicity of less than 14 symbols. Agreements: - For CA with up to N CCs, maximum number of PDCCH blind decodes per slot for a UE depends on the number of configured CCs. - All UEs supporting CA with the same set of CCs supports the same maximum number of PDCCH blind decodes. - No explicit UE capability signaling to inform the maximum number of PDCCH blind decodes is reported. - For CA with more than N CCs, maximum number of PDCCH blind decodes for a UE depends on the explicit UE capability. - All UEs supporting CA with the same set of CCs supports at least the same number of PDCCH blind decodes. - FFS: the value of N (no more than 8). Agreements: - For each SCS, whether or not separate UE capabilities for PDCCH monitoring periodicities are needed is concluded at RAN1#91. SCS. 15kHz; 30kHz; 60kHz; 120kHz. Need of separate UE capabilities for (i) PDCCH monitoring of 14 or more symbols and (ii) PDCCH monitoring of less than 14 symbols; Y/N; Y/N; Y/N; Y/N. Agreements: - Take the same hash function of LTE EPDCCH as the hash functuion for NR-PDCCH - Further refinements can be further considered till next meeting if necessary Agreements: - One set of the following parameters determines a set of search spaces - A set of aggregation levels - The number of PDCCH candidates for each aggregation level - PDCCH monitoring occasion for the set of search spaces Agreements: - At least for cases other than initial access, to identify a set of search spaces, following parameters are configured by UE-specific RRC signaling: - The number of PDCCH candidates for each aggregation level of {1, 2, 4, 8, [16]} - One value from {0, 1, 2, 3, 4, 5, 6, 8} - PDCCH monitoring occasion for the set of search spaces - One value of from {1-slot, 2-slot, [5-slot], [10-slot], [20-slot]} (at least 5 values) - One or more value(s) from 1st symbol, 2nd symbol, …, 14th symbol within a monitored slot - Each set of search spaces associates with a CORESET configuration by RRC signaling Agreements: - For PDCCH monitoring occasion of 1-slot, 2-slot, [5-slot], [10-slot], and [20-slot], - Slot-level offset value for PDCCH monitoring occasion is also supported. - For N-slot monitoring occasion, the offset is one from [0, N-1]. - Note: symbol-level bit-map of monitoring occasion within a slot agreed at RAN1#90bis is still available. Agreements: - For the DMRS of NR-PDCCH in a CORESET, - The QCL configuration/indication is on a per CORESET basis (Alt.1). Agreements: - The value(s) of TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, and/or TPC-SRS-RNTI, are provided by RRC signaling. - The association between at least each of the following RNTIs and a DCI format is specified in the specification. - C-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI. FFS: other RNTI(s). - The value of C-RNTI is obtained as part of random access procedure. - The association between a DCI format and a type of search space (UE-common search space and UE-specific search space) is specified in the specification. - UE-common search space contains a DCI format of C-RNTI, RNTI(s) for SPS/grant-free, TPC-PUSCH-RNTI, TPC-PUCCH,RNTI, TPC-SRS-RNTI, and INT-RNTI. - UE-specific search space contains a DCI format of C-RNTI and RNTI(s) for SPS/grant-free. Agreements: - By PBCH, a UE obtains at least one CORESET configuration at least for PDCCH scheduling RMSI associated with a given SS block. - The set of aggregation levels and candidates per aggregation level for PDCCH scheduling RMSI is specified in the specification. - FFS the indication of the support of aggregation level 16 in the cell - FFS: Set of search spaces for OSI, random access, and paging. - By RMSI, the UE can be configured with at least one CORESET configuration at least for PDCCH for random access. - If not configured by RMSI, the CORESET configuration(s) for random access is/are the one(s) configured by PBCH. - FFS: whether the CORESET configuration can be configured outside of the initial active DL BWP. - By UE-specific RRC signalling, the UE can be configured with one or more CORESET configuration(s) at least for PDCCH scheduling UE-specific data. - Each CORESET configuration is associated with one or more sets of search spaces. - Note: each set of search spaces is associated with one CORESET configuration.

RAN1#91

Agreements: - For each CORESET configured by PBCH, physical cell ID is used for DMRS sequence initialization - Working assumption: For each CORESET configured by RMSI, it can be configured with a configurable ID for DMRS sequence initialization via RMSI (if not configured, physical cell ID is used for DMRS sequence initialization) - Working assumption: The value range of the configurable ID is the same as that for physical cell ID - For each CORESET configured by UE-specific RRC signalling, a UE is configured with a configurable ID for DMRS sequence initialization - Working assumption: The value range of the configurable ID is the same as that for physical cell ID in Rel-15 NOTE: there is subsequent agreement made in 7.2 which replaces working assumption Agreements: - No new RRC parameter is necessary to identify the reference point for DMRS generation for the given CORESET. - For a CORESET configured by UE-specific RRC signaling, a configurable ID for cyclic shift of the interleaving unit. - The value range of the configurable ID is {0 - 274}. - For a CORESET configured by PBCH/RMSI, physical cell ID is used for the cyclic shift of the interleaving unit. Agreements: - The same length-31 Gold sequence with LTE is used for scrambling PDCCH and DMRS for PDCCH. Agreements: - Reference point for DMRS generation for PDCCH is, - PRB 0 of common PRB indexing for UE-specific CORESET - PRB 0 of the initial active DL BWP for CORESET configured by PBCH/RMSI Agreements: - RRC parameter “CORESET-start-symb” is deleted from the RRC parameter list. Agreements: - For NR PDCCH associated with the CORESET(s) configured by PBCH, AL=16 is supported. - For NR PDCCH associated with the CORESET(s) configured by RMSI or UE-specific RRC signaling, AL=16 is supported - Note: additional complexity, if any, for NR PDCCH channel estimation is to be discussed separately - Discuss further offline whether or not AL=16 is associated with wideband RS only Agreements: - CORESET configured by RMSI is confined within the initial active DL BWP Agreements: - For a CORESET configured by UE-specific RRC signaling, DL BWP-specific RB indexing + RB-offset are used to configure frequency-domain resource. - The length of the bit-map is Floor((N_RB – (ceil(BWP_start/6)*6-BWP_start))/6) - CORESET starting RB is ceil(BWP_start/6)*6 - For a CORESET configured by PBCH/RMSI, RB indexing is for the initial DL BWP. Agreements: - C-SS in each DL BWP of the PCell/PScell - On C-SS, Yp ,kp= 0. - In Rel.15, - For scheduling RMSI, OSI, Paging, UE monitors common search space in the PCell only - In addition, for random access and fall back, UE monitors common search space in the PCell and PSCell only - Working assumption: The UE is not expected to be configured without C-SS on the PCell (PSCell) in the active DL BWP - NOTE: RAN1 does not expect additional impact on the UE behavior due to not having PRACH resource in the BWP - Working assumption: In Rel.15, - A UE is expected to monitor C-SS (if configured) in the activated BWP - Full functionalities of C-SS (scheduling RMSI, OSI, Paging, random access, etc) are supported by the C-SS configured by UE-specific RRC signaling. - All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling. Agreements: - C-SS (at least for SFI/PI if configured) in a Scell: - On C-SS, Yp ,kp= 0. - Working assumption: All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling. Agreements: - Introduce a linkage between search space set and CORESET via an index to the CORESET configuration - CORESET is removed from the search space configuration - In Rel-15, the max no. of CORESETs configurable for a BWP in a cell for a UE is [3] - In Rel-15, the max no. of search space sets configurable for a BWP in a cell for a UE is [10] Agreements: - Scrambling for PDCCH (after channel coding) is supported. - No additional RRC parameter is necessary. - Re-use the ID for DMRS initialization. Conclusion: - It is clarified that Mp,maxL is the maximum of “configured” number of PDCCH candidates for the given aggregation level L across all serving cells scheduled by the search space Agreements: - UE is not expected to receive PDSCH type A in the same slot if the PDCCH monitoring is after the first two or three symbols of a slot - Note: PUSCH mapping is up to MIMO decision Agreements: - For information, the following cases are clarified: - Case 1: PDCCH monitoring periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring periodicity of less than 14 symbols - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot - The numbers in bracket in the following table can be further adjusted but not to be increased - X<=16, Y<=8 - FFS whether or not to have case 2’, where the values of X and/or Y can be smaller than case 2 Max no. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Case 1-1; 44; 36; 22; 20. Case 1-2; [44]; -. Case 2; [44+X]; [36+Y]; [22+Y]; [20]. Working assumption: - For PDCCH monitoring for receiving RMSI, the number of PDCCH candidates are following: - 4 candidates for AL = 4 - 2 candidates for AL = 8 - DCI size for RMSI scheduling and DCI size for OSI scheduling are the same - FFS: Paging and fallback Conclusion: - RAN1 common understanding is that the PDCCH channel estimation complexity is not negligible at least in some cases. - FFS: Possible solutions to resolve the channel estimation complexity issue together with the impact on PDCCH blocking probability - Opt.1: Define the limits of “the number of CCEs for PDCCH channel estimation which refers to the union of the sets of CCEs for PDCCH candidates” - Note: the overlapped CCEs associated with different CORESETs are counted separately. - FFS: CCEs for the same precoder-granularity are counted as one channel estimation - FFS: whether/how to handle the variation on the actual number of CCEs for PDCCH channel estimation and BDs over time - Application of overbooking is considered - Strive for not having specific UE capability to report the maximum number of CCEs for PDCCH channel estimation. - Study the solutions considering the cases 1-1, 1-2, 2, and 2’. - Opt.2: Modify the hashing function - Opt.3: Increase the size of the precoder granularity

RAN1#NR-AH1801

Agreements: - Adopt following text proposal for PDCCH scrambling (TS38.211 Section 7.3.2.3) == The scrambling sequence generator shall be initialized with cinit=nRNTI∙216+nID mod 231 where - nID∈{0, 1, …, 216-1} equals the higher-layer parameter Control-scrambling-Identity if configured and RNTI is equal to C-RNTI, otherwise, and - nRNTI is C-RNTI for PDCCH in USS, while is 0 for PDCCH in CSS. Companies can check whether the above agreements are consistent with previous coding discussion/agreements or not Agreements: - DM-RS sequence for PDCCH is initialized with an equation with at least symbol index, slot index, and the ID. - FFS: further randomization to avoid consistent collisions over frames. - DMRS sequence for PDCCH is generated per symbol. Agreements: - The TP in R1-1801278 for 38.211 is endorsed - The last sentence is subject to editor’s refinement Agreements: - The maximum number of CORESETs per BWP per cell is 3 - For the 3rd CORESET, i.e., for p=2, Ap=39839. - Confirm 10 as the maximum number of search space sets per BWP per cell. Agreements: - CORESET ID of the CORESET configured by PBCH is 0. - Search space ID of the search space configured by PBCH is 0. Agreements: - A UE can be configured with a search space configuration by UE-specific RRC signaling which includes following: - CORESET ID (range: 0-11, to indicate which CORESET the search space is mapped to) - The search space can be associated with any CORESET configuration - When the CORSET ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH - Search space ID (range: 0-39) - When the search space ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH Agreements: - DCI format 0_1 and 1_1 are monitored only in USS. - DCI format 0_0 and 1_0 are monitored in CSS. - DCI format 0_0 and 1_0 can be monitored in USS. - They have the same DCI payload size. - One of the following is configured by RRC signaling for the USS: - Monitoring DCI format 0_1 and 1_1 only - Monitoring DCI format 0_0 and 1_0 only Agreements: - NR supports a DCI format having the same size as the DCI format 1_0 to be used for scheduling RMSI/OSI, for Paging, and for random access. Agreements: - For each search space configuration configured by UE-specific RRC signaling, the UE is informed whether the search space configuration is CSS or USS, together with the following information, as part of the search space configuration: - Which DCI format(s) to monitor - For a CSS, - DCI format 0_0 and DCI format 1_0 - In which case, the UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), SP-CSI-RNTI (if configured), RA-RNTI, TC-RNTI, P-RNTI, SI-RNTI - DCI format 2_0 - In which case, the UE monitors the DCI format with CRC scrambled by SFI-RNTI, and the SFI-related parameters SFI-PDCCH is provided as part of the search space configuration - FFS: how to select one or two decoding candidates if the configured PDCCH candidates are larger than the value - DCI format 2_1 - In which case, the UE monitors the DCI format with CRC scrambled by INT-RNTI, and the PI-related parameters Preemp-DL is provided as part of the search space configuration - DCI format 2_2 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI - DCI format 2_3 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-SRS-RNTI - Monitoring of multiple DCI formats can be configured for one CSS - For USS, - A UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), TC-RNTI (if a certain condition is met), and SP-CSI-RNTI (if configured) - Further discussion offline the association of the RNTIs with DCI formats - Monitoring of multiple DCI formats can be configured for one USS Agreements: - For a search space configuration, monitoring periodicity of slot(s) is updated as follows: - For all SCS, {1, 2, 4, 5, 8, 10, 16, 20} slots - For INT-RNTI, a subset of {1,2,4} slots is applied - FFS: the case when concatenated semi-static DL/UL assignments is configured Working assumption: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for 48 CCEs for a given slot per scheduled cell - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2 Agreements - For the following previous agreement, N=4 Agreements: - For CA with up to N CCs, maximum number of PDCCH blind decodes per slot for a UE depends on the number of configured CCs. - All UEs supporting CA with the same set of CCs supports the same maximum number of PDCCH blind decodes. - No explicit UE capability signaling to inform the maximum number of PDCCH blind decodes is reported. - For CA with more than N CCs, maximum number of PDCCH blind decodes for a UE depends on the explicit UE capability. - All UEs supporting CA with the same set of CCs supports at least the same number of PDCCH blind decodes. - FFS: the value of N (no more than 8).

RAN1#92

Agreements: - Adopt the following TP for 38.211, 7.4.1.1.1 === Start === The pseudo-random sequence generator shall be initialized with cinit=(217(14ns,f + l + 1)(2NID+1) + 2 NID) mod 231 Where l is the OFDM symbol number within the slot, ns,fμ ns,f is the slot number within a frame, and - NID{0, 1, …, 65535} is given by the higher-layer parameter PDCCH-DMRS-Scrambling-ID if provided NID=NIDcell otherwise. === End === Conclusion: - Regarding the issue of cyclic shift for PDCCH interleaver as discussed in Section 2 of R1-1803298: - No spec update is necessary Agreements: - For RMSI CORESET, interleaved CCE-to-REG mapping with - REG-bundle size of 6 - R=2 - All RMSI CORESET configurations are with precoder granularity = REG-bundle size. Agreements: - To adopt the following TP (Section 7.3.2.3, 38.211) === Start === The scrambling sequence generator shall be initialized with where - For a PDCCH in a UE-specific search space, nID{0, 1, …, 65535} equals the higher-layer parameter Control-scrambling-Identity if configured, - otherwise and where is given by the C-RNTI for a PDCCH in a UE-specific search space if Control-scrambling-Identity is configured and otherwise for a PDCCH in a common search space. === End === Agreements: - If a PDCCH decoding candidate having a CCE overlapped, even partially, with the configured SSB, the UE is not required to monitor the PDCCH with the decoding candidate. Agreements: - To adopt the following TP for 38.211: === Start === 7.3.2.5 Mapping to physical resources The UE shall assume the block of complex-valued symbols to be scaled by a factor and mapped to resource elements not used for the associated PDCCH DMRS in increasing order of first , then , in the resource-element groups used for the monitored PDCCH. The antenna port . === End === Agreements: - To adopt the following TP for 7.4.1.3.2 of 38.211 === Start === 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence is mapped to resource elements according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter CORESET-precoder-granularity equals CORESET-REG-bundle-size, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter CORESET-precoder-granularity equals the size of the CORESET in the frequency domain. === End === Agreements: - Adopt following TP: === Start === For interleaved CCE-to-REG mapping, for and for where is configured by the higher-layer parameter CORESET-REG-bundle-size. The interleaver is defined by where is given by the higher-layer parameter CORESET-interleaver-size and where - is a function of for a PDCCH transmitted in a CORESET configured by the PBCH or RMSI - is a function of the higher-layer parameter CORESET-shift-index Agreements: - Confirm the value for Case 1-2. X=0 and Y=0 for Case 2. No consensus on additional Case 2’. Max no. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Case 1-1; 44; 36; 22; 20. Case 1-2; [44]; -. Case 2; [44+X]; [36+Y]; [22+Y]; [20]. Agreements: - To adopt the TP for TS38.213 Section 10.1 - Also add one sentence in the spec saying “when the number of REGs is not sufficient for a given aggregation level, the UE is not required to monitor candidates of the given aggregation level” - Up to spec editor for final wording === Start === Table 10.1-1: CCE aggregation levels and max number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. === End === Agreements: - The number of CCEs for PDCCH channel estimation which refers to the union of the sets of CCEs for PDCCH candidates to be monitored, regardless of which REG-bundle size or precoder granularity. - Overlapped CCEs associated with different CORESETs are counted separately. - Overlapped CCEs associated with different PDCCH starting symbols with the same or different search space sets with the same CORESET are counted separately. - Overlapped CCEs associated with same or different search space sets with the same PDCCH starting symbol associated with the same CORESET are counted one. - Note: in the above, the overlapping CCEs for candidates for a given search space set with different starting symbols are assumed to be supported. Agreements: - Change Y_{p, kp} to Y_{p, ns,f ns,fμ} ns,fμ Yp,ns,fμ in the search space hashing function in subclause 10.1 of 38.213, where the index ns,f ns,fμ is the slot number. - (Working assumption) The reset of the update is per radio frame Agreements: - The UE capability signaling for PDCCH BDs in CA is integer value from {4, …, 16}. - Discuss further whether or not to restrict the combination of the number of CCs that a UE can support vs. the number of PDCCH BDs indicated via UE capability signalling Agreements: - Mp,maxL is Mp,s,maxL which is the maximum number of PDCCH candidates for the given aggregation level L across all serving cells for the given search space set s for the given CORESET p Agreements: - Specify PDCCH candidate mapping rules. - PDCCH candidates are mapped to search-space-sets until either or both limit(s) of (number of blind decodes, CCEs for channel estimation) is/are met at least with the following rule - SS type order, e.g. CSS before USS - FFS: further rule within a search space set/type Agreements: - Confirm the following working assumption, with updates: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for following numbers of 48 CCEs for a given slot per scheduled cell - 56 CCEs for SCS = 15kHz and 30kHz - 48 CCEs for SCS = 60kHz - 32 CCEs for SCS = 120kHz - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2 Agreements: - Adopt following TP (38.213) === Start === A UE determines a PDCCH monitoring occasion from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For the search space set s in the control resource set p, the UE monitors PDCCH in a slot where nfNslotframe,μ+ns,fμ-op,smod kp,s=0 is satisfied, with nf nfbeing the frame number. === End ===

RAN1#92bis

Agreements: - NW and UE maintain the same understanding on SSB/CORESET#0/SS#0 in connected_mode at least for non-broadcast PDCCH - Solutions FFS - For the broadcast PDCCH, it is up to UE which common search space to monitor based on which SSB in both connected, in-active, and idle modes - Unicast PDSCH can be scheduled by a DCI associated with the CORESET #0 Agreements: - The size of DCI 1_1 is the same (by zero-padding when necessary) across CORESETs of a BWP of a serving cell Agreements: - For WB RS CORESET, the UE is expected to be configured with the CORESET only with the following possibilities: - Up to 4 clusters, each cluster consisting of contiguous RBs. - It is clarified that the counting rule of CCE for channel estimation is the same between NB RS CORESET and WB RS CORESET, which implies following: - The number of CCEs for channel estimation in case of WB RS CORESET is counted as the union of the sets of CCEs for PDCCH candidates to be monitored and hence BW of the WB RS CORESET is not restricted by CCE limit for channel estimation. - Note: it is up to RAN4 whether UE may perform channel estimation within a BW smaller than the maximum number of contiguous RBs within the WB RS CORESET Agreements: - For Rel.15 December 2017 version of Case 2, number of CCEs for channel estimation per slot is {56, 56, 48, 32} CCEs for SCS {15kHz, 30kHz, 60kHz, 120kHz} Agreements: - UE is not expected to be configured with PDCCH monitoring in CSS(s) for more than what UE can monitor in terms of numbers of BDs/CCEs Agreements: - Following working assumption is made: - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of PDCCH blind decodes per slot the UE shall support is X*M, and; - For a UE supporting CA with up to Y DL-CCs with the same numerology Y > 4, maximum number of PDCCH blind decodes per slot the UE shall support is y*M, where; - M = {44, 36, 22, 20} for SCS = {15kHz, 30kHz, 60kHz, 120kHz} - y is an integer from {4, …, 16} and is reported as UE capability to the network. - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of CCEs for channel estimation per slot the UE shall support is X*N, and; - For a UE supporting CA with up to Y DL-CCs with the same numerology with Y > 4, maximum number of CCEs for channel estimation per slot the UE shall support is y*N, where; - N = {56, 56, 48, 32} for SCS = {15kHz, 30kHz, 60kHz, 120kHz} - y is an integer from {4, …, 16} and is reported as UE capability to the network. Agreements: - The working assumption as part of the agreements below is confirmed: - Change Y_{p, kp} to Y_{p, ns,f } in the search space hashing function in subclause 10.1 of 38.213, where the index ns,f is the slot number. - (Working assumption) The reset of the update is per radio frame Agreements: - A UE is not expected to be configured to monitor PDCCH across a slot boundary. Working assumption: - For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with C-RNTI is monitored in non-DRX occasions after C-RNTI is available. - For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with CS-RNTI is monitored in non-DRX occasions after CS-RNTI is available. Agreements: - Following is to be clarified in 213: - Type0-CSS is configured by RMSI-PDCCH-Config provided by MIB or is configured by searchSpace-SIB1 provided by PDCCH-ConfigCommon - Type0A-CSS is configured by searchSpace-OSI provided by PDCCH-ConfigCommon - Type1-CSS is configured by ra-SearchSpace provided by PDCCH-ConfigCommon - Type2-CSS is configured by pagingSearchSpace provided by PDCCH-ConfigCommon - Type3-CSS is configured by a SearchSpace with the searchSpaceType “common” provided by PDCCH-Config Working assumption: - At least for Case 1-1 and Case 1-2, map all candidates of USS search-space-set with lower SS set ID before candidates of USS with higher ID - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped) - Case 2 FFS

- References

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R1-1805880 | Remaining issues on PDCCH structure | Huawei, HiSilicon | 7.1.3.1.1 |
| R1-1806055 | Remaining issues on PDCCH CORESET | vivo | 7.1.3.1.1 |
| R1-1806127 | PDCCH structure remaining issues | ZTE | 7.1.3.1.1 |
| R1-1806141 | Remaining details on PDCCH structure | Nokia, Nokia Shanghai Bell | 7.1.3.1.1 |
| R1-1806288 | On PDCCH structure | CATT | 7.1.3.1.1 |
| R1-1806361 | Discussion on remaining issues for unicast PDCCH in CORESET#0 | CMCC | 7.1.3.1.1 |
| R1-1806362 | Discussion on PDCCH repetition for URLLC | CMCC | 7.1.3.1.1 |
| R1-1806408 | Remaining issues on PDCCH structure | Spreadtrum Communications | 7.1.3.1.1 |
| R1-1806615 | Remaining issues on PDCCH structure | LG Electronics | 7.1.3.1.1 |
| R1-1806728 | Corrections on PDCCH structure | Samsung | 7.1.3.1.1 |
| R1-1806778 | Reamining issues on PDCCH structure | MediaTek Inc. | 7.1.3.1.1 |
| R1-1806915 | Collision between SSB and PDCCH | Panasonic | 7.1.3.1.1 |
| R1-1807160 | Remaining issues on CORESET#0 | ITRI | 7.1.3.1.1 |
| R1-1807246 | Remaining issues of PDCCH | Ericsson | 7.1.3.1.1 |
| R1-1807352 | Remaining issues on PDCCH structure | Qualcomm Incorporated | 7.1.3.1.1 |
| R1-1807413 | Remaining issues on PDCCH structure | Qualcomm Incorporated | 7.1.3.1.1 |
| R1-1805881 | Remaining issues on search space | Huawei, HiSilicon | 7.1.3.1.2 |
| R1-1806019 | Search Space Design for URLLC | Ericsson | 7.1.3.1.2 |
| R1-1806056 | Remaining issues on Search Space | vivo | 7.1.3.1.2 |
| R1-1806128 | Remaining issues on search space | ZTE | 7.1.3.1.2 |
| R1-1806142 | Remaining details on search space | Nokia, Nokia Shanghai Bell | 7.1.3.1.2 |
| R1-1806289 | Remaining details of NR PDCCH search space | CATT | 7.1.3.1.2 |
| R1-1806550 | Remaining details on CORESET and search space | Intel Corporation | 7.1.3.1.2 |
| R1-1806616 | Remaining issues on search space | LG Electronics | 7.1.3.1.2 |
| R1-1806655 | On the PDCCH limits on the number of CCEs for channel estimation and BDs for URLLC | Nokia, Nokia Shanghai Bell | 7.1.3.1.2 |
| R1-1806666 | Clarification on UE procedure for receiving OSI | ETRI | 7.1.3.1.2 |
| R1-1806729 | Corrections on Search Space Design | Samsung | 7.1.3.1.2 |
| R1-1806779 | Reamining issues on PDCCH search space | MediaTek Inc. | 7.1.3.1.2 |
| R1-1806879 | Channel estimation complexity and search space | Panasonic Corporation | 7.1.3.1.2 |
| R1-1807016 | On remaining aspects of search space | InterDigital, Inc. | 7.1.3.1.2 |
| R1-1807058 | Search space | NTT DOCOMO, INC. | 7.1.3.1.2 |
| R1-1807059 | Search space for June drop URLLC | NTT DOCOMO, INC. | 7.1.3.1.2 |
| R1-1807238 | Remaining details on search space | ASUSTEK COMPUTER (SHANGHAI) | 7.1.3.1.2 |
| R1-1807247 | Remaining issues of search space | Ericsson | 7.1.3.1.2 |
| R1-1807353 | Remaining issues on control resource set and search space | Qualcomm Incorporated | 7.1.3.1.2 |