3GPP TSG RAN WG1 Meeting 93		 R1-1807761

Busan, Korea, May 21st – 25th, 2018 	Revision of R1-1807616

Agenda item:	7.1.7

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for MIMO feature list

Document for:	Discussion/Decision

# Discussion

In this document, we discuss proposed changes to the NR UE feature list.

Note that this contribution only addresses eMBB related capabilities (not URLLC).

## TCI states

Should be clarified that the indicated number of TCI states also includes control QCL states

2-4 TCI states for PDSCH

| 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per CC, including control and data 2. maximum number of configured TCI states, | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  |  |  | Component-1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-4a | Additional control QCL state | Support one additional active QCL state for control in addition to the supported number of active TCI states for PDSCH Applicable if Component-1 of 2-4 is set to 1 | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  | Note: Only applicable if Component-1 of 2-4 is set to 1 |  | Mandatory with capability signaling |

Proposal 1:

Clarify that the number of active TCI states per CC includes the number of non-identical active QCL configurations for both control and data. Add Feature 2-4a to indicate support of one additional active QCL state for control

## Number of CSI-RS resources

Needs to be clarified that the number of CSI-RS resources and number of ports used are counted multiple times (n times) if the same resource is used in multiple (n) simultaneous CSI calculations.

2-36, 2-40, 2-41, 2-43

| 2-36 | Type I single panel codebook | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s) | 2-35 | Yes | No additional Type I codebook configurations other than the basic CSI feedback (2-32) is supported | Type 3 | No need |  |  | Note: simultaneously doesn’t mean in the same slot Note: When determining the number of resources and total number of ports for Component-1, a given resource or port is counted multiple times when multiple reports are calculated simultaneously based on the same resource or port |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2} Down-select: Alt.1 Mode-1 as mandatory Alt.2: Both Mode-1 and Mode-2 are mandatory (in this case, this component is not needed) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-40 | Type I multi-panel codebook | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s): 3. Supported number of panels, Ng, | 2-35 | Yes | multi-panel Type I codebook is not supported | Type 3 | N.A. | N.A. |  | Note: simultaneously doesn’t mean in the same slot Note: When determining the number of resources and total number of ports for Component-1, a given resource or port is counted multiple times when multiple reports are calculated simultaneously based on the same resource or port |  | Optional Component-1: the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} |
| 2-41 | Type II codebook | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Support amplitude subset restriction level | 2-35 | Yes | Type II codebook is not supported | Type 3 | N.A. | N.A. |  | Note: simultaneously doesn’t mean in the same slot Note: When determining the number of resources and total number of ports for Component-1, a given resource or port is counted multiple times when multiple reports are calculated simultaneously based on the same resource or port |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {no restriction, subset restriction}, “no restriction” is mandatory and RAN1 hasn’t decide whether “subset restriction” is mandatory or not, if it’s mandatory then this component is not needed. |
| 2-43 | Type II codebook with port selection | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type |  | Yes | Type II codebook with port selection is not supported | Type 3 | N.A. | N.A. |  | Note: simultaneously doesn’t mean in the same slot Note: When determining the number of resources and total number of ports for Component-1, a given resource or port is counted multiple times when multiple reports are calculated simultaneously based on the same resource or port |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} |

Proposal 2:

For each of Type I, Type I multi-panel, Type II, Type II with port selection codebooks, clarify that the number of CSI-RS resources and total number of ports used are counted multiple times (n times) if the same resource or port is used in multiple (n) simultaneous CSI calculations.

## Beam management

2-24 SSB/CSI-RS for beam measurement

| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS 4. Supported max # of configured NZP CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) per CCs to measure L1-RSRP 5. When intra-band NR CA is configured in a band, the same beam management is applied across all CCs in the band | 2-21, 2-22 or 2-23, 2-23a | Yes | RSRP measurement is not supported | Type 1 | No need | Yes |  |  |  | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 Component-4: candidate value set is {0, 4, 8, 16, 32, 64, 128} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3:

Need to have a limit on configurable CSI-RS resources for L1-RSRP.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

## Semi-persistent resources

There is already a capability for semi-persistent CSI report; however, there is no capability for rate matching around semi-persistent resources for UEs that don’t support semi-persistent CSI report.

2-33b SP CSI-RS and SP CSI-IM

| 2-33b | SP CSI-RS and SP CSI-IM | 1. Support of SP-CSI-RS and SP-CSI-IM | 2-1 | Yes |  | Type 1 | N.A. | N.A. |  |  | RAN1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4:

Introduce capability for handling semi-persistent resources.

## Codebook based UL MIMO

There is already capability defined for number of SRS resources transmitted simultaneously but there is no capability for number of SRS resources supported.

2-14 Codebook based PUSCH MIMO transmission

| 2-14 | Codebook based PUSCH MIMO transmission | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers 2. supported maximal number of SRS resources for codebook based transmission | 2-13 | Yes | Uplink codebook based MIMO (with >1 Tx port) transmission is not supports | Type 3 | N.A. | N.A. |  |  |  | Component-1 candidate value set: {no-codebook based MIMO, 1, 2, 4} Component-2 candidate value set: {1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:

Introduce capability indication for maximum number of SRS resources supported.

## CSI report

In order to merge Type A and Type B UE CSI capability, we propose to introduce a capability of simultaneously active resources. Active resources consist of those that the UE is either currently processing or storing for later triggered A-CSI report.

2-35 CSI report framework

| 2-35 | CSI report framework | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process] 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. 6. Supported CSI computation type 7. A list of supported combinations, each combination is {max number of active NZP CSI-RS resources, max total number of ports in all active NZP CSI-RS} that the UE can process simultaneously across all CCs. FFS: whether X should also count the SRS precoder derivation in case of reciprocity based SRS Tx | 2-32 | Yes | CSI report is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-5: candidate values: {from 5 to 32} Componet-6: Candidate values: {Type-A, Type-B} Componet-7: The candidate values for the max # of active NZP CSI-RS resources: {from 1 to 64} The candidate value set of the max # of ports in all active resources is: {from 2 to 256} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6:

Define the memory resource available for CSI processing by (Kmaxμ,Pmaxμ) as a UE capability, where Kmaxμ is the maximum number of active NZP CSI-RS resources across all CCs with SCS index μ, Pmaxμ is the maximum total number of ports in all active NZP CSI-RS resources across all CCs with SCS index μ.  An NZP CSI-RS resource is active in a duration of time defined as follows:

- For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS.

- For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied.

- For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released.

# Conclusions

In this contribution, the following proposals have been made:

Proposal 1:

Clarify that the number of active TCI states per CC includes the number of non-identical active QCL configurations for both control and data.

Proposal 2:

For each of Type I, Type I multi-panel, Type II, Type II with port selection codebooks, clarify that the number of CSI-RS resources and total number of ports used are counted multiple times (n times) if the same resource or port is used in multiple (n) simultaneous CSI calculations.

Proposal 3:

Need to have a limit on configurable CSI-RS resources for L1-RSRP.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

Proposal 4:

Introduce capability for handling semi-persistent resources.

Proposal 5:

Introduce capability indication for maximum number of SRS resources supported.

Proposal 6:

Define the memory resource available for CSI processing by (Kmaxμ,Pmaxμ) as a UE capability, where Kmaxμ is the maximum number of active NZP CSI-RS resources across all CCs with SCS index μ, Pmaxμ is the maximum total number of ports in all active NZP CSI-RS resources across all CCs with SCS index μ.  An NZP CSI-RS resource is active in a duration of time defined as follows:

- For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS.

- For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied.

- For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released.

# References

- R1-1805728, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1800886, “NR features and capabilities”, Qualcomm

- R1-1802849, “NR features and capabilities”, Qualcomm