3GPP TSG RAN WG1 #93	R1-1807777

May 21st  – May 25th, 2018

Busan, Korea

Agenda item:	7.6

Source: 	Qualcomm Incorporated

Title: 	Summary of offline discussion on NR-U

Document for:	Discussion and Decision

# Background

The following agreements were reached before.

RAN1 #92, Feb 2018

R1-1803272 is approved with the following changes:

- Remove duplicate section 4.4

- Move section 4.4 as a subsection of section 4.1

Agreement:

- The study targets identification of additional functionality needed for a PHY layer design (except channel access procedures) for operation in unlicensed spectrum that may be applicable over a particular frequency range (e.g., sub-7 GHz, 7-52.6 GHz, > 52.6 GHz).

  - FFS: The definition of the frequency ranges

  - Note: Optimizations for a particular frequency band may be necessary.

  - Note: Channel bandwidths below 5 MHz are not targeted

- The study targets the design of channel access procedures for frequency bands based on coexistence and regulatory considerations applicable to the band.

- Note: The study includes identification of procedures for technology neutral channel access for frequency bands that may become available subject to regulations.

  - The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands.

Agreement:

- 5GCM in 38.802 is used for NR-U simulation evaluation

- NR-unlicensed simulation evaluation considers the following scenarios

  - Indoor sub-7GHz, 2 operators

  - Outdoor Sub-7 GHz, 2 operators

  - Indoor mmW, 2 Operators

  - Outdoor mmW, 2 operators

  - Stadium scenario for sub-7GHz, 2 operators, can be optionally considered by interested companies.

  - Note: RAN1 prioritizes the simulation for sub-7 GHz band. It does not preclude evaluation for above 7 GHz.

- Deployment scenarios to simulate

  - CA between NR licensed cell and NR unlicensed cell

  - DC (with LTE and with NR)

  - SA

  - An NR cell with DL in unlicensed band and UL in licensed band

  - Note: A single set of evaluations may be applicable to multiple scenarios

  - Note: Only unlicensed cell(s) is simulated.

  - Note: The licensed cell may not be explicitly modeled in the simulation. Necessary assumptions regarding the presence of the licensed carriers can be made and provided.

- Coexistence with other networks (e.g. WiFi, LAA LTE, NR-U)

  - When coexistence with WiFi is evaluated, only consider deployed WiFi systems (e.g. 11ac for 5 GHz)

    - Fairness criterion for coexistence with 11ax can be further discussed at plenary level

    - The coexistence evaluation applies to 5GHz band (11ac) and 60GHz (11ad)

    - From SID: NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier

  - For sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (eg. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations

    - Note: The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands

Note (for the minutes): Some companies believe that a prioritization among the agreed simulation scenarios may be necessary.

Agreement:

The following network topologies are included in the evaluations:

- Indoor sub7GHz, choose one of the following options

  - Option 1: Reuse 38.802 indoor hotspot topology and allocating half of the gNBs to each operator (6+6)

  - Option 2: Reuse 38.802 indoor hotspot topology but further reduce gNB density (3+3)

  - Option 3: Based on IEEE indoor enterprise model with modifications

- Outdoor sub7GHz

  - NR dense urban scenario with two layers, but only consider the micro layer

  - Randomly drop one micro layer per operator

- Indoor mmW

  - Reuse indoor sub7GHz topology

  - Parameter changes may be needed and submitted together with simulation results

- Outdoor mmW

  - Reuse outdoor sub7GHz topology

  - Parameter changes may be needed and submitted together with simulation results

Agreement:

Study the additional functionality needed beyond the specifications for operation in licensed spectrum in the following deployment scenarios.

- Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)

  - NR-U SCell may have both DL and UL, or DL-only.

- Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

- Stand-alone NR-U

- An NR cell with DL in unlicensed band and UL in licensed band

- Dual connectivity between licensed band NR (PCell) and NR-U (PSCell)

Note (for the minutes): Some companies believe that a prioritization among the agreed deployment scenarios may be necessary.

Agreement:

- From RAN1 design perspective, the study is not limited to a particular unlicensed band

- Note: This does not have any implications on prioritizations between unlicensed bands

- Note: The study does not target sub-1GHz unlicensed bands

RAN1 #92bis, April 2018

Agreement:

In the discussions in the NR-U study item, references to sub-7 GHz are intended to include unlicensed bands in the 6 GHz region that are being discussed in regulatory discussions which may have some region exceeding 7 GHz (e.g., 7.125 GHz)

Agreement:

- For sub7 indoor simulation evaluation:

  - Scenario: Option 2 (3+3) with indoor mixed office model

    - Target to reach 10%-15% serving links below -72dBm

    - Further layout parameter fine tuning may be needed. An example procedure for fine tuning is the following sequence.

      - Currently a-b-a=15-20-15

      - If not reaching target, try a-b-a=15-30-15 and a-b-a=20-40-20

      - If not reaching target, apply a scaling factor to the layout with a-b-a=20-40-20

  - Other parameters: Default is NR parameters in 38.901 and 38.802 with the exception of the following

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Email discussion on further layout parameter fine tuning until May 3, 2018 (Jing, Qualcomm)

Agreement: (outcome of email discussion)

- Adopt layout as in Figure 1 with a=20 meters, b=40 meters, c=20 meters, and d=40 meters for indoor sub7GHz NR-U evaluation.

Figure 1. Indoor sub7 simulation office layout

Agreement:

- For sub7 outdoor simulation evaluation:

  - Select one of the following for the Outdoor sub-7 GHz scenario

    - Alt 1: Each operator randomly drop [1 or 2] micro-layer TRPs within each macro cell with minimum dibstance between gNBs as in NR

      - Use NR dense Urban option 1 (gNB dropped at the center of the hot-spot)

      - Independent dropping between two operators

      - Use the NR current [57.9] meters intra-operator minimum distance

      - Use [10] meters as the inter-operator minimum distance

      - UE randomly dropped within [28.9] meters within the serving cell

    - Alt 2: Drop [1 or 2 or 3] hot spots as in NR urban option 1

      - Within each hot-spot, randomly drop one gNB from each operator within a circle of radius [10] meters centered at the center of the hot-spot

        - The minimum inter-gNB distance is [10] meters

      - Within each hot-spot, drop UE within [28.9] meters from the hot-spot center

  - Parameters: Use the indoor sub7 table as baseline, with further fine tunes possible

Agreement:

- For calibration for sub-7 GHz indoor and outdoor scenarios, companies should submit for the baseline scenario:

  - Cdf of received signal power from serving cell

  - Optional: Cdf of received signal power from each of the all non-serving cells (including the cells from the other operator)

Agreement:

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- For sub-7 GHz, NR-U study the SCSs, 15/30/60KHz

  - Study performance difference between different SCS

  - Study if changes to UL design are needed to meet the PSD and OCB requirements

  - Study if an SS block design/RMSI/OSI with 60KHz SCS is needed

  - Impact on MIB and SIB1 content

  - Need for use of ECP for 60KHz

  - RACH design with 60KHz SCS in addition to options currently part of NR

  - Other considerations are not precluded.

  - Impact on support of different BWs with different SCS

- Study supporting more than one switching points within a TxOP

  - FFS the LBT requirement for each DL/UL data/control burst in the TxOP

Proposal:

- Study FBE (as in the ETSI BRAN specifications) based frame structure

Agreements:

- Study the design changes needed to support the following channels /signals in NR-U

  - PDCCH/PDSCH

  - PUCCH/PUSCH

  - PSS/SSS/PBCH

  - PRACH

  - DL and UL reference signals applicable to the operational frequency range

Agreement:

- Study possible enhancements for HARQ operation

- Study changes needed for Configured Grant support in NR-U

- Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer  multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT : RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

- Note: Other aspects are not precluded from being included

# Discussion

7.6.1	Simulation Methodology for NR-U operation

For sub7 GHz outdoor, we have the following 2 alternatives for evaluation methodology:

- Alt 1: Each operator randomly drop [1 or 2] micro-layer TRPs within each macro cell with minimum dibstance between gNBs as in NR

  - Use NR dense Urban option 1 (gNB dropped at the center of the hot-spot)

  - Independent dropping between two operators

  - Use the NR current [57.9] meters intra-operator minimum distance

  - Use [10] meters as the inter-operator minimum distance

  - UE randomly dropped within [28.9] meters within the serving cell

- Alt 2: Drop [1 or 2 or 3] hot spots as in NR urban option 1

  - Within each hot-spot, randomly drop one gNB from each operator within a circle of radius [10] meters centered at the center of the hot-spot

    - The minimum inter-gNB distance is [10] meters

  - Within each hot-spot, drop UE within [28.9] meters from the hot-spot center

From our evaluation, consider the macro layer is 200 meters site-to-site distance, the deployment is quite dense. If we drop UEs uniformly within the macro sector, with high probability the UE received RSSI will not be lower than -82dBm threshold. In that case, the UE serving RSSI distribution will be the same between alternative 1 and alternative 2. The main difference between alternative 1 and alternative 2 will be the gNB to gNB RSSI distribution, where in alternative 2, if the gNB between two operators are dropped within a small radius from the hot-spot center, the gNB to gNB RSSI will be typically high.

In the submitted papers, 7 companies support Alt 1 and 5 companies support Alt 2.

Proposal:

- For sub7 GHz outdoor scenario, adopting the following

  - Each operator randomly drop 2 micro-layer TRPs within each macro cell with minimum distance between gNBs as in NR

    - Use the NR current 57.9 meters intra-operator minimum distance

  - Use NR dense Urban option 1 (gNB dropped at the center of the hot-spot)

  - Independent dropping between two operators

    - Use 10 meters as the inter-operator minimum distance

  - UE randomly dropped in the system with a minimum serving cell RSSI of -82dBm

  - For UE to UE link

    - If one of them is indoor, add X dB penetration loss

    - If both of them are indoor, add 2X dB penetration loss

  - For gNB to gNB link

    - FFS: Ways to

Proposal:

- For sub7 GHz outdoor scenario, adopting the following

  - Macro deployment with ISD=200×A meters

  - Each operator randomly drops 1 micro-layer TRP within each macro cell sector with minimum distance between micro-layer TRPs equals 57.9×A meters

  - Independent dropping between two operators

    - Use 10×A meters as the inter-operator micro-layer TRP minimum distance

  - UE randomly dropped within macro cell sector with a minimum serving cell RSSI of -82dBm

  - All UEs dropped outdoor

  - Try A>=1 and find the A that satisfies serving cell received power distribution satisfies (10+X)% to (15+X)%] UEs below -72dBm

  - Other parameters follow the table below

  - An email discussion will be requested on calibration on parameter A and X

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

For mmW evaluations, not many contributions talk about these. However, in order to move forward on LBT discussion, we still need a baseline. The baseline needs to provide enough coverage (serving cell RSSI)

Proposal:

- mmW indoor:

  - Adopt 12+12 in a 50 meter by 120 meter room as in the following figure as the deployment model for evaluation

  - For the simulation parameters, adopt the following:

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 60GHz |
| Channel Bandwidth | 2.16GHz |
| Channel Model | InH Open office model in TR 38.901 Chapter 7.4.1 |
| BS Tx Power | 14 dBm EIRP limit for 60GHz indoor is 40dBm EIRP=Tx power + antenna gain + antenna array gain e.g. 14dBm+5dBi+10log10(8*16)~=40dBm for BS in 60GHz |
| UE Tx Power | 8 dBm 8 dBm Tx Power is proposed considering power consumption and PA efficiency. |
| BS Antenna gain | 5 dBi |
| UE Antenna gain | 5 dBi |
| BS Noise Figure | 7 dB |
| UE Receiver Noise Figure | 13 dB |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ for 60 GHz |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ for 60 GHz |

Proposal:

- mmW outdoor:

  - Randomly drop 9 TRPs per operator with a [X] meters minimum distance between TRPs

  - Independent dropping between two operators

    - Use [Z] meters as inter-operator minimum distance

  - UE randomly dropped in the system with a minimum serving cell RSSI of –[Y] dBm

7.6.2	Frame structure for NR-U operation

Most companies favour supporting SCS of 30 and 60KHz for NR-U due to reduced overhead and improved channel access.

Proposal:

- Sub7: NR-U supports SCS of 30KHz and 60Khz for sub-7 Ghz

  - FFS SCS of 15KHz

Many companies highlighted benefits of multiple switching points within a TxOP such as in reducing latency, reducing the number of HARQ processes and allowing fast CQI feedback.

Proposal:

- NR-U supports multiple switching points within TxOP:

  - FFS: Any constraints on the switches (e.g number of switches/combinations, gap between switches)

Several contributions highlighted challenges in UE detecting the start of the TxOP and the impact on UE power.

Proposal:

- Study the following techniques for start of TxOP indication:

  - PDCCH monitoring

  - Wake Up Signal

- Introduce mechanisms to indicate the TxOP structure.

Channel access mechanisms are defined in ETSI BRAN specification for two types of equipment - Load Based Equipment (LBE) and Frame Based Equipment (FBE). LBE is already being considered for the SI. FBE is an equipment where the transmit/receive structure has a periodic timing with a periodicity equal to the Fixed Frame Period within the range of 1 ms to 10 ms. Due to its simpler channel access rules, it can provide significant benefits over LBE especially when deployed in controlled environments such as for industrial automation use cases.

Proposal:

- Study FBE (as in the ETSI BRAN specifications) based frame structure

From online:

Proposal:

- Support for single and multiple DL to UL and UL to DL switching within a shared gNB COT is identified to be beneficial

  - LBT requirement to support single or multiple switching points, including

    - For gap of less than 16us, no-LBT can be used per regulation

      - Restrictions/conditions on when no-LBT option can be used will be further identified, in consideration of fair coexistence.

    - For gap of above 16us but does not exceed 25us, one-shot LBT can be used per regulation

      - Restrictions/conditions on when one-shot LBT option can be used will be further identified, in consideration of fair coexistence.

    - For single switching point, for the gap from DL transmission to UL transmission exceeds 25us, one-shot LBT is used

      - Further study needed on how many one-shot LBT attempts is allowed for granted UL transmission

    - FFS: For multiple switching points, for the gap from DL transmission to UL transmission exceeds 25us, one-shot LBT is used

Proposal:

- PDCCH monitoring procedures due to LBT constraints

Also discuss

Proposal:

- Study the following techniques for start of TxOP indication:

  - PDCCH monitoring

  - Wake Up Signal

- Introduce mechanisms to indicate the TxOP structure.

Proposal:

- Study FBE (as in the ETSI BRAN specifications) based frame structure

Proposal:

Benefit of introducing preamble in TxOP and design options

7.6.3.1	DL Signals and Channels

For the basic DL channels, NR design can be reused. However, many companies brought up the point that the FDM multiplexing of SS/PBCH block with other channels should be considered.

Proposal:

- Introduce FDM multiplexing pattern for SS/PBCH block and RMSI PDCCH/PDSCH for sub7 GHz band

  - FFS: Details

For bands with PSD limitations, typically the scheduler can handle the resource allocation to satisfy the PSD and OCB limitation. However current NR has the design that the RBG size depends on the number of RBs in a BWP, and in some cases, when BWP is wide, the RBG can be quite large.

Proposal:

- For band with PSD limitations, the NR decision on RBG size as a function of number of RBs in BWP needs  be revisited

From online:

Proposal:

- NR-U should have a discovery reference signal (DRS) that serves similar functions as in Rel-13 LAA

- The DRS design should consider the following characteristics specific to unlicensed band operation

- There are no gaps in the transmission between a single transmission of the signals in the DRS, e.g., SS/PBCH and RMSI

- The occupied channel bandwidth is satisfied (although this may not be a requirement)

- The channel occupancy time for SS/PBCH blocks and RMSI transmission is minimized

- Characteristics that may facilitate higher priority channel access, e.g.:

  - One-shot LBT with a duty cycle that in time that is small, e.g., 5%

  - High priority Cat 4 LBT

7.6.3.2	UL Signals and Channels

Agreement:

- An interlaced waveform can have benefits in some scenarios including

  - Link budget limited cases with given PSD constraint

  - As one option to efficiently meet the occupied channel bandwidth requirement.

- A waveform contiguous in frequency may be adequate in some scenarios

  - To inherit legacy contiguous allocation designs.

Note: It is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied.

Agreement:

- Support for Rel-15 NR PUCCH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that block-interlaced based PUSCH can be beneficial.

- It is beneficial to use the same interlace structure for PUCCH and PUSCH.

- The following aspects can be considered for interlace waveform based PUCCH design:

  - Flexible number of OFDM symbols

  - Flexible payload size

  - User multiplexing

  - Number of formats

Agreement:

- Support for Rel-15 NR PRACH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that interlaced based PRACH can be beneficial.

- The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

  - Interlacing based on PRB or REs

  - Targeted cell sizes

  - Targeted PRACH capacity

  - Targeted false alarm and detection rates

  - Targeted timing estimation accuracy

  - Number of formats

  - Multiplexing with other channels such as block interlaced PUCCH and PUSCH

7.6.4	Potential physical layer procedures

7.6.4.1	Channel access procedures

Agreement:

- LTE-LAA channel access mechanism is adopted as baseline for 5GHz

  - Further enhancements not precluded

- LTE-LAA channel access mechanism is adopted as starting point of the design for 6GHz

  - Further enhancements not precluded

- For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation.

  - Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence.

- No-LBT option can be applied to 6GHz band if allowed by regulation

  - Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band

Note: Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges.

Agreement:

- Initial active DL/UL BWP is approximately 20MHz for 5GHz band

  - The final value will be quantized to number of PRBs

- Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band

- FFS: Initial active DL/UL BWP for other applicable bands, including 60GHz

Proposal:

- Consider using one-shot LBT or high priority cat 4 LBT for PRACH transmission

Proposal:

- LBT considerations for PRACH, SRS, PUCCH, DRS

Proposal:

- Wide BWP with subband LBT supported

  - FFS wideband BWP

7.6.4.2	Initial access and mobility

Agreement:

The following modifications to initial access procedures are beneficial

- Modifications to initial access procedures considering limitations on access to the channel based on LBT

  - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

- Enhancement to 4-step RACH

  - Mechanisms to handle reduced msg 1/2/3/4 transmission opportunities due to LBT failure

- 2-step RACH potentially has benefit for channel access

Agreement:

Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied

Agreement:

Modifications to paging procedures due to reduced transmission opportunities for paging due to LBT failure are beneficial and should be identified and studied

7.6.4.3	Potential HARQ enhancements

Due to LBT requirements UE may miss transmitting HARQ-ACK in its assigned resource. Since gNB cannot distinguish between missed PDCCH (DL grant) and missed HARQ-ACK Tx due to LBT it will end up retransmitting the PDSCH which results in significant wastage of resources. Providing multiple opportunities for ACK feedback (in time/frequency) helps reduce the wastage.

Proposal :

- Mechanisms are introduced to support multiple time domain opportunities for HARQ-ACK feedback of the same TB

- NR-U supports configuring of muliptle PUCCH resources spanning different frequency resources and UE transmitting PUCCH in one of them based on LBT outcome

Many companies observed that due to uncertaintly in HARQ-ACK feedback it is desirable to have a single HARQ-ACK feedback for all HARQ processes. However, some concerns were rasised on the overhead of such feedback especially when considering CBG based HARQ-ACK feedback.

Proposal :

- NR-U supports group HARQ-ACK feedback potentially containing HARQ-ACK feedback for all HARQ processes

  - FFS mechanism for triggering and determining the UL resources

  - FFS mechanisms to reduce UL overhead. For example, mechanisms to indicate subset of HARQ-IDs for which feedback is requested.

In UL heavy traffic, scheduling multiple UL TTIs each with its own grant would involve having a DL portion before UL portion just for sineding control or involve frequent switching between DL and UL to accommodate the UL grants. These result in high overlead and potentially loss of medium due to LBT when switching. Multi-TTI grants help solve this issue. They were also introduced in eLAA for similar reasons.

Proposal :

- NR-U supports scheduling of multiple TTIs for PUSCH using a single UL grant

From online:

Proposal :

- Mechanisms are introduced to support multiple time domain opportunities for HARQ-ACK feedback of the same TB

- NR-U supports configuring of muliptle PUCCH resources spanning different frequency resources and UE transmitting PUCCH in one of them based on LBT outcome

Proposal :

- NR-U supports group HARQ-ACK feedback potentially containing HARQ-ACK feedback for all HARQ processes

  - FFS mechanism for triggering and determining the UL resources

  - FFS mechanisms to reduce UL overhead. For example, mechanisms to indicate subset of HARQ-IDs for which feedback is requested.

Proposal :

- NR-U supports scheduling of multiple TTIs for PUSCH using a single UL grant

7.6.4.4	Potential enhancements to configured grants

Multiple companies propose to support the LTE-AUL feature in NR-U as an enhancement to the current NR configured grant UL.

Proposal:

- LTE-LAA type AUL is supported in NR

  - AUL-UCI in PUSCH, at least include HARQ process ID, NDI, RVID

  - AUL-DFI is supported, at least include explicit A/N bits for HARQ processes configured for AUL

  - FFS: Other enhancements

From online

Proposal:

- The following modifications to the configured grant procedures are beneficial

  - Removing dependencies of HARQ process information to the timing and frequency of used resources

  - Increased flexibility in time domain for the configured grant transmissions

  - AUL retransmissions

  - Inclusion of HARQ process ID, NDI, RVID in UCI on PUSCH

  - Modifications to the activation/deactivation DCI for Type 2 configured grants

  - CBG based transmission for configured grants

# Proposal for online