3GPP TSG RAN WG1 Meeting #93			R1-1807905

Busan, Korea, May 21st – May 25th, 2018

Title:	[Draft] LS on signalling of transmitter-side DC location

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	MediaTek, [RAN1]

To:	RAN2

CC:	RAN4

Contact Person:

#### Name:	Pei-Kai Liao

E-mail Address:	pk.liao@mediatek.com

1. Overall Description:

It was noticed by RAN1 that there is an agreement regarding on Tx DC location informed from the transmitter to the receiver by a semi-static signalling. However, such signalling wasn’t implemented in RAN2’s RRC spec. yet. RAN1 kindly asks RAN2 to implement such signalling with signalling details agreed in RAN1 as follows.

- For UL, a 13-bit indicator is used to indicate the Tx DC location corresponding to a UL BWP within a carrier

  - This information is provided for each UL BWP configuration

  - It’s also RAN1’s understanding that this information can be provided in RRCReconfigurationComplete from a UE to the network when a UL BWP is configured

  - 1 bit is used to indicate whether there is 7.5 KHz shift

    - “0” means no 7.5 KHz shift

    - “1” means 7.5 KHz shift

  - 12 bits are used to indicate the Tx DC location with the following table

| Subcarrier index (12 bits) within the carrier corresponding to the numerology of the UL BWP | Tx DC location |
| --- | --- |
| 0 | 0 |
| ⁞ | ⁞ |
| 3299 | 3299 |
| 3300 | Outside the carrier |
| 3301 | Undetermined position within the carrier |
| 3302 | Reserved |
| 3303 | Reserved |
| ⁞ | Reserved |
| 4095 | Reserved |

- For DL, 12-bit indicator is used to indicate the Tx DC location corresponding to a numerology within a carrier

  - This information is provided for each numerology potentially used for the carrier

  - It’s RAN1’s understanding that this information can be optionally provided in SCS-SpecificCarrier from the network to a UE

  - It’s RAN1’s understanding that the UE can assume that Tx DC location is outside the carrier if this information is not provided – up to RAN4 to handle performance related issues

  - 12 bits are used to indicate the Tx DC location with the following table

| Subcarrier index (12 bits) within the carrier corresponding to a numerology | Tx DC location |
| --- | --- |
| 0 | 0 |
| ⁞ | ⁞ |
| 3299 | 3299 |
| 3300 | Outside the carrier |
| 3301 | Reserved |
| 3302 | Reserved |
| 3303 | Reserved |
| ⁞ | Reserved |
| 4095 | Reserved |

2. Actions:

To RAN2 group

ACTION:

RAN1 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next RAN1 Meetings:

RAN1#94	20th – 24th August 2018	Gothenburg, Sweden

RAN1#94bis	8th – 12th October 2018	Chengdu, China