3GPP TSG RAN WG1 Meeting #94		R1-1808187

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.7.2

Source:	ZTE

Title:	Consideration on Evaluation of Mobility of IMT-2020 self-evaluation

Document for:	Discussion

Introduction

There is an approved Study Item on Self Evaluation towards IMT-2020 submission in the RAN#75 meeting, which mainly focuses on evaluated RAN technologies based on Rel-15 and beyond to satisfy all ITU-R IMT-2020 requirements including eMBB scenario.

At RAN#77 meeting, the general work plan of self evaluation is approved [1]. Moreover, “[ITU-R AH 01] Calibration for self-evaluation”[2] has been set up for discussion on calibration and collection of calibration results. It is observed that the calibration results are well aligned according to the results collected so far for InH_x, UMa_x and RMa_x in all IMT-2020 defined test environments.

Moreover, At RAN1 #92, some features of system-level simulation in eMBB have been approved. In this contribution, some evaluation results on Mobility are listed for part of UL system-level simulation.

Evaluation results for Mobility

System-level simulation

Based on [3][4], we simulated uplink SINR CDF distribution for obtaining values of 50 percentile CDF by system-level simulation in all three eMBB test environments. Moreover, we also evaluated the influence of power control parameters to 50 percentile point of uplink SINR CDF distribution in each test environment.

More importantly, it is necessary to keep inconsistency system-level configuration between Spectrum efficiency and mobility, although ITU-R does not mentioned it. The advantage is more reasonable self-evaluation results in eMBB usage scenario.

Proposal 1: For mobility system-level simulation, related configuration(s) should keep same with other eMBB evaluation by system simulation.

- Indoor hotspot-eMBB test environment

Figure 2-1 the Uplink SINR distribution of Indoor hotspot-eMBB test environment

| Test environment | Antenna pattern | Antenna configuration for pre-processing SINR | Power control parameters | 50% CDF (dB) |
| --- | --- | --- | --- | --- |
| Indoor Hotspot–eMBB Config A | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) (4,4,2,1,1) | 8TXRU (Mp,Np,P,Mg,Ng) (4,1,2,1,1) | <P0= -106dBm, Alpha=1> | 7.65 |
|  |  |  | <P0= -81dBm, Alpha=0.8> | 11.82 |
|  |  |  | <P0= -60dBm, Alpha=0.6> | 12.40 |

Table 2-1 UL SINR distribution of Indoor hotspot-eMBB test environment

Observation 1: Applying about 12.4dB to save the respective 50th-percentile SINR value in the indoor hotspot-eMBB test environment.

Proposal 2: For indoor hotspot-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

- Dense urban-eMBB test environment

Figure 2-2 the Uplink SINR distribution of Dense Urban-eMBB test environment

| Test environment | Antenna pattern | Antenna configuration for pre-processing SINR | Power control parameters | 50% CDF (dB) |
| --- | --- | --- | --- | --- |
| Dense Urban–eMBB Config A | 192Rx cross-polarized antenna (M,N,P,Mg,Ng) (12,8,2,1,1) | 8TXRU (Mp,Np,P,Mg,Ng) (4,1,2,1,1) | <P0= -106dBm, Alpha=1> | 9.55 |
|  |  |  | <P0= -81dBm, Alpha=0.8> | 11.37 |
|  |  |  | <P0= -60dBm, Alpha=0.6> | 13.20 |

Table 2-2 UL SINR distribution of Dense Urban-eMBB test environment

Observation 2: Applying about 13.2dB to save the respective 50th-percentile SINR value in the Dense Urban-eMBB test environment.

Proposal 3: For Dense Urban-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

- Rural-eMBB test environment

Figure 2-3 the Uplink SINR distribution of Rural-eMBB ConfigA test environment

Figure 2-4 the Uplink SINR distribution of Rural-eMBB ConfigB test environment

| Test environment | Antenna pattern | Antenna configuration for pre-processing SINR | Power control parameters | 50% CDF (dB) |
| --- | --- | --- | --- | --- |
| Rural–eMBB Config A | 16Rx cross-polarized antenna (M,N,P,Mg,Ng) (8,1,2,1,1) | 2TXRU (Mp,Np,P,Mg,Ng) (1,1,2,1,1) | <P0= -106dBm, Alpha=1> | 6.00 |
|  |  |  | <P0= -81dBm, Alpha=0.8> | 8.01 |
|  |  |  | <P0= -60dBm, Alpha=0.6> | 9.15 |
| Rural–eMBB Config B | 192Rx cross-polarized antenna (M,N,P,Mg,Ng) (12,8,2,1,1) | 8TXRU (Mp,Np,P,Mg,Ng) (4,1,2,1,1) | <P0= -106dBm, Alpha=1> | 9.86 |
|  |  |  | <P0= -81dBm, Alpha=0.8> | 10.91 |
|  |  |  | <P0= -60dBm, Alpha=0.6> | 11.07 |

Table 2-3 UL SINR distribution of Rural-eMBB test environment

Observation 3: Applying about 9.15dB to save the respective 50th-percentile SINR value in the Rural-eMBB ConfigA test environment.

Observation 4: Applying about 11.07dB to save the respective 50th-percentile SINR value in the Rural-eMBB ConfigB test environment.

Proposal 4: For Rural-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

- The mean value of ZoD spread

Based on UE dropping in system level simulation，we also obtain the CDF of the mean value of ZoD spread for both Dense Urban– eMBB and Rural – eMBB usage scenario for UL mobility, which can be found below table, related simulation assumption also can be found the attachment.

Table 2-4 ZoD spread for all three test environments

| Test environment | mean value of LOS ZoD spread |  | mean value of NLOS ZoD spread |  |
| --- | --- | --- | --- | --- |
| Channel model | a | b | a | b |
| Dense urban–eMBB Config A | 3.055 | 3.06 | 4.25 | 4.31 |
| Rural–eMBB Config A | 1.26 | 1.264 | 1.41 | 1.42 |
| Indoor-eMBB Config A | 1.02 |  | 1.08 |  |

Observation 5: 1.02degree/ and 1.08degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in both model a and model b of Indoor-eMBB with 4GHz

Observation 6: 3.05/3.06degree/ and 4.25/4.31degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in model a/b of Dense urban-eMBB with 4GHz

Observation 7: 1.26/1.264 degree/ and 1.41/1.42degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in model a/b of Rural-eMBB with 700MHz

Conclusion

In this document, we provide our considerations on both Mobility and Reliability of evaluation towards IMT-2020 submission.

Observation 1: Applying about 12.4dB to save the respective 50th-percentile SINR value in the indoor hotspot-eMBB test environment.

Observation 2: Applying about 13.2dB to save the respective 50th-percentile SINR value in the Dense Urban-eMBB test environment.

Observation 3: Applying about 9.15dB to save the respective 50th-percentile SINR value in the Rural-eMBB ConfigA test environment.

Observation 4: Applying about 11.07dB to save the respective 50th-percentile SINR value in the Rural-eMBB ConfigB test environment.

Observation 5: 1.02degree/ and 1.08degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in both model a and model b of Indoor-eMBB with 4GHz

Observation 6: 3.05/3.06degree/ and 4.25/4.31degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in model a/b of Dense urban-eMBB with 4GHz

Observation 7: 1.26/1.264 degree/ and 1.41/1.42degree are 50%-tile point of LOS CDF, and NLOS CDF, respectively, in model a/b of Rural-eMBB with 700MHz

Proposal 1: For mobility system-level simulation, related configuration(s) should keep same with other eMBB evaluation by system simulation.

Proposal 2: For indoor hotspot-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

Proposal 3: For Dense Urban-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

Proposal 4: For Rural-eMBB test environment, P0= -60dBm, Alpha=0.6 is appreciated for evaluated Mobility in the part of system-level simulation.

References

[1] RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al.

[2] Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

[3] R1-1804029, “Consideration on Evaluation methodology of IMT-2020 self-evaluation”, RAN1 #92bis,ZTE, Sanechips, Sanya

[4] R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, RAN1 #92bis, Huawei, Sanechips, Sanya

- Appendix

Table A-1 UL system level configuration parameters for Mobility

| Parameter | Value | Value | Value |  |
| --- | --- | --- | --- | --- |
| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB |  |
| Evaluation configuration | Configuration A | Configuration A | Configuration A/B |  |
| Channel model | InH_B | UMa_B | RMa_B |  |
| Subcarrier spacing | 30 kHz | 30 kHz | 30 kHz |  |
| Symbols number per slot | 14 | 14 | 14 |  |
| ISD | 20 m | 200 m | 1732 m |  |
| Carrier Frequency | 4 GHz | 4 GHz | 700 MHz/4GHz |  |
| Total transmit power per TRxP | 21 dBm for 10MHz bandwdith | 41 dBm for 10MHz bandwdith | 46 dBm for 10MHz bandwdith |  |
| Mechanic tilt | 180deg in GCS (pointing to the ground) | 90 deg in GCS (pointing to the horizontal direction) | 90 deg in GCS (pointing to the horizontal direction) |  |
| Electronic tilt | 90deg in LCS | 105 deg in LCS | 100 deg in LCS |  |
| UT attachment | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 |  |
| Number of antenna elements per TRxP | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) (4,4,2,1,1) | 192Rx cross-polarized antenna (M,N,P,Mg,Ng) (12,8,2,1,1) | 16Rx cross-polarized antenna (M,N,P,Mg,Ng)= (8,1,2,1,1) | 192Rx cross-polarized antenna (M,N,P,Mg,Ng)= (12,8,2,1,1) |
| Number of RXRU per TRxP | 8RXRU (Mp,Np,P,Mg,Ng) (4,1,2,1,1) | 8RXRU (Mp,Np,P,Mg,Ng) (4,1,2,1,1) | 2RXRU (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) | 8RXRU (Mp,Np,P,Mg,Ng) = (4,1,2,1,1) |
| Number of antenna elements per UE | 2Tx with 0°,90° polarization | 2Tx with 0°,90° polarization | 2Tx with 0°,90° polarization |  |
| Device deployment | 100% indoor | 80% indoor, 20% outdoor (in car) | 50% indoor, 50% outdoor (in car) |  |
|  | Randomly and uniformly distributed over the area | Randomly and uniformly distributed over the area under Macro layer | Randomly and uniformly distributed over the area |  |
| UE speeds of interest | 10 km/h | 30 km/h | 120 km/h |  |
| BS noise figure | 5 dB | 5 dB | 5 dB |  |
| UE noise figure | 7 dB | 7 dB | 7 dB |  |
| UE power class | 23 dBm | 23 dBm | 23 dBm |  |
| UL PC parameter | P0= -106，Alpha=1 P0= -81，Alpha=0.8 P0= -60，Alpha=0.6 | P0= -106，Alpha=1 P0= -81，Alpha=0.8 P0= -60，Alpha=0.6 | P0= -106，Alpha=1 P0= -81，Alpha=0.8 P0= -60，Alpha=0.6 |  |