3GPP TSG RAN WG1 Meeting #94	R1-1808792

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.2.7.2

Source: 	Samsung

Title: 	Evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation

Document for:	Discussion and Decision

# Introduction

In RAN#77 [1], work plan to provide self-evaluation against eMBB, mMTC and URLLC test enviroments/KPIs is agreed. Based on the work plan, details on evaluation assumptions including details such as power back-off model and overhead modelling has been continuously discussed until RAN1#93. In this contribution, evaluation results of Indoor Hotspot-eMBB for self-evaluation are provided.

# Evaluation methodology and requirement

In order to evaluate the spectral efficiency using system-level simulation and area traffic capacity, following methodology for area traffic capacity are provided in [1].

Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). 
The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6).

Carea = ρ × W × SEavg	(6)

The target value for Area traffic capacity in downlink is 10 Mbit/s/m2 in the Indoor Hotspot – eMBB test environment.

The minimum requirement for the 5th percentile user spectral efficiency and average spectral efficiency for each scenario is provided as follow [1].

Table 1. 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) |
| --- | --- |
| Indoor Hotspot – eMBB | 0.3 |
| Dense Urban-eMBB | 0.225 |
| Rural-eMBB | 0.12 |

Table 2. Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) |
| --- | --- |
| Indoor Hotspot – eMBB | 9 |
| Dense Urban-eMBB | 7.8 |
| Rural-eMBB | 3.3 |

# Evaluation results

For Indoor Hotspot-eNBB evaluation, evaluation assumptions for self-evaluation are agreed in RAN1#92 [2]. Based on the agreed evaluation assumption, further clarifications on parameters such as PRB bundling, coherent channel bandwidth and scaling factor of guard band ratio are made in RAN1#93 [3]. Additionally, detailed parameters for multi-band and/or multi-layer, power-back off model and PDCCH overhead modelling for larger bandwidth are agreed in this meeting. In order to provide evaluation results for self-evaluation, detailed evaluation assumptions are decoded based on the agreed evaluation assumptions and provided in Appendix. For purpose of comparison, minimum performance requirements for Indoor Hotspot-eNBB are provided in Table 3.

Table 3. Minimum performance requirements for Indoor Hotspot-eNBB (4GHz)

| Performance Metric | Requirement (DL) | Evaluation result (DL) |
| --- | --- | --- |
| Average Spectral Efficiency (bits/Hz/TRxP) | 9 | 13.16 |
| 5% user spectral efficiency for downlink (bits/Hz/UE) | 0.3 | 0.33 |

Table 4 provides the area traffic capacity for Indoor Hotspot -eMBB. For area traffic capacity, TRxP density (TRxP/m2) needs to be calculated. For Indoor Hotspot-eMBB test environment, a building where TRxPs are located has 120m x 50m = 6000m2. Therefore, for a TRxP per site (i.e. total 12 TRxP) the ρ is calculated as 12/6000 = 0.002 (TRxP/m2). The area traffic capacity for Indoor Hotspot-eMBB is provided in Table 4.

Table 4. Area traffic capacity for Indoor Hotspot -eMBB (4GHz)

| Bandwidth (MHz) | 12 TRxP | Requirement (Mbit/s/m2) |
| --- | --- | --- |
| 350 | 9.21 | 10 |
| 400 | 10.53 |  |
| 450 | 11.84 |  |

Based on the evaluation results, we draw following observation:

Observation:

- For Indoor Hotspot-eMBB, NR achieves minimum performance requirements of average TRP and 5% UE spectral efficiency for ITU-R self-evaluation.

- For Indoor Hotspot-eMBB, NR achieves minimum performance requirements of area traffic capacity for ITU-R self-evaluation if the bandwidth is larger than 400MHz.

# Conclusion

In this contribution, performance requirements and evaluation results with detailed evaluation assumptions are provided. Based on the evaluation results, we draw following observation:

Observation:

- For Indoor Hotspot-eMBB, NR achieves minimum performance requirements of average TRP and 5% UE spectral efficiency for ITU-R self-evaluation.

- For Indoor Hotspot-eMBB, NR achieves minimum performance requirements of area traffic capacity for ITU-R self-evaluation if the bandwidth is larger than 400MHz.

# References

- Guidelines for evaluation of radio interface technologies for IMT-2020[IMT-2020.EVAL], Working Party 5D, ITU

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei

- R1-1807758, “Summary on offline discussion on eMBB evaluation method and parameters for IMT-2020 self-evaluation”, Huawei.

# Appendix

Table 2. Evaluation assumptions for DL Indoor Hotspot-eMBB scenario

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | FDD |
| Carrier Frequency |  | 4 GHz |
| System BW |  | 10MHz |
| Subcarrier spacing |  | 15 kHz |
| Symbols number per slot |  | 14 |
| BS Antenna Configuration |  | 12site: 32 TXRUs (M,N,P,Mg,Ng, Mp, Np) = (4,4,2,1,1,4,4) |
| Number of antenna elements per UE |  | 4Rx with 0°,90° polarization |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-MIMO with PF |
| ACK/NACK delay |  | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 12 |
| Guard band ratio |  | 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay |
|  |  | NR Type II CSI |
| Overhead | PDCCH | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | 32 ports per 5 slots |
|  | SSB | 1 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20 ms, 1 port, 50 PRB |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |