3GPP TSG-RAN WG1 Meeting #94	R1-1808863

Gothenburg, Sweden, 20th - 24th August 2018

Agenda item:		7.2.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On Evaluation of the Mobility Parameter for IMT-2020

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we discuss the evaluation of the mobility performance indicator for the self-evaluation of NR for IMT-2020 and provide complementary performance results to our contribution in 3GPP RAN1#92bis [1].  Table 1 shows minimum requirements for spectra efficiency in each of the IMT-2020 evaluation environments [2].  The residual BLER requirement is below 1%.  This contribution is an updated version of R1-1807283.

Table 1.  Spectral Efficiency Requirements in IMT-2020 for Mobility evaluations.

| Environment | UE Speed | SE [bit/s/Hz] |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 10 km/h | 1.5 |
| Dense Urban – eMBB | 30 km/h | 1.12 |
| Rural – eMBB | 120 km/h 500km/h | 0.8 0.45 |

# 2	Discussion of Evaluation Methods

For the IMT-2020 self-evaluation of NR, the mobility performance indicator will be evaluated for the Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments [2].  The evaluation methodology described by ITU-R in [3] provides a 5 step process for evaluating the mobility in these three environments.  A simplified version of these steps follows :

- Run uplink system simulations using the same configurations used for average and 5th percentile spectral efficiency simulations, but using the appropriate speeds specified for mobility evaluation.  From these simulations, obtain the CDF of the uplink SINR.

- Extract the 50th percentile SINR values from the SINR CDF’s.

- Run link simulations to obtain the link data rate and residual packet error rates as a function of SINR, covering the 50th percentile values extracted in step 2.

- Calculate the uplink spectral efficiencies at the 50th percentile SINR values and compare with the requirements in [2].

- The requirements are met if the calculated uplink spectral efficiency is greater than the requirement and the residual packet error rate is less than 1%.

The evaluation methodology does not specify the exact SINR to use in the connection between the system and link level simulations.  In [4], the preprocessing SINR was derived and considered to be used for this connection.  This contribution also included a CDF for the Rural – eMBB environment at 700 MHz and 120 km/h outdoor users.  In [5], link level results were presented to indicate the required SINR’s for Rural – eMBB at 700 MHz and 4GHz for speeds of 120 km/h and 500 km/h, but without the corresponding system-level SINR CDF’s.

The use of the pre-processing SINR as proposed in [4] raises concerns, particularly about its applicability to UL MU-MIMO.  Since the link-level simulation simulates a single user [3], while the pre-processing SINR captures interference from other paired users, it does not take into account the effects of receiver processing of that interference.  In addition, the interference seen on DM-RS symbols is different than the interference on data symbols, but the preprocessing SINR treats the interference the same in the link simulation context.  Furthermore, use of the pre-processing SINR requires the link simulation to perform link adaptation to set the rank and precoding for the UE being simulated.

Observation 1:  Using the pre-processing SINR with MU-MIMO transmission does not properly account for interference experienced with MU-MIMO.

If the post-processing SINR is used in the system level CDF’s, the effect of processed MU-MIMO cross-talk is already included in the SINR.  The UE simulated at link level will then use rank 1, assuming the post-processing SINR has been collected per data layer.  In addition, the link simulation will be SISO since the effects of the antenna arrays have been taken into account during the system level simulation.  The resulting throughput is then scaled by the average number of transmission layers.

For both SINR’s, the effect of the channel model, HARQ, and noise is included in the link simulation.

Proposal 1:  Use the post-procesing SINR distribution as the interface between the system and link simulations for evaluating the mobility performance.

# 3	Results

Using the above methodology, we performed system simulations for the three eMBB test environments using the configurations defined in [3] and the simulation parameters shown in Table 3.  The indoor  and dense urban simulations were performed at the required UE speeds of 10 km/h and 30 km/h (for outdoor users), respectively.  The rural simulations were performed using both 120 km/h and 500 km/h (for outdoor users). The post-processing SINR distributions are shown in Figure 1 through Figure 3 for the three environments. Step 2 above indicates that the 50th percentile SINR is required as input to the link simulations. The extracted 50th percentile SINR values are tabulated in Table 2.

Observation 2:  The Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments have post-processing SINR 50th percentile values of 7.08 dB (10 km/h), 7.93 dB (30 km/h), 6.96 dB (120km/h) and 6.03 dB (500 km/h), respectively.

Figure 1.  Indoor Hotspot – eMBB post-processing SINR distribution at 10 km/h (4 GHz carrier frequency).

Figure 2.  Dense Urban – eMBB post-processing SINR distribution at 30 km/h (4 GHz carrier frequency).

Figure 3.  Rural – eMBB post-processing SINR distribution for 700 MHz and 4 GHz carrier frequencies.

In Step 3 and Step 4, link simulations are performed according to simulation assumptions shown in Table 4. Spectral efficiency (SE) values from link simulations for each scenario are included in Table 2 with the corresponding residual BLER. The link simulations are conducted with the FDD duplex mode. The link spectral efficiency values in the FDD case include two symbols overhead due to DMRS and one symbol additional overhead for control in the Indoor Hotspot-eMBB and Dense Urban-eMBB scenarios. In the Rural-eMBB scenario, it is assumed that two double-symbol DMRSs’ are needed to support MU-MIMO transmission up to 12 layers with high mobility. Furthermore one symbol additional overhead for control is assumed also in the Rural-eMBB scenario. Thus, in the FDD case 11 symbols out of 14 symbols are available for data transmission in the Indoor Hotspot-eMBB and Dense Urban-eMBB scenarios, while 9 symbols out of 14 symbols is available in the Rural-eMBB scenario. Link spectral efficiency values for the TDD case are calculated assuming that only 9 symbols out of 14 symbols are available for data transmission in the Indoor Hotspot-eMBB and Dense Urban-eMBB scenarios, while only 7 symbols out of 14 symbols are available for data transmission in the Rural-eMBB scenario.

In Step 5 we compare Spectral Efficiency values from Table 2 against the Table 1 requirements and we can conclude that NR meets requirements in all evaluated environments, in both FDD and TDD modes. Furthermore it is noted that the residual BLER is well below 1%.

Proposal 2:  IMT-2020 requirements for spectral efficiency, and residual BLER are met in both FDD and TDD with the Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments.

Table 2.  Extracted 50th Percentile SINR , Link SE, and Residual BLER Values.

| Environment | UE Speed | 50th Percentile SINR | Link SE [bit/s/Hz] |  | Residual BLER [%] |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |  |
| Indoor Hotspot – eMBB | 10 km/h | 7.08 dB | 1.84 | 1.50 | 0.2 |
| Dense Urban – eMBB | 30 km/h | 7.93 dB | 1.85 | 1.52 | < 0.1 |
| Rural – eMBB | 120 km/h 500km/h | 6.96 dB 6.03 dB | 1.02 0.87 | 0.80 0.68 | 0.1 0.1 |

# 4	Conclusions

In this contribution, we have discussed the evaluation methods to use for evaluating the mobility performance indicator for the IMT-2020 self-evaluation of NR and provided simulation results using the proposed methods.  The observations and proposals in this contribution may be summarized as:

Observation 1:  Using the pre-processing SINR with MU-MIMO transmission does not properly account for interference experienced with MU-MIMO.

Proposal 1:  Use the post-procesing SINR distribution as the interface between the system and link simulations for evaluating the mobility performance.

Observation 2:  The Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments have post-processing SINR 50th percentile values of 7.08 dB (10 km/h), 7.93 dB (30 km/h), 6.96 dB (120 km/h) and 6.03 dB (500 km/h), respectively.

Proposal 2:  IMT-2020 requirements for spectral efficiency and residual BLER are met in FDD and TDD with the Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments.

# References

- R1-1805276, “NR Mobility Self-Evaluation for IMT-2020”, Nokia, Nokia Shanghai Bell , RAN1#92bis, Sanya, China, 16th - 20th April 2018.

- “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” Report ITU-R M.2410‑0, November 2017.

- “Guidelines for evaluation of radio interface technologies for IMT-2020,” Report ITU-R M.2412-0, October 2017.

- R1-1801793, “Consideration on self evaluation of IMT-2020 for high mobility”, Huawei, RAN1#92, Athens, Greece, 26 February-2 March, 2018.

- R1-1802952, “IMT-2020 self-evaluation: Mobility evaluations for NR”, Ericsson, RAN1#92, Athens, Greece, 26 February-2 March, 2018.

# Appendix

Table 3.  System Simulation Parameters

| Parameter | Value |
| --- | --- |
| Environments | - Indoor Hotspot – eMBB, Configuration A (4GHz), Channel model A, 12 TRxP’s - Dense Urban – eMBB, Configuration A (4GHz), Channel model A - Rural – eMBB, Configuration B (4GHz/1732m), Channel model A - Rural – eMBB, Configuration A (700MHz/1732m), Channel model A |
| System bandwidth | 10MHz |
| Subcarrier spacing | 15kHz |
| Duplex mode | FDD |
| Traffic model | Full buffer |
| UE transmit power | 23 dBm |
| gNB antenna element configuration | - (M,N,P) = (8,4,2). (dV,dH) = ( 0.5, 0.5 ) λ, 180° mechanical tilt in GCS - (M,N,P) = (16,8,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt in GCS - (M,N,P) = (8,16,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt in GCS - (M,N,P) = (4,8,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt in GCS +45°, -45° polarization in all cases |
| gNB antenna virtualization | - (M,N,P) = (8,4,2), Electrical tilt = 90° in LCS - (M,N,P) = (4,8,2), Electrical tilt = 100° in LCS - (M,N,P) = (2,16,2), Electrical tilt = 100° in LCS - (M,N,P) = (4,8,2), Electrical tilt = 90° in LCS |
| UE antenna config. | (M,N,P) = (1,2,2), dH = 0.5 λ, 0°, 90° polarization for all environments |
| UE antenna pattern | Omni |
| UL power control | α=0.8 and - P0=-101 dBm - P0=-95 dBm - P0=-95 dBm - P0=-95 dBm |
| Receiver | MMSE with channel estimation error and interference modeling. Indoor Hotspot channel estimation was ideal. |
| CSI feedback | Codebook-based transmission, CSI feedback period is 10ms, CSI feedback delay is 5 ms |
| MIMO transmission scheme | MU-MIMO with maximum UE rank of 2 and maximum number of paired users is 6 (Indoor, Rural) or 3 (Dense Urban) |
| Scheduler | PF with wideband scheduling |

Table 4.  Link Simulation Parameters

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz (52RBs with 15kHz, 24RBs with 30kHz) |
| Channel model | Indoor Hotspot - eMBB: TDL-i 39ns, 10 km/h Dense Urban – eMBB: TDL-iii 363ns, 30 km/h Rural – eMBB: TDL-iii 37ns, 120 km/h Rural – eMBB: TDL-iii 37ns, 500km/h |
| Subcarrier spacing | 15kHz (700MHz), 30kHz (4GHz) |
| Waveform | CP-OFDMA |
| Duplex mode | FDD |
| gNB antenna element configuration | 1 RX antenna |
| UE antenna config. | 1 TX antenna |
| Channel coder | LDPC |
| Receiver | MMSE with channel estimation error |
| HARQ | max. 4 HARQ transmissions |
| Link adaptation | yes |
| Rank | 1 |
| DMRS | 2 symbols, config. 1 with 3dB power boost |
| Additional overhead | 1 symbol |