3GPP TSG RAN WG1 Meeting #94	R1-1809115

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Sharp

Title:	Initial evaluation results for IMT2020 self-evaluation Reliability

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Background

In RAN#77, the following steps have been agreed for self-evaluation toward ITU submission [1].

- The following work plan is proposed according to the agreed IMT-2020 submission timeplan - Step 1: From Sep 2017 to Dec 2017, discussions in RAN ITU-R Ad-Hoc - Calibration for self evaluation - Prepare and finalize initial description template information that is to be submitted to ITU-R WP 5D#29. - Step 2: From early 2018 to Sep 2018, targeting “update & self eval” submission in Sep 2018 - Performance evaluation against eMBB, mMTC and URLLC requirements and test environments for NR and LTE features. - Update description template and prepare compliance template according to self evaluation results. - Provide description template, compliance template, and self evaluation results based on Rel-15 in Sep 2018. - Step 3: From Sep 2018 to June 2019, targeting “Final” submission in June 2019 - Performance evaluation update by taking into account Rel-16 updates in addition to Rel-15 - Update description template and compliance template to take into account Rel-16 updates in addition to Rel-15 - Provide description template, compliance template, and self evaluation results based on Rel-15 and Rel-16 in June 2019.

The planned deadline of step2 evaluations is approaching. In this contribution, we show our initial evaluation results of URLLC.

Discussion

System level evaluation

According to [2], 5th percentile geometry for DL and UL is evaluated first by the system level evaluation. To calculate geometry for DL and UL, the following agreement was made [3] at RAN1#93 meeting.

Agreements: - The proposals in Section 2 of R1-1807760 are agreed with the following update: - “…pre-processing SINR is encouraged to be used in system….”

We use the pre-precessing SINR agreed in RAN1#93 as geometry.

Table 1 shows 5th percentile pre-processing SINR values both for DL and UL and each configuration. The evaluation assumptions are listed in Tables A1 and A2 in annex, and C.D.F.s of DL and UL geometry is shown in Figures B1 – B4.

- Table 1. 5th percentile pre-processing SINR values

|  | UMa_A channel model |  | UMa_B channel model |  |
| --- | --- | --- | --- | --- |
|  | Config. A | Config. B | Config. A | Config. B |
| DL [dB] | -2.84 | -2.65 | -2.61 | -2.62 |
| UL [dB] | 1.151 | 0.4415 | 0.6115 | 0.4718 |

Link level evaluation

Channel design to satisfy the requirement.

According to [4], the following requirements are required for the reliability.

## 4.10 Reliability ## Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. ## Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. ## This requirement is defined for the purpose of evaluation in the URLLC usage scenario. The minimum requirement for the reliability is 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-URLLC test environment, assuming small application data (e.g. 20 bytes application data + protocol overhead).

We assume the following structure for DL and UL, and Figure 1 shows the structures and timelines for DL and UL respectively

Figure 1: Transmission timeline

DL (Figure 1 (a)):

- SCS: 30 kHz

- PDCCH: 1-OFDM symbol CORESET, AL = 8

- PDSCH: 4-OFDM symbols (Each transmission is assigned by a PDCCH)

- PUCCH: 2-OFDM symbol with frequency hopping to transmit HARQ-ACK bit for the first transmission

- 5-OFDM symbols PDSCH processing time (according to PDSCH processing capability 2 of Table 5.3-2 in [5]) and 6-OFDM symbols PDSCH preparation time

UL (Figure 1 (b)):

- SCS: 30 kHz

- PDCCH: 1-OFDM symbol CORESET, AL = 8

- PUSCH: 7-OFDM symbols (First transmission is scheduled by configured grant and retransmission is scheduled by a PDCCH)

- 5-OFDM symbols PUSCH processing time and 6-OFDM symbols PUSCH preparation time (according to PUSCH timing capability 2 of Table 6.4-2 [5])

Link level evaluation results

Table 2 shows link level performance for PDCCH, PDSCH, PUCCH, and PUSCH under 5 percentile pre-processing SINR values for each configuration. For PDSCH and PUSCH, up to two HARQ retransmission is considered. Table A3 to A6 shows evaluation assumptions for PDCCH, PDSCH, PUCCH, and PUSCH. Figure B5 to B8 shows link level performance curve for PDCCH, PDSCH, PUCCH, and PUSCH.

- Table 2. Link level performance under 5th percentile pre-processing SINR for each configuration

|  | UMa_A channel model |  | UMa_B channel model |  |
| --- | --- | --- | --- | --- |
|  | Config. A | Config. B | Config. A | Config. B |
| PDCCH | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| PDSCH | 0.00055 | 0.00030 | 0.00026 | 0.00028 |
| PUCCH (NACK to ACK ) | 0.000573 | 0.000573 | 0.000573 | 0.000573 |
| PUCCH (NACK to DTX ) | 0.009745 | 0.016624 | 0.015191 | 0.015477 |
| PUSCH | 0.000002 | 0.00003 | 0.00004 | 0.00003 |

Reliability evaluation results

To obtain the final results, the following formula can be used for DL reliability evaluation [3].

pt=n=1Ki=1nn-1n-i1-p1p4n-ip1p2,ij=1i-1p1p31-p2,j

Where pt is total success rate, p1 is PDCCH success probability, p2,x is PDSCH/PUSCH success probability with x transmissions, p3 is PUCCH NACK detection success probability, p4 is PUCCH DTX detection success probability, and K is the maximum number of PDSCH/PUSCH transmissions.

On the other hand, the following formula can be used for UL reliability evaluation [3].

pt=p2,M+(1-p2,M)k=2Kp1p2,kNi=2k-1(1-p1p2,iN)

In our reliability evaluation, soft-combining is not assumed. Thus, PDSCH/PUSCH success probability p2,x = p2,1 for x = 1, 2.

Table 3 shows the reliability evaluation results for each configuration assuming 2 PDSCH/PUSCH transmissions. According to Table 3, followings are observed.

- Table 3. Reliability evaluation results

|  | UMa A |  | UMa B |  |
| --- | --- | --- | --- | --- |
|  | Config.A | Config.B | Config.A | Config.B |
| DL | 8.35E-07 | 6.07E-07 | 5.31E-07 | 5.6E-07 |
| UL | 4.40E-11 | 1.50E-09 | 2.40E-09 | 1.50E-09 |

Observation 1: NR downlink satisfies the reliability requirement in case of 1-symbol PDCCH with AL=8, 4-OFDM symbol PDSCH with up to two transmissions and 2-OFDM symbol short PUCCH.

Observation 2: NR uplink satisfies the reliability requirement in case of 1-symbol PDCCH with AL=8, 7-OFDM symbol PUSCH with up to two transmissions.

Conclusion

In this contribution, we have the following observations:

Observation 1: NR downlink satisfies the reliability requirement in case of 1-symbol PDCCH with AL=8, 4-OFDM symbol PDSCH with up to two transmissions and 2-OFDM symbol short PUCCH.

Observation 2: NR uplink satisfies the reliability requirement in case of 1-symbol PDCCH with AL=8, 7-OFDM symbol PUSCH with up to two transmissions.

References

- RP-172101, “WF on Work plan of Self-Evaluation SI,” Sapporo, Japan, Sept. 2017.

- ITU-R M2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Oct. 2017

- Chairman’s note RAN1 #93, May 2018

- ITU-R M2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Oct. 2017

- 3GPP TS38.214V15.2.0, June 2018

- 3GPP TR36.873V12.7.0, “Study on 3D channel model for LTE”, Jan., 2018

Annex A: Evaluation assumptions

- Table A1. System level evaluation assumptions both for DL and UL

|  | - Configuration A | - Configuration B |
| --- | --- | --- |
| - Cell layout | - Hexagonal grid layout - 19 sites with 3-sectored TRP | - Hexagonal grid layout - 19 sites with 3-sectored TRP |
| - Wrap around model | - Geographical distance based | - Geographical distance based |
| - Carrier frequency | - 4 GHz | - 700 MHz |
| - Simulation bandwidth | - 10 MHz | - 10 MHz |
| - Transmit power per TRP | - 46 dBm | - 46 dBm |
| - Inter-site distance | - 500 m | - 500 m |
| - BS antenna height | - 25 m | - 25 m |
| - Number of antenna elements per TRP | - 64 Tx/Rx - (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.8 λ) - Polarization: 45, −45 (degree) | - 16 Tx/Rx - (M, N, P, Mg, Ng) = (8, 1, 2, 1, 1) - (dH, dV) = (N/A, 0.8 λ) - Polarization: 45, −45 (degree) |
| - Number of TXRUs per TRP | - 8 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 4, 2, 1, 1) | - 2 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 1) |
| - BS antenna gain | - 8 dBi | - 8 dBi |
| - BS antenna radiation pattern | - Table 8-6 in [2] - θ3dB = 65 (degree), SLAV = 30 (dB) - φ3dB = 65 (degree), Am = 30 (dB) | - Table 8-6 in [2] - θ3dB = 65 (degree), SLAV = 30 (dB) - φ3dB = 65 (degree), Am = 30 (dB) |
| - BS mechanical downtilt | - 90° in GCS | - 90° in GCS |
| - BS electrical downtilt | - 99° in LCS | - 99° in LCS |
| - BS array boresight | - 30°, 150°, 270° - → x | - 30°, 150°, 270° - → x |
| - UE antenna height | - 1.5 m | - 1.5 m |
| - Number of antenna elements per UE | - 4 Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5 λ, N/A) - Polarization: 0, 90 (degree) | - 2 Tx/Rx - (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) - Polarization: 0, 90 (degree) |
| - Number of TXRUs per UE | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) | - 2 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 1) |
| - UE antenna gain | - 0 dBi | - 0 dBi |
| - UE antenna radiation pattern | - Omni-directional | - Omni-directional |
| - UE array orientation | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° |
| - Polarized antenna model | - Model 2 [6] | - Model 2 [6] |
| - Device deployment | - 20 % indoor, 80 % outdoor - Randomly and uniformly distributed over the area under macro layer | - 20 % indoor, 80 % outdoor - Randomly and uniformly distributed over the area under macro layer |
| - Percentage of high loss and low loss building type | - 100% low loss - (applicable only to UMa_B) | - 100% low loss - (applicable only to UMa_B) |
| - UE mobility model | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| - UE speed of interest | - Indoor UE: 3 km/h - Outdoor UE: 30 km/h | - Indoor UE: 3 km/h - Outdoor UE: 30 km/h |
| - Inter-site interference modeling | - Explicitly modeled | - Explicitly modeled |
| - UE noise figure | - 7 dB | - 7 dB |
| - Channel model | - UMa_A, UMa_B | - UMa_A, UMa_B |
| - Traffic model | - Full buffer | - Full buffer |
| - Thermal noise level | - −174 dBm/Hz | - −174 dBm/Hz |
| - UE density | - 10 UEs per TRP | - 10 UEs per TRP |
| - Min. distance between TRP and UE | - d2D_min = 10 m | - d2D_min = 10 m |
| - UE attachment | - RSRP based from port 0 [6] | - RSRP based from port 0 [6] |
| - Handover margin | - 0 dB | - 0 dB |

- Table A2. Additional system level evaluation assumptions for UL

|  | - Configuration A | - Configuration B |
| --- | --- | --- |
| - BS noise figure | - 5 dB | - 5 dB |
| - Scheduler | - Random scheduling - (5 RBs allocated for each UE) | - Random scheduling - (5 RBs allocated for each UE) |
| - TPC parameters (P0, alpha) | - (-106 dBm, 1) | - (-106 dBm, 1) |

- Table A3. Link level evaluation assumptions for PDCCH

| - Carrier frequency | - 4 GHz |
| --- | --- |
| - Bandwidth | - 52 PRBs |
| - Subcarrier spacing | - 30 kHz |
| - Waveform | - OFDM with normal CP |
| - Channel model | - TDL-C with delay spread 363 ns |
| - Antenna element setting | - 2 TX 2 RX |
| - UE speed | - 3 km |
| - DCI payload size including CRC | - 57 bits |
| - Encoder | - Polar coding |
| - Decoder | - List decoding with list size 8 |
| - Transmission scheme | - Precoder cycling |
| - Aggregation level | - 8 |
| - REG bundle size | - 6 REGs |
| - CCE-to-REG mapping scheme | - Non-interleaved |
| - CORESET duration | - 1 OFDM symbols |
| - Channel estimation | - 2D-MMSE |
| - Receiver | - MMSE |

- Table A4. Link level evaluation assumptions for PDSCH

| - Carrier frequency | - 4 GHz |
| --- | --- |
| - Bandwidth | - 52 PRBs |
| - Subcarrier spacing | - 30 kHz |
| - Waveform | - OFDM with normal CP |
| - Channel model | - TDL-C with delay spread 363 ns |
| - Antenna element setting | - 2 TX 2 RX |
| - UE speed | - 3 km |
| - Transport block size | - 256 bits |
| - CRC size | - 16 bits |
| - Encoder | - LDPC coding |
| - Decoder | - Sum product with 50 iterations |
| - Transmission scheme | - Precoder cycling |
| - Modulation scheme | - QPSK |
| - Nominal coding rate | - 120/1024 |
| - PRG size | - 2 PRBs |
| - Frequency domain resource allocation | - Continuous allocation (32 PRBs) |
| - Time domain resource allocation | - 4 OFDM symbols |
| - Channel estimation | - 2D-MMSE |
| - Receiver | - MMSE |
| - DMRS configuration | - DMRS configuration type 1 with 3 dB power boost |

- Table A5. Link level evaluation assumptions for PUCCH

| - Carrier frequency | - 4 GHz |
| --- | --- |
| - Bandwidth | - 52 PRBs |
| - Subcarrier spacing | - 30 kHz |
| - Channel model | - TDL-C with delay spread 363 ns |
| - Antenna element setting | - 1 TX 2 RX |
| - UE speed | - 3 km |
| - PUCCH format | - PUCCH format 0 with frequency hopping |
| - Payload size | - 1 bit |
| - Frequency hopping configuration | - Each hop on the band edge |
| - Target DTX to NACK error rate | - 0.01 |

- Table A6. Link level evaluation assumptions for PUSCH

| - Carrier frequency | - 4 GHz |
| --- | --- |
| - Bandwidth | - 52 PRBs |
| - Subcarrier spacing | - 30 kHz |
| - Waveform | - OFDM with normal CP |
| - Channel model | - TDL-C with delay spread 363 ns |
| - Antenna element setting | - 1 TX 2 RX |
| - UE speed | - 3 km |
| - Transport block size | - 256 bits |
| - CRC size | - 16 bits |
| - Encoder | - LDPC coding |
| - Decoder | - Sum product with 50 iterations |
| - Modulation scheme | - QPSK |
| - Nominal coding rate | - 120/1024 |
| - Frequency domain resource allocation | - Continuous allocation (32 PRBs) |
| - Time domain resource allocation | - 4 OFDM symbols |
| - Channel estimation | - 2D-MMSE |
| - Receiver | - MMSE |
| - DMRS configuration | - DMRS configuration type 1 with 3 dB power boost |

Annex B: Evaluation results

Figure B1 to Figure B4 shows the system level evaluation results.

Figure B1: C.D.F. for DL pre-processing SINR for UMa A Scenario

Figure B2: C.D.F. for DL pre-processing SINR for UMa B Scenario

Figure B3: C.D.F. for UL pre-processing SINR for UMa A Scenario

Figure B4: C.D.F. for UL pre-processing SINR for UMa B Scenario

Figure B5 to Figure B8 shows the link level evaluation results.

Figure B5:PDCCH BLER performance

Figure B6: PDSCH BLER performance

Figure B7: C.D.F. for PUCCH NACK to ACK error and NACK to DTX error performance

Figure B8: PUSCH BLER performance