3GPP TSG RAN WG1 Meeting #94		R1-1809189

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:   7.2.7.2 Others

Source:	ITRI

Title:	Evaluation results of Rural-eMBB spectral efficiency for IMT-2020 self-evaluation

Document for:	Discussion

Introduction

In RAN #77 meeting, the general work plan which are related to ITU self evaluation have been made in [1]. The IMT-2020 submission time plan has also been agreed [2]. It was agreed that:

Agreements:

- Step 1: From Sep 2017 to Dec 2017, discussions in RAN ITU-R Ad-Hoc

  - Calibration for self evaluation.

  - Prepare and finalize initial description template information that is to be submitted to ITU-R WP 5D#29.

- Step 2: From early 2018 to Sep 2018, targeting “update & self eval” submission in Sep 2018

  - Performance evaluation against eMBB, mMTC and URLLC requirements and test environments for NR and LTE features.

  - Update description template and prepare compliance template according to self evaluation results.

  - Provide description template, compliance template, and self evaluation results based on Rel-15 in Sep 2018.

- Step 3: From Sep 2018 to June 2019, targeting “Final” submission in June 2019

  - Performance evaluation update by taking into account Rel-16 updates in addition to Rel-15.

  - Update description template and compliance template to take into account Rel-16 updates in addition to Rel-15.

  - Provide description template, compliance template, and self evaluation results based on Rel-15 and Rel-16 in June 2019.

In this contribution, we provide our initial evaluation results of Rural-eMBB for ITU IMT-2020 self-evaluation.

ITU Evaluation Methodology and Requirement for Spectral Efficiency

ITU-R report M.2142 [3] shows the average spectral efficiency (SE) can be estimated by running system-level simulations over number of drops Ndrops, then the average SE result is given by

SEavg= j=1NdropsR(j) (T)Ndrops T.W.M=  j=1Ndropsi=1NRij(T)Ndrops T.W.M  ,

where Rij (T) is the simulated total number of correctly received bits for user i in drop j. W denote the channel bandwidth and T the time over which the data bits are received. The 5th percentile user spectral efficiency is defined as 5th percentile point of the cumulative distribution function (CDF) of the normalized user throughput that can be given by

ri(j)= Rij (T)Ti.W ,

where Ti is the scheduling time duration of user i. It is noted that Ti equal to total simulation time T in the case of full buffer traffic model.

The minimum requirement for the 5th percentile user spectral efficiency and average spectral efficiency for Rural-eMBB sceanrio is provided as follow [4].

Table 1 Average Spectral Efficiency and 5th Percentile User Spectral Efficiency

| Test Environment |  | Average spectral efficiency (bit/s/Hz/TRxP) | 5th percentile user spectral efficiency (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Rural – eMBB | Downlink | 3.3 | 0.12 |
|  | Uplink | 1.6 | 0.045 |

Evaluation Results of DL Spectral Efficiency

In this section, an initial downlink evaluation results of DL average spectral efficiency and 5th percentile user spectral efficiency are listed in Table 2 and 3. The detailed evaluation assumptions can be found in Annex. In addition, Table 4 shows the details of number of DL transmission layer.

Table 2 DL Average Spectral Efficiency

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UE Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8T: (8,4,2,1,1;1,4) | For 2R: (1,1,2,1,1; 1,1) | 7.287 | 3.3 |
| Rural – eMBB Config. B | For 16T: (8,8,2,1,1;1,8) | For 4R: (1,2,2,1,1; 1,2) | 10.212 |  |
|  | For 32T: (8,8,2,1,1;2,8) | For 4R: (1,2,2,1,1; 1,2) | 10.760 |  |
| Rural – eMBB Config. C | For 8T: (8,4,2,1,1;1,4) | For 4R: (1,2,2,1,1; 1,2) | 11.732 |  |

Table 3 DL 5th Percentile User Spectral Efficiency

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UE Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8T : (8,4,2,1,1;1,4) | For 2R: (1,1,2,1,1; 1,1) | 0.142 | 0.12 |
| Rural – eMBB Config. B | For 16T: (8,8,2,1,1;1,8) | For 4R: (1,2,2,1,1; 1,2) | 0.159 |  |
|  | For 32T: (8,8,2,1,1;2,8) | For 4R: (1,2,2,1,1; 1,2) | 0.221 |  |
| Rural – eMBB Config. C | For 8T: (8,4,2,1,1;1,4) | For 4R: (1,2,2,1,1; 1,2) | 0.329 |  |

Table 4 DL Transmission Layer Information

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UE Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | 1 Layer | 2 Layer | 3 Layer | 4 Layer |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8T : (8,4,2,1,1;1,4) | For 2R: (1,1,2,1,1; 1,1) | 56.85 % | 43.15 % | 0 % | 0 % |
| Rural – eMBB Config. B | For 16T: (8,8,2,1,1;1,8) | For 4R: (1,2,2,1,1; 1,2) | 46.5 % | 18.94 % | 26.49 % | 8.07 % |
|  | For 32T: (8,8,2,1,1;2,8) | For 4R: (1,2,2,1,1; 1,2) | 20.0 % | 44.39 % | 29.29 % | 6.32 % |
| Rural – eMBB Config. C | For 8T: (8,4,2,1,1;1,4) | For 4R: (1,2,2,1,1; 1,2) | 20.7 % | 46.49 % | 26.14 % | 6.67 % |

Observation 1: NR can fulfill the minimun DL average spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 2: NR can fulfill the minimun DL 5th percentile user spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 3: The increasing of TXRU can effectively increase the layer two transmission probability and spectral efficiency for Rural-eMBB Config. C test environment.

Evaluation Results of UL Spectral Efficiency

In this section, an initial downlink evaluation results of UL average spectral efficiency and 5th percentile user spectral efficiency are listed in Table 5 and 6. In addition, Table 7 shows the details of number of DL transmission layer.

Table 5 UL Average Spectral Efficiency

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UE Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8R: (8,4,2,1,1; 1,4) | For 2T: (1,1,2,1,1; 1,1) | 8.715 | 1.6 |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 5.378 |  |
| Rural – eMBB Config. B | For 16R: (8,8,2,1,1;1,8) | For 4T: (1,2,2,1,1; 1,2) | 9.614 |  |
|  | For 32R: (8,8,2,1,1; 2,8) | For 1T: (1,1,1,1,1; 1,1) | 5.093 |  |
| Rural – eMBB Config. C | For 8R: (8,4,2,1,1; 1,4) | For 4T: (1,2,2,1,1; 1,2) | 8.995 |  |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 4.749 |  |

Table 6 UL 5th Percentile User Spectral Efficiency

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UEAntenna Configuration (M,N,P,Mg,Ng; Mp,Np) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8R: (8,4,2,1,1; 1,4) | For 2T: (1,1,2,1,1; 1,1) | 0.052 | 0.045 |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 0.058 |  |
| Rural – eMBB Config. B | For 16R: (8,8,2,1,1;1,8) | For 4T: (1,2,2,1,1; 1,2) | 0.015 |  |
|  | For 32R: (8,8,2,1,1; 2,8) | For 1T: (1,1,1,1,1; 1,1) | 0.063 |  |
| Rural – eMBB Config. C | For 8R: (8,4,2,1,1; 1,4) | For 4T: (1,2,2,1,1; 1,2) | 0.045 |  |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 0.081 |  |

Table 7 UL Transmission Layer Information

| Test Environment | TRP Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | UE Antenna configuration (M,N,P,Mg,Ng; Mp,Np) | 1 Layer | 2 Layer | 3 Layer | 4 Layer |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config. A | For 8R: (8,4,2,1,1; 1,4) | For 2T: (1,1,2,1,1; 1,1) | 57.54 % | 42.46 % | 0 % | 0 % |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 100 % | 0 % | 0 % | 0 % |
| Rural – eMBB Config. B | For 16R: (8,8,2,1,1;1,8) | For 4T: (1,2,2,1,1; 1,2) | 39.65 % | 10.0 % | 29.13 % | 21.22 % |
|  | For 32R: (8,8,2,1,1; 2,8) | For 1T: (1,1,1,1,1; 1,1) | 100 % | 0 % | 0 % | 0 % |
| Rural – eMBB Config. C | For 8R: (8,4,2,1,1; 1,4) | For 4T: (1,2,2,1,1; 1,2) | 37.01 % | 25.09 % | 32.11 % | 5.79 % |
|  | For 8R: (8,4,2,1,1; 1,4) | For 1T: (1,1,1,1,1; 1,1) | 100 % | 0 % | 0 % | 0 % |

Observation 4: NR can fulfill the minimun UL average spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 5: NR can fulfill the minimun UL 5th percentile user spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Summary

In this contribution, we provide our preliminary evaluation results of Rural-eMBB, we have following observations:

Observation 1: NR can fulfill the minimun DL average spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 2: NR can fulfill the minimun DL 5th percentile user spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 3: The increasing of TXRU can effectively increase the layer two transmission probability and spectral efficiency for Rural-eMBB Config. C test environment.

Observation 4: NR can fulfill the minimun UL average spectral efficiency requirements for Rural-eMBB Config. A, B and C.

Observation 5: NR can fulfill the minimun UL 5th percentile user spectral efficiency requirements for Rural-eMBB Config. A, B and C.

References

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia, September 2017.

- RP-172098, “3GPP submission towards IMT-2020”, ITU-R Ad-Hoc Contact person, September 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Annex: Evaluation assumptions

In this contribution, the downlink and uplink evaluation assumptions for Rural-eMBB are provided in the following table.

| Rural – eMBB |  | Config A | Config B | Config C (LMLC) |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation |  | 700 MHz | 4 GHz | 700 MHz |
| Total transmit power per TRxP |  | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class |  | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type |  | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance |  | 1732 m | 1732 m | 6000 m |
| Number of antenna elements per TRxP |  | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP |  | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements |  | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) , (dH,dV) = (N/A, N/A)λ 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE |  | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Device deployment |  | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 40% indoor, 40% outdoor (pedestrian), 20% outdoor (in-car) Randomly and uniformly distributed over the area |
| UE speeds of interest |  | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (pedestrian): 3 km/h; Outdoor users (in-car): 30 km/h |
| Traffic model |  | Full buffer | Full buffer | Full buffer |
| UE density |  | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height |  | 1.5 m | 1.5 m | 1.5 m |
| Electronic tilt |  | [100°] in LCS | [100°] in LCS | [96°] in LCS |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |  |  |
| Wrapping around method |  | Geographical distance based wrapping |  |  |
| Polarized antenna model |  | Model-2 in TR36.873 |  |  |
| Scheduler |  | DL: SU PF UL: SU PF |  |  |
| Duplexing |  | FDD |  |  |
| Modulation |  | Up to 256 QAM |  |  |
| Downlink Overhead | SS/PBCH block | 1 SS/PBCH block per 20ms |  |  |
|  | PDCCH | 2 OFDM symbols per slot |  |  |
|  | DMRS (Type II) | 4, 8, 12 ports |  |  |
|  | CSI-RS for CM | 4, 8, 16, 32 ports with periodicity of 5ms |  |  |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |  |  |
|  | TRS | 20ms period; maximal bandwidth with 52 PRB; burst length with 2 slots 12 RE/PRB/20ms |  |  |
| Uplink Overhead | PUCCH | 2 PRBs and 14 OS for the slots without SRS transmission; 2 PRBs and 12 OS for the slots with SRS |  |  |
|  | DMRS | Type II, 2 complete symbols |  |  |
|  | SRS | 2 OFDM symbols per 5 slots |  |  |
| Receiver |  | MMSE-IRC |  |  |
| CSI feedback |  | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |  |  |
| Channel estimation |  | Non-ideal |  |  |