3GPP TSG RAN WG1 Meeting #94	R1-1809290

3GPP TSG RAN WG1 Meeting #94	R1-18xxxxx

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	7.2.7.2

Source:	CAICT

Title:	Self evaluation on peak spectral efficiency of FR1 towards IMT-2020

Document for:	Discussion and decision

Introduction

In this contribution, the self evaluation on peak spectral efficiency of FR1 towards IMT-2020 is provided.

The peak data rate and peak spectral efficiency have been defined in subsection 4.1 and 4.2 of Report ITU M.2410 [1], respectively. In [1], the minimum requirements of peak data rate and peak spectral efficiency are presented as follows.

Table 1-1 The minimum requirements of peak data rate and peak spectral efficiency.

| Performance requirements | Downlink | Uplink |
| --- | --- | --- |
| Peak data rate (Gbit/s) | 20 | 10 |
| Peak spectral efficiency (bit/s/Hz) | 30 | 15 |

Evaluation methodology of peak spectral efficiency and peak data rate

Peak data rate

If bandwidth is aggregated across  bands then the total peak data rate is defined in [1] as follows

where  and  () are the component bandwidths and spectral efficiencies, respectively.

In 3GPP, the calculation formula of peak data rate has also been discussed [2].As presented in [2], the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. The peak data rate can be computed as the maximum of the approximate data rates computed using the below formula for each of the supported band or band combinations.

Data rate (in Mbit/s) =

where

- is the number of aggregated component carriers in a band or band combination,

- is the maximum target code rate as defined in Table 5.1.3.1-2 in [3],

- For thecomponent carriers,

  - is the maximum number of layers,

  - is the maximum modulation order, which is set to 8 according to [3],

  - is the scaling factor, which is set to 1,

  - is the numerology, which is defined in [1],

  - is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum RB allocation in bandwidth  with numerology ,

  - is the overhead, which should be computed with the evaluation assumption.

Peak spectral efficiency

The peak spectral efficiency has been defined in subsection 4.2 of Report ITU M.2410 [1]. With the definition, the peak spectral efficiency can be obtained by

Analysis of overhead

The overhead should be considered with different scenarios and evaluation assumptions, which may be the most complicated in the evaluation. In this section, some examples for overhead calculation are given for clarity.

FR1 FDD

When the overhead of FDD downlink is analyzed, DMRS/CSI-RS/CSI-IM/TRS/SSB/PDCCH should be taken into account. The example of FR1 FDD downlink (15kHz, 50MHz, 10ms) is given as below.

Table 3-1 The example for overhead analysis of FR1 FDD downlink system.

| FR1 FDD Downlink (15kHz, 50MHz, 10ms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | RE Number in 1 PRB and 1 Slot (RE/PRB/Slot) | PRB Number | Slot Number in 10ms | Total RE Number |
| Available RE | 168 | 270 | 10 | 453600 |
| Overhead |  |  |  |  |
| DMRS | 16 | 270 | 10 | 43200 |
| CSI-RS | 8 | 270 | 0.5 | 1080 |
| CSI-IM | 4 | 270 | 0.5 | 540 |
| TRS | 12 | 52 | 0.125 | 78 |
| SSB | 960 |  | 0.5 | 480 |
| PDCCH | 144 |  | 10 | 1440 |
| Total Overhead |  |  |  | 46818 |
| Overhead ratio |  |  |  | 10.32% |
| Remarks |  |  |  |  |
| DMRS | SU transmission, maximum 8 layers, 16 RE/PRB/slot |  |  |  |
| CSI-RS | 8 ports, 8 RE/PRB/period with 20ms period |  |  |  |
| CSI-IM | 4 RE/PRB/period with 20ms period |  |  |  |
| TRS | 12 RE/PRB/period with 80ms period |  |  |  |
| SSB | 2 SSB, 960 RE/period with 20ms period |  |  |  |
| PDCCH | 2 CCE, 144 RE/period with 1ms period |  |  |  |

When the overhead of FDD uplink is analyzed, DMRS/SRS/PUCCH should be taken into account. The example of FR1 FDD uplink (15kHz, 50MHz, 10ms) is given as below.

Table 3-2 The example for overhead analysis of FR1 FDD uplink system.

| FR1 FDD Uplink (15kHz, 50MHz, 10ms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | RE Number in 1 PRB and 1 Slot (RE/PRB/Slot) | PRB Number | Slot Number in 10ms | Total RE Number |
| Available RE | 168 | 270 | 10 | 453600 |
| Overhead |  |  |  |  |
| DMRS | 8 | 270 | 10 | 21600 |
| SRS | 12 | 270 | 0.5 | 1620 |
| PUCCH | 12 | 1 | 10 | 120 |
| Total Overhead |  |  |  | 23340 |
| Overhead ratio |  |  |  | 5.15% |
| Remarks |  |  |  |  |
| DMRS | SU transmission, maximum 4 layers, 8 RE/PRB/slot |  |  |  |
| SRS | 1 symbols, 12 RE/PRB/period with 20ms period |  |  |  |
| PUCCH | 1 symbols and 1 PRB, 12 RE/period with 1ms period |  |  |  |

FR1 TDD

When the overhead of TDD downlink is analyzed, besides DMRS/CSI-RS/CSI-IM/TRS/SSB/PDCCH, the frame structure and the downlink/uplink ratio should be taken into account. In our opinion, the GP needs to be taken into account. The example of FR1 TDD downlink (DDDSU, S (11D, 1G, 2U)) (15kHz, 50MHz, 10ms) is given as below.

Table 3-3 The example for overhead analysis of FR1 TDD downlink system.

| FR1 TDD Downlink (DDDSU, S(11D,1G,2U), 15kHz, 50MHz, 10ms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | RE Number in 1 PRB and 1 Slot (RE/PRB/Slot) | PRB Number | Slot Number in 10ms | Total RE Number |
| Downlink RE | 168 | 270 | 6 | 272160 |
| Special RE | 144 | 270 | 2 | 77760 |
| Available RE |  |  |  | 349920 |
| Overhead |  |  |  |  |
| GP | 12 | 270 | 2 | 6480 |
| DMRS | 16 | 270 | 8 | 34560 |
| CSI-RS | 8 | 270 | 0.5 | 1080 |
| CSI-IM | 4 | 270 | 0.5 | 540 |
| TRS | 12 | 52 | 0.125 | 78 |
| SSB | 960 |  | 0.5 | 480 |
| PDCCH | 144 |  | 8 | 1152 |
| Total Overhead |  |  |  | 44370 |
| Overhead ratio |  |  |  | 12.68% |
| Remarks |  |  |  |  |
| DMRS | SU transmission, maximum 8 layers, 16 RE/PRB/slot |  |  |  |
| CSI-RS | 8 ports, 8 RE/PRB/period with 20ms period |  |  |  |
| CSI-IM | 4 RE/PRB/period with 20ms period |  |  |  |
| TRS | 12 RE/PRB/period with 80ms period |  |  |  |
| SSB | 2 SSB, 960 RE/period with 20ms period |  |  |  |
| PDCCH | 2 CCE, 144 RE/period with 1ms period |  |  |  |

When the overhead of TDD uplink is analyzed, besides DMRS/SRS/PUCCH, the frame structure and the downlink/uplink ratio should be taken into account. In our opinion, the GP needs to be taken into account. The example of FR1 TDD uplink (DDDSU, S (11D, 1G, 2U)) (15kHz, 50MHz, 10ms) is given as below.

Table 3-4 The example for overhead analysis of FR1 TDD uplink system.

| FR1 TDD Uplink (DDDSU, S(11D,1G,2U), (15kHz, 50MHz, 10ms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | RE Number in 1 PRB and 1 Slot (RE/PRB/Slot) | PRB Number | Slot Number in 10ms | Total RE Number |
| Available RE | 168 | 270 | 2 | 90720 |
| Special RE | 24 | 270 | 2 | 12960 |
| Available RE |  |  |  | 103680 |
| Overhead |  |  |  |  |
| DMRS | 8 | 270 | 4 | 8640 |
| SRS | 12 | 270 | 0.5 | 1620 |
| PUCCH | 12 | 1 | 2 | 24 |
| Total Overhead |  |  |  | 10284 |
| Overhead ratio |  |  |  | 9.92% |
| Remarks |  |  |  |  |
| DMRS | SU transmission, maximum 4 layers, 8 RE/PRB/slot |  |  |  |
| SRS | 1 symbols, 12 RE/PRB/period with 20ms period |  |  |  |
| PUCCH | 1 symbols and 1 PRB, 12 RE/period with 1ms period |  |  |  |

Evaluation assumption and results

The values of the common parameters for peak spectral efficiency evaluation are presented as below.

Table 4-1 Common parameters value of FR1 peak spectral efficiency evaluation.

| Parameters | DL | UL |
| --- | --- | --- |
|  | Values | Values |
|  | 8 | 4 |
|  | 8 | 8 |
|  | 1 | 1 |
|  | 0.9258 | 0.9285 |

For the convenience of the results discussion, the overhead results are listed.

Table 4-2 Overhead results of FR1.

| SCS | BW | FDD |  | TDD |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | DL | UL |
|  |  | OH | OH | OH | OH |
| 15 | 5 | 13.40% | 5.40% | 15.86% | 10.15% |
|  | 10 | 12.17% | 5.26% | 14.73% | 10.02% |
|  | 15 | 11.39% | 5.21% | 13.86% | 9.97% |
|  | 20 | 11.00% | 5.19% | 13.44% | 9.95% |
|  | 25 | 10.78% | 5.17% | 13.18% | 9.94% |
|  | 30 | 10.62% | 5.16% | 13.02% | 9.93% |
|  | 40 | 10.43% | 5.15% | 12.80% | 9.92% |
|  | 50 | 10.32% | 5.15% | 12.68% | 9.92% |
|  | 60 | - | - | - | - |
|  | 80 | - | - | - | - |
|  | 100 | - | - | - | - |
| 30 | 5 | 17.54% | 5.59% | 18.25% | 9.68% |
|  | 10 | 13.32% | 5.24% | 13.87% | 9.38% |
|  | 15 | 12.00% | 5.13% | 12.51% | 9.28% |
|  | 20 | 11.71% | 5.08% | 12.27% | 9.24% |
|  | 25 | 11.28% | 5.05% | 11.81% | 9.21% |
|  | 30 | 11.01% | 5.03% | 11.52% | 9.19% |
|  | 40 | 10.67% | 5.01% | 11.15% | 9.17% |
|  | 50 | 10.47% | 4.99% | 10.94% | 9.16% |
|  | 60 | 10.33% | 4.98% | 10.79% | 9.15% |
|  | 80 | 10.17% | 4.97% | 10.62% | 9.14% |
|  | 100 | 10.08% | 4.97% | 10.51% | 9.14% |
| 60 | 5 | - | - | - | - |
|  | 10 | 17.43% | 5.50% | 18.10% | 9.29% |
|  | 15 | 14.40% | 5.25% | 14.96% | 9.07% |
|  | 20 | 13.21% | 5.15% | 13.72% | 8.98% |
|  | 25 | 12.40% | 5.08% | 12.89% | 8.93% |
|  | 30 | 11.89% | 5.04% | 12.36% | 8.89% |
|  | 40 | 11.46% | 4.99% | 11.95% | 8.85% |
|  | 50 | 11.06% | 4.96% | 11.53% | 8.82% |
|  | 60 | 10.80% | 4.94% | 11.25% | 8.80% |
|  | 80 | 10.49% | 4.92% | 10.92% | 8.78% |
|  | 100 | 10.31% | 4.90% | 10.73% | 8.77% |

FR1 downlink results

With the evaluation assumption above, the FR1 downlink peak spectral efficiency is presented as below.

Table 4-3 FR1 Downlink peak spectral efficiency (bit/s/Hz).

| SCS[kHz] |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 DL | FDD | 15 | 43.1 | 45.5 | 46.5 | 47 | 47.2 | 47.4 | 48.1 | 48.2 | - | - | - |
|  |  | 30 | 36.1 | 41.4 | 44.4 | 44.8 | 45.9 | 46.1 | 47.1 | 47.4 | 48.2 | 48.5 | 48.9 |
|  |  | 60 | - | 36.2 | 40.9 | 41.5 | 43.2 | 44.4 | 44.9 | 46 | 46.8 | 47.7 | 48.2 |
|  | TDD | 15 | 41.9 | 44.1 | 45.2 | 45.7 | 46 | 46.2 | 46.9 | 46.9 | - | - | - |
|  |  | 30 | 35.8 | 41.2 | 44.1 | 44.5 | 45.6 | 45.8 | 46.9 | 47.2 | 48 | 48.3 | 48.6 |
|  |  | 60 | - | 35.9 | 40.6 | 41.2 | 43 | 44.2 | 44.7 | 45.8 | 46.5 | 47.4 | 48 |

FR1 uplink results

With the evaluation assumption above, the FR1 uplink peak spectral efficiency is presented as below.

Table 4-4 FR1 Uplink peak spectral efficiency (bit/s/Hz).

| SCS[kHz] |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 UL | FDD | 15 | 23.5 | 24.5 | 24.8 | 25 | 25.1 | 25.2 | 25.5 | 25.5 | - | - | - |
|  |  | 30 | 20.7 | 22.6 | 23.9 | 24.1 | 24.6 | 24.6 | 25.1 | 25.2 | 25.5 | 25.7 | 25.8 |
|  |  | 60 | - | 20.7 | 22.6 | 22.7 | 23.4 | 23.9 | 24.1 | 24.6 | 24.9 | 25.3 | 25.6 |
|  | TDD | 15 | 22.4 | 23.3 | 23.6 | 23.8 | 23.8 | 23.9 | 24.2 | 24.2 | - | - | - |
|  |  | 30 | 19.8 | 21.6 | 22.9 | 23 | 23.5 | 23.5 | 24 | 24.1 | 24.4 | 24.5 | 24.7 |
|  |  | 60 | - | 19.9 | 21.7 | 21.7 | 22.5 | 23 | 23.1 | 23.6 | 23.9 | 24.3 | 24.5 |

Conclusions

In this document, the peak spectral efficiency of NR FR1 is evaluated. According to the evaluation results, the following observations are made.

Observation 1: NR FR1 can fulfill the peak spectral efficiency requirements of DL and UL.

- References

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

RP-172172, “Reply to LS on NR UE Category”, Ericsson,Intel

3GPP TS 38.214, “NR; Physical layer procedures for data (Release 15)”, v15.2.0, 2018-06

3GPP TS 38.211, “NR; Physical channels and modulation (Release 15)”, v15.2.0, 2018-06