3GPP TSG-RAN WG1 meeting #94			Tdoc R1-1809374

Gothenburg, Sweden, August 20th – 24th, 2018

Title:	[Draft] LS Reply on Further Agreements on Paging Occasions

Response to:	R1-1808172

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Nokia

To:	TSG RAN WG2

Cc:	-

Contact Person:

#### Name:	Jorma Kaikkonen

E-mail Address:	jorma.kaikkonen(at)nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-1808172 (R2-1810954) providing update on agreements on the Paging Occasions. RAN1 discussed the information provided by RAN2 and would like to provide following observations.

RAN1 noted that RAN2 had agreed that for default association with Ns=2, UEs would be distributed to first half-frame (i_s=0) and second half-frame (i_s=1) based on  UE_ID. RAN1 felt that if Type0-PDCCH monitoring pattern is used as a default PDCCH monitoring pattern for paging, it would not be possible to distribute the UE’s in the PF based on UE_ID as the monitoring occasion associated to a given SS/PBCH block would not necessarily occur is said half-frame.  Also RAN1 would like to point that in RAN1 terminology the association related only in determination of the linking between SSB and PDCCH monitoring occasion for QCL.

For defining useful paging PDCCH monitoring occasions as monitoring occasions, RAN1 felt that in order to ensure that also paging message could be delivered in the same PO, it would be good also to account number of symbols required for PDSCH allocation to deliver the message. This could be set to be a fixed value, e.g. 4 symbols, preventing need for signalling.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information in to account.

3. Date of Next RAN1 Meetings:

TSG RAN1 Meeting #94bis		8 – 12 October, 2018		Chengdu, China

TSG RAN1 Meeting #95		 12 – 16 November, 2018	Spokane, USA