3GPP TSG RAN WG1 Meeting 94		 R1-1809433

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.1.7

Source: 	Qualcomm Incorporated

Title: 	NR Features and Capabilities

Document for:	Discussion/Decision

# Discussion

In this document, we discuss proposed changes to the NR UE feature list.

## URLLC capabilities

No change is required other than adding back HARQ timing Capability #2.

| 5-5a | Advanced UE processing time N1 for downlink | 1. Carrier aggregation support 2. Bandwidth limitation |  | Yes |  | Type 3 | N.A. | N.A. |  | If a UE indicates BW limitation, then the maximum #RBs per CC is limited to 136. If a UE indicates BW limitation and CA support, then the total #RBs across all CCs is limited to 300 for 15kHz or 136 for 30kHz. | RAN1 | Optional with capability signaling. Component-1 candidate values: {“non-CA only”, “CA”} Component-2 candidate values: {“BW limited”, “no BW limitation”} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-5b | Advanced UE processing time N2 for uplink | 1. Carrier aggregation support 2. Bandwidth limitation |  | Yes |  | Type 3 | N.A. | N.A. |  | If a UE indicates BW limitation, then the maximum #RBs per CC is limited to 136. If a UE indicates BW limitation and CA support, then the total #RBs across all CCs is limited to 300 for 15kHz or 136 for 30kHz. | RAN1 | Optional with capability signaling. Component-1 candidate values: {“non-CA only”, “CA”} Component-2 candidate values: {“BW limited”, “no BW limitations”} |

Proposal 1:

Add capability corresponding to UE Capability # 2 for processing time with CA and/or BW limitation.

## PDSCH beam switching

Clarify that when the UE supports a single active TCI state for PDSCH, there will be interruption beyond the time value defined for 2-2.

| 2-2 | PDSCH beam switching | 1. Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. | 2-1 | Yes |  | Type-3 | No need | Applicable only for FR2 |  | Note: Effective beam switching interruption is per RAN4 requirements based on a combination of 2-2 and 2-4 capabilities | RAN1 | Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2:

Add note to feature 2-2 to clarify that when the UE supports a single active TCI state for PDSCH, there will be interruption beyond the time value defined for 2-2.

## Semi-persistent TRS

Include SP TRS capability indication.

2-33b SP CSI-RS

| 2-33b | SP CSI-RS | 1. Support SP CSI-RS 2. Support SP TRS | 2-1 | Yes |  | Type 4 | No | Yes |  |  | RAN1 | FFS on the mandatory or optional | Mandatory with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3:

Include capability for handling semi-persistent TRS in 2-33b.

## Codebook based UL MIMO

It is optional to support codebook based UL MIMO. The codebook coherency subset indication should be conditional based on codebook based MIMO support.

2-13 PUSCH codebook coherency subset

| 2-13 | PUSCH codebook coherency subset | Proposal: 1. Supported codebook coherency subset type: Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} | 2-12 | Yes | Only non-coherent codebook subset is supported | Type 1 | N.A. | N.A. |  | Note: Not included if 2-14 is set to no-codebook based MIMO |  | Candidate values: {non-coherent, partial-non-coherent, full-coherent} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4:

Add a note saying that PUSCH codebook coherency subset is not included when the UE indicates no codebook based MIMO capability.

## CSI-RS processing for SRS precoding

Currently there is no limit on CSI-RS processing capability for SRS.

2-15a Association between CSI-RS and SRS

| 2-15a | Association between CSI-RS and SRS | Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRS-AssocCSIRS” 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | 2-15 | Yes | Association between CSI-RS and SRS is not supported | Type 3 | N.A. | N.A. |  |  |  | Optional Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-15b | CSI-RS processing framework for SRS | 1. Maximum number of periodic SRS resources associated with CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4. UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5. UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. | 2-15a | Yes | Association between CSI-RS and SRS is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |

Proposal 5:

Add limits on CSI-RS processing capability for 2-15a, add 2-15b.

## Beam correspondence

Optional/mandatory for beam correspondence is undecided.

| 2-20 | Beam correspondence | 1. Support Beam correspondence |  | Yes | Beam correspondence is not supported | Type 1 | N.A. | N.A. |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  | Mandatory with capability signaling for FR2 Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

## SSB/CSI-RS for beam measurements

There are no limits on simultaneous measurement capability for beam management.

| 2-21 | Periodic beam report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot 3. Support beam report based on SSB and CSI-RS for FR2 4. Support beam report based on SSB for FR1 |  | Yes | No support of periodic L1-RSRP report | Type 1 | N.A. | N.A. |  |  |  | Mandatory with capability signaling for FR2 Optional with capability signaling for FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-21a | FR1 Periodic beam report based on CSI-RS | 1. Support periodic beam report based on CSI-RS for FR1 | 2-21 | Yes | No support of FR1 periodic L1-RSRP report based on CSI-RS | Type 1 | N.A. | N.A. |  |  |  | Optional with capability signaling |
| 2-22 | Aperiodic beam report | 1. Support report on PUSCH 3. Support beam report based on SSB and CSI-RS for FR2 |  | Yes | No support of aperiodic L1-RSRP report | Type 1 | N.A. | N.A. |  |  |  | Mandatory with capability signaling for FR2 Not supported in FR1 |
| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP simultaneously shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP simultaneously shall not exceed MB_2 3. Supported density of CSI-RS 4. Supported max # of configured NZP CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) per CCs to measure L1-RSRP 5. Supported max # of configured NZP CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP 6. When intra-band NR CA is configured in a band, the same beam management is applied across all CCs in the band | 2-21, 2-21a, 2-22, or 2-23, 2-23a | Yes | RSRP measurement is not supported | Type 1 | N.A. | N.A. |  | Note: simultaneously doesn’t mean in the same slot |  | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 Component-4: candidate value set is {0, 4, 8, 16, 32, 64, 128} Component-5: candidate value set is {0, 4, 8, 16, 32, 64, 128} |

Proposal 7:

Define limits for CSI-RS processing capability for L1-RSRP.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

## Beam reporting timing

For the case of 30 kHz SCS, the current beam reporting timing is too tight, change the upper limit to a larger value.

2-25 Beam reporting timing

| 2-25 | Beam reporting timing | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | Yes | Beam reporting time capability is not known by gNB | Type 1 | N.A. | N.A. |  |  | RAN1/4 | Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8, 14, 28} X4 is {14, 28, 56} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

## No analog beamforming in FR1

2-26 and 2-27 are analog beamforming related and are not applicable to FR1.

| 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | Yes | Rx beam switching is not supported | Type 1 | N.A. | N.A. |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-27 | Beam switching | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | 2-24 | Yes. | No restriction on the maximum number of Tx+Rx beam change for a slot | Type 1 | N.A. | N.A |  |  |  | Only applicable to FR2 Candidate value set: {4, 7, 14} |

Proposal 9:

For FR1, there is no need to define features 2-26 and 2-27.

## Limiting number of CSI reports per CC

There is already a capability associated with the number of reports supported across all CCs. There should be a limit on reports per CC as well, to accommodate UEs that process different CCs in different hardware blocks and have a processing limitation per hardware block.

2-35 CSI report framework

| 2-35 | CSI report framework | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process]4. UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. FFS: whether X and Y should also count the SRS precoder derivation in case of reciprocity based SRS Tx | 2-32 | Yes | CSI report is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 10:

Introduce a capability for number of simultaneous reports in a CC.  Resolve the FFS in 2-35 by including SRS related precoder derivation in the limits.

## Missing value range for PDCCH Case 2 with a span gap

PDCCH monitoring Case 2 with a span gad has been decided to be optional; however, the candidate value range for the span gap is missing.

3-5b Case 2 with a span gap

| 3-5b | For type 1 with dedicated RRC configuration, type 3, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | For type 1 with dedicated RRC configuration, type 3 and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation of X between the start of two spans, where each span is of length up to Y |  | Yes |  | Type 3 | N.A. | N.A. |  |  |  | Optional with capability signaling Candidate value set for (X, Y): {[(7, 3)], [(4, 3) and (7, 3)], [(2, 2) and (4, 3) and (7, 3)]} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11:

Add candidate value set for Feature 3-5b (PDCCH monitoring Case 2 with a span gap).

## Single Tx / Dynamic power sharing capability

Single Tx and dynamic power sharing capabilities should be Type 3 instead of Type 2.

| 6-12 | Support 2 simultaneous UL transmissions for problematic cases |  |  | Yes | 2 simultaneous UL transmissions are not supported for problematic cases | Type 3 | N.A. | N.A. |  | RAN2/4 to decide This is a UE feature for LTE for a LTE/NR dual connectivity UE | RAN2/4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) HARQ subframe offset |  | Yes |  | Type 3 | N.A. | N.A. |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  |
| 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | Yes |  | Type 3 | N.A. | N.A. |  | RP-172833 | RAN |  |

Proposal 12:

Change Features 6-12, 6-13, 8-1 from Type 2 to Type 3.

## Number of groups for UCI multiplexing

Need to define capability limitation for number of groups per slot for which UE performs UCI multiplexing.

| 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) Multiplexing of SR and HARQ-ACK on a PUCCH 8) HARQ-ACK piggyback on PUSCH 9) Semi-static beta-offset configuration for HARQ-ACK 10) One group of overlapping channels per slot per CC for control multiplexing |  | Yes |  | N.A. | N.A. | N.A. |  | RAN4 to check feasibility of frequency hopping for PUCCH formats for FR2 |  | Mandatory without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-12 | Up to 2 PUSCHs per slot for different TBs | Up to 2 groups of overlapping channels per slot per CC for control multiplexing |  | Yes |  | Type 3 | N.A. | N.A. |  | Up to 2 unicast PUSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |
| 5-12a | Up to 7 PUSCHs per slot for different TBs | Up to 7 groups of overlapping channels per slot per CC for control multiplexing |  | Yes |  | Type 3 | N.A. | N.A. |  | Up to 7 unicast PUSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |
| 5-12b | Up to 4 PUSCHs per slot for different TBs | Up to 4 groups of overlapping channels per slot per CC for control multiplexing |  | Yes |  | Type 3 | N.A. | N.A. |  | Up to 4 unicast PUSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |

Proposal 13:

Add limitation on number of groups per slot for which the UE performs UCI multiplexing, based on the number of PUSCH per slot supported by the UE.

## Remaining decisions on optional/mandatory

In the feature list, there are missing definitions for optional/mandatory for several features. We propose to add the following.

| 1-8 | RLM based on a mix of SS block and CSI-RS signals |  | 1-4 and 1-7 | Yes | UE does not support RLM based on a mix of SS block and CSI-RS signals | Type 4 | No need | No need |  |  | RAN1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-30 | Uplink beam management | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | Yes | Uplink beam management is not supported | Type1 | N.A. | N.A. |  |  |  | Optional, with capability signaling Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
| 2-34 | NZP-CSI-RS based interference measurement | 1. Support NZP-CSI-RS based interference measurement FFS whether to add the number of NZP-CSI-RS port for interference measurement | 2-33 | Yes | NZP-CSI-RS based interference measurement is not supported | Type 4 | No need | No need |  |  |  | Optional with capability signaling |
| 2-44 | Basic DL PTRS | Support 1 port of PTRS |  | Yes | DL PTRS is not supported | Type 4 | N.A. | Yes |  |  |  | Mandatory with UE capability signaling for FR2 Optional with capability signaling for FR1 |
| 2-55 | SRS Tx switch | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, | 2-53 | Yes | SRS Tx Switch is not supported | Type 3 | N.A. | N.A. |  | Component-2 is agreed with conditioned to RAN4’s decision.— | RAN1/4 | Optional with capability signaling Component-1 is a list of TRx pairs, candidates are {“1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “T=R”} Component-2: Candidate value set:, {yes, no}, |
| 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) | Multiplexing of CSI with SR/HARQ-ACK once per slot |  | Yes | CSI is always dropped when collision with SR/HARQ-ACK occurs | Type 4 | No need | Yes |  |  | RAN1 | Optional with capability signaling |
| 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH |  |  | Yes |  | Type 4 | No need | No need |  |  |  | Optional with capability signaling |
| 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  |  | Yes |  | Type 4 | No need | No need |  | RAN1 will discuss this feature |  | Mandatory with capability signaling |
| 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | Yes | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | N.A. | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |
| 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | Yes |  | Type 3 | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |

Proposal 14:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# Conclusions

In this contribution, the following proposals have been made:

Proposal 1:

Add capability corresponding to UE Capability # 2 for processing time with CA and/or BW limitation.

Proposal 2:

Add note to feature 2-2 to clarify that when the UE supports a single active TCI state for PDSCH, there will be interruption beyond the time value defined for 2-2.

Proposal 3:

Include capability for handling semi-persistent TRS in 2-33b.

Proposal 4:

Add a note saying that PUSCH codebook coherency subset is not included when the UE indicates no codebook based MIMO capability.

Proposal 5:

Add limits on CSI-RS processing capability for 2-15a, add 2-15b.

Proposal 6:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

Proposal 7:

Define limits for CSI-RS processing capability for L1-RSRP.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

Proposal 8:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

Proposal 9:

For FR1, there is no need to define features 2-26 and 2-27.

Proposal 10:

Introduce a capability for number of simultaneous reports in a CC.  Resolve the FFS in 2-35 by including SRS related precoder derivation in the limits.

Proposal 11:

Introduce a capability for number of simultaneous reports in a CC.  Resolve the FFS in 2-35 by including SRS related precoder derivation in the limits.

Proposal 12:

Change Features 6-12, 6-13, 8-1 From Type 2 to Type 3.

Proposal 13:

Add limitation on number of groups per slot for which the UE performs UCI multiplexing, based on the number of PUSCH per slot supported by the UE.

Proposal 14:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# References

- R1-1805728, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1800886, “NR features and capabilities”, Qualcomm

- R1-1802849, “NR features and capabilities”, Qualcomm

- R1-1807374, “NR features and capabilities”, Qualcomm