3GPP TSG RAN WG1 Meeting #94		 R1-1809469

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Intel Corporation

Title:	DL quality reporting in Msg3 for eMTC

Agenda item:	6.2.1.5

Document for:	Discussion/Decision

# Introduction

At RAN #80 meeting, a new work item (WI) on Rel-16 MTC enhancements was approved [1]. One objective of the WI is to improve DL transmission efficiency by:

Specify quality report in MSG3 at least for EDT [RAN1, RAN2].

With DL quality report in Msg3 at least for EDT, the eNB can have a better knowledge of DL channel status, which benefits the efficient scheduling of DL transmissions after Msg2, especially when the channel quality of DL and UL is quite different. In this contribution, we share our views on the design options for the support of DL channel quality report in Msg3, including aspects on configuration of this feature, quality report metric, measurement reference resource and Msg3 design.

# Design of DL quality report in Msg3

## Configuration and UE capability

For configuration of DL quality report in Msg3 for eMTC, it can be configured semi-statically by higher layer signaling in a cell-specific manner.

The support of DL quality report should be UE capability. However, for initial access, the UE has not reported its capability yet during the random access procedure. Therefore, whether or how to indicate the UE capability should be studied. One option is to configure dedicated PRACH resources for UE supporting DL quality report in Msg3. However, considering the need of PRACH partitioning for EDT and the existence of legacy eMTC UEs, this option may impact the system capacity and thus is not preferred. On the other hand, the UL grant can be designed to enable the scheduling of both Msg3 without DL quality report and Msg3 with DL quality report, as elaborated in Section 2.4. It can be up to UE to select which one to follow. With this method, there would be no need of UE capability indication for support of DL quality report in Msg3.

Observation 1:

- Indication of UE capability for support of DL quality report in Msg3 via PRACH partitioning would impact system capacity.

Proposal 1:

- The DL quality report in Msg3 is configured semi-statically by higher layer signaling in a cell-specific manner.

- It is UE capability whether the DL quality report in Msg3 is supported.

  - The design of UL grant scheduling Msg3 should enable UE to choose whether DL quality report is transmitted in Msg3 or not, which requires no report of UE capability.

## Quality report metric

Recall that in Rel-13 eMTC, CQI feedback is supported in CE mode A. The same report metric can be extended to DL quality report in Msg3. The existing CQI table in Rel-13 or the CQI tables introduced in Rel-15 can be reused. The parameter RCSI, if used, can be indicated by higher layer signaling (e.g. SIB) or be determined as a predefined mapping from Rmax configured for type-2 CSS. When frequency hopping is configured for MPDCCH in type-2 CSS, wideband CQI is reported, otherwise, subband CQI as defined in Rel-13 eMTC is reported.

Alternatively, similar to the DL quality report in Msg3 for Rel-14 NB-IoT, the number of repetitions that the UE needs to decode hypothetical MPDCCH with BLER of 1% can be adopted as the quality report metric.

Proposal 2:

- Consider to denote the DL channel quality as CQI, or number of repetitions that the UE needs to decode hypothetical MPDCCH with BLER of 1%.

## Measurement reference resource

If the DL channel quality report is denoted by CQI, similar to Rel-13 eMTC CSI measurement, reference resource for channel quality measurement can be defined.

On the other hand, if the DL channel quality report is denoted by the number of repetitions for MPDCCH, similar to Rel-14 NB-IoT, a reference resource for MPDCCH may not be defined.

Proposal 3:

- If DL channel quality report is denoted by CQI, define measurement reference resource similar as in Rel-13 eMTC.

- If DL channel quality report is denoted by the number of repetitions for MPDCCH, measurement reference resource is not defined.

## Design of Msg3 carrying the DL quality report

Recall that in Rel-14 NB-IoT, reserved bits in RRC message are used to carry the DL quality report in Msg3. Different from NB-IoT with sufficient number of reserved bits available in RRC message, the number of reserved bits available in RRC message is limited for eMTC. To carry the DL quality report in Msg3, the RRC message size can be extended, or a dedicated MAC CE can be included in the MAC PDU of Msg3. Inputs from RAN2 may be needed regarding the detailed design of how to carry the DL quality report in Msg3. From RAN1 perspective, it is expected that a larger TBS is needed for Msg3 with DL quality report.

As discussed in Section 2.1, it is not preferred to indicate UE capability via further PRACH partitioning. For the cases where eNB is not aware of UE capability for the support of DL quality report, the following two design options can be considered.

- Option 1. For cases without EDT (if supported), the eNB always schedules a larger TBS for Msg3 if the DL quality report in Msg3 is enabled. For UEs not supporting the DL quality report, padding bits would be added to the Msg3 transmission. Depending on the number of bits needed for DL quality report, the efficiency on UL transmission may be impacted for UEs not supporting the DL quality report. For cases with EDT, as the scheduling of multiple TBS values are already supported, the same mechanism can be reused. It is up to UE to select the desirable TBS to carry the data and DL quality report. With this option, the maximum number of bits that can be supported for EDT may be reduced for UEs supporting the DL quality report. To indicate whether DL quality report is carried or not in Msg3, the reserved bit in RRC message can be used.

- Option 2. Similar to the EDT design, the UL grant can be used to indicate multiple TBS values, where part of the TBS values are for Msg3 without DL quality report and the remainder of the TBS values are for Msg3 with DL quality report. It is up to UE to choose which TBS value to be used depending on its capability for support of DL quality report in Msg3. With this option, the eNB would blindly detect whether the DL quality report is carried in the Msg3. Considering the case with EDT where eNB already needs to perform blind detection for multiple TBS values, to limit the complexity at eNB, the number of permitted actual transmitted TBS can be configured to be limited (e.g. 2 values for EDT and up to 4 values for joint consideration of EDT and DL quality report) when both EDT and DL quality report in Msg3 are enabled.

Observation 2:

- It is expected that a larger TBS is needed for Msg3 with DL quality report.

Proposal 4:

- When eNB is not aware of UE capability for support of DL quality report in Msg3, consider the following design options for scheduling of Msg3:

  - Option 1. For cases without EDT (if supported), eNB always schedules a larger TBS considering the need to carry DL quality report in Msg3, and it is up to UE to carry DL quality report or add padding bits in Msg3. For case with EDT, existing EDT scheduling mechanism is reused and it is up to UE to choose preferred TBS value and to decide whether DL quality report is carried in Msg3.

  - Option 2. Support scheduling of multiple TBS values similar as EDT design, where part of the TBS values are for Msg3 without DL quality report while the rest are for Msg3 with DL quality report. It is up to UE to choose preferred TBS value and to decide whether DL quality report is carried in Msg3.

# Conclusion

In this contribution, we discuss the design for the support of DL quality report in Msg3 for eMTC. Based on the discussions, we make the following observations and proposals:

Observation 1:

- Indication of UE capability for support of DL quality report in Msg3 via PRACH partitioning would impact system capacity.

Observation 2:

- It is expected that a larger TBS is needed for Msg3 with DL quality report.

Proposal 1:

- The DL quality report in Msg3 is configured semi-statically by higher layer signaling in a cell-specific manner.

- It is UE capability whether the DL quality report in Msg3 is supported.

  - The design of UL grant scheduling Msg3 should enable UE to choose whether DL quality report is transmitted in Msg3 or not, which requires no report of UE capability.

Proposal 2:

- Consider to denote the DL channel quality as CQI, or number of repetitions that the UE needs to decode hypothetical MPDCCH with BLER of 1%.

Proposal 3:

- If DL channel quality report is denoted by CQI, define measurement reference resource similar as in Rel-13 eMTC.

- If DL channel quality report is denoted by the number of repetitions for MPDCCH, measurement reference resource is not defined.

Proposal 4:

- When eNB is not aware of UE capability for support of DL quality report in Msg3, consider the following design options for scheduling of Msg3:

  - Option 1. For cases without EDT (if supported), eNB always schedules a larger TBS considering the need to carry DL quality report in Msg3, and it is up to UE to carry DL quality report or add padding bits in Msg3. For case with EDT, existing EDT scheduling mechanism is reused and it is up to UE to choose preferred TBS value and to decide whether DL quality report is carried in Msg3.

  - Option 2. Support scheduling of multiple TBS values similar as EDT design, where part of the TBS values are for Msg3 without DL quality report while the rest are for Msg3 with DL quality report. It is up to UE to choose preferred TBS value and to decide whether DL quality report is carried in Msg3.

# References

- RP-181450, “New WID on Rel-16 MTC enhancements for LTE”, Ericsson, 3GPP TSG RAN meeting #80, La Jolla, USA, June 2018.