3GPP TSG RAN WG1 Meeting #94	R1-1809508

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency

Document for:	Discussion

Introduction

In this contribution, the evaluation results for eMBB spectral efficiency (average and the 5th percentile user spectral efficiency) for NR and LTE are provided.

Test environments for eMBB spectral efficiency

According to Report ITU-R M.2410, average spectral efficiency and 5th percentile user spectral efficiency are used to evaluate the achievement for three times spectral efficiency improvement in ITU-R vision (see Recommendation ITU-R M.2083). The eMBB test environments and their related evaluation configurations are employed in Report ITU-R M.2412 to test the fulfillment of the spectral efficiency requirements. In this contribution, a subset of the eMBB test environments and the related evaluation configurations as given in Table 1 is considered for spectral efficiency evaluation.

TABLE 1 Evaluation configuration for spectral efficiency evaluation

| Test env. | Evaluation configuration | Carrier frequency | ISD | Remark |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | Macro layer only |
| Rural - eMBB | Config. A | 700 MHz | 1732 m |  |
|  | Config. B | 4 GHz | 1732 m |  |
|  | Config. C (LMLC) | 700 MHz | 6000 m |  |

Evaluated technical features

The IMT-2020 eMBB spectral efficiency requirement is three times higher compared to IMT-Advanced. Therefore, it is a challenging requirement and the NR features that could contribute to the improvement of spectral efficiency should be selected.

On the other hand, it is recalled that for LTE-Advanced evaluation towards IMT-Advanced submission, the evaluation is basically applied based on duplexing schemes, i.e., to FDD and TDD, respectively. This is due to the fact that, duplexing scheme is one of the fundamental features among the other features that impact spectral efficiency performance.

Duplexing scheme

In NR design, the flexible duplexing scheme is available, e.g.,

- Different transmission directions in either part of a paired spectrum,

- TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing.

In this document, the following duplexing schemes are considered

- FDD (for paired spectrum): NR features that can be applied to FDD operation are invoked.

- TDD (for unpaired spectrum): NR features that can be applied to TDD operation are invoked.

Spectral efficiency calculation for duplexing scheme

The spectral efficiency of different duplexing schemes can be calculated according to Report ITU-R M.2412.

For DL average spectral efficiency and 5th percentile spectral efficiency,

- In case of FDD, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by

(1)

where W is the DL bandwidth of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control and DL reference signals on the DL bandwidth of 10 MHz is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For TDD, the simulation bandwidth is 20 MHz for DL and UL. The DL average spectral efficiency is given by (1), where W is the effective DL bandwidth that accounts for the time-frequency resource used for DL transmission (including GP symbols); Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal on the DL effective bandwidth is taken into account; and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

For UL average spectral efficiency and 5th percentile spectral efficiency, similar way is employed to derive the evaluation results for these two metrics.

Spectral efficiency calculation for different bandwidth

To reflect the benefit of reduced guard band ratio and overhead for larger bandwidth in NR, i.e. when the system bandwidth is larger than simulation bandwidth (10 MHz in FDD and 20 MHz in TDD), the spectral efficiency can be derived from Eq. (2)

(2)

where  and is the guard band ratio and the overhead at given number of RB N, respectively, and  is calculated by Eq. (1). For FDD,  for 10 MHz simulation bandwidth and 15 kHz subcarrier spacing. For TDD,  for 20 MHz simulation bandwidth and 30 kHz subcarrier spacing.

The overhead reduction for the larger bandwidth mainly comes from the PDCCH. In addition, SSB and TRS overhead will be reduced slightly. By assuming M0 OFDM symbols for PDCCH at the bandwidth BW0, the number of OFDM symbol for PDCCH at bandwidth BW could be

M= BW0/ BW×M0                                                                                                  (3)

For example, if we assume M0 = 2 for 20 MHz bandwidth system, then M=1 for 40 MHz bandwidth system. The value of M could be a non-integer since NR supports PDCCH sharing with PDSCH.

For the evaluation results in Section 4, the guard band ratio and PDCCH overhead reduction model for larger bandwidth based on Eq. (2) is considered in DL.

Evaluation results

Based on the above discussion, evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated. The evaluation is applied to both NR and LTE. The detailed evaluation assumptions can be found in Appendix.

NR spectral efficiency evaluation results

The detailed evaluation assumptions for NR DL and UL spectral efficiency evaluation can be found in Appendix A.

DL spectral efficiency evaluation results

The DL spectral efficiency evaluation results for NR are given in Table 2 to Table 6.

TABLE 2 FDD DL spectral efficiency evaluation for different system bandwidth in FR1 
(Channel model A)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. |
| Indoor Hotspot | Config. A 32T4R | 11.287 | 12.803 | 13.838 | 9 | 0.356 | 0.404 | 0.436 | 0.3 |
| Dense Urban | Config. A 32T4R | 11.45 | 12.942 | 13.962 | 7.8 | 0.376 | 0.425 | 0.458 | 0.225 |
| Rural | Config. A 8T2R | 6.594 | 7.383 | 7.927 | 3.3 | 0.138 | 0.155 | 0.166 | 0.12 |
|  | Config. A 16T2R | 7.476 | - | - |  | 0.151 | - | - |  |
|  | Config. B 32T4R | 13.6 | 15.376 | 16.59 |  | 0.354 | 0.4 | 0.432 |  |
|  | Config. C 8T4R | 7.597 | 8.51 | 9.138 |  | 0.18 | 0.202 | 0.217 |  |
|  | Config. C 16T4R | 8.134 | - | - |  | 0.203 | - | - |  |

TABLE 3 FDD DL spectral efficiency evaluation for different system bandwidth in FR1 
(Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. |
| Indoor Hotspot | Config. A 32T4R | 11.307 | 12.826 | 13.862 | 9 | 0.344 | 0.39 | 0.422 | 0.3 |
| Dense Urban | Config. A 32T4R | 11.417 | 12.904 | 13.921 | 7.8 | 0.379 | 0.428 | 0.462 | 0.225 |
| Rural | Config. A 8T2R | 6.537 | 7.32 | 7.858 | 3.3 | 0.13 | 0.146 | 0.156 | 0.12 |
|  | Config. A 16T2R | 7.456 | - | - |  | 0.156 | - | - |  |
|  | Config. B 32T4R | 13.533 | 15.299 | 16.506 |  | 0.345 | 0.39 | 0.421 |  |
|  | Config. C 8T4R | 7.607 | 8.521 | 9.151 |  | 0.183 | 0.205 | 0.22 |  |
|  | Config. C 16T4R | 8.137 | - | - |  | 0.201 | - | - |  |

TABLE 4 TDD DL spectral efficiency evaluation for different system bandwidth in FR1 (Channel model A)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | Config. A (30KHz SCS); 32T4R | 12.725 | 14.888 | 16.368 | 9 | 0.37 | 0.433 | 0.476 | 0.3 |
|  | Config. A (15KHz SCS); 32T4R | 12.536 | - | - |  | 0.387 | - | - |  |
| Dense Urban | Config. A (30KHz SCS); 32T4R | 12.8 | 14.904 | 16.346 | 7.8 | 0.375 | 0.437 | 0.479 | 0.225 |
|  | Config. A (30KHz SCS); 64T4R | 15.8 | 18.489 | 20.328 |  | 0.485 | 0.568 | 0.624 |  |
|  | Config. A (15KHz SCS); 32T4R | 12.711 | - | - |  | 0.381 | - | - |  |
|  | Config. A (15KHz SCS); 64T4R | 15.417 | - | - |  | 0.475 | - | - |  |
| Rural | Config. A (30KHz SCS); 8T2R | 7.672 | 8.821 | 9.612 | 3.3 | 0.189 | 0.217 | 0.237 | 0.12 |
|  | Config. A (30KHz SCS); 16T2R | 9.041 |  |  |  | 0.21 |  |  |  |
|  | Config. A (15KHz SCS); 8T2R | 7.515 | - | - |  | 0.161 | - | - |  |
|  | Config. A (15KHz SCS); 16T2R | 8.732 | - | - |  | 0.174 | - | - |  |
|  | Config. B (30KHz SCS); 32T4R | 15.061 | 17.54 | 19.238 |  | 0.374 | 0.436 | 0.478 |  |
|  | Config. B (15KHz SCS); 32T4R | 15.026 | - | - |  | 0.361 | - | - |  |
|  | Config. C (30KHz SCS); 8T4R | 7.63 | 8.774 | 9.562 |  | 0.182 | 0.209 | 0.228 |  |
|  | Config. C (30KHz SCS); 16T4R | 8.447 | - | - |  | 0.195 | - | - |  |
|  | Config. C (15KHz SCS); 8T4R | 7.857 | - | - |  | 0.192 | - | - |  |
|  | Config. C (15KHz SCS); 16T4R | 8.669 |  |  |  | 0.215 |  |  |  |

TABLE 5 TDD DL spectral efficiency evaluation for different system bandwidth in FR1 (Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | Config. A (30KHz SCS); 32T4R | 12.78 | 14.953 | 16.439 | 9 | 0.385 | 0.45 | 0.495 | 0.3 |
|  | Config. A (15KHz SCS); 32T4R | 12.53 | - | - |  | 0.394 | - | - |  |
| Dense Urban | Config. A (30KHz SCS); 32T4R | 12.581 | 14.646 | 16.061 | 7.8 | 0.385 | 0.448 | 0.491 | 0.225 |
|  | Config. A (30KHz SCS); 64T4R | 15.542 | 18.184 | 19.991 |  | 0.475 | 0.556 | 0.611 |  |
|  | Config. A (15KHz SCS); 32T4R | 12.506 | - | - |  | 0.395 | - | - |  |
|  | Config. A (15KHz SCS); 64T4R | 15.172 | - | - |  | 0.483 | - | - |  |
| Rural | Config. A (30KHz SCS); 8T2R | 7.604 | 8.742 | 9.526 | 3.3 | 0.185 | 0.213 | 0.232 | 0.12 |
|  | Config. A (30KHz SCS); 16T2R | 9.049 | - | - |  | 0.215 | - | - |  |
|  | Config. A (15KHz SCS); 8T2R | 7.433 | - | - |  | 0.156 | - | - |  |
|  | Config. A (15KHz SCS); 16T2R | 8.783 | - | - |  | 0.18 | - | - |  |
|  | Config. B (30KHz SCS); 32T4R | 14.946 | 17.405 | 19.09 |  | 0.37 | 0.431 | 0.473 |  |
|  | Config. B (15KHz SCS); 32T4R | 14.901 | - | - |  | 0.359 | - | - |  |
|  | Config. C (30KHz SCS); 8T4R | 7.641 | 8.787 | 9.576 |  | 0.178 | 0.205 | 0.223 |  |
|  | Config. C (30KHz SCS); 16T4R | 8.429 | - | - |  | 0.19 | - | - |  |
|  | Config. C (15KHz SCS); 8T4R | 7.869 | - | - |  | 0.192 | - | - |  |
|  | Config. C (15KHz SCS); 16T4R | 8.653 | - | - |  | 0.211 | - | - |  |

TABLE 6 TDD DL spectral efficiency evaluation for FR2

(Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=80 MHz | BW=100 MHz | BW=200 MHz | Req. | BW=20 MHz | BW=100 MHz | BW=200 MHz | Req. |
| Indoor Hotspot | Config. B (60KHz SCS); 32T8R | 11.384 | 11.984 | 12.998 | 9 | 0.302 | 0.318 | 0.345 | 0.3 |

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 2: NR with above 6 GHz can fulfill DL spectral efficiency requirements in Indoor Hotspot –  eMBB test environment with configuration B.

UL spectral efficiency evaluation results

The UL spectral efficiency evaluation results for NR are given in Tables 7 to Table 11.

TABLE 7 FDD UL spectral efficiency evaluation in FR1 (Channel model A)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. |
| Indoor Hotspot | Config. A 2T32R | 8.608 | 8.774 | 9.111 | 6.75 | 0.54 | 0.550 | 0.572 | 0.21 |
|  | Config. A 4T32R | 9.157 | - | - |  | 0.571 | - | - |  |
| Dense Urban | Config. A 2T16R | 6.599 | 6.726 | 6.985 | 5.4 | 0.248 | 0.253 | 0.262 | 0.15 |
|  | Config. A 2T32R | 7.91 | 8.062 | 8.372 |  | 0.378 | 0.385 | 0.400 |  |
|  | Config. A 4T32R | 8.599 | - | - |  | 0.44 | - | - |  |
| Rural | Config. A 1T8R | 4.17 | 4.250 | 4.414 | 1.6 | 0.134 | 0.137 | 0.142 | 0.045 |
|  | Config. A 2T8R | 6.181 | - | - |  | 0.14 | - | - |  |
|  | Config. B 1T32R | 4.253 | 4.335 | 4.502 |  | 0.163 | 0.166 | 0.173 |  |
|  | Config. B 4T32R | 7.498 | - | - |  | 0.253 | - | - |  |
|  | Config. C 2T8R | 4.038 | 4.116 | 4.274 |  | 0.081 | 0.083 | 0.086 |  |
|  | Config. C 4T8R | 4.685 | - | - |  | 0.1 | - | - |  |

TABLE 8 FDD UL spectral efficiency evaluation in FR1 (Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. |
| Indoor Hotspot | Config. A 2T32R | 8.643 | 8.809 | 9.148 | 6.75 | 0.537 | 0.547 | 0.568 | 0.21 |
|  | Config. A 4T32R | 9.198 | - | - |  | 0.572 | - | - |  |
| Dense Urban | Config. A 2T16R | 6.499 | 6.624 | 6.879 | 5.4 | 0.213 | 0.217 | 0.225 | 0.15 |
|  | Config. A 2T32R | 7.874 | 8.025 | 8.334 |  | 0.315 | 0.321 | 0.333 |  |
|  | Config. A 4T32R | 8.568 | - | - |  | 0.365 | - | - |  |
| Rural | Config. A 1T8R | 4.184 | 4.264 | 4.428 | 1.6 | 0.125 | 0.127 | 0.132 | 0.045 |
|  | Config. A 2T8R | 6.204 | - | - |  | 0.133 | - | - |  |
|  | Config. B 1T32R | 4.161 | 4.241 | 4.404 |  | 0.142 | 0.145 | 0.150 |  |
|  | Config. B 4T32R | 7.364 | - | - |  | 0.209 | - | - |  |
|  | Config. C 2T8R | 3.999 | 4.076 | 4.233 |  | 0.073 | 0.074 | 0.077 |  |
|  | Config. C 4T8R | 4.675 | - | - |  | 0.091 | - | - |  |

TABLE 9 TDD UL spectral efficiency evaluation in FR1 (Channel model A)

| Test env. | Evaluation Config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | Config. A (30KHz SCS); 2T32R | 7.551 | 7.847 | 8.401 | 6.75 | 0.42 | 0.436 | 0.467 | 0.21 |
|  | Config. A (30KHz SCS); 4T32R | 8.234 | - | - |  | 0.471 | - | - |  |
|  | Config. A (15KHz SCS); 2T32R | 7.545 | - | - |  | 0.419 | - | - |  |
|  | Config. A (15KHz SCS); 4T32R | 8.279 | - | - |  | 0.459 | - | - |  |
| Dense Urban | Config. A (30KHz SCS); 2T32R | 6.662 | 6.923 | 7.412 | 5.4 | 0.3 | 0.312 | 0.334 | 0.15 |
|  | Config. A (30KHz SCS); 2T64R | 7.633 | 7.932 | 8.492 |  | 0.386 | 0.401 | 0.429 |  |
|  | Config. A (30KHz SCS); 4T32R | 7.312 | - | - |  | 0.359 | - | - |  |
|  | Config. A (15KHz SCS); 2T32R | 6.91 | - | - |  | 0.292 | - | - |  |
|  | Config. A (15KHz SCS); 2T64R | 7.876 | - | - |  | 0.377 | - | - |  |
|  | Config. A (15KHz SCS); 4T32R | 7.57 | - | - |  | 0.34 | - | - |  |
| Rural | Config. A (30KHz SCS); 2T8R | 5.159 | 5.361 | 5.740 | 1.6 | 0.113 | 0.117 | 0.126 | 0.045 |
|  | Config. A (15KHz SCS); 2T8R | 5.517 | - | - |  | 0.104 | - | - |  |
|  | Config. B (30KHz SCS); 1T32R | 3.457 | 3.593 | 3.846 |  | 0.123 | 0.128 | 0.137 |  |
|  | Config. B (30KHz SCS); 4T32R | 6.219 | - | - |  | 0.193 | - | - |  |
|  | Config. B (15KHz SCS); 1T32R | 3.719 | - | - |  | 0.126 | - | - |  |
|  | Config. B (15KHz SCS); 4T32R | 6.627 | - | - |  | 0.195 | - | - |  |
|  | Config. C (30KHz SCS); 2T8R | 3.62 | 3.762 | 4.028 |  | 0.066 | 0.069 | 0.073 |  |
|  | Config. C (30KHz SCS); 4T8R | 4.166 | - | - |  | 0.079 | - | - |  |
|  | Config. C (15KHz SCS); 2T8R | 3.747 | - | - |  | 0.052 | - | - |  |
|  | Config. C (15KHz SCS); 4T8R | 4.283 | - | - |  | 0.063 | - | - |  |

TABLE 10 TDD UL spectral efficiency evaluation in FR1 (Channel model B)

| Test env. | Evaluation Config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | Config. A (30KHz SCS); 2T32R | 7.604 | 7.902 | 8.460 | 6.75 | 0.423 | 0.440 | 0.471 | 0.21 |
|  | Config. A (30KHz SCS); 4T32R | 8.277 | - | - |  | 0.469 | - | - |  |
|  | Config. A (15KHz SCS); 2T32R | 7.631 | - | - |  | 0.428 | - | - |  |
|  | Config. A (15KHz SCS); 4T32R | 8.363 | - | - |  | 0.476 | - | - |  |
| Dense Urban | Config. A (30KHz SCS); 2T32R | 6.629 | 6.889 | 7.375 | 5.4 | 0.271 | 0.282 | 0.302 | 0.15 |
|  | Config. A (30KHz SCS); 2T64R | 7.601 | 7.899 | 8.457 |  | 0.353 | 0.367 | 0.393 |  |
|  | Config. A (30KHz SCS); 4T32R | 7.329 | - | - |  | 0.336 | - | - |  |
|  | Config. A (15KHz SCS); 2T32R | 6.813 | - | - |  | 0.259 | - | - |  |
|  | Config. A (15KHz SCS); 2T64R | 7.861 | - | - |  | 0.31 | - | - |  |
|  | Config. A (15KHz SCS); 4T32R | 7.554 | - | - |  | 0.303 | - | - |  |
| Rural | Config. A (30KHz SCS); 2T8R | 5.171 | 5.374 | 5.753 | 1.6 | 0.107 | 0.111 | 0.119 | 0.045 |
|  | Config. A (15KHz SCS); 2T8R | 5.483 | - | - |  | 0.094 | - | - |  |
|  | Config. B (30KHz SCS); 1T32R | 3.384 | 3.517 | 3.765 |  | 0.103 | 0.107 | 0.115 |  |
|  | Config. B (30KHz SCS); 4T32R | 6.257 | - | - |  | 0.141 | - | - |  |
|  | Config. B (15KHz SCS); 1T32R | 3.643 | - | - |  | 0.104 | - | - |  |
|  | Config. B (15KHz SCS); 4T32R | 6.349 | - | - |  | 0.157 | - | - |  |
|  | Config. C (30KHz SCS); 2T8R | 3.593 | 3.734 | 3.997 |  | 0.059 | 0.061 | 0.066 |  |
|  | Config. C (30KHz SCS); 4T8R | 4.153 | - | - |  | 0.071 | - | - |  |
|  | Config. C (15KHz SCS); 2T8R | 3.751 | - | - |  | 0.045 | - | - |  |
|  | Config. C (15KHz SCS); 4T8R | 4.293 | - | - |  | 0.056 | - | - |  |

TABLE 11 TDD UL spectral efficiency evaluation for FR2 (Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=80 MHz | BW=100 MHz | BW=200 MHz | Req. | BW=20 MHz | BW=100 MHz | BW=200 MHz | Req. |
| Indoor Hotspot | Config. B (60KHz SCS); 8T16R | 6.382 | 6.418 | 6.455 | 6.75 | 0.245 | 0.246 | 0.248 | 0.21 |
|  | Config. B (30KHz SCS); 8T32R | 7.392 | 7.434 | 7.477 |  | 0.425 | 0.427 | 0.43 |  |

Observation 3: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 4: NR with above 6 GHz can fulfill UL spectral efficiency requirements in Indoor Hotspot – eMBB test environment using 8TXU UEs.

LTE spectral efficiency evaluation results

The detailed evaluation assumptions for LTE DL and UL spectral efficiency evaluation can be found in Appendix B.

DL spectral efficiency evaluation results

The DL spectral efficiency evaluation results for LTE are given in Table 12 and Table 13.

TABLE 12 FDD DL spectral efficiency evaluation for LTE (Channel model A)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | Req. | BW=10 MHz | Req. |
| Rural | Config. A; 8T2R | 4.644 | 3.3 | 0.12 | 0.12 |
|  | Config. C; 8T4R | 5.964 |  | 0.148 |  |

TABLE 13 FDD DL spectral efficiency evaluation for LTE (Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | Req. | BW=10 MHz | Req. |
| Rural | Config. A; 8T2R | 4.628 | 3.3 | 0.12 | 0.12 |
|  | Config. C; 8T4R | 5.968 |  | 0.149 |  |

Observation 5: LTE can fulfill DL spectral efficiency requirements in Rural – eMBB test environment with configuration A and C.

UL spectral efficiency evaluation results

The UL spectral efficiency evaluation results for LTE are given in Table 14 and Table 15.

TABLE 14 FDD UL spectral efficiency evaluation for LTE (Channel model A)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | Req. | BW=10 MHz | Req. |
| Rural | Config. A; 1T8R | 3.494 | 1.6 | 0.10 | 0.045 |
|  | Config. C; 2T8R | 3.272 |  | 0.067 |  |

TABLE 15 FDD UL spectral efficiency evaluation for LTE 
(Channel model B)

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | Req. | BW=20 MHz | Req. |
| Rural | Config. A; 1T8R | 3.501 | 1.6 | 0.10 | 0.045 |
|  | Config. C; 2T8R | 3.225 |  | 0.062 |  |

Observation 6: LTE can fulfill UL spectral efficiency requirements in Rural – eMBB test environment with configuration A and C.

Conclusion

In this document, the evaluation of eMBB spectral efficiency for NR operating on frequency range 1 (FR1, i.e. below 6 GHz) and frequency range 2 (FR2, i.e., above 6 GHz) is provided. In addition, the evaluation of eMBB spectral efficiency for LTE is provided. Based on the evaluation, the following observations are made.

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 2: NR with above 6 GHz can fulfill DL spectral efficiency requirements in Indoor Hotspot – eMBB test environment with configuration B.

Observation 3: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 4: NR with above 6 GHz can fulfill UL spectral efficiency requirements in Indoor Hotspot – eMBB test environment using 8TXU UEs.

Observation 5: LTE can fulfill DL spectral efficiency requirements in Rural – eMBB test environment with configuration A and C.

Observation 6: LTE can fulfill UL spectral efficiency requirements in Rural – eMBB test environment with configuration A and C.

- Reference

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, February 2018.

- Appendix A: Evaluation assumptions for NR

Table A-1 Evaluation assumptions for NR DL

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration |  | Configuration A/B | Configuration A | Configuration A/B/C |
| Channel model |  | InH_A/InH_B | UMa_A/UMa_B | RMa_A/RMa_B |
| ISD |  | 20 m (12TRxP) | 200 m | Configuration A/B: 1732 m Configuration C: 6000 m |
| TDD frame structure |  | DDDSU | DDDSU | DDDSU |
| Carrier Frequency |  | Configuration A: 4 GHz Configuration B: 30 GHz | 4 GHz | Configuration A: 700 MHz Configuration B: 4 GHz Configuration C: 700 MHz |
| System bandwidth |  | TDD: Configuration A: 20MHz ; Configuration B: 80MHz | TDD: 20MHz | TDD: 20MHz |
|  |  | FDD:10MHz | FDD:10MHz | FDD:10MHz |
| Subcarrier spacing |  | FDD: 15 kHz TDD: 15kHz and 30 kHz for configuration A: 60kHz for configuration B | FDD: 15 kHz TDD: 15kHz and 30 kHz | FDD: 15 kHz TDD: 15kHz and 30 kHz |
| Symbols number per slot |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP |  | Configuration A/B: 32Tx cross-polarized antennas (M,N,P,Mg,Ng) = (4,4,2,1,1); | For 32Tx: 128Tx cross-polarized antennas (M,N,P,Mg,Ng) = (8,8,2,1,1) For 64Tx: 192Tx cross-polarized antennas (M,N,P,Mg,Ng) = (12,8,2,1,1) | Configuration A/C: 64Tx cross-polarized antennas (M,N,P,Mg,Ng) = (8,4,2,1,1); Configuration B: 128Tx cross-polarized antennas (M,N,P,Mg,Ng) = (8,8,2,1,1) |
| Number of TXRU per TRxP |  | Configuration A/B: 32TXRU: Vertical 1-to-1 | 32TXRU: Vertical 2-to-8 64TXRU: Vertical 4-to-12 | Configuration A/C: 8TXRU Vertical 1-to-8; 16TXRU Vertical 2-to-8. Configuration B: 32TXRU Vertical 2-to-8 |
| Number of antenna elements per UE |  | Configuration A : 4Rx with 0°,90° polarization Configuration B : 8Rx with 0°,90° polarization (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 4Rx with 0°,90° polarization | Configuration A: 2Rx Configuration B/C: 4Rx with 0°,90° polarization |
| Transmit power per TRxP |  | TDD: Configuration A: 24 dBm Configuration B: 23 dBm | TDD: 44 dBm | TDD: 49 dBm |
|  |  | FDD: 21 dBm | FDD: 41 dBm | FDD: 46 dBm |
| TRxP number per site |  | 1 | 3 | 3 |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | Configuration A: 90deg in LCS Configuration B: According to Zenith angle in "Beam set at TRxP" | 105deg in LCS | Configuration A/B: 100deg in LCS Configuration C: 92deg in LCS |
| Beam set at TRxP |  | Configuration B: Azimuth angle φi = [0], Zenith angle θj = [pi/2] | N/A | N/A |
| Beam set at UE |  | Configuration B: Azimuth angle φi = [-pi/4, pi/4]; Zenith angle θj = [pi/4, 3*pi/4] | N/A | N/A |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF | MU-PF | MU-PF |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user; Configuration A: Maximum MU layer = 12; Configuration B: Maximum MU layer = 6 | MU-MIMO with rank 2/4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 2/4 adaptation per user; Maximum MU layer = 8 for 8Tx and maximum MU layer = 12 for 16Tx and 32Tx; |
| Guard band ratio |  | TDD: Configuration A: 8.2% for 30kHz SCS and 4.6% for 15kHz SCS (for 20 MHz); Configuration B: 5.5% (for 80 MHz); | TDD: 8.2% for 30kHz SCS and 4.6% for 15kHz SCS (for 20 MHz) | 8.2% for 30kHz SCS and 4.6% for 15kHz SCS (for 20 MHz) |
|  |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI feedback |  | 5 slots period based on non-precoded CSI-RS with delay | For 32Tx: 5 slots period based on non-precoded CSI-RS with delay For 64Tx: 5 slots period based on precoded CSI-RS with delay | 5 slots period based on non-precoded CSI-RS with delay |
| SRS transmission |  | Non-precoded SRS for 4Tx ports; Period: 5 slots; 2 symbols for 30kHz SCS; 4 symbols for 15kHz SCS; | Non-precoded SRS for 4Tx ports; Period: 5 slots; 2 symbols for 30kHz SCS; 4 symbols for 15kHz SCS; | Non-precoded SRS for 2/4 Tx ports for 2/4 Rx; Period: 5 slots; 4 symbols per 5 slots for configuration A/B for 15kHz and 30kHz; 2 symbols for 30kHz SCS and 4 symbols for 15kHz SCS for configuration C; |
| Precoder derivation |  | TDD: SRS based | TDD: SRS based | TDD: SRS based |
|  |  | FDD: NR Type II codebook (4 beams, WB+SB quantization, 8 PSK) | FDD: NR Type II codebook (4 beams, WB+SB quantization, 8 PSK) | FDD: NR Type II codebook (4 beams, WB+SB quantization, 8 PSK) |
| Overhead | PDCCH | 2 complete symbols | 2 complete symbols | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | FDD: 32 ports per 5 slots | FDD: 32 ports per 5 slots | FDD: 8/16/32 ports for 8Tx/16Tx/32Tx |
|  |  | TDD: 32 ports per 5 slots | TDD: For 64Tx, 4 ports per UE per 5 slots; For 32Tx, 32 ports per 5 slots | TDD: 8/16/32 ports for 8Tx/16Tx/32Tx |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |
|  | SSB | 1 SSB per 20 ms | 1 SSB per 20 ms | 1 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20 ms, 1 port, maximal 52 PRBs | 2 consecutive slots per 20ms, 1 port, maximal 52 PRBs | 2 consecutive slots per 20 ms, 1 port, maximal 52 PRBs |
|  | PTRS | Configuration B: 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density | N/A | N/A |
| Channel estimation |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM | OFDM |

Table A-2 Evaluation assumptions for NR UL

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration |  | Configuration A/B | Configuration A | Configuration A/B/C |
| Channel model |  | InH_A/InH_B | UMa_A/UMa_B | RMa_A/RMa_B |
| Subcarrier spacing |  | FDD: 15 kHz TDD: 15kHz and 30 kHz for configuration A: 60kHz for configuration B | FDD: 15 kHz TDD: 15kHz and 30 kHz | FDD: 15 kHz TDD: 15kHz and 30kHz |
| TDD frame structure |  | DDDSU | DDDSU | DDDSU |
| Symbols number per slot |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP |  | Configuration A: 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1); Configuration B: 64Rx cross-polarized antenna for 16TXRU, (M,N,P,Mg,Ng) = (4,8,2,1,1); 32Rx cross-polarized antenna for 32TXRU, (M,N,P,Mg,Ng) = (4,4,2,1,1); | For 32Rx: 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) For 64Rx: 192Rx cross-polarized antenna (M,N,P,Mg,Ng) = (12,8,2,1,1) | Configuration A/C: 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1); Configuration B: 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) |
| Number of TXRU per TRxP |  | Configuration A/B: 32TXRU Vertical 1-to-1; Configuration B: 16TXRU Vertical 2-to-4, Horizontal 4-to-8 | 32TXRU: Vertical 2-to-8 64TXRU: Vertical 4-to-12 | Configuration A/C: 8TXRU Vertical 1-to-8 Configuration B: 32TXRU Vertical 2-to-8 |
| Number of antenna elements per UE |  | Configuration A : 2Tx/4Tx with 0°,90° polarization Configuration B : 8Tx with 0°,90° polarization (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 2Tx/4Tx with 0°,90° polarization | Configuration A: 1Tx for FDD, 2Tx with 0°,90° polarization ; Configuration B: 1Tx/4Tx with 0°,90° polarization Configuration C: 2Tx/4Tx with 0°,90° polarization |
| UE power class |  | 23 dBm | 23 dBm | 23 dBm |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | Configuration A: 90deg in LCS Configuration B: According to Zenith angle in "Beam set at TRxP" | 105deg in LCS | Configuration A/B: 100deg in LCS Configuration C: 92deg in LCS |
| Beam set at TRxP |  | Configuration B: For 32Rx, Azimuth angle φi = [0], Zenith angle θj = [pi/2]; For 16Rx, Azimuth angle φi = [-pi/4,pi/4], Zenith angle θj = [pi/2]; | N/A | N/A |
| Beam set at UE |  | Configuration B: Azimuth angle φi = [-pi/4, pi/4]; Zenith angle θj = [pi/4, 3*pi/4] | N/A | N/A |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF | SU-PF | SU-PF |
| MIMO mode |  | Configuration A: SU-MIMO with rank 2 adaptation; Configuration B: SU-MIMO with rank 4 adaptation; | SU-MIMO with rank 2 adaptation | SIMO for 1Tx; SU-MIMO with rank 2 adaptation for 2Tx/4Tx |
| BS receiver type |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α=0.9， P0=-86 dBm | α=0.6， P0=-60 dBm | Configuration A: α=0.8， P0=-76 dBm; Configuration B: α=0.6， P0=-60 dBm; Configuration C: α=0.6， P0=-60 dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 38.101 in Section 6.2.2; Non-continuous RB allocation: additional 2 dB reduction | Continuous RB allocation: follow TS 38.101 in Section 6.2.2; Non-continuous RB allocation: additional 2 dB reduction | Continuous RB allocation: follow TS 38.101 in Section 6.2.2; Non-continuous RB allocation: additional 2 dB reduction |
| Overhead | PUCCH | 2 RBs and 14 OFDM symbols for FDD and TDD 30kHz SCS; 4 RBs and 14 OFDM symbols for TDD 15kHz and 60kHz SCS; | 2 RBs and 14 OFDM symbols for FDD and TDD 30kHz SCS; 4 RBs and 14 OFDM symbols for TDD 15kHz SCS; | 2 RBs and 14 OFDM symbols for FDD and TDD 30kHz SCS; 4 RBs and 14 OFDM symbols for TDD 15kHz SCS; |
|  | DMRS | Type II, 2 symbols (including one additional DMRS symbol), multiplexing with PUSCH | Type II, 2 symbols (including one additional DMRS symbol), multiplexing with PUSCH | Type II, 2 symbols (including one additional DMRS symbol), multiplexing with PUSCH |
|  | SRS | 2 symbols per 5 slots, | 2 symbols per 5 slots, | 2 symbols per 5 slots, |
|  | PTRS | Configuration B: 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density | N/A | N/A |
| Channel estimation |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM | OFDM |

- Appendix B: Evaluation assumptions for LTE

Table B-1 Evaluation assumptions for LTE DL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Rural - eMBB |
| Evaluation configuration |  | Configuration A/C |
| Channel model |  | RMa_A/RMa_B |
| ISD |  | Configuration A: 1732 m Configuration C: 6000 m |
| frame structure |  | Full downlink |
| Carrier Frequency |  | 700 MHz |
| System bandwidth |  | FDD:10MHz |
| Subcarrier spacing |  | 15 kHz |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1); |
| Number of TXRU per TRxP |  | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE |  | Configuration A: 2Rx with 0°,90° polarization Configuration C: 4Rx with 0°,90° polarization |
| Transmit power per TRxP |  | FDD: 46 dBm |
| TRxP number per site |  | 3 |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | Configuration A: 100deg in LCS Configuration C: 92deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF |
| ACK/NACK delay |  | N+4 |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user; Maximum MU layer = 8 |
| Guard band ratio |  | 10% |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay |
| Precoder derivation |  | LTE advanced CSI codebook (2 beams, WB quantization, QPSK) |
| Overhead | PDCCH | 2 complete symbols |
|  | DMRS | 12 REs/PRB/TTI |
|  | CSI-RS | 8 ports per 5 ms |
|  | CRS | 2 ports |
|  | PSS/SSS | 288 REs per 10 ms |
|  | PBCH | 240 REs per 10 ms |
|  | MBSFN | 6 MBSFN in 10 subframes |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |

Table B-2 Evaluation assumptions for LTE UL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Rural - eMBB |
| Evaluation configuration |  | Configuration A/C |
| Channel model |  | RMa_A/RMa_B |
| Subcarrier spacing |  | 15 kHz |
| Frame structure |  | Full uplink |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | Configuration A/C: 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1); |
| Number of TXRU per TRxP |  | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE |  | Configuration A: 1Tx Configuration C: 2Tx with 0°,90° polarization |
| UE power class |  | 23 dBm |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | Configuration A: 100deg in LCS Configuration C: 92deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF |
| MIMO mode |  | SIMO for 1Tx; SU-MIMO with rank 2 adaptation for 2Tx |
| BS receiver type |  | MMSE-IRC |
| UE precoder scheme |  | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay |
| Power control |  | Configuration A: α=0.8， P0=-76 dBm; Configuration C: α=0.6， P0=-60 dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 36.101 in Section 6.2.3; |
| Overhead | PUCCH | 2 RBs, 14 OFDM symbols |
|  | DMRS | 2 complete symbols |
|  | SRS | 1 symbol per 5 ms, 8 RBs per symbol |
| Channel estimation |  | Non-ideal |
| Waveform |  | DFT-S-OFDM |