3GPP TSG RAN WG1 Meeting 94 			R1-1809570

Gothenburg, Sweden, Aug 20th – Aug 24th, 2018

Source:	Huawei, HiSilicon

Title:	Draft summary of MPDCCH performance improvement

Agenda Item:	6.2.1.6

Document for:	Discussion and Decision

Introduction

The following tdoc provides a summary of the open issues and proposals based submitted tdoc’s to the “MPDCCH performance improvement” agenda item for RAN1#94. The following is the WID objective:

Improved DL transmission efficiency and/or UE power consumption:

- Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

- Specify quality report in MSG3 at least for EDT [RAN1, RAN2]

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

The WID states “Specify MPDCCH performance improvement by using CRS at least for connected mode”. This section includes issues related to using CRS for MPDCCH performance improvement.

Using CRS for improving channel estimation on USS MPDCCH

Companies basically have the common view that CRS+DMRS would be beneficial for improving channel estimation performance on USS MPDCCH.

Proposal 1: CRS for improving channel estimation on USS MPDCCH is supported

Supported by: Ericsson, Huawei, HiSilicon, LGE, Nokia, Sony, ZTE, Qualcomm, Intel, Samsung

Re-use of DMRS REs for MPDCCH REs

Sony and ZTE proposed that DMRS REs can be used for MPDCCH mapping in view of the channel estimation of MPDCCH can be based on CRS. Ericsson thought significant specification impact would be expected for using DMRS REs on MPDCCH.

Proposal 2: For further study on the impact of “Re-use of DMRS REs for MPDCCH REs”

Relation between the MPDCCH DMRS and CRS

Issue: If CRS+DMRS are also used for MPDCCH channel estimation, the relation between the MPDCCH DMRS and CRS should be defined. At least, the following solutions can be considered:

- Explicit relationship configured by RRC

- Predefined the relation

Proposal 3: The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation.

- Supported by: Ericsson, LGE, Nokia, Sony, ZTE, Huawei, HiSilicon, Qualcomm, Samsung

Relation between the CRS ports and DMRS ports

Issue: If CRS+DMRS are also used for MPDCCH channel estimation, the relation between the CRS ports and DMRS ports should be defined. The relation can be configured in cell-specific manner or predefined. Qualcomm proposed that “A given CRS port is the same as a given MPDCCH port” or “A set of CRS ports are linearly combined into a given MPDCCH port”.

Proposal 4: The relation between the CRS ports and DMRS ports is defined. FFS the details.

- Supported by: Ericsson, Nokia, Qualcomm, Samsung, Huawei, HiSilicon

Enable/disable “CRS for improving channel estimation on MPDCCH”

Issue: The feature of CRS for improving channel estimation on MPDCCH should be optional. Some companies proposed the eNB can enable/disable the feature by RRC signaling. At least, the following options can be considered for enabling/disabling “CRS for improving channel estimation on MPDCCH”:

- Dedicated RRC signaling

- SIB/SI signaling

- Per CEL configuration

Proposal 5: eNB enables/disables “CRS for improving channel estimation on MPDCCH” via RRC signaling. FFS the details on the signaling.

- Supported by: Ericsson, Nokia, Qualcomm, Samsung, Huawei, HiSilicon

CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH

Issue: Whether CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported. Ericsson and Qualcomm proposed that CRS for improving channel estimation can be applied on Type0-MPDCCH CSS.

Proposal 6: CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported.

- Supported by: Ericsson, Qualcomm, Intel (possibly)

CRS for improving channel estimation on Type1-MPDCCH CSS MPDCCH and Type2-MPDCCH CSS MPDCCH

Issue: For UEs in idle state, whether CRS for improving channel estimation can be applied on Type1-MPDCCH CSS (for paging) and Type1-MPDCCH CSS (for Msg2/3/4 in random access procedure).

Ericsson, Qualcomm, Sony and ZTE proposed that CRS for improving channel estimation on Type1-MPDCCH CSS MPDCCH and Type2-MPDCCH CSS MPDCCH can be supported via SIB/SI indication. Samsung is FFS on this point. Nokia does not support point.

Proposal 7: FFS on CRS for improving channel estimation on Type1-MPDCCH CSS MPDCCH and Type2-MPDCCH CSS MPDCCH.

Power offset between CRS and DMRS antenna ports of MPDCCH

Issue: If the joint CRS and DMRS-based operation for MPDCCH is enabled, the power offset between CRS and DMRS antenna ports is needed to match each other CRS-based channel estimates and DMRS-based channel estimates.

Intel, Samsung and ZTE proposed that power offset between CRS and DMRS antenna ports of MPDCCH is indicated to the UE by RRC.

Proposal 8: Power offset between CRS and DMRS antenna ports of MPDCCH is indicated to the UE by RRC

- Supported by: Intel, Samsung, ZTE

SFBC on MPDCCH

Issue: If MPDCCH demodulation is based on CRS, whether SFBC is supported on MPDCCH.

Nokia and ZTE proposed SFBC can be supported on MPDCCH. However, some limitations of applying SFBC on MPDCCH may be expected.

Proposal 9: Whether MPDCCH transmission using SFBC is FFS.

Tdocs

- R1-1808039 MPDCCH performance improvement in LTE-MTC Ericsson

- R1-1808120 On MPDCCH performance improvement Huawei, HiSilicon

- R1-1808657 MPDCCH performance improvement Intel Corporation

- R1-1808470 Discussion on MPDCCH performance improvement, LG Electronics

- R1-1808435 MPDCCH performance improvement Nokia, Nokia Shanghai Bell

- R1-1809027 Usage of CRS for MPDCCH Qualcomm Incorporated

- R1-1808734 Discussion on MPDCCH performance improvement by using CRS for MTC Samsung

- R1-1808352 On MPDCCH performance improvement Sony

- R1-1808636 Consideration on MPDCCH improvement by using CRS ZTE