3GPP TSG RAN WG1 Meeting #94	R1-18xxxxx

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	6.2.3.1.2

Source:	Huawei, HiSilicon

Title:	Feature summary on virtual cell ID for SRS

Document for:	Discussion and decision

Introduction

In this contribution, the discussion on additional SRS symbols for LTE [2-12] are summarized. Some suggested proposals are also provided.

WID objectives

At RAN#80 meeting, the following objectives were agreed for enhancement in LTE Rel-16 [1], with the objective for virtual cell ID highlighted.

The work item aims to specify the enhancements identified for further improving network performance. The detailed objectives are as follows.

- Enhance SRS capacity and coverage [RAN1]

  - Introduce more than one symbol for SRS for one UE or for multiple UEs on a UL normal subframe

      - Baseline: the minimum SRS resource allocation granularity for a cell is one slot, when more than one symbol in a normal subframe is allocated for SRS for the cell

      - Enhancements on PUCCH and PUSCH are not in scope

  - Introduce virtual cell ID for SRS

- Specify higher layer support of enhancements listed above [RAN2]

- Specify RF requirement of UE for the enhancements listed above [RAN4]

Summary

Configuration of virtual cell ID

The views of companies are listed in following table

| Sourcing | Related proposals/comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: For SRS, nIDRS=nIDSRSif nIDSRSis configured by higher layer, otherwise nIDRS=NIDcell. |
| ZTE | Therefore, for SRS can be configured by higher layer signaling in order to achieve interference randomization especially when traffic load is heavy. Certainly, the can still be configured as the same as to achieve orthogonality for intra-cell SRS transmission, it is up to Base Station’s implementation. Proposal 1: Support independent SRS sequence ID for trigger type 0 and type 1. Support SRS sequence ID per SRS configuration. |
| Mitsubishi Electric | Observation 1: The number of usable SRS sequences is limited by the requirement on orthogonality among the sequences Observation 2: Inter cell interference is a critical issue in Rel. 16 LTE Observation 3: Flexible assignment and quasi-orthogonal generation of SRS sequences are needed Proposal 1: Support virtual cell ID based SRS sequence generation Proposal 2: Support higher layer based configuration for virtual cell ID |
| LGE | Proposal 1: The configurable range of virtual cell ID for SRS sequence is the same as LTE physical cell ID. |
| Lenovo, Motorola Mobility | Observation: virtual cell ID for SRS can increases SRS capacity in the code/spatial domain. Proposal 1: VCID for PUSCH can be used for SRS without RRC impact. Proposal 2: eNB could configure multiple VCIDs for SRS for UE for different scenarios. |
| Intel | Proposal : Introduce a UE specific higher layer configurable virtual cell ID for SRS with default value being cell ID. |
| Samsung | Observation: - Support of virtual cell ID for SRS can provide SRS capacity and corresponding performance enhancements. Proposal: Support dynamic selection of higher layer virtual cell ID for SRS. |
| MTK | Proposal 1: Virtual Cell ID for SRS is configurable via higher layer, where - nIDRS is a per-UE configuration - nIDRS is a per-antenna-port configuration - nIDRS is a per-antenna-port-group configuration nIDRS=NIDcell otherwise if no value for nIDRS is configured by high layers Observation 1: Introducing Virtual Cell ID for SRS is an effective way of increasing SRS capacity. |
| QC | Proposal 1: The vertical cell ID is a unicast RRC configured parameter that replaces the cell ID to generating SRS sequence. - The cell ID is still used as the SRS sequence ID if not configured by higher layers. Proposal 2: Consider symbol/symbol-group specific configuration of virtual cell ID or cell ID for SRS sequence ID. |
| Ericsson | Proposal 1 A virtual Cell ID for SRS () has the same value range as the physical Cell ID. Proposal 2 Independent higher layer virtual cell ID parameters for SRS (), is supported for periodic and aperiodic SRS respectively |
| Nokia | Proposal 1: UE-specific virtual cell ID for SRS with the range [0, 503] is introduced. Proposal 2: Study further if virtual cell ID configuration should be SRS symbol or subframe specific. |

Based on the inputs, all companies agree to have higher layer configured virtual cell ID for SRS. On the range of virtual cell ID, three companies proposed to be the same as LTE cell ID range. And companies have diverse views on the granularity of virtual cell ID configuration. Therefore, we have following proposals:

Proposal 1: For SRS, nIDRS=nIDSRSif nIDSRSis configured by higher layer, otherwise nIDRS=NIDcell, where nIDRS is within the range from 0 to 503.

Proposal 2: FFS on the virtual cell ID configuration is

- Per trigger type (periodic or aperiodic)

- per-UE configuration

- per-antenna-port configuration

- per-antenna-port-group configuration

- per symbol/symbol-group

- per subframe

- dynamically indicated

Other issues

Some other issues were also discussed as listed in following table:

| Sourcing | Related proposals/comments |
| --- | --- |
| ZTE | Proposal 3: For SRS pseudo-random sequence, support the same symbol-level generation of the pseudo-random sequence for both and as NR. |
| QC | Proposal 3: Consider time-varying SRS sequence for more than one SRS symbol in a UL normal subframe. FFS other enhancement/modification to the SRS sequence. |
| Huawei, HiSilicon | Observation 1: Root sequence index for SRS with KTC =2 and KTC =4 should be carefully selected to avoid collision when virtual cell ID is introduced. Proposal 2: If the comb number for SRS is configured as 4, the root sequence index q is multiplied by 4. |

Based on the inputs, the following proposals are made for discussion:

Proposal 3: Consider time-varying/symbol-wise SRS sequence generation.

Proposal 4: Consider whether enhancement for sequence collision is supported.

Conclusion

In this contribution, the inputs are summarized and following proposals are made based on the inputs:

Proposal 1: For SRS, nIDRS=nIDSRSif nIDSRSis configured by higher layer, otherwise nIDRS=NIDcell, where nIDRS is within the range from 0 to 503.

Proposal 2: FFS on the virtual cell ID configuration is

- Per trigger type (periodic or aperiodic)

- per-UE configuration

- per-antenna-port configuration

- per-antenna-port-group configuration

- per symbol/symbol-group

- per subframe

- dynamically indicated

Proposal 3: Consider time-varying/symbol-wise SRS sequence generation.

Proposal 4: Consider whether enhancement for sequence collision is supported.

- References

RP-181485, “DL MIMO efficiency enhancements for LTE”, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11 - 14, 2018.

R1-1808137	On virtual cell ID for SRS sequence generation	Huawei, HiSilicon

R1-1808193	Discussion on virtual cell ID for SRS	ZTE

R1-1808446	Views on SRS virtual cell ID	Mitsubishi Electric Co.

R1-1808481	Discussion on VCID for SRS sequence	LG Electronics

R1-1808548	Virtual cell ID for SRS	Lenovo, Motorola Mobility

R1-1808663	Virtual cell ID support for SRS	Intel Corporation

R1-1808743	Discussions on virtual cell ID for SRS	Samsung

R1-1808957	Virtual Cell ID for SRS Capacity Enhancements	MediaTek Inc.

R1-1809038	Virtual cell ID for SRS	Qualcomm Incorporated

R1-1809192	On virtual cell ID for SRS	Ericsson

R1-1809377	Introduction of virtual cell id for SRS	Nokia, Nokia Shanghai Bell