3GPP TSG RAN WG1 Meeting #94 	R1-1809622

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0001 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC parameters for FeLAA in 37.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The interpretation of the bit value for ‘COT sharing indication’ field, and the association relationship between the AUL-UCI subframe (i.e., subframe n) and the applicable subframe (i.e., subframe n+X) is not reflected in 37.213. In addition, the RRC parameters of COT sharing indication offset, AUL ending position indication and CWS adjustment timer are not reflected in 37.213 or not aligned with 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct 37.213 to reflect the association relationship between the AUL-UCI subframe and the applicable subframe. In addition, correct RRC parameter names for COT sharing indication offset, AUL ending position indication and CWS adjustment timer in 37.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The association relationship between AUL-UCI indicating the sharing and start of DL transmission shared by AUL is not clear. The names for RRC parameters are not reflected or not aligned with 36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3, 4.2.1, 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

4.1.3	Channel access procedure for transmissions including PDCCH and not including PDSCH

If ‘COT sharing indication’ in AUL-UCI in subframe n indicates ‘1’, an eNB may transmit a transmission in subframe n+X, where X is subframeOffsetCOT-Sharing, including PDCCH but not including PDSCH on the same carrier immediately after sensing the channel to be idle for at least a sensing interval , if the duration of the PDCCH is less than or equal to two OFDM symbols length and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received.  consists of a duration immediately followed by one slot duration  and includes an idle slot duration at start of . The channel is considered to be idle for  if it is sensed to be idle during the slot durations of .

< Unchanged parts are omitted >

4.2.1	Channel access procedure for uplink transmission(s)

< Unchanged parts are omitted >

If the UE is scheduled by a UL grant received on a carrier to transmit a PUSCH transmission(s) starting from subframe  on the same carrier using Type 1 channel access procedure and if at least for the first scheduled subframe occupies  resource blocks  and the indicated 'PUSCH starting position is OFDM symbol zero, and if the UE starts autonomous UL transmissions before subframe  using Type 1 channel access procedure on the same carrier, the UE may transmit UL transmission(s) according to the received UL grant from subframe  without a gap, if the priority class value of the performed channel access procedure is larger than or equal to priority class value indicated in the UL grant, and the autonomous UL transmission in the subframe preceding subframe  shall end at the last OFDM symbol of the subframe regardless of the higher layer parameter endingSymbolAUL. The sum of the lengths of the autonomous UL transmission(s) and the scheduled UL transmission(s) shall not exceed the maximum channel occupancy time corresponding to the priority class value used to perform the autonomous uplink channel access procedure. Otherwise, the UE shall terminate the ongoing autonomous UL transmission at least one subframe before the start of the UL transmission according to the received UL grant on the same carrier.

< Unchanged parts are omitted >

4.2.2	Contention window adjustment procedure

< Unchanged parts are omitted >

-	If there exist one or more previous transmissions {, … , } using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer,  burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, for each transmission ,  is adjusted as following:

-	increase  for every priority class  to the next higher allowed value;

-	The  is adjusted once

-	Else if the UE transmits transmissions using Type 1 channel access procedure before N subframes have elapsed from the start of previous UL transmission burst using Type 1 channel access procedure and neither UL grant nor AUL-DFI is received,

-	the  is unchanged.

-	If the UE receives an UL grant or an AUL-DFI indicates feedback for one or more previous transmissions {, … , } using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer,  burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, the UE may recompute  as follows:

-	The UE reverts  to the value used to transmit at  using Type 1 channel access procedure.

-	The UE updates  sequentially in the order of the transmission {, … , }

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref' is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref' received in the earliest AUL-DFI after +3 indicates ACK.

-	For every priority class set

-	Otherwise, increase  for every priority class to the next higher allowed value

< Unchanged parts are omitted >