3GPP TSG RAN WG1 Meeting #94	                             	R1-1809719

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self-evaluation on Peak Data Rate and Peak Spectrum Efficiency for LTE

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

According to Report ITU-R M.2410 [2], the peak data rate and peak spectral efficiency is defined as in subsection 4.1 and 4.2:

The minimum requirements for peak data rate and peak spectral efficiency are provide in Report ITU-R M.2410 [2] and summarized in  Table 1-1.

Table 1-1  the requirements of peak data rate and peak spectral efficency

| Performance Measure | DL | UL |
| --- | --- | --- |
| Peak data rate | 20 Gb/s | 10 Gb/s |
| Peak spectral efficiency | 30 bps/Hz | 15 bps/Hz |

Also in Report ITU-R M.2412[3], more detailed evaluation methodologies are defined, where it is noted that proponents should demonstrate that the peak spectral efficiency requirement can be met for, at least, one of the carrier frequencies assumed in the test environments under the eMBB usage scenario, and proponents should demonstrate that the peak data rate requirement can be met for, at least, one carrier frequency or a set of aggregated carrier frequencies (where it is the case), assumed in the test environments under the eMBB usage scenario.

In this document, the peak spectrum efficiency and peak data rate evaluation for LTE are considered .

Methodology

Peak spectrum efficiency

The generic formula for peak spectral efficiency for FDD and TDD for a specific component carrier (say j-th CC) is given by

(1)

wherein

- Rmax = 948/1024

- For the j-th CC,

  - is the maximum number of layers

  - is the maximum modulation order

  - is the scaling factor

      - The scaling factor can at least take the values 1 and 0.75.

      - is signalled per band and per band per band combination as per UE capability signalling

  - is the average OFDM symbol duration in a subframe, i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum RB allocation in bandwidth  , as given in TS 36.104 section 5.6, where  is the UE supported maximum bandwidth in the given band or band combination.

  - is the overhead  calculated as the average ratio of the number of REs occupied by L1/L2 control, Synchronization Signal, PBCH and reference signals etc. with respect to the total REs in effective bandwidth  time product as given by .

-is the normalized scalar considering the downlink/uplink ratio; for FDD =1 for DL and UL; and for TDD for DL and UL is calculated based on the frame structure.

Peak data rate

The DL/UL peak data rate for FDD and TDD over Q component carriers can be calculated as below

(2)

where and  are the effective bandwidth and spectral efficiencies on component carrier , respectively, is the normalized scalar on component carrier  considering the downlink/uplink ratio on that component carrier; for FDD  =1 for DL and UL; and for TDD for DL and UL is calculated based on the frame structure, and is the carrier bandwidth of component .

The above formulas are applied to the following sessions for calculating peak data rate and peak spectral efficiency.

Evaluation results

For Peak data rate and peak spectrum efficiency evaluation , there is one UE which is expected to experience good channel environment so that the overhead will be different from that to spectrum efficiency evaluation. Overhead calculation should include PBCH, PSS,SSS,PDCCH,URS, CRS, CSI-RS for DL and SRS, PUCCH, DMRS for UL. Also ratio of overhead depends on bandwidth of a component carrier, which needs to be considered. When MBSFN subframe is configured ,the CRS can only be transmitted in the non-MBSFN region of MBSFN subframe, the overhead caused by CRS can be fewer. The evaluation results of FDD and TDD are both provided, for TDD, the configuration DSUUD is assumed and there are 2 MBSFN subframe in a radio frame , for FDD, there are 6 MBSFN subframe in a radio frame. for DL is 0.5571 and for UL is 0.4286.

Proposal 1: Overhead for peak data rate/spectrum efficiency

- The overhead for peak data rate/peak spectrum efficiency should be different from those used for spectrum efficiency

- Including PBCH, PSS,SSS, PDCCH,, URS, CRS, CSI-RS, for DL and SRS, PUCCH, DMRS  for UL

Proposal 2: MBSFN subframe can decrease the DL overhead

DL evaluation assumption

The values of the key parameters are needed and their assumed values for DL peak data rate are provided as below:

Table 3-1   Parameters for DL peak data rate

| Parameters | Values | Remarks |
| --- | --- | --- |
|  | 8 | LTE supports up to 8 layers for a single user for DL |
|  | 10 | LTE supports up to 1024QAM for DL |
| f | 1 | The value of 1 is chosen as scaling factor for DL peak spectral efficiency evaluation. |
|  | 0.9258 | LTE supports highest coding rate as =948/1024. |

The maximum transmission bandwidth configuration NRB for each BS channel bandwidth is specified in Section 5.6 of TS36.104 [4]. Considering the requirement of peak data rate, the following bandwidths of a component carrier are firstly investigated.

Table 3-2  DL maximum spectrum utilization (in PRB) for various bandwidth

|  | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 15 | 25 | 50 | 75 | 100 |

For the assessment of DL peak data rate, overhead due to control channel and reference signal must be considered. PBCH,PSS,SSS,PDCCH,URS, CRS, CSI-RS assumed are summarized in Table 3-3. In an MBSFN subframe, cell-specific reference signals shall only be transmitted in the non-MBSFN region of the MBSFN subframe ,so we provide the results of with MBSFN subframe and without MBSFN subframe. It is also noted that overhead assumed in this table is only for reasonable peak data rate calculation, but not meant it also applies to self evaluation for e.g. spectral efficiency.

Table 3-3 DL evaluation assumption for peak data rate and spectral efficiency

| Parameter | Setting |
| --- | --- |
| PBCH | 288 RE per 10ms |
| PSS/SSS | 288 RE per 10ms |
| PDCCH | 1 complete symbols |
| CSI-RS | 8 CSI-RS ports with 1 RE/PRB/port, with periodicity of 20ms |
| CRS | FDD: 20RE per PRB for the non-MBSFN subframe and 0RE for the MBSFN subframe TDD: 20RE per PRB for the non-MBSFN subframes 16RE per PRB for the DwPTS and 0RE for the MBSFN subframes |
| URS | 8ports, 24RE per PRB |

DL evaluation results

Based on the above assumptions and considerations, the evaluation results of peak spectral efficiency are provided in Table 3-4 . Considering the spectrum efficiency with MBSFN subframe can be higher than those without MBSFN subframe, so the results of peak data rate are provided with MBSFN subframe. For peak data rate, we assume the CC bandwidth is the maximum bandwidth and we use this CC bandwidth to evaluate. For example, we assume the CC bandwidth is 20 MHz.There are two options to calculate the peak data rate:

Option1:Take max BW per CC and max number of CC(32) to calculate the maximum peak data rate, the results are provided inTable 3-5.

Option2: Find certain BWs per CC and a number of CC to reach peak data rate requirement and show how much aggregated BW provided  , the results are provided in Table 3-6.

Table 3-4  evaluation results of DL peak spectral efficiency(bit/s/Hz) and overhead  under a certain component carrier

|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD with MBSFN | OH | 32.14% | 29.29% | 27.80% | 27.11% | 26.89% | 26.77% | 30 |
|  | SE | 36.35 | 35.35 | 45.12 | 45.55 | 45.69 | 45.76 |  |
| FDD without MBSFN | OH | 39.29% | 36.43% | 34.94% | 34.26% | 34.03% | 33.91% |  |
|  | SE | 35.52 | 31.78 | 40.66 | 41.08 | 41.23 | 41.30 |  |
| TDD with MBSFN | OH | 41.45% | 36.32% | 33.66% | 32.43% | 32.02% | 31.81% |  |
|  | SE | 31.36 | 31.84 | 41.46 | 42.23 | 42.48 | 42.61 |  |
| TDD without MBSFN | OH | 45.73% | 40.60% | 37.93% | 36.70% | 36.29% | 36.09% |  |
|  | SE | 29.07 | 29.69 | 38.79 | 39.56 | 39.82 | 39.94 |  |

Table 3-5 evaluation results of DL peak data rate of option 1

| Duplexing | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 20 | 0.91 | 32 | 640 | 29.12 | 20 |
| TDD | 20 | 0.48 | 32 | 640 | 15.36 |  |

Table 3-6  evaluation results of DL peak data rate of option 2

| Duplexing | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 20 | 0.91 | 22 | 440 | 20.02 | 20 |
| TDD | 20 | 0.48 | 42 | 840 | 20.16 |  |

Observation 1:  LTE can fulfill the peak spectral efficiency requirements of DL with MBSFN subframe.

Observation 2:  LTE can not fulfill the peak spectral efficiency requirements of DL with the component carrier bandwidth of 1.4/3MHz without MBSFN subframe.

Observation 3: LTE FDD can fulfill the peak data rate requirements of DL.

Observation 4: LTE TDD cannot fulfill the peak data rate requirements of DL under current overhead assumption and UL/DL configuration applied in this contribution.

UL evaluation assumption

The values of the key parameters are the same compared to DL in Table 3-1 except the maxim number of layer  is 4.

The maximum transmission bandwidth configurations NRB are the same as those for DL in Table 3-2.

For the assessment of UL peak data rate, overhead due to control channel and reference signal must be considered. SRS, PUCCH, DM-RS assumed for are summarized in Table 3-7. It is noted that overhead assumed in this table is only for reasonable peak data rate calculation, but it does not meant that it also applies to self evaluation for e.g. spectral efficiency.

Table 3-7 UL evaluation assumption for peak data rate and spectral efficiency

| Parameter | Setting |
| --- | --- |
| SRS | Resources ensuring the whole bandwidth measured within 20ms |
| PUCCH | 2 PRB pair every subframe |
| DM-RS | 24RE per PRB |

UL evaluation results

Based on the above assumptions and considerations, the evaluation results of peak spectral efficiency are provided in Table 3-8.  For peak data rate, we assume the CC bandwidth is the maximum bandwidth and we use this CC bandwidth to evaluate. For example, we assume the CC bandwidth is 20 MHz.There are two options to calculate the peak data rate:

Option1:Take max BW per CC and max number of CC(32) to calculate the maximum peak data rate, the results are provided inTable 3-9.

Option2: Find certain BWs per CC and a number of CC to reach peak data rate requirement and show how much aggregated BW provided , the results are provided in Table 3-10.

Table 3-8 evaluation results of UL peak spectral efficiency(bit/s/Hz) and overhead under a certain component carrier

|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | OH | 47.98% | 31.31% | 22.64% | 18.64% | 17.31% | 22.64% | 15 |
|  | SE | 11.15 | 13.74 | 19.34 | 20.34 | 20.67 | 19.34 |  |
| TDD | OH | 51.94% | 36.39% | 28.30% | 24.57% | 23.32% | 22.70% |  |
|  | SE | 10.30 | 12.72 | 17.92 | 18.86 | 19.17 | 19.32 |  |

Table 3-9 evaluation results of UL peak data rate of option1

| Duplexing | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 20 | 0.39 | 32 | 640 | 12.38 | 10 |
| TDD | 20 | 0.17 | 32 | 640 | 5.30 |  |

Table 3-10 evaluation results of UL peak data rate of option2

| Duplexing | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 20 | 0.39 | 26 | 520 | 10.14 | 10 |
| TDD | 20 | 0.17 | 61 | 1220 | 10.37 |  |

Observation 5:  LTE can fulfill the peak spectral efficiency requirements of UL with the component carrier bandwidth of 5/10/15/20MHz

Observation 6:  LTE cannot fulfill the peak spectral efficiency requirements of UL with the component carrier bandwidth of 1.4/3MHz

Observation 7: LTE FDD can fulfill the peak data rate requirements of UL.

Observation 8: LTE TDD cannot fulfill the peak data rate requirements of UL under current overhead assumption and UL/DL configuration applied in this contribution.

Conclusion

In this document, the peak spectral efficiency and peak data rate of LTE f are analysed and we have the following proposals:

Proposal 1: Overhead for peak data rate/spectrum efficiency

- The overhead for peak data rate/peak spectrum efficiency should be different from those used for spectrum efficiency

- Including PBCH, PSS,SSS, PDCCH,, URS, CRS, CSI-RS, for DL and SRS, PUCCH, DMRS  for UL

Proposal 2: MBSFN subframe can decrease the DL overhead

Observation 1:  LTE can fulfill the peak spectral efficiency requirements of DL with the component carrier bandwidth of 5/10/15/20MHz

Observation 2:  LTE cannot fulfill the peak spectral efficiency requirements of DL with the component carrier bandwidth of 1.4/3MHz

Observation 3: LTE FDD can fulfill the peak data rate requirements of DL.

Observation 4: LTE TDD cannot fulfill the peak data rate requirements of DL under current overhead assumption and UL/DL configuration applied in this contribution.

Observation 5:  LTE can fulfill the peak spectral efficiency requirements of UL with the component carrier bandwidth of 5/10/15/20MHz

Observation 6:  LTE cannot fulfill the peak spectral efficiency requirements of UL with the component carrier bandwidth of 1.4/3MHz

Observation 7: LTE FDD can fulfill the peak data rate requirements of UL.

Observation 8: LTE TDD cannot fulfill the peak data rate requirements of UL under current overhead assumption and UL/DL configuration applied in this contribution.

Reference

  - RP-172172, “Reply to LS on NR UE Category”, Ericsson,Intel

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - TS36.104 V15.3.0, “Base Station (BS) radio transmission and reception”,June,2018