3GPP TSG RAN WG1 Meeting #94	R1-1809753

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.1.1

Source:	Huawei, HiSilicon

Title:	Discussion on the design of SCMA

Document for:	Discussion and decision

- Introduction

In Rel-15 NR Non-orthogonal Multiple Access (NOMA) SID, the following objectives have been identified for the study of NoMA transmitter side signal processing [1]:

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

In RAN1#93, it is further agreed that

- Detailed transmission schemes particularly MA signature design per scheme will be captured in TR. Performance and complexity comparisons and observation/conclusion should at least be made scheme-wise.

In this contribution, we discuss the design of NoMA transmitter design. In particular, we introduce the design of Sparse Code Multiple Access (SCMA), and explain the benefit and flexibility in the design.

- SCMA Transmitter Side Design

A NoMA transmitter directly maps a sequence of the coded binary bits of a UE to a number of the available transmission resources by some user-specific operations to help a receiver separate the superposed multi-user signals with a reasonable complexity.

Since NR has already supported a UE-specific scrambling operation, we can describe a SCMA transmitter into two logical mapping functions: bits-to-symbols and symbols-to-REs.

Through the mapping operations, a SCMA design creates a more “structured” symbol dependency among a sequence of coded bits by jointly modulating and repetition in the bits-to-symbols mapping operation, and a sparsity against inter-user interference over the allocated REs (resource element) by repetition and layering in the symbols-to-REs mapping one. In this way, SCMA design enables a NoMA receiver to benefit from both the symbol-level dependency and the sparse mapping for UE separation, especially when the NoMA overloading ratio is high.

A SCMA transmitter strictly follows the unified framework agreed in RAN1 #86bis [2][3].

Figure 1. Transmit side design of SCMA based on the unified framework agreed in RAN1 #86bis

  - Bits-to-Symbols Mapping of SCMA

The symbol-level dependency is attributed to the flexibility to optimize the constellations across multiple symbols. Figure 2 shows an example of 16-point SCMA constellation over two symbols. By adjusting the labeling of the input binary bits to the constellation points for each RE (resource element), a NoMA transmitter can optimize the overall distances (Euclidean/product).

In contrast, a linear spreading with symbol-independent modulation (sequence-based nor repetition based spreading) has no means to improve the overall distance property. This explains the link-level performance gain of the 16-point SCMA modulation against linear spreading over 16QAM over two REs in Figure 3 (More details about the simulation parameters can be found in table A-1 in the appendix). Although the gain stands regardless of the spreading factor, larger spreading factor would bring about more degrees of freedom to optimize the distances, thereby leading to larger gain over linear spreading.

Figure 2. Example of 2-symbol mapping for 16-point constellation

| Figure 3. Example of single user performance comparison between the 2-symbol mapping and linear spreading (repetition or sequence based) in both AWGN and fading channels |  |
| --- | --- |

Observation 1: The structured symbol dependency in the bits-to-symbols mapping of SCMA provides more flexibility and better distance properties than sequence or repetition based linear spreading.

The M-bit to N-symbol mapping of SCMA transmitter can be represented by a N×2M table in which each column represents the symbol sequence in term of an index of the input bit stream. These tables for 8-point, 16-point and 64-point modulations with mapping length of 2 are given in tables A-2 to A-4 in the appendix.

The same mapping function can also be presented by a formula expressing the relation between the input bit stream b and the output symbol sequence x. For example, the formula of 8-point table in table A-2 in the appendix is

x=23-12j0-120j(1-2b).

Compared with a linear spreading over one symbol QAM modulation, the 8-point constellation modulation of SCMA maps three binary bits over two symbols, as shown in Figure 4 (correspondent table A-2). This equips NoMA scheme with a finer granularity on the optimization and configuration over the modulation sizes and FEC coding rates, the benefit of which is shown in Figure 5: an 8-point SCMA modulation outperforms a linear spreading over both QPSK and 16QAM modulations. Details about the simulation parameters are in Table A-5.

Figure 4. Example of joint mapping of three bits to two REs

| Figure 5. Example of single user performance of 8-point joint mapping vs. linear spreading (sequence or repetition based) in AWGN and fading channels |  |
| --- | --- |

Observation 2: The bits-to-symbols mapping design in SCMA provides more modulation granularity than the sequence or repetition based linear spreading.

For further user separation, more choices of the joint mapping sequences for a given bit stream (equivalently more M-bits to N-symbols mapping matrices) can be generated by casting some N-by-N transform matrix. For instance of N = 2, this 2-by-2 transform matrix can be one of

1001, 100-1, 100j, 100-j.

  - Symbols-to-REs mapping of SCMA

SCMA utilizes the symbols-to-REs mapping to further mitigate the inter-user interference and facilitate the low complexity implementation of the NoMA receivers. UE specific sparse patterns at configurable sparsity level are introduced as follows:

- Sparsity level is designated as the ratio of the non-zero elements to all available REs in a given mapping block. The SCMA transmitter configures it for optimal performance by adjusting the channel coding gain and the MUD mitigation capability.

- Sparse pattern defines the locations of the non-zero elements in the group of REs of a mapping block.  The pattern can be repeated throughout the available bandwidth. For example, the sparse patterns of sparse level 50% over the mapping block of size 4 are

1100,0011,1010,0101,1001,0110.

In practice, the symbols-to-REs mapping can also be integrated with the bits-to-symbols mapping to generate the symbol sequence (including zeros) directly from a sequence of coded bits and then map them to the available REs sequentially from an input bit stream.

Observation 3: UE-specific sparse mapping of SCMA provides good tradeoff between the coding gain and the multi-user interference mitigation capability.

  - UE Separation

As discussed above, SCMA can adopt UE-specific operations in the two mapping operations in favor of UE separation:

- UE-specific bits-to-symbols mapping: UE-specific transform matrix

- UE-specific symbols-to-REs mapping: UE-specific sparse pattern

The UE separation herein mainly facilitates the MUD receiver, and is different from the UE identification and activity detection that are jointly performed with other procedures such as DMRS based UE detection. More details can be found in [4].

- Features and Benefits of SCMA Transmitter

  - Flexibility and Scalability

SCMA transmitter provides sufficient flexibility and configurability for various NR scenarios such as mMTC, URLLC, and eMBB, specifically in the following four dimensions:

- Bits-to-Symbols mapping dimension: SCMA comes up with the constellations with larger distances and finer granularity than symbol-independent one. It offers the flexibility to extend the mapping length above 2.

- Symbols-to-REs mapping dimension: SCMA sparse repetition mapping comes up with the optimal tradeoff between the coding gain and multi-user interference mitigation capability by adjusting the size of mapping block, and the UE-specific sparsity levels and patterns.

- Layered transmission dimension: SCMA accommodates multiple layers (rate splitting) for a single UE to achieve higher SE (spectrum efficiency) more efficiently. The diagram of multi-layer SCMA is given in Figure 6, where each layer is configured with layer-specific bits-to-symbols mapping and symbols-to-REs mapping. Moreover, UEs configured with different layers can transmit together.

Figure 6. Diagram of SCMA transmitter side design with multi-layer transmission.

Figure 7. Diagram of SCMA transmitter side design with DFT-s-OFDM waveform.

- Mixed waveform dimension: SCMA supports both CP-OFDM and DFT-s-OFDM. An example of a SCMA-enabled DFT-s-OFDM is to have a transform precoding (DFT precoding) matrix between the modulated symbol sequence generator and symbol-to-RE mapping blocks, as shown in Figure 7. Neither the MA signature design nor receiver needs to be changed, e.g., the mapping functions in Table A-2 to A-4 can be directly applied to the modulated symbol sequence generator module. Moreover, very low PAPR mapping function for DFT-s-OFDM UEs transmitting in the coverage limited case can be designed (examples provided in table A-6).

Moreover, with SCMA scheme, the UEs configured with CP-OFDM waveform can be multiplexed with those configured with DFT-s-OFDM waveform in the same resource, alleviating the scheduling burden of the associated gNB. For some multiplexed users, the transform precoding is enabled (corresponding to DFT-s-OFDM waveform); and for others, it is disabled (corresponding to CP-OFDM waveform). Such flexibility is beneficial for most practical and typical application scenarios in which the coverage is modest and the majority of the UEs are configured with CP-OFDM waveform. It would be inefficient to dedicate a separate resource for a very limited number of UEs configured with DFT-s-OFDM waveform from the whole network perspective.

In summary, by configuring the design parameters, one can adapt the SCMA transmission scheme based on the system requirements in each scenario.

Observation 4: SCMA provides flexible and scalable solution for NoMA transmission to address versatile KPIs in different application scenarios.

  - Performance

The structured symbol dependency in SCMA offers the best and finest tradeoffs between the coding gain and spreading gain, and between the good distance properties and the inter-user interference mitigation capability, which can be utilized by the NoMA receiver (which includes multi-user detector and channel decoder as described in [5]) to improve the overall performance. Moreover, SCMA design allows for near-optimal multi-user decoding such as MPA with reasonable implementation cost and using low-complexity receivers (like EPA [5]) even for high overloading scenarios. The following curves in Figures 8, 9 and 10 show some examples of SCMA performance compared to other NoMA schemes. More detailed simulation results capturing the performance of SCMA in link-level can be found in [6]-[8].

In Figure 10, two options of RSMA have been considered:

- Option A: UE specific spreading and cell-specific scrambling with spreading length & layer adaptation

- Option B: UE specific spreading and UE-specific scrambling with spreading length & layer adaptation

The curves show that SCMA can provide large SNR gain over sequence spreading based NoMA schemes. The PAPR comparison of SCMA, RSMA, and QPSK baseline for DFT-s-OFDM waveform is presented in Figure 11. The results show that multi-layer RSMA has more than 1dB higher PAPR than QAM baseline, and the PAPR of SCMA is less than or equal to the QAM baseline.

Moreover, in the cases of higher overloading factor, the performance of SCMA, MUSA, RSMA are compared in Figure 12. The results show that with 24 UEs, SCMA still has better BLER performance than the sequence based linear spreading NoMA schemes for both ICE and RCE cases.

As a summary of the features and benefits described above, SCMA is a suitable candidate for NoMA transmission in NR to satisfy different performance metrics for all application scenarios.

Observation 5: With both CP-OFDM and DFT-s-OFDM waveform, SCMA can provide large SNR gain over sequence spreading based NoMA schemes with layer and/or spreading factor adaptation in all the cases simulated.

Observation 6: Multi-layer sequence spreading based NoMA schemes without sparsity increase the PAPR for DFT-s-OFDM, compared to QAM baseline.

Proposal 1: SCMA should be a candidate NoMA scheme of Rel-16 NoMA SI and captured in the TR.

| (a). mMTC, TDLA-30ns, 1T2R, ICE, Equal SNR | (b). mMTC, TDLA-30ns, 1T4R, ICE, Equal SNR |
| --- | --- |
| (c). mMTC, TDLA-30ns, 1T2R, ICE, Unequal SNR | (d). mMTC, TDLA-30ns, 1T4R, ICE, Unequal SNR |
| (e). URLLC, TDLA-30ns, 1T2R, ICE, Equal SNR | (f). URLLC, TDLA-30ns, 1T4R, ICE, Equal SNR |
| (g). eMBB, TDLA-30ns, 1T4R, ICE, Equal SNR | (h). eMBB, TDLA-30ns, 1T4R, ICE, Unequal SNR |

Figure 8. Performance comparison of SCMA and some other NoMA schemes (CP-OFDM)

| (a). TDLA-30ns, 1T2R, ICE, Equal SNR | (b). TDLA-30ns, 1T4R, ICE, Equal SNR |
| --- | --- |
| (c). TDLA-30ns, 1T2R, RCE, Equal SNR | (d). TDLA-30ns, 1T4R, RCE, Equal SNR |
| (e). TDLA-30ns, 1T2R, ICE, Unequal SNR | (f). TDLA-30ns, 1T4R, ICE, Unequal SNR |
| (g). TDLA-30ns, 1T2R, RCE | (h). TDLA-30ns, 1T4R, RCE |
| Figure 9. Performance comparison of SCMA and some other NoMA schemes (DFT-s-OFDM) |  |

| (a). mMTC, TDLA-30ns, 1T2R, CP-OFDM | (b). mMTC, TDLC-300ns, 1T2R, CP-OFDM |
| --- | --- |
| (c). mMTC, TDLA-30ns, 1T2R, DFT-s-OFDM | (d). mMTC, TDLC-300ns, 1T2R, DFT-s-OFDM |
| Figure 10. Performance comparison of SCMA and RSMA (a). QPSK baseline; (b). 16QAM baseline. Figure 11. PAPR comparison of SCMA, RSMA, and QAM baseline |
| (a). TDLA-30ns, 1T2R, ICE, 20Bytes, 24UE | (b). TDLA-30ns, 1T2R, RCE, 20Bytes, 24UE |

Figure 12. Performance comparison of SCMA and some other NoMA schemes (CP-OFDM)

Conclusions

In this contribution, we introduce the detailed transmitter design for SCMA and made the following observations and proposals.

Observation 1: The structured symbol dependency in the bits-to-symbols mapping of SCMA provides more flexibility and better distance properties than sequence or repetition based linear spreading.

Observation 2: The bits-to-symbols mapping design in SCMA provides more modulation granularity than the sequence or repetition based linear spreading.

Observation 3: UE-specific sparse mapping of SCMA provides good tradeoff between the coding gain and the multi-user interference mitigation capability.

Observation 4: SCMA provides flexible and scalable solution for NoMA transmission to address versatile KPIs in different application scenarios.

Observation 5: With both CP-OFDM and DFT-s-OFDM waveform, SCMA can provide large SNR gain over sequence spreading based NoMA schemes with layer and/or spreading factor adaptation in all the cases simulated.

Observation 6: Multi-layer sequence spreading based NoMA schemes without sparsity increase the PAPR for DFT-s-OFDM, compared to QAM baseline.

Proposal 1: SCMA should be a candidate NoMA scheme of Rel-16 NoMA SI and captured in the TR.

References

RP-170829, “New SID on Non-Orthogonal Multiple Access (NoMA) for NR”, RAN#75, Dubrovnik, Croatia, March 6 - 9, 2017.

RAN1 Chairman’s notes, RAN1#86b, Lisbon, Portugal, October, 10-15, 2016.

3GPP TR 38.802: “Study on New Radio (NR) Access Technology; Physical Layer Aspects”.

R1-1808051, “Discussion on NoMA related procedures”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

R1-1808050, “Discussion on the design of NoMA receiver”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

R1-1808054, “Evaluation results for mMTC scenario”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

R1-1808055, “Evaluation results for uRLLC scenario”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

R1-1808056, “Evaluation results for eMBB scenario”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

R1-1807377, “Transmitter Side Signal Processing Schemes for NOMA”, RAN1#92bis, Sanya, China, April 16th – April 20th, 2018.

R1-1804826, “Link and System Level Performance Evaluation for NOMA”, RAN1#92bis, Sanya, China, April 16th – April 20th, 2018.

Appendix

Table A-1. Simulation Assumptions for Figure 3.

| Parameter | Value |
| --- | --- |
| Evaluated schemes | Two-dimensional 16-point shown in Figure 3 vs. linear spreading of 16QAM over two symbols |
| Channel model | SISO AWGN and SIMO 1x2 TDLA-30 channel |
| Channel coding | Turbo code with rate 0.5 |
| Number of data tones | 864 (equivalent to 6 LTE RBs, including the DM-RS overhead) |
| Transport Block size | 864 (including CRC bits, Spectral Efficiency = 1) |

Table A-2. Mapping function for the 8-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| Output symbol sequence | -a8+jb8-a8+jb8 | a8+jb8a8+jb8 | -a8-jb8-a8+jb8 | a8-jb8a8+jb8 | -a8+jb8-a8-jb8 | a8+jb8a8-jb8 | -a8-jb8-a8-jb8 | a8-jb8a8-jb8 |

with a8=0.5774 and b8=0.8165.

Table A-3. Mapping function for the 16-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 |
| Output Symbol sequence | 3+3j1+j | -1+3j3+j | 1+3j-3+j | -3+3j-1+j | 3-j1+3j | -1-j3+3j | 1-j-3+3j | -3-j-1+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| Output Symbol sequence | 3+j1-3j | -1+j3-3j | 1+j-3-3j | -3+j-1-3j | 3-3j1-j | -1-3j3-j | 1-3j-3-j | -3-3j-1-j |

Table A-4. Mapping function for the 64-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 000 | 000 001 | 000 010 | 000 011 | 000 100 | 000 101 | 000 110 | 000 111 |
| Output symbol sequence | 1+j1+j | 1+3j1+3j | 1-j1+j | 1-3j1+3j | 3+j3+j | 3+3j3+3j | 3-j3+j | 3-3j3+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 001 000 | 001 001 | 001 010 | 001 011 | 001 100 | 001 101 | 001 110 | 001 111 |
| Output symbol sequence | -1+j1+j | -1+3j1+3j | -1-j1+j | -1-3j1+3j | -3+j3+j | -3+3j3+3j | -3-j3+j | -3-3j3+3j |
| Sequence index | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Corresponding bit sequence | 010 000 | 010 001 | 010 010 | 010 011 | 010 100 | 010 101 | 010 110 | 010 111 |
| Output symbol sequence | 1+j1-j | 1+3j1-3j | 1-j1-j | 1-3j1-3j | 3+j3-j | 3+3j3-3j | 3-j3-j | 3-3j3-3j |
| Sequence index | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| Corresponding bit sequence | 011 000 | 011 001 | 011 010 | 011 011 | 011 100 | 011 101 | 011 110 | 011 111 |
| Output symbol sequence | -1+j1-j | -1+3j1-3j | -1-j1-j | -1-3j1-3j | -3+j3-j | -3+3j3-3j | -3-j3-j | -3-3j3-3j |
| Sequence index | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| Corresponding bit sequence | 100 000 | 100 001 | 100 010 | 100 011 | 100 100 | 100 101 | 100 110 | 100 111 |
| Output Symbol sequence | 1+j-1+j | 1+3j-1+3j | 1-j-1+j | 1-3j-1+3j | 3+j-3+j | 3+3j-3+3j | 3-j-3+j | 3-3j-3+3j |
| Sequence index | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| Corresponding bit sequence | 101 000 | 101 001 | 101 010 | 101 011 | 101 100 | 101 101 | 101 110 | 101 111 |
| Output Symbol sequence | -1+j-1+j | -1+3j-1+3j | -1-j-1+j | -1-3j-1+3j | -3+j-3+j | -3+3j-3+3j | -3-j-3+j | -3-3j-3+3j |
| Sequence index | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| Corresponding bit sequence | 110 000 | 110 001 | 110 010 | 110 011 | 110 100 | 110 101 | 110 110 | 110 111 |
| Output Symbol sequence | 1+j-1-j | 1+3j-1-3j | 1-j-1-j | 1-3j-1-3j | 3+j-3-j | 3+3j-3-3j | 3-j-3-j | 3-3j-3-3j |
| Sequence index | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |
| Corresponding bit sequence | 111 000 | 111 001 | 111 010 | 111 011 | 111 100 | 111 101 | 111 110 | 111 111 |
| Output Symbol sequence | -1+j-1-j | -1+3j-1-3j | -1-j-1-j | -1-3j-1-3j | -3+j-3-j | -3+3j-3-3j | -3-j-3-j | -3-3j-3-3j |

Table A-5. Simulation Assumptions for Figure 5.

| Parameter | Value |
| --- | --- |
| Evaluated schemes | Single-UE performance for Joint mapping of 8-point shown in Figure 6 to two tones with two zero tones vs. linear spreading of QPSK and 16QAM and over 4 symbols |
| Channel model | SISO AWGN and SIMO 1x2 TDLA-30 channel |
| Spectral Efficiency | 0.25 per tone |
| Number of data tones | 864 (equivalent to 6 LTE RBs, including the DM-RS overhead) |
| Transport Block size | 216 (including the CRC bits) |

Table A-6. Three mapping functions with 4-point low PAPR modulated symbol of length 4 
(no sparsity is applied, only used when transform precoding is enabled in coverage limited case)

| Sequence index | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1111 | 1-j-1j | -1j1-j | -1-1-1-1 |
| Sequence index | 1 | 2 | 3 | 4 |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1111 | 1j-1-j | -1-j1j | -1-1-1-1 |
| Sequence index | 1 | 2 | 3 | 4 |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1j-1-j | 1-j-1j | -1j1-j | -1-j1j |