3GPP TSG RAN WG1 Meeting #94	R1-1809762

Gothenburg, Sweden, August 20-24, 2018

Source:	Qualcomm

Title:	Summary of DMRS Issues

Agenda Item:	7.1.2.4

Document for:	Discussion and Decision

# Collision of DMRS of PDSCH Type B with CORESET

[ZTE, Sanechips] For PDSCH mapping type B, the DMRS symbol should be shifted after a CORESET. However, since the configuration of CORESET does not include the time domain behavior, we propose to include the associated search space information. In addition, one typo should be corrected as follows.

TP4 for section 7.4.1.1.2 in 38.211

##### 7.4.1.1.2 Mapping to physical resources The UE shall assume the PDSCH DM-RS being mapped to physical resources according to configuration type 1 or configuration type 2 as given by the higher-layer parameter dmrs-Type. The UE shall assume the sequence is scaled by a factor to conform with the transmission power specified in [6, TS 38.214] and mapped to resource elements according to where , , and are given by Tables 7.4.1.1.2-1 and 7.4.1.1.2-2 and the following conditions are fulfilled: - the resource elements are within the common resource blocks allocated for PDSCH transmission ... For PDSCH mapping type B - if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and

| Company | Comment |
| --- | --- |
| ZTE, Sanechips, | OK |
| Qualcomm | OK |
| Nokia | OK |
| LGE | OK |
| Ericsson | OK |
| vivo | OK |
| Huawei, HiSilicon | Fine for the TP |
| DOCOMO | OK |
| Intel | Agree with ZTE |
| CATT | OK |

# Correction on the EPRE ratio of DMRS & PxSCH

[LGE] In the current TS38.214 [1], there is typo in the equation for the DM-RS scaling factor, which is derived from the ratio of PxSCH EPRE to DM-RS EPRE. The DM-RS scaling factor in the current TS38.214 [1], i.e., βPxSCHDMRS=10-βDMRS20, needs to be modified to βPxSCHDMRS=10βDMRS20or βPxSCHDMRS=10-1201βDMRS.

#### Text proposals on TS 38.214 4.1 Power allocation for downlink ---- Unchanged parts omitted --- For downlink DM-RS associated with PDSCH, the UE may assume the ratio of DM-RS EPRE to PDSCH EPRE ([dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Subclause 5.1.6.2. The DM-RS scaling factor specified in Subclause 7.4.1.1.2 of [4, TS 38.211] is given by βPDSCHDMRS=10-βDMRS20. Table 4.1-1: The ratio of DM-RS EPRE to PDSCH EPRE () Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; 3 dB; 3 dB. 3; -; 4.77 dB. ---- Unchanged parts omitted --- 6.2.2 UE DM-RS transmission procedure ---- Unchanged parts omitted --- For uplink DM-RS with PUSCH, the UE may assume the ratio of DM-RS EPRE to PUSCH EPRE (1/βDMRS [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by βPUSCHDMRS=10-βDMRS20.. Table 6.2.2-1: The ratio of DM-RS EPRE to PUSCH EPRE (βDMRS) Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; 3 dB; 3 dB. 3; -; 4.77 dB.

On the same topic OPPO provided a different TP which effectively seems to be doing the same required changes:

[OPPO] According to the values in table 4.1-1 for DL DMRS, the  [dB] is ratio of PDSCH EPRE to DM-RS EPRE, so the inverse is not needed in Subclause 4.1. Similar case in Subclause 6.2.2 for UL DMRS.

/************************ Start of Text Proposal for 38.214 **************************/ ## 4.1 Power allocation for downlink For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE ([dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Subclause 5.1.6.2. The DM-RS scaling factor specified in Subclause 7.4.1.1.2 of [4, TS 38.211] is given by . - Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; -3 dB; -3 dB. 3; -; -4.77 dB. /***************************** Omitted part *******************************/ ### 6.2.2 UE DM-RS transmission procedure For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by . Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; -3 dB; -3 dB. 3; -; -4.77 dB. /************************* End of Text Proposal for 38.214***************************/

Please provide your view on either one proposals shown above, or for another proposal on this topic.

| Company | Comment |
| --- | --- |
| Qualcomm, Ericsson | Some preference for the TP from OPPO since it requires fewer changes with respect to current text. |
| ZTE | The same view with Qualcomm |
| Nokia | Share view with Qualcomm and ZTE |
| LGE | When we consider the commonality with PT-RS section, the first one would be better. However, OPPO’s proposal is also good for simple modifications. |
| Huawei, HiSilicon | Fine with OPPO’s TP |
| DOCOMO | Agree with OPPO’s TP because of minimum change. |
| Intel | Both TPs are fine |
| CATT | Fine with OPPO’s TP. |

# Parameter consistency of DMRS text between 38.331 in 38.211

[OPPO] The candidate values for DMRS configuration used in 38.211 is not consistent with current values in 38.211 and should be corrected.

/************************ Start of Text Proposal for 38.211 **************************/ ##### 6.4.1.1.3 Precoding and mapping to physical resources For PUSCH mapping type A, the case dmrs-AdditionalPosition equal to ‘ pos3’ is only supported when dmrs-TypeA-Position is equal to ‘ pos2’. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for dmrs-TypeA-Position equals 2. The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6: - if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to ‘len2’ and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is not set to ‘pos0’ and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Tables 6.4.1.1.3-6 and 6.4.1.1.3-7 shall be used assuming dmrs-AdditionalPosition is equal to ‘pos1’ for each hop. /***************************** Omitted part *******************************/ ##### 7.4.1.1.2 Mapping to physical resources The reference point for and the position of the first DM-RS symbol depends on the mapping type: - for PDSCH mapping type A: - is defined relative to the start of the slot - if the higher-layer parameter dmrs-TypeA-Position is equal to ‘pos3’ and otherwise - for PDSCH mapping type B: - is defined relative to the start of the scheduled PDSCH resources - The position(s) of the DM-RS symbols is given by and - for PDSCH mapping type A, the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot - for PDSCH mapping type B, the duration is the number of OFDM symbols of the scheduled PDSCH resources as signalled and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case dmrs-AdditionalPosition equal to ‘pos3’ is only supported when dmrs-TypeA-Position is equal to ‘pos2’. For PDSCH mapping type A, duration of 3 and 4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to ‘pos2’. For PDSCH mapping type B - if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the PDSCH allocation collides with resources reserved for a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESETand - if the PDSCH duration is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol, - if the PDSCH duration is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix, - the UE is not expected to receive the first DM-RS beyond the fourth symbol, and - if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted. - if the PDSCH duration is 2 or 4 OFDM symbols, only single-symbol DM-RS is supported. The time-domain index and the supported antenna ports are given by Table 7.4.1.1.2-5 where - single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured, - single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to ‘len2’ In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain. /************************* End of Text Proposal for 38.211***************************/

/************************ Start of Text Proposal for 38.214 **************************/ ### 6.2.2 UE DM-RS transmission procedure If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition='pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. nIDDMRS,i For the UE-specific reference signals generation as defined in Subclause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B. When transmitting PUSCH scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI or CS-RNTI, - the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Subclause 6.4.1.1 of [4, TS 38.211]. - the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig. - if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be ‘pos0’, ‘pos1’ , ‘pos2’ or ‘pos3’. - if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'. - and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Subclause 6.4.1.1.3 of [4, TS 38.211]. /************************* End of Text Proposal for 38.214***************************/

| Company | Comment |
| --- | --- |
| OPPO | OK |
| Qualcomm | OK |
| ZTE | OK |
| Nokia | OK |
| LGE | OK |
| Ericsson | OK |
| Huawei, HiSilicon | Seems OK |
| DOCOMO | OK |
| CATT | OK |

# DMRS scrambling initialization for DCI format 0_0 with CS-RNTI.

- [Ericsson]In 38.211, fix the missing DMRS scrambling initialization for DCI format 0_0 scrambled with CS-RNTI.

A text proposal implementing this proposal is as follows:

<<< Begin TP for 38.211 >>>> 6.4.1.1.1.1 Sequence generation when transform precoding is disabled If transform precoding for PUSCH is not enabled, the sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1; - is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI or CS-RNTI; - otherwise. The quantity nSCID∈0,1 is indicated by the DM-RS initialization field, if present, in the DCI associated with the PUSCH transmission if DCI format 0_1 in [4, TS 38.212] is used, otherwise nSCID=0. <<< End TP for 38.211 >>>>

| Company | Comment |
| --- | --- |
| Ericsson | OK |
| Qualcomm | OK |
| ZTE | OK |
| Nokia | OK |
| LGE | OK |
| Huawei, HiSilicon | Fine for the TP |
| DOCOMO | OK |
| Intel | OK with TP |
| CATT | OK |

# Clarification on PUSCH DMRS position

[Sasung] For PUSCH, intra-slot frequency hopping and inter-slot frequency hopping are supported and these can be configured by higher layer. In TS 38.331 the higher layer parameter frequencyHopping is used in order to configure one of two supported frequency hopping mode. The current specification captures as follows:

PUSCH-Config ::= 						SEQUENCE {

dataScramblingIdentityPUSCH				INTEGER (0..1023)							OPTIONAL,	-- Need M

txConfig								ENUMERATED {codebook, nonCodebook}			OPTIONAL, 	-- Need S

dmrs-UplinkForPUSCH-MappingTypeA		SetupRelease { DMRS-UplinkConfig }				OPTIONAL,	-- Need M

dmrs-UplinkForPUSCH-MappingTypeB		SetupRelease { DMRS-UplinkConfig }				OPTIONAL,	-- Need M

pusch-PowerControl						PUSCH-PowerControl						OPTIONAL, -- Need M

frequencyHopping						ENUMERATED {mode1, mode2}				OPTIONAL, -- Need M

...

}

In the above, the mode1 for frequencyHopping indicates intra-slot frequency hopping only while the mode2 for frequencyHopping indicates inter-slot frequency hopping only. If this is not configured in higher layer, frequency hopping is disabled. On the other hand, if one of two supported frequency hopping modes is configured, then one bit in DCI format 0_1 is used for enable or disable PUSCH frequency hopping. However, in the current version of TS 38.211, PUSCH DMRS position tables (Tables 6.4.1.1.3-3 through 6.4.1.1.3-6) are provide considering only intra-slot frequency hopping. Therefore, we need to clarify this aspect. Specifically, PUSCH DMRS positions in Tables 6.4.1.1.3-3 through 6.4.1.1.3-6 are defined within a slot and Table 6.4.1.1.3-6 is for PUSCH DMRS positions when intra-slot frequency hopping is enabled. Also, Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 can be used for PUSCH DMRS positions when intra-slot frequency hopping is disabled or inter-slot frequency hopping is enabled. We provide a text proposal in Table 4 reflecting these clarifications with minor typo error.

Proposal 4: Agree on the following text proposal for TS 38.211 v15.2.0 Section 6.4.1.1.1.3 The position(s) of the DM-RS symbols is given by and - the signalled duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if inter-slot frequency hopping is used or intra-slot frequency hopping is not used, or - the signalled duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if inter-slot frequency hopping is used or intra-slot frequency hopping is not used, or - the signalled duration per hop according to Table 6.4.1.1.3-6 if intra-slot frequency hopping is used. - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and intra-slot frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212] and by higher layer, Tables 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is 1 for each hop. For PUSCH mapping type A, the case dmrs-AdditionalPosition equal to 3 is only supported when dmrs-TypeA-Position is equal to 2. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for dmrs-AdditionalPosition equals 2. The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-35. through 6.4.1.1.3-6: - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Tables 6.4.1.1.3-6 and 6.4.1.1.3-7 shall be used assuming dmrs-AdditionalPosition is 1 for each hop. Table 6.4.1.1.3-3: PUSCH DM-RS positions within slot for single-symbol DM-RS and intra-slot frequency hopping disabled and frequency hopping disabled. Duration in symbols; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. dmrs-AdditionalPosition; dmrs-AdditionalPosition. 0; 1; 2; 3; 0; 1; 2; 3. <4; -; -; -; -. 4. 5; , 4; , 4; , 4. 6; , 4; , 4; , 4. 7; , 4; , 4; , 4. 8; , 7; , 7; , 7; , 6; , 3, 6; , 3, 6. 9; , 7; , 7; , 7; , 6; , 3, 6; , 3, 6. 10; , 9; , 6, 9; , 6, 9; , 8; , 4, 8; , 3, 6, 9. 11; , 9; , 6, 9; , 6, 9; , 8; , 4, 8; , 3, 6, 9. 12; , 9; , 6, 9; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. 13; , 11; , 7, 11; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. 14; , 11; , 7, 11; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. Table 6.4.1.1.3-4: PUSCH DM-RS positions within slot for double-symbol DM-RS and intra-slot frequency hopping disabled and frequency hopping disabled. Duration in symbols; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. dmrs-AdditionalPosition; dmrs-AdditionalPosition. 0; 1; 2; 3; 0; 1; 2; 3. <4; -; -; -; -. 4; -; -. 5. 6. 7. 8; , 5. 9; , 5. 10; , 8; , 7. 11; , 8; , 7. 12; , 8; , 9. 13; , 10; , 9. 14; , 10; , 9. Table 6.4.1.1.3-6: PUSCH DM-RS positions within slot for single-symbol DM-RS and intra-slot frequency hopping enabled. Duration in symbols; DM-RS positions l. PUSCH mapping type A; PUSCH mapping type B l0=0. l0=2; l0=3. dmrs-AdditionalPosition; dmrs-AdditionalPosition; dmrs-AdditionalPosition. 0; 1; 0; 1; 0; 1. 1st hop; 2nd hop; 1st hop; 2nd hop; 1st hop; 2nd hop; 1st hop; 2nd hop; 1st hop; 2nd hop; 1st hop; 2nd hop. ≤3; -; -; -; -; -; -; -; -; 0; 0; 0; 0. 4; 2; 0; 2; 0; 3; 0; 3; 0; 0; 0; 0; 0. 5, 6; 2; 0; 2; 0, 4; 3; 0; 3; 0, 4; 0; 0; 0, 4; 0, 4. 7; 2; 0; 2, 6; 0, 4; 3; 0; 3; 0, 4; 0; 0; 0, 4; 0, 4.

| Company | Comment |
| --- | --- |
| Samsung | OK |
| Qualcomm | We are in general OK with the proposal, but the text may need some offline re-wording |
| ZTE | Generally OK |
| Nokia | OK in general |
| LGE | OK |
| Ericsson | In principle we are fine to clarify, but with the current TP it seems that the case when neither inter nor intra-slot hopping is used is excluded. The TP needs to be revised. |
| DOCOMO | We are fine to clearly, but the case of frequency hopping disabled should be specified (similar view with Ericsson). |
| Intel | OK with TP |
| CATT | OK with TP |

# DM-RS configuration for MU-MIMO

For the MU-MIMO pairing, the configuration for DMRS is required with number of front-loaded DMRS symbols, number of additional DMRS, and DMRS symbol location, configuration type as following IN Section 5.1.6.2 of TS 38.214:

“The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the number of additional DM-RS, the DM--RS symbol location, and DM-RS configuration type as described in Subclause 7.4.1.1 of [4, TS 38.211].”

But for MU pairing, the number of DMRS CDM group(s) without data for different UEs also should be the same, otherwise it will waste of resources or introduce additional interference on DMRS. So, we have the following text proposal:

In Section 5.1.6.2 of 38.214 The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the number of additional DM-RS, the DM--RS symbol location, number of DMRS CDM group(s) without data and DM-RS configuration type as described in Subclause 7.4.1.1 of [4, TS 38.211].

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Support the TP |
| Intel | Need more discussion |
| Qualcomm | We are OK with the TP |
| CATT | OK with the TP. |

# n_SCID for DL in DCI format 1_1

[ZTE, Sanechips] For uplink DMRS, as the editor replied in the email discussion, the existence nSCID in DCI is based on the condition of CP-OFDM. So there is a condition ‘if present’ in the current 38.211 as follows.

The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1 in [4, TS 38.212] is used, otherwise nSCID=0.

However, this condition is unnecessary for DL DMRS because of no DFT-s-OFDM in DL.  In other words, nSCID field always exists in DCI format 0_1 and 1_1 for CP-OFDM. Therefore, we made the corresponding corrections in the following TPs.

TP1 for section 7.4.1.1.1 in 38.211 ##### 7.4.1.1.1 Sequence generation … The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field in the DCI associated with the PDSCH transmission if DCI format 1_1 in [4, TS 38.212] is used, otherwise nSCID=0.. TP2 for section 7.3.1.1.2 in 38.212 ##### 7.3.1.1.2 Format 0_1 ... - DMRS sequence initialization – 0 bit if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled.. TP3 for section 7.3.1.2.2 in 38.212 ##### 7.3.1.2.2 Format 1_1 ... - DMRS sequence initialization – 1 bit..

A similar proposal was shown in vivo (8222):

TS 38.212 V15.2.0 (2018-06) 7.3.1.1.2 Format 0_1 < Unchanged parts are omitted > - DMRS sequence initialization – 0 if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled and both scramblingID0 and or scramblingID1 are is configured in DMRS-UplinkConfig, for selection defined in Subclause 6.4.1.1.1.1 of [4, TS 38.211]. < Unchanged parts are omitted > 7.3.1.2.2 Format 1_1 < Unchanged parts are omitted > - DMRS sequence initialization – 1 bit if and both scramblingID0 and or scramblingID1 are is configured in DMRS-DownlinkConfig for selection defined in Subclause 7.4.1.1.1 of [4, TS 38.211]; 0 bit otherwise. < Unchanged parts are omitted >

Please provide your view on either one proposals shown above, or for another proposal on this topic.

| Company | Comment |
| --- | --- |
| ZTE | The motivation of the two TPs are the same, and the above one is simpler |
| vivo | The two TPs are slightly different. The first one is saying the field is always present. The second one is stating that when no scrambling ID are configured, the field is not present. |

# DM-RS sequence generation for Configured Grant

[NTT DOCOMO] In TS 38.211, DM-RS sequence is generated by using the higher-layer parameters scramblingID0 and scramblingID1 in the DMRS-UplinkConfig IE. In Rel. 15, Configured-Grant is supported. For a UE configured with Configured-Grant, both PUSCH-Config and ConfiguredGrantConfig that have DMRS-UplinkConfig are provided. Hence, when the UE generates DM-RS sequence, UE cannot know which parameter of DMRS-UplinkConfig the UE shall use. We believe this issue is an essential for Configured-Grant, and a TP is required.

- PUSCH-Config information element PUSCH-Config ::= SEQUENCE { […] dmrs-UplinkForPUSCH-MappingTypeA SetupRelease { DMRS-UplinkConfig } OPTIONAL, -- Need M dmrs-UplinkForPUSCH-MappingTypeB SetupRelease { DMRS-UplinkConfig } OPTIONAL, -- Need M […] ... }

- ConfiguredGrantConfig information element ConfiguredGrantConfig ::= SEQUENCE { […] cg-DMRS-Configuration DMRS-UplinkConfig, ... } OPTIONAL -- Need R }

Based on the above discussion, we provide the following TP.

6.4.1.1.1.1 Sequence generation when transform precoding is disabled If transform precoding for PUSCH is not enabled, the sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by PDCCH using DCI format 0_1 with the CRC scrambled by C-RNTI or in the cg-DMRS-configuration IE if provided and the PUSCH is the uplink transmission without grant according to [TS38.214 Section 6.1.2.3]; - is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by using DCI format 0_0 with the CRC scrambled by C-RNTI or in the cg-DMRS-configuration IE if provided and the PUSCH is the uplink transmission without grant according to [TS38.214 Section 6.1.2.3]; - otherwise. The quantity nSCID∈0,1 is indicated by the DM-RS initialization field, if present, in the DCI associated with the PUSCH transmission if DCI format 0_1 in [4, TS 38.212] is used or is configured by dmrs-SeqInitialization in the ConfiguredGrantConfig for the uplink transmission without grant according to [TS38.214 Section 6.1.2.3], otherwise nSCID=0.

6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where is given by clause 5.2.2 with and for a PUSCH transmission dynamically scheduled by DCI. The sequence group , where is given by - if is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE for the PUSCH scheduled by a DCI, or in the cg-DMRS-Configuration for the uplink transmission without grant according to [TS38.214 Section 6.1.2.3], and the PUSCH is not a msg3 PUSCH according to clause 8.3 in [5, TS 38.213]. - nIDRS=NIDcell otherwise

Following is an updated the TP: == Begin == 6.4.1.1 Demodulation reference signal for PUSCH 6.4.1.1.1 Sequence generation 6.4.1.1.1.1 Sequence generation when transform precoding is disabled If transform precoding for PUSCH is not enabled, the sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE in the PUSCH-Config if provided and if the PUSCH is scheduled by DCI format 0_1 with the CRC scrambled by C-RNTI, or in the cg-DMRS-configuration IE in the ConfiguredGrantConfig if provided and if the PUSCH is configured or is scheduled by DCI format 0_1 with the CRC scrambled by CS-RNTI; - is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE in the PUSCH-Config if provided and if the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, or in the cg-DMRS-configuration IE in the ConfiguredGrantConfig if provided and if the PUSCH is configured or is scheduled by DCI format 0_0 with the CRC scrambled by CS-RNTI; - otherwise. The quantity nSCID∈0,1 is indicated by the DM-RS initialization field, if present, in the DCI associated with the PUSCH transmission if DCI format 0_1 in [4, TS 38.212] is used, or by the higher layer parameter dmrs-SeqInitialization, if present, in the rrc-ConfiguredUplinkGrant IE, otherwise nSCID=0. […] 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where is given by clause 5.2.2 with and for a PUSCH transmission dynamically scheduled by DCI. The sequence group , where is given by - if is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE in the PUSCH-Config for the PUSCH scheduled by a DCI with CRC scrambled by C-RNTI, or in the cg-DMRS-Configuration for the uplink transmission without grant according to [TS38.214 Section 6.1.2.3], and the PUSCH is not a msg3 PUSCH according to clause 8.3 in [5, TS 38.213]. - nIDRS=NIDcell otherwise == End ==

| Company | Comment |
| --- | --- |
| NTT DOCOMO | OK. We believe the changed parts for DCI format 0_1 are needed. Please double-check to your grant-free delegates. Type 2 configured grant is RRC + DCI activation, the DCI can be DCI format 0-0 or DCI format 0-1. Type 1 configured grant is purely rely on RRC configuration. |
| Qualcomm | We understand the motivation and are in general supportive, but for re-Tx (NDI=1 for SPS traffic) when DCI format 0_1 is used we could use the PUSCH-Config and not the cg-DMRS-configuration. Either way, it may be more appropriate the UL SPS/grant-free delegates to make this decision. |
| ZTE | The changed parts for DCI format 0_1 are unnecessary |
| Nokia | Similar view with ZTE. |
| LGE | The same view with Qualcomm |
| vivo | Agree with ZTE |
| Intel | OK with DOCOMO proposal |
| CATT | OK with DOCOMO proposal. |

# Correction on the group hopping initialization

[Samsung] The group and sequence hopping are supported for reference signal sequence generation both in SRS and DMRS for PUSCH when transform precoding is enabled. In case of DMRS for PUSCH, the pseudo-random sequence for the parameter v shall be initialized with cinit=nIDRS if sequence hopping but not group hopping shall be used. On the other hand, the pseudo-random sequence for the parameter fgh shall be initialized with cinit=nIDRS/30 if group hopping but not sequence hopping shall be used. In the second case, dividing nIDRS by 30 has no apparent reasons but reduces the randomness and potentially increases the chances of worse cross-correlation. For the same reason, the following agreement was made in RAN1#93 for SRS when group hopping but not sequence hopping shall be used:

Agreement The following text proposal is agreed. ============= Text Proposal in 6.4.1.4.2 in 38.211 ====================== if SRS-GroupSequenceHopping equals ’groupHopping’, group hopping but not sequence hopping shall be used and where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame. ============= End of Text Proposals in 38.211 ====================== is replaced with .

Therefore, we propose to apply the same principle for DMRS for PUSCH when transform precoding is enabled as well.

Proposal 3: Agree on the following text proposal for TS 38.211 v15.2.0 Section 6.4.1.1.1.2 Table 3. A text proposal for TS 38.211 v15.2.0 Section 6.4.1.1.1.2 - if neither group, nor sequence hopping shall be used - if group hopping but not sequence hopping shall be used where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with cinit=nIDRS/30 cinit=nIDRS at the beginning of each radio frame

| Company | Comment |
| --- | --- |
| Samsung | OK |
| Qualcomm | We have a preference to leave the spec as is. Actually, the change for SRS was not required either, and it may have been better if we use the initial agreement |
| ZTE | Prefer to keep the current specification since the changed TP may cause the same value of u for different two N_ID, e.g. for two values N_ID = 1 and 31. |
| Nokia | Same view with Qualcomm. Original text looks clearer than new one, because it shows the level of randomization is upto 30 sequences. Hard to figure out what output will be obtained when no ceiling is. |
| Ericsson, vivo | Not needed, not essential. |
| Huawei, HiSilicon | No need for the TP. Similar view with Qualcomm. |
| DOCOMO | Agree with Samsung’s TP. This change leads different c(i) for N_ID=1 and 31, which would be lower cross-correlation. Also, this should be aligned with SRS. |
| CATT | Not an essential issue. |

# DMRS port mapping

[vivo] In current specification TS 38.211, the description for DMRS port mapping order for both UL and DL is unclear. Based on RAN1 discussion, only single TRP transmission case is supported for DMRS port definition, and DMRS port mapping order is according TS 38.212. To make it more clear, we propose to further clarify the related description.

Proposal (vivo): - The following text proposals for DMRS port mapping are supported. TS 38.211 V15.2.0 (2018-06) 6.3.1.5 Precoding < Unchanged parts are omitted > The block of vectors , shall be precoded according to where , . The set of antenna ports shall be determined according to the procedure in [6, TS 38.214] and DMRS ports are mapped to layer 0,…, in the order described by Tables 7.3.1.1.2-8/9/10/11/12/13/14/15/16/17/18/19/20/21/22/23 in [7, TS 38.212]. < Unchanged parts are omitted > 7.3.1.4 Antenna port mapping < Unchanged parts are omitted > The block of vectors , shall be mapped to antenna ports according to where , . The set of antenna ports shall be determined according to the procedure in [6, TS 38.214] and DMRS ports are mapped to layer 0,…, in the order described by Tables 7.3.1.2.2-1/2/3/4 in [7, TS 38.212]. < Unchanged parts are omitted >.

| Company | Comment |
| --- | --- |
| Vivo | OK Reply to Nokia: For DL, 212 just gives the order of DMRS ports. But the mapping order from DMRS ports to layers 0~v-1 need to be explicitly stated. |
| ZTE | OK |
| Nokia | For DL, it is described in 212 already. Only change need is change reference from 214 to 212. The set of antenna ports shall be determined according to the procedure in [4, TS 38.212] For UL, there are still open issue with antenna port mapping. After finalizing the issue, it can be specified in 212 and 214 |
| Ericsson | Agree with Nokia |
| Huawei, HiSilicon | Antenna port mapping had been described in 38.212 (7.3.1.2.2) as “The antenna ports shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4”. So the second TP is unnecessary. For consistency, we suggest to add PUSCH DMRS port mapping in 38.212 as well. |

# DM-RS reception procedure

[Nokia, NSB] Allow DM-RS and an SS/PBCH block to be multiplexed in the same OFDM symbol, but with non-overlapping PRBs, and the UE prioritize PDSCH reception.

================= Text proposal for TS38.214 =========================== #### 5.1.6.2 DM-RS reception procedure When receiving PDSCH scheduled by DCI format 1_0 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH < Unchanged parts are omitted > If the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may receive the DM-RS for PDSCH, and the UE may or may not receive SS/PBCH block according to UE’s capability. Furthermore, the UE shall not expect to receive DM-RS in resource elements that overlap with those of the SS/PBCH block, and the UE can expect that the same or different subcarrier spacing is configured for the DM-RS and SS/PBCH block in a CC except for the case of 240 kHz where only different subcarrier spacing is supported. ================= End of Text proposal ===========================

| Company | Comment |
| --- | --- |
| Nokia, NSB | OK This is not only related to multi-panel issue. And, parallel discussion exists in simultaneous reception, PDSCH and SSB. (at least outside SMTC window). Current specification, not allow DMRS multiplexing even outside SMTC window. |
| Qualcomm | Do not support. There was an agreement on this early on, and any multi-panel consideration should be post-poned for Rel-16. |
| Ericsson, vivo | Current spec seems overly restrictive, especially for FR1 which seems to preclude this as well, so some more elaborate wording is possible. We think this depends on the outcome of the simultaneous reception discussion (e.g. outside SMTC window). |
| Huawei, HiSilicon | The issue already captured in RS multiplexing summary. |
| DOCOMO | Do not support the above proposal. Scheduling availability on symbols carrying SSB has already discussed and concluded in RAN4. Discussion is not needed in RAN1. |