3GPP TSG-RAN WG1 Meeting #94 	R1-1809776

August 20th  – 24th 2018

Gothenburg, Sweden

Agenda item:	7.1.3.3

Source: 	Qualcomm Incorporated

Title: 	Summary for DL/UL data scheduling maintenance

Document for:	Discussion/Decision

# Introduction

For this meeting, contributions [1-20] highlighted some remaining issues related to maintenance of NR Rel-15. We summarize those and provide some potential proposals for this meeting to correct and complete specification. This is a revision of R1-1809745.

# Causality for CA with cross-carrier scheduling

From the previous RAN1 #93, the following points were noted.

Agreements:

- Cross-carrier scheduling should at least satisfy the causality constraints between scheduling PDCCH and PDSCH as for self-scheduling, also taking into carrier timing difference

  - Note: in the case of mixed numerology, limitations on the number of symbols to buffer need to be taken into account

Agreements:

- For cross-carrier scheduling across different numerology

  - FFS: how to specify additional constraints related to K0 to address the number of symbols which may need to be buffered

In this meeting, several companies [MTK, HW, QC, Intel] provided proposals for cross carrier scheduling causality to reducing e.g., buffering requirements on UE side. Note that current values in the RRC 38.331 for K0 and K1 now include [0…32], so having minimum K0 scheduling requirements would not hurt the flexibility of NR. So such a requirement would improve implementations. Many companies considered a fixed threshold in specification, while [QC] proposed a UE capability.

The following principle is proposed:

Proposal:

- A minimum k0 threshold should be introduced for cross-carrier scheduling with CIF.

- This should be a constraint k0 ≥ X used for scheduling such that the end of the PDCCH symbol on the scheduling carrier is before the start of the corresponding PDSCH on the scheduled carrier.

Proposal:

- Consider adoption of the following table with values X FFS

| Table 1. Proposed minimum values X |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scheduling CC Scheduled CC | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| 15 kHz | 0 |  |  |  |
| 30 kHz | FFS | 0 |  |  |
| 60 kHz | FFS | FFS | 0 |  |
| 120 kHz | FFS | FFS | FFS | 0 |

Additional considerations should be included for FR1 scheduling FR2, as raised by [Qualcomm]

Proposal:

- For the case where SCC in FR2 does not have a configured CORESET, and is cross carrier scheduled via DCI from a different carrier, the DCI shall carry the TCI state information for the reception of PDSCH. In addition, the offset between the reception of DL DCI and DL PDSCH should be greater than the threshold Threshold-Sched-Offset.

  - The minimum k0 threshold should be greater than or equal to Threshold-Sched-Offset.

For k2, there is already a hard requirement for casuality. K2 should accommodate N2 which is defined from the end of the PDCCH on the scheduling carrier, to the start of the PUSCH on the scheduled carrier; It should also accommodate the maximum timing difference between the scheduling and scheduled carriers.

# Simultaneous decoding

The following was captured from RAN1 #93.

Agreements:

- For UE behavior in RRC_IDLE, if PDSCHs among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell

  - NR supports UE to decode up to two PDSCH simultaneously in Pcell, but not more than two (if more than two are received, the decoding prioritization is up to UE implementation)

  - Note: the above assumes that all the PDSCHs to be decoded by the UE have non-overlapped PRBs

Among the submitted contributions to this meeting, [Huawei, Qualcomm, Panasonic] all supported to extend the above agreement to RRC_INACTIVE. It is noted also by Panasonic that UE is not expected to decode two PDSCHs partially or fully overlapping in time for any cell other than PCell. Since the UE capabilities in Release 15 also ensure this operation, it could also be captured in 38.214.

Note that this has implications for both specifications 38.214 and 38.202.

Proposal (offline consensus): The following agreement is extended to include RRC_INACTIVE. Text proposals for 38.214 and 38.202 should also be adopted to capture additional related specification details and corrections.

- For UE behavior in RRC_IDLE and RRC_INACTIVE, if PDSCHs among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell

  - NR supports UE to decode up to two PDSCH simultaneously in Pcell, but not more than two (if more than two are received, the decoding prioritization is up to UE implementation)

  - Note: the above assumes that all the PDSCHs to be decoded by the UE have non-overlapped PRBs

-------------------------------------------------------- Begin Text Proposal 38.214 ----------------------------------------------------------

## 5.1	UE procedure for receiving the physical downlink shared channel

<Unchanged text omitted>

The UE in RRC_IDLE and RRC_INACTIVE mode shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs.

<Unchanged text omitted>

---------------------------------------------------------- End Text Proposal 38.214 ----------------------------------------------------------

-------------------------------------------------------- Begin Text Proposal 38.202 ----------------------------------------------------------

<Unchanged text omitted>

## 6.2	Downlink

The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities, and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported.

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| A | PBCH | N/A | BCH |  |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH | Note 1 |
| C0 | PDCCH | P-RNTI | N/A | Note 2 |
| C1 | PDCCH+PDSCH | P-RNTI | PCH | Note 1 |
| D0 | PDCCH+PDSCH | RA-RNTI or Temporary C-RNTI | DL-SCH | Note 1 |
| D1 | PDCCH+PDSCH | C-RNTI, CS-RNTI, [new RNTI] | DL-SCH |  |
| E | PDCCH | C-RNTI | N/A | Note 3 |
| F | PDCCH | C-RNTI, CS-RNTI, [new RNTI] | UL-SCH |  |
| G | PDCCH | SFI-RNTI | N/A |  |
| H | PDCCH | INT-RNTI | N/A |  |
| J0 | PDCCH | TPC-PUSCH-RNTI | N/A |  |
| J1 | PDCCH | TPC-PUCCH-RNTI | N/A |  |
| J2 | PDCCH | TPC-SRS-RNTI | N/A |  |
| K | PDCCH | SP-CSI-RNTI | N/A |  |
| Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: This corresponds to PDCCH-ordered PRACH. |  |  |  |  |

Table 6.2-2: Downlink "Reception Type" combinations

| UE capability | Supported Combinations |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | PCell | PSCell | SCell |  |
| 1. RRC_IDLE |  |  |  |  |
|  | A + (B and/or C1 and/or D0) |  |  | Note 1 |
| 2. RRC_INACTIVE |  |  |  |  |
|  | A + (B and/or C1 and/or D0) |  |  | Note 1 |
| 3. RRC_CONNECTED |  |  |  |  |
|  | A + C0 + (B and/or (D0 or D1)) + E + F + G + H + J0 + J1 + J2 + K | A + D1 + E + F + G + H + J0 + J1 + J2 + K | D1 + F + G + H + J0 + J1 + J2 + K | Note 2 |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. |  |  |  |  |

---------------------------------------------------------- End Text Proposal 38.202 ----------------------------------------------------------

During Monday morning offline discussions, there was also a proposal to capture the following text in 38.214.

“On other than PCell, the UE is not expected to decode more than one PDSCH if partially or fully overlap in time”.

The was raised because of the following

- For cells other than PCell, there is no broadcast PDSCH expected to be overlapping in time with unicast PDSCH.

- For unicast PDSCH, from UE perspective multiple unicast PDSCH are not expected to be overlapping in time (i.e., for UE capabilities currently defined, e.g. RP-181483 from RAN #80, there is no capability to allow for overlapping PDSCH)

  - Some companies thought that for URLLC, this should be allowed. However, such a capability would need be agreed in RAN1 for addition into the UE feature list.

The specification should ideally capture this text, however if not captured the same understanding is still the current status of Rel-15.

# UE Processing Time Corrections

The following corrections are provided to align specification with agreements.

## Corrections related to HARQ-ACK

A number of corrections for processing time of HARQ-ACK were reported this meeting. We provide a quick summary here and a text proposal below which may resolve these issues.

- Huawei and Nokia indicating corrections needed to in relation the allocation duration.

- Huawei, Spreadtrum, Nokia, and Samsung dditionally indicated that Capability #2 was not agreed to be defined for case of additional DMRS configured, and so should be captured consistently in specification.

- Ericsson and CATT both noted that there may be some inconsistencies between the references to processing time in 38.213 subclause 9.2.5 and 38.214 subclase 5.3.

Offline Notes from Monday:

The following specification alignment is needed between 38.213 and 38.214, and further refinement of the text should be done to capture the following aspects:

- Current 38.213 uses 38.214 timelines for pre-multiplexed UCI channels separately to determine whether certain overlapping channels should be multiplexed, however the some of the timelines in 38.214 assume that channels have been multiplexed. Therefore, the Tproc approach below may need to be refined so as not to be circular.

- A-CSI needs to be captured for d1,1

- Tproc,CSI for CSI computation time may need to be captured for reference within 38.213 from CSI computation time which include impact of d, and also Tproc for SPS Release

  - Check for any further cross reference to N1,N2,…

- SFI text in Section 11 of 38.213 should also be checked for consistency, as mentioned from [CATT].

Note the following proposals will be revised based on the above notes.

Proposal: Adopt the following text proposals for 38.214 and 38.213 to align specification with agreements made from previous meetings.

-----------------------------------------------------Begin Text Proposal for 38.214 ----------------------------------------------------------

<Unchanged text omitted>

## 5.3	UE PDSCH processing procedure time

If the first uplink symbol of the physical channel which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUSCH or PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after  after the end of the last symbol of the PDSCH carrying the TB being acknowledged.

-	N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.1 of [4, TS 38.211].

-	If HARQ-ACK is transmitted on PUCCH, then d1,1 = 0,

-	If HARQ-ACK is transmitted on PUSCH, then d1,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol which carries the HARQ-ACK information further includes the effect of timing difference between the component carriers as given in [11, TS 38.133].

-	For PDSCH is mapping type A as given in subclause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,2 = 7 - i, otherwise d1,2 = 0.

-	For UE processing capability 1: If the PDSCH is mapping type B as given in subclause 7.4.1.1 of [4, TS 38.211], and

- 	if the number of PDSCH symbols allocated is 7, then d1,2 = 0,

-	if the number of PDSCH symbols allocated is 4, then d1,2 = 3,

-	if the number of PDSCH symbols allocated is 2, thend1,2 = 3+d, where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2: If the PDSCH is mapping type B as given in subclause 7.4.1.1 of [4, TS 38.211],

- 	if the number of PDSCH symbols allocated is 7, then d1,2 = 0,

- 	if the number of PDSCH symbols allocated is 2 or 4, then d1,2 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2 with scheduling limitation when µ = 1, if the scheduled RB allocation exceeds 136 RBs, the UE defaults to capability 1 processing time.

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of Tproc,1 is used both in the case of normal and extended cyclic prefix.

<Unchanged text omitted>

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 8 | 13 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |  |
| 0 | 3 |  |
| 1 | 4.5 |  |
| 2 | 9 for frequency range 1 |  |

-----------------------------------------------------End Text Proposal for 38.214 ------------------------------------------------------------

-----------------------------------------------------Begin Text Proposal for 38.213 ----------------------------------------------------------

### 9.2.5	UE procedure for reporting multiple UCI types

<Unchanged text omitted>

If a UE would transmit multiple overlapping PUCCHs without repetitions in a slot or overlapping PUCCH(s) without repetitions and PUSCH(s) in a slot and, when applicable as described in Subclauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met.

If one of the PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE expects that the first symbol of the earliest PUCCH or PUSCH, among the overlapping PUCCHs and PUSCHs in the slot, is not less than Tproc,1 after a last symbol of any corresponding PDSCH or SPS PDSCH release, and is not less than Tproc,2 after a last symbol of any corresponding PDCCH  where Tproc,1 and Tproc,2 are defined in [5.3 and 6.4, TS 38.214]. A UE does not expect a PUCCH or a PUSCH that is in response to a DCI format detection to overlap with any other PUCCH or PUSCH that does not satisfy the above Tproc,1 and Tproc,2 timing conditions.

If a UE would transmit multiple PUCCHs in a slot that include HARQ-ACK/SR and CSI and PUCCHs with HARQ-ACK satisfies the above Tproc,1 and Tproc,2 timing conditions and does not overlap with any other PUCCH or PUSCH that does not satisfy the above Tproc,1 and Tproc,2timing conditions, the UE multiplexes HARQ-ACK/SR and CSI and determines corresponding PUCCH(s) for transmission in the slot according to the following pseudo-code.

-----------------------------------------------------End Text Proposal for 38.213 ------------------------------------------------------------

## Corrections related to CSI report multiplexing

In the previous meeting RAN1 #93, the following was agreed.

Agreements:

In the case of multiplexing CSI with uplink data on PUSCH, the MIMO session should update agreements related to CSI multiplexing with UL-SCH (i.e., data on PUSCH), and NR should adopt the framework below:

- Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective

  - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session

- N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d symbols

  - FFS value of d <= N2

    - E.g., d=1

    - E.g., d=1 for 15kHz, 2 for 30kHz, 3 for 60kHz, and 4 for 120kHz

    - E.g., d=N2

- UE is not expected transmit the CSI multiplexed with uplink data if the network set the values of K2 and SLIV without leaving sufficient time for UE processing

Agreements:

- d in the above agreement is updated to the following

- 2 for 15kHz

- 2 for 30kHz

- 3 for 60kHz

- 4 for 120kHz

Agreements:

In the case of multiplexing CSI, HARQ-ACK, and uplink data on PUSCH

- N1’ the number of OFDM symbols required for UE processing from the end of PDSCH to the earliest possible start of the corresponding ACK/NACK transmission on PUSCH from UE perspective

  - N1’ ≥ N1 + d where N1 is based on the UE capability for ACK-only

- Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective

  - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session

- N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d symbols

  - 2 for 15kHz

  - 2 for 30kHz

  - 3 for 60kHz

  - 4 for 120kHz

- UE is not expected transmit the CSI multiplexed with HARQ-ACK and with uplink data if the network set the values of K1, K2 and SLIV without leaving sufficient time for UE processing

Both ZTE and MTK noted that current specification was not completely in line with the above agreements for PUSCH preparation procedure time, so the following proposal is considered from MTK. Samsung also raises issues in the specification capturing agreements from the MIMO session regarding CSI computation time, however we leave these up to the feature lead for CSI reporting agenda item (note that summary has already captured the concerns from Samsung).

Proposal: The following text proposal for 38.214 should be adopted to align specification with agreements.

-----------------------------------------------------Begin Text Proposal for 38.214 ----------------------------------------------------------

## 6.4	UE PUSCH preparation procedure time

< Unchanged parts are omitted >

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	 If A-CSI is multiplexed on PUSCH, d2,2  = 2/2/3/4 for SCS = 15/30/60/120Hz, respectively; else if HARQ-ACK is multiplexed on PUSCH without A-CSI, d2,2  = 1; otherwise d2,2  = 0.

-----------------------------------------------------End Text Proposal for 38.214 ------------------------------------------------------------

# PUCCH resource adaptation and UE processing time

The following issue has been raised by MTK and should be concluded this meeting. The PUCCH resource determination is based on a PUCCH resource indicator field in the last DL assignment. When HARQ-ACKs of different PDSCH of different time instance are to be transmitted within one slot, the PUCCH resource may be updated from UE perspective. In some cases, as shown in Figure 7 from [MTK], this may arise in very fast processing times somewhat inconsistent with the UE’s capability, i.e., by the time the DCI has been decoding from Slot #(n+1), the PUCCH resource 1 has been transmitted and so the UE cannot fulfill the specification requirement of one HARQ-ACK PUCCH per slot (which should be sent on PUCCH resource 2 following the latest DCI).

[MTK] Figure 7. Example of the critical issue in case there is no any restriction on adaptation of PUCCH resource

Noting the proposals in [MTK], one natural proposal to at least budget some time for PDCCH decoding and ensure this is met otherwise this is treated as an error condition. After offline discussion Monday morning, most companies agreed that this was an issue which needed to be resolved. Moreover, given the state of Rel-15, the down-selected proposal to use the same as “N1” is provided below since it had the most support.

Proposal (discussed jointly with UCI agenda):

- If a first DCI has indicated a first PUCCH resource for HARQ-ACK bits associated to the PDSCH scheduled by this DCI

  - If due to a second DCI, additional HARQ-ACK bits are to be multiplexed on the same slot as the first PUCCH resource, the UE is not expected to multiplex additional HARQ-ACK bits if the last symbol of second DCI is not earlier than N OFDM symbols before first symbol of the first PUCCH resource, where for UE Capability #1 the N values are:

    - 15 kHz case: 8

    - 30 kHz case: 10

    - 60 kHz case: 17

    - 120 kHz case: 20

    - FFS values to be used for Capability #2 (e.g., 3, 4.5, and 9 for 15kHz, 30kHz, and 60kHz respectively)

Note that similar timing (slightly larger) was decided for DL SPS release.

Agreements:

The following working assumption is confirmed with updates:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1:

  - 15 kHz case: 8 + 2 5

  - 30 kHz case: 10 + 2 5

  - 60 kHz case: 17 + 5

  - 120 kHz case: 20 + 5

# Capability #2 open items

The following agreements were made in RAN1 #93 with regard to Capability #2 advanced UE processing time. Below we address some of the FFS within these agreements.

Agreements:

The Capability #2 for (Aggressive) UE processing time in Rel-15 is supported under the following conditions

- Non-CA

  - Note: this does not preclude EN-DC

- FFS CA case with Capability #2 supported on only one or more of the carriers, and potential handling of some special cases

- Single numerology for PDCCH, PDSCH, and PUSCH for the serving cell

- PDSCH/PUSCH allocation with mapping Type A and Type B

  - For PDSCH mapping type A with last PDSCH symbol ending in symbol ‘i' of a slot, where i < 7

    - N1 processing time is increased by (7-i) relative to the case where i=7.

  - (Working assumption) For PDSCH mapping type B with 4 or 2 symbols

    - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

      - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH

- No UCI multiplexing

  - FFS: whether similar multiplexing rule as with Capability #1 may be included

- For C-RNTI

  - FFS: simultaneous reception with broadcast PDSCH

- Note: The UE signals whether Capability #2 is supported for each SCS, and separately for uplink and downlink

Agreements:

The Capability #2 UE processing times are given below, where the PDSCH allocation length is assumed to be at least 7 symbols.

- N1

  - 15kHz: N1 = 3

  - 30kHz: N1 = 4.5

    - Note: as part of UE capability,

      - UE may report support with maximum scheduled RB allocation of 136 RBs

        - If RB allocation by scheduling exceeds the maximum signaled, UE defaults to Capability #1 processing time

      - OR UE may report support with no restriction on maximum scheduled RB allocation

  - 60kHz for FR1: 9

- N2

  - 15kHz: N2 = 5 for CP-OFDM & For DFT-S-OFDM

  - 30kHz: N2 = 5.5

  - 60kHz for FR1: 11

  - If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2.

## PDSCH with 2-symbols and 4-symbols

Both Huawei and Intel proposed to confirm the working assumption and also address the case of 3-symbol CORESET overlap by adding an extra symbol of processing time. Ericsson proposed to confirm but wanted to remove any special handling for 3-symbol CORESETs. Given the complications on the receive side, as well as how useful the case of a control duration finishing after the data allocation in a low latency service, we suggest taking the proposal from Intel below with some modification.

An important note was that the exception for 3-symbol CORESET raised questions regarding how this should be handled when there is also a 2-symbol CORESET. For instance, UE processing does not necessarily reflect the shortest CORESET (since search spaces and candidates are not exactly reflective of CORESET lengths and sizes).

Proposal: Confirm the following working assumption for N1 with Capability #2 and the following change

  - (working assumption) For PDSCH mapping type B with 4 or 2 symbols

    - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

      - For the case 2-symbol PDSCH, the N1 processing time is increased by d symbols relative to the case of PDSCH with 7 symbols, where d counts the largest overlap between the PDSCH and any CORESET which can schedule the PDSCH.

Given the above, we introduce an analog for the uplink.

Proposal: For N2 with Capability #2, the UE processing time N2 is counted from the end of the latest CORESET search space which can schedule the PDSCH, rather than from the end of the scheduling PDCCH.

## CORESET Bandwidth and #TB limitations

Qualcomm mentioned that having a limit on scheduled bandwidth in the previous meeting should be extended to the CORESET bandwidth so that there is some consistency in the limitations. The following simplified proposal is included here for discussion.

Proposal: If UE has indicated support for Capability #2 with a maximum number of scheduled RBs = 136

- Then the UE does not expect any CORESET larger than 136 RBs to schedule with Capability #2 processing time. The UE follows Capability #1 otherwise.

MTK mentioned that it would be important to now separately report the UE capability for #TBs per slot between Capability #1 and Capability #2, now that there has been adoption of Capability #2. The following is proposed for discussion.

Proposal 3:

- The supported #PDSCH per slot can be reported for separately for UE processing time capability#1 and capability#2.

- The supported #PUSCH per slot can be reported for separately for UE processing time capability#1 and capability#2.

## Support for CA

Several companies mentioned support for extended Capability #2 to CA, though there were more proposals limiting the number of carriers to 1 at most for Capability #2 [Intel, MTK, Panasonic], while [Qualcomm, Huawei, Ericsson] provided various other proposals to allow multiple carriers with Capability #2. For instance, Huawei provides a limitation on the number of carriers to support while Qualcomm indicates an aggregate bandwidth. Panasonic makes the point that at least there may be need for FR1 + FR2 capable devices to support CA across FR1 and FR2, while allowing also for URLLC operation in FR1.

Given the inputs across companies this meeting, the following is proposed as a compromise among the companies for discussion this meeting.

Proposal:

The following UE capabilities should be reported to the network

- The number of carriers that can be scheduled with Capability #2

  - Note: this is a separate capability from the # of carriers supported with Capability #1 only

  - This capability should be signaled as Type 3

  - This should be separately signaled for the UL and DL

  - No scheduling restriction on number of scheduled RB on any CC

  - The UE only supports CC self-scheduling with Capability #2

  - FFS: if UE signals additional # of CC’s that can support Capability #1

    - FFS: if CC must be configured for Capability #2 operation

## Dynamic operation between Capability #1 and Capability #2

Both Intel and Qualcomm raised issues that in cases where there is a BW limitation for Capability #2, is it important to ensure efficient pipelining and no hardware contention for such operation. Therefore, the following compromise proposal is considered below to reduce the scope.

Proposal (working assumption): For a UE indicating Capability #2 N1 support with maximum scheduled RB allocation of 136 RBs

- The UE is not expected to provide a valid HARQ-ACK with Capability #2 processing time until all HARQ processes under Capability #1 have been completed

  - i.e., until N symbols after last PDSCH scheduled with more than 136 RBs, where the value of N depends on the SCS of the PDSCH

    - 15 kHz case: [8]

    - 30 kHz case: [10]

    - 60 kHz case: [17]

    - 120 kHz case: [20]

(Alternative Proposal)

- The UE may be allowed to drop the processing of a number of PDSCHs that may be scheduled to follow Capability #1 such that they are scheduled within a window ‘W_N1’ before the start of a PDSCH that is scheduled to follow Capability #2.

  - The window W_N1 is defined as: W_N1 = {1, 2, …, N1_cap1 (=10)}, with the exact value of W_N1 indicated by the UE to the network as a UE capability.

  - Additional margin to N1_cap1 considering special cases like short PDSCH durations with/without overlaps with scheduling PDCCH may be considered as well.

## Simultaneous reception with broadcast

Panasonic proposed to require simultaneous reception of broadcast messages while maintaining Capability #2 operation, while Qualcomm proposed to follow the procedure for FR2 and prioritize C-RNTI or SI-RNTI. Given the added complexity for handling processing time with Capability #2, and the complications already addressed in the case of Capability #1 from prior meetings, the proposal from Qualcomm is provided below for discussion.

Proposal: The following agreement is amended to include the new case of Capability #2 unicast traffic in FR1:

- While UE acquires SI upon being triggered by Paging DCI

  - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol

- In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH

- The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2976 for FR1 and the UE is only scheduled C-RNTI PDSCH under operation with Capability #1 processing time, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH

    - Note: the above assumes that the PDSCHs to be decoded by the UE have non-overlapped PRBs

- The first two bullets always apply in FR2

## Additional items

Regarding UCI multiplexing, there were differing view e.g., from Ericsson and Panasonic. Ericsson preferred no relaxation while Panasonic suggested relaxation, and in some cases restricted the operation of UCI multiplexing for new MCS or new RNTI.

For this meeting, further discussion may be needed on the following proposal below which takes the same relaxation as with Capability #1.

Proposal:  For Capability #2, the processing time for UCI multiplexing should be extended by d symbols using the same values and conditions as used for Capability #1.

Regarding SPS, the previous RAN1 #93 agreement is extended to Capability #2.

Agreements:

The following working assumption is confirmed with updates:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1:

  - 15 kHz case: 8 + 2 5

  - 30 kHz case: 10 + 2 5

  - 60 kHz case: 17 + 5

  - 120 kHz case: 20 + 5

Proposal:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #2:

  - 15 kHz case: [5]

  - 30 kHz case: [5.5]

  - 60 kHz case (FR1 only): [11]

-----------------------------------------------------Begin Text Proposal for 38.213 ----------------------------------------------------------

## 10.2	PDCCH validation for DL SPS and UL grant Type 2

<Unchanged text omitted>

A UE with capability 1 [6, TS 36.214] is expected to be able to provide HARQ-ACK information in response to a DL SPS scheduling release after N symbols from the last symbol of a PDCCH providing the DL SPS scheduling release where, for the subcarrier spacing of the PDCCH reception, N=10 for 15 kHz, N=12 for 30 kHz, N=22 for 60 kHz, and N=25 for 120 kHz.

A UE with capability 2 [6, TS 36.214] is expected to be able to provide HARQ-ACK information in response to a DL SPS scheduling release after N symbols from the last symbol of a PDCCH providing the DL SPS scheduling release where, for the subcarrier spacing of the PDCCH reception, N=5 for 15 kHz, N=5.5 for 30 kHz, N=11 for 60 kHz.

-----------------------------------------------------End Text Proposal for 38.213 ------------------------------------------------------------

# Configurability of K1 and K0/K2

This meeting, Ericsson proposed to amend K1 values to address some TDD configurations for which the current set of RRC parameters would be insufficient. Currently, the set is only limited to the values from 0 to 8, though the field may be larger.

From the last meeting, there was an observation that the set of RRC values for K0 are also limited and cannot properly address all configurations for CA with mixed numerology. Therefore, is was proposed to add values 6 and 7 to be complete and aligned with K2.

This meeting we combine these two proposals.

Proposal:

Send LS to RAN2 to requesting if they can make [0..15] applicable for K1.

- Note that from 38.331 vf2, RAN2 already has [0…15] as values.

dl-DataToUL-ACK							SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)											OPTIONAL,	-- Need M

- But in the field description below, the range is limited. Removing this limitation in the field description will enable the requested change.

PUCCH-Config field descriptions

dl-DataToUL-ACK List of timing for given PDSCH to the DL ACK. In this version of the specification only the values [0..8] are applicable. Corresponds to L1 parameter 'Slot-timing-value-K1' (see TS 38.213, section FFS_Section).

Proposal:

The following are noted by RAN1 as already supported in 38.331

- The scheduling parameters K0 and K2 may be configured from the set of values {0,1,2,3,…,32}

# Rx-to-Tx Switch Times

From the previous meeting, and LS from RAN4 was provided stating that an upper bound must be placed on UE Rx/Tx and Tx/Rx transition times. These times impact which slot formats may be supported by a UE, for a given SCS. As such, a table was provided below.

| Transition/FR | FR1 | FR2 |
| --- | --- | --- |
| TRX2TX | 13s | 7s |
| TTX2RX | 13s | 7s |

RAN4 requested RAN1 to use the above UE transition times while determining slot format and/or any other dependent system level parameters. However, at the previous RAN1 #93 only the following was agreed with only focus on Rx to Tx.

Agreements:

- UE is not required to receive on a downlink symbol and then transmit on a uplink symbol if those two symbols are not separated by at least Rx2Tx us on unpaired spectrum for a given serving cell, from the UE perspective

  - Discuss further whether it’s an error case or to specify a UE behavior

  - Note that the exact value of Rx2Tx has been specified in RAN4 [R4-1805766]

Moreover the resulting text in 38.211 capturing the agreement did not reference the appropriate specification for this these values. An except is given below.

“A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by [TS 38.101].”

For this meeting, it was noted by several companies [Intel,Qualcomm] that this needed correction, while others also brought proposals that this should be confirmed as an error case [Panasonic, Huawei, Qualcomm]. Addition concerns were raised by [DCM] to provide more specific reference to CA, while [WILUS] asked for reference to PRACH, although the text within 38.211 does in some sense still readily apply to those cases.

Note that in the 7.1.3.3 summary on resource allocation, while is lead by the editor of 38.211, this same issue is also addressed with a text proposal which reflects the contribution of [Intel].

Proposal: Discuss text proposals for Rx-to-Tx and Rx-to-Rx switching times in resource allocation agenda of 7.1.3.3.

# References

- R1-1808105	Remaining issues on data scheduling and HARQ	Huawei, HiSilicon

- R1-1808210	Remaining Issues of NR scheduling and HARQ process	ZTE

- R1-1808226	Remaining issues on scheduling and HARQ	vivo

- R1-1808261	Remaining issues of DLUL data scheduling and HARQ procedure	MediaTek Inc.

- R1-1808380	Corrections to DL/UL scheduling and HARQ management	CATT

- R1-1808492	Discussion on DL/UL data scheduling and HARQ procedure	LG ELECTRONICS

- R1-1808674	NR scheduling and HARQ procedures	Intel Corporation

- R1-1808755	Remaining Issues on UL/DL Scheduling	Samsung

- R1-1808808	Remaining issues on DL/UL data scheduling and HARQ procedure	Spreadtrum Communications

- R1-1808870	Maintenance for DL/UL data scheduling and HARQ procedure	Panasonic Corporation

- R1-1808887	Text proposal for DL/UL data scheduling and HARQ procedure	Guangdong OPPO Mobile Telecom

- R1-1809051	38.214 Text Proposal to align PUSCH-AllocationList and PDSCH-AllocationList to ASN.1	SPRINT Corporation

- R1-1809060	Remaining details on PDSCH time domain resource allocation	AT&T

- R1-1809110	Remaining issues on Msg3 PUSCH frequency resource allocation	Sharp

- R1-1809143	Maintenance for DL/UL data scheduling and HARQ procedure	NTT DOCOMO, INC.

- R1-1809297	Discussion on Remaining issues on HARQ-ACK bundling and multiplexing	China Telecommunications

- R1-1809322	Remining issues on DL/UL data scheduling and HARQ procedure	WILUS Inc.

- R1-1809392	Remaining issues on resource allocation and UE processing times	Nokia, Nokia Shanghai Bell

- R1-1809407	Maintenance issues of DL/UL data scheduling and HARQ procedure	Ericsson

- R1-1809428	Maintenance for DL/UL data scheduling and HARQ procedure	Qualcomm Incorporated

- Chairman’s Notes, RAN1 #92bis, April 2018, Sanya, China.

- Chairman’s Notes, RAN1 #93, May 2018, Busan, Korea.

- R4-1805766, “LS on UE Rx/Tx Transition Times”, RAN4 LS to RAN1