3GPP TSG-RAN WG1 Meeting #94bis	R1-1811128

# Chengdu, China, October 8th – 12th, 2018

Agenda Item:	7.2.9.2.2

Source:	Apple Inc.

Title:	UE-assisted Approaches for UE Power Saving

Document for:	Discussion, Decision

# Introduction

To reduce the power consumption at UE side, UE can take advantage of the information available inside handset such as file size, traffic arrival rate, application layer traffic’s throughput/delay requirements, implementation specific power consumption characteristics of RF and BB for different frequency, bandwidth, number of carriers, scheduling time line etc. Some of this information could be directly used at UE side for reducing power, but in most cases this information should be transferred to network to help network configure/schedule UE such that the UE consumes less power. This is because the power consumption of UE highly depends on UE configuration, and the configuration is done by the network.

In this paper, we discuss UE-assisted techniques for UE power saving.

# UE-assisted Techniques for Power Saving

In NR, we have new features to support higher data rates applications (eMBB) entailing higher power consumption. To overcome increased power consumption, techniques such as BWP and periodic PDCCH monitoring were introduced.

## Bandwidth Part

NR supports higher data rate with wider bandwidth (up to 400MHz for single carrier). This requires much higher rate of data processing in both RF side and baseband side; supporting higher sampling rate, smaller slot lengths, higher encoder/decoder throughput, faster clocks, higher voltage (with higher leakage), larger memory, and LTE-NR dual connectivity require significantly higher power consumption. Especially, the support of mmWave requires additional power consumption for beam forming operation and beam management.

The BWP and cross-slot scheduling were introduced to overcome the increased power consumption. Network can use BWPs to adapt bandwidth to the demand for data transmission/reception. Network can configure narrow BWP for monitoring PDCCH only and wider BWP for receiving large-sized data. Channel bandwidth adaptation using BWP is a quite effective power saving technique.

However, whether UE can expect power saving or not is totally up to network configuration / scheduling decision. If network does not configure multiple BWPs with different sizes or if network does not change BWPs considering traffic demand and UE power consumption, then, even with multiple different BWPs configured, there will be no power saving at UE side (e.g., the case when BWP is used just for load balancing purpose). Similar example in LTE is DRX configuration. If only very simple configuration is used due to network complexity, then it is hard to expect intended power saving gain at UE side. UE having better knowledge on channel status and traffic demand/pattern/usage may help network to make better decisions for both BWP configuration and scheduling in both CONNECTED mode and IDLE mode as shown in Figure 1.

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for both BWP configuration.

Figure 1 Current BWP framework (left) and improved BWP framework 
with UE preferred BWP configuration (right)

### UE-assisted BWP Configuration

If UE can indicate its preferred BWP configuration, then UE can potentially achieve higher power saving. It is clear that UE has more information about device owner’s preference on applications, types/amount/duration of traffics to be received/transmitted, etc. This kind of information could be used in conjunction with internal power consumption statistics to determine the right size of BWP. Another reason is that different UE could have different RF capabilities and different implementations. This means optimal BWP for a certain traffic could be different for different UEs.

For example, the set of supported sampling rates could be different across UEs. This means that it can happen that, e.g., a UE A is configured with a 6MHz BWP with sampling rate of 8MHz, while another UE B is configured of a 6MHz BWP with sampling rate of 10MHz. This implies that the power consumption of using the 6MHz BWP could be different between UE A and UE B and it might be better for UE B to be configured with a BWP larger than 6MHz (e.g., 10MHz) since the same sampling rate is used anyway. UE have additional knowledge on current traffic from application layer and their arrival characteristics. In this case, UE could indicate its preferred BWP configurations which may be suitable for such traffic.

Furthermore, we see that there are strong motivation for network that it configures larger BWP (and larger number of component carriers (CC)). It is because that, with larger BWPs and larger number of CCs, network can keep its scheduling flexibility high to improve network capacity. Thus, in many cases, we see that component carriers are active without much data transmission. In such case, UE should be able to request to use a smaller BWP or turn off unused/idling component carriers.

Proposal 1

- NR support that UE sends UE-preferred BWP configuration to network for UE power saving.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices.

  - UE can indicate its preferred BWP switching timer.

## PDCCH Monitoring

Another important new feature in NR is the support of periodic PDCCH monitoring before UE enters CDRX state. With this feature, UE can monitor PDCCH less frequently and accordingly the chance that UE receives PDCCH (with grant) increases. Consequently, it can reduce the amount of power wasted for PDCCH only monitoring. This kind of approaches of controlling the so-called duty cycle of UE’s ON state could be promising approach for UE power saving.

### Periodicity

With the introduction of mmWave, the importance of periodic PDCCH monitoring gets even higher since it is power consuming operation to monitor slots with short length continuously in mmWave. The current R15 NR supports up to 20 slots of PDCCH monitoring periodicity which corresponds to 20ms with SCS=15kHz. However, since the periodicity was defined in the unit of slot, it corresponds to only 2.5ms with SCS of 120kHz. Due to the design/decision, the maximum time that UE can skip PDCCH monitoring in absolute time depends on SCS and it is not desirable/enough for NR in FR2 to achieve similar power saving gain as NR in FR1.

In Figure 2, we show the PDCCH monitoring periodicity of 10 slots as an example for two different SCS=15kHz and 120kHz. For 15kHz case, UE monitors the second slot of one periodicity and takes a deep sleep for the rest of the period with no PDSCH scheduled. While in 120kHz case, UE has to wake up every 1.25ms (10 slots in 120kHz). Due to associated overhead (RFIC ramp up & RFIC ramp down) and short sleep time (which allows UE to enter shallow sleep state only), the expected power saving gain is much lower than the case of 15kHz.

Observation 2

- In FR2, it is hard to achieve similar power saving gain as FR1 with currently defined PDCCH monitoring (in general duty cycle) periodicities.

Proposal 2

- Increase the maximum PDCCH monitoring periodicity to support longer monitoring periodicity in FR2.

Figure 2 Expected power consumption with PDCCH monitoring periodicity of 10 slots for SCS=15kHz (up) and 120kHz. For 15kHz case, UE can enter deeper sleep state. While for 120kHz case, UE cannot enter deep sleep state due to too short PDCCH monitoring periodicity.

Since the monitoring periodicity has direct impact on UE power consumption, UE can indicate preferred PDCCH monitoring periodicity based on its expected traffic arrival rate and power consumption preference.

Proposal 3

- NR support the UE indication of its preferred PDCCH monitoring periodicity for UE power saving.

Note that the selection of monitoring periodicities limits UE’s PDCCH monitoring slots and accordingly limits the chance that UE can receive PDSCH, which could potentially limit the supported throughput. The problem of throughput limitation and solution is discussed in our companion paper [5].

## Scheduling Technique for UE Power Saving

NR has very flexible HARQ scheduling timing. Unlike LTE with fixed HARQ timing, the transmission timing for NR’s PDSCH and corresponding ACK can be dynamically determined and indicated by DCI. The increased flexibility makes network scheduler much easier to serve multiple UEs with different processing capability, different traffic demand, different latency requirement, etc.

We see that self-contained slot scheduling could potentially help UE power saving. For example, note that a UE which receives PDCCH in slot N, PDSCH in slot N+4, and sends ACK in slot N+8 can enter micro sleep state between slot N and N+4 and between N+4 and N+8. If network can schedule this UE with PDCCH, PDSCH, and ACK all in the same slot N (self-contained slot scheduling), then the UE can quickly finish data reception and enter deeper sleep mode for longer time, which save more energy as shown in Figure 4. Since there is power cost for ramp up and ramp down, it is better to reduce those power cost and make modem sleeps longer.

Figure 3 Reduced power consumption of self-contained slot scheduling compared slot scheduling with K0>0 and K1>0 illustrating the reduced power consumption (dark green) and lower power consumption during sleep

Another important factor to consider is potential PDSCH buffering. If K0=0 could be potentially scheduled (or configured to UE in time domain resource allocation table), then UE has to buffer PDSCH symbols until PDCCH decoding finishes which is typically around 6~7th OFDM symbols (for 15kHz) in a slot. This is because the UE cannot determine whether there is potential PDSCH being transmitted or not in the current slot. Figure 4 shows the two cases when K0=0 could be/ could not be potentially signaled.

Figure 4 The potential PDSCH buffering is done until PDCCH decoding finishes if K0=0 could be potentially signaled (left). While, if K0=0 is not RRC-configured in UE’s resource allocation table, then UE can enter sleep state early (right).

To avoid unnecessary buffering, it is recommended to avoid having K0=0 in the RRC-configured table. To realize this, we think UE should be able to indicate network its preferred K0, K1, K2 values. One example set of those values would be K0=2, K1=0, K2=0; where K0=1 allows UE to avoid unnecessary PDSCH buffering, K1=0 makes to avoid unnecessary power consumption due to ramp down and ramp up between PDSCH and ACK, and K2=0 reduces unnecessary ramp down and ramp up time between DCI and PUSCH. Figure 3 illustrates the case with K0=2, K1=0 with PDCCH monitoring periodicity of 3. PDSCH reception and ACK transmission is delayed to the last slot of PDCCH monitoring period to avoid unnecessary ramp up and ramp down.

Figure 5 Cross slot scheduling with immediate ACK followed by PDCCH monitoring (K0=2, K1=0, P=3)

Proposal 4

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

In our companion paper[5], we have related discussion on temporary K0 value restriction for UE power saving.

## UE-assisted DRX configuration

DRX mechanisms is one of the key component mechanism for UE power saving, but it is less utilized in LTE. For example, it has been observed that many LTE networks use only limited set of DRX parameters and they are hardly changed. Long inactivity timer value (~200ms) is one of such examples. Another important observation we made is that in many cases the same DRX configuration is applied to all the UEs implying that network does not usually take into account different characteristics of UE’s traffic. The only exception we have observed is VoLTE case. When VoLTE call is going on, network changes DRX parameters to support it in more power efficient manner. Considering that NR DRX design is essentially is not different from that of LTE, we expect that NR network would have similar configurations as LTE network.

However, since the scope of NR use case / scenarios are much wider than that of LTE, we expect much more variation in traffics arrival / demand in NR. For power efficient operation at UE, it is critical to quickly adapt to such variations. The DRX parameters are one of key parameters which enables such adaptation. But, as pointed out above, it is quite static and not fully utilized in most of cases. Thus, we propose to utilize it further for adaptation and to support better, we think UE can provide assistance information to network based on the knowledge inside UE (such as traffic arrival statistics, user behavior statistics, power consumption statistics).

Proposal 5

- NR supports that UE’s indication of its preferred DRX parameters.

In above discussion, DRX cover both Connected mode DRX (CDRX) and Idle mode DRX. But CDRX is much more relevant in the context of adaptation to dynamically varying traffic.

Considering the most common use case of BWP is power saving, we see the connection between BWP and DRX. Given that both features can be configured for quick adaptation to traffic arrival while trying to reduce power consumption, we can consider the joint operation of the two features. For example, NR could support the per BWP DRX configuration. For example, a narrow bandwidth part BWP1 and wide bandwidth part BWP2 could be configured with different values for short/long DRX cycle timer, on duration timer, inactivity timer, etc. The values for BWP1 could be configured as different from than that for BWP2 to further reduce power consumption when there is not much traffic coming in BWP1. This kind of association between BWP and DRX parameters could provide increased flexibility enabling better adaptation.

Proposal 6

- NR supports the association between a BWP and a set of DRX parameter configuration.

# Conclusions

In this contribution, we have identified a few important UE-assisted power saving mechanisms in the area of bandwidth part, PDCCH monitoring, cross slot scheduling and BWP-DRX parameter association. We made following observations and proposals based on the discussion.

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for both BWP configuration.

Proposal 1

- NR support that UE sends UE-preferred BWP configuration to network for UE power saving.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices.

  - UE can indicate its preferred BWP switching timer.

Observation 2

- In FR2, it is hard to achieve similar power saving gain as FR1 with currently defined PDCCH monitoring (in general duty cycle) periodicities.

Proposal 2

- Increase the maximum PDCCH monitoring periodicity to support longer monitoring periodicity in FR2.

Proposal 3

- NR support the UE indication of its preferred PDCCH monitoring periodicity for UE power saving.

Proposal 4

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

Proposal 5

- NR supports that UE’s indication of its preferred DRX parameters.

Proposal 6

- NR supports the association between a BWP and a set of DRX parameter configuration.

# References

[1] R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

[2] R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

[3] RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

[4] R1- 1714092 Control Information for UE Power Saving, Apple

[5] R1-1811127 Network-indication based Approaches for UE power saving, Apple, Oct 2018