3GPP TSG RAN WG1 Meeting #95	 R1-1812671

Spokane, USA, November 12th – November 16th, 2018

Agenda Item:	7.2.2.1

Source:	Huawei, HiSilicon

Title:	Coexistence evaluation of NR unlicensed bands

Document for:	Discussion and decision

# Introduction

In RAN1 #94bis meeting [1], the following agreements on simulation methodology for NR-U operation were made:

Agreement:

- When submitting the evaluation result, the following has to be provided as well

  - the TxOP assumptions of WiFi and NR-U

  - Is RTS/CTS enabled for WiFi

  - PD/ED threshold assumptions

  - Max modulation order supported in each technology

  - MIMO scheme and number of MIMO layers used for both technologies

  - WiFi MAC layer A-MPDU/A-MSDU aggregation level, MPDU size

  - NR-U SCS,

  - WiFi guard interval

  - NR UE processing time capability (#1 or #2)

  - NR PDSCH/PUSCH mapping type, PDCCH monitoring configuration

  - Link adaptation assumptions

  - NR assumption on self-scheduling or using cross carrier scheduling

- For the coexistence evaluation of sub-7GHz bands other than 5GHz band, previously agreed to use technology neutral assumptions. Companies are encouraged to provide simulation results together with assumption on the technology neutral channel access mechanism.

Based on the above agreements, in this document we provide the coexistence results between NRU and 802.11ac in both indoor and outdoor scenarios assuming simultaneous DL and UL traffic and with different SCS. Moreover, coexistence result between 802.11ax and NRU in 6GHz are given with different PD/ED threshold setting.

# Coexistence performance for DL+UL NR-U and Wi-Fi in indoor/outdoor scenarios

In this section, 2-operator scenario is assumed. It is regarded as fair coexistence if the UPT of Wi-Fi operator is not degraded when the coexisting operator is replaced with NRU.  The simulation assumptions for indoor and outdoor scenarios conform to the agreements in the previous meetings, as listed in Appendix A and Appendix B. For Wi-Fi networks, we evaluate with 802.11ac system, the detail simulation assumption is listed in Appendix C. The coexistence are evaluated when two operators deploy Wi-Fi or NRU on the same 20MHz CC in NSA mode. Buffer occupancy of WiFi +WiFi with BO=10%, 35% and 55% represent light, medium, and high traffic load, respectively. Each STA/UE has 50% DL traffic and 50% UL traffic. Typical configurations for processing delay agreed in RAN4 are adopted, including HARQ-feedback delay and UL grant delay. The UL Buffer report latency is not taken into account.  For control and RS overhead, only the front-loaded DMRS is considered. CCA-ED for NRU is -72dBm, and CCA-ED=-62dBm, CCA-PD = -82dBm for Wi-Fi. We provide the simulation results of SCS 30 kHz and 60 KHz with 26RBs. For DL transmission, multiple starting point with 7OS mini-slot is used for 30 kHz and 60 kHz SCS. The following figure 1 to 3 are the simulation results for co-existence scenarios of WiFi+WiFi, WiFi+NRU and NRU+NRU.

(a)

(b)

Figure 1. Average UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor scenario

(a)

(b)

Figure 2. Average UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1

(a)

(b)

Figure 3. Average UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2

It is observed from figure 1 to 3 that when the coexisting operator is changed from Wi-Fi to NR-U, rather than being degraded, the DL and UL UPT of Wi-Fi operator is increased in all SCS assumptions. The major reasons that make NR-U system a good neighbor to Wi-Fi system are summarized as below:

- Wi-Fi transmitters uses packet detection with threshold of -82dbm for coexisting with ongoing Wi-Fi transmission while using  energy detection with threshold of -62dbm for coexisting with ongoing NR-U transmission which increases the probability of channel access for Wi-Fi nodes when coexisting with NRU nodes.

- NR-U has better spectrum efficiency than 802.11ac which will result in less time occupancy to transmit same amount of traffic than 802.11ac. It gives more opportunities for Wi-Fi system to access the medium.

- NR-U is a scheduled system as compared to Wi-Fi system which will reduce the number of contending nodes within the cell, while Wi-Fi uses DCF coordination to access the channel both in AP and STA nodes.

Observation 1: In indoor/outdoor scenarios with symmetric DL+UL traffic at low, medium and high traffic loads in 20 MHz channel, coexistence evaluations of user perceived throughput show that NR-U can fairly coexist with 802.11ac.

# Coexistence performance for DL+UL NR-U and Wi-Fi (11ax) in indoor scenarios in 6GHz with different coexistence mechanism

During the RAN1#92[2], some agreements were made that for sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (e.g. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations. In this section, we provide the results assuming 802.11ax and NRU using same ED threshold. For 802.11ax system, same PD and ED level is assumed which enable NAV mechanism among different transmitting nodes. It can be observed that performance of both Wi-Fi and NRU can be improved when common ED threshold is adopted. More spatial reuse gain can be achieved by increasing the common ED threshold.

Observation 2: The performance of Wi-Fi and NRU increase while using higher common ED threshold.

Figure 1. Average UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor scenario

# Conclusion

In this contribution, we made the following observation and proposals from the results of indoor/outdoor coexistence evaluation for NR-U and Wi-Fi:

Observation 1: In indoor/outdoor scenarios with symmetric DL+UL traffic at low, medium and high traffic loads in 20 MHz channel, coexistence evaluations of user perceived throughput show that NR-U can fairly coexist with 802.11ac.

Observation 2: The performance of Wi-Fi and NRU increase while using higher common ED threshold.

# References

Chairman’s notes for RAN1 #94bis meeting

Chairman’s notes for RAN1#92bis meeting

# Appendix A

Table A: Summary of simulation assumptions for indoor Sub-7GHz

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30KHz and 60KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

# Appendix B

Table B: Summary of simulation assumptions for outdoor Sub-7GHz

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Micro layer: Random drop(all micro BSs are outdoor) 1 micro BSs per operator per macro BS |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30kHz and 60kHz |
| Macro ISD | 200 m (opt 1) and 300 m (opt 2) |
| Maximum distance inter-operator | 30 m (opt 1) |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE |
| TXOP | 8ms |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback and OLLA |
| Scheduling | Proportional fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

# Appendix C

Table c: Summary of simulation assumptions for 802.11ac/ax

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| MCS |  |  | 802.11ac/ax MCS with 256QAM |
| Carrier Frequency |  |  | 5GHz |
| Carrier Channel Bandwidth |  |  | 20MHz |
| Number of carriers |  |  | 1 |
| Number of users per operator |  |  | Exactly 5 per gNB per 20MHz |
| MCOT |  |  | 4 ms |
| Frame aggregation |  |  | A-MPDU |
| MPDU size |  |  | 1500B MSDU + 14 B header |
| Guard interval |  | 11ac | 0.8us |
|  |  | 11ax | 0.8us |
| Channel Model |  |  | NR UMi street canyon |
| AP Tx Power |  |  | 23dBm |
| STA Tx Power |  |  | 18dBm |
| MAC | Coordination |  | DCF |
|  | SIFS/DIFS |  | 16us/43us |
|  | Detection |  | Energy detection & preamble detection |
|  | RTS/CTS |  | No |
|  | Contention |  | EDCA |
| CCA-PD |  |  | -82dbm for 11ac |
| CCA-ED |  |  | -62dbm for 11ac |
| ACK Modeled |  |  | Immediate ACK |
| OFDM symbol length |  |  | 4 us for 11ac; 13.6us for 11ax |

# Appendix D

Table 1: coexistence results of DL and UL performance for SCS 30KHz of NRU in indoor scenario

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 15.66 | 21.56 | 31.42 | 36.39 | 7.53 | 15.89 | 26.17 | 30.57 | 3.11 | 13.19 | 23.9 | 26.84 |
|  | 50% | 48.84 | 66.4 | 87.54 | 97.08 | 35.61 | 56.64 | 75.05 | 87.45 | 27.16 | 51.13 | 67.41 | 81.65 |
|  | 95% | 77.93 | 78.23 | 103.9 | 104.8 | 77.3 | 78.02 | 103.9 | 104.8 | 75.23 | 77.93 | 103.7 | 104.8 |
|  | Mean | 49.69 | 59.5 | 78.05 | 83.71 | 38.52 | 53.61 | 71.56 | 78.44 | 30.88 | 50.07 | 67.9 | 75.28 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.08 | 0.06 | 0.05 | 0.04 | 0.11 | 0.08 | 0.05 | 0.05 | 0.15 | 0.08 | 0.06 | 0.05 |
|  | 95% | 0.25 | 0.19 | 0.13 | 0.11 | 0.51 | 0.25 | 0.15 | 0.13 | 1.13 | 0.30 | 0.17 | 0.15 |
|  | Mean | 0.11 | 0.08 | 0.06 | 0.05 | 0.18 | 0.10 | 0.07 | 0.06 | 0.32 | 0.11 | 0.07 | 0.07 |
| UL: UPT CDF [Mbps] | 5% | 18.43 | 26.83 | 18.46 | 29.85 | 6.98 | 18.39 | 13.13 | 24 | 1.49 | 13.13 | 9.97 | 20.6 |
|  | 50% | 53.63 | 72.26 | 56.06 | 76.68 | 38.48 | 59.77 | 42.8 | 66.15 | 29.2 | 56.98 | 37.55 | 60.15 |
|  | 95% | 79.2 | 79.44 | 72.22 | 83.73 | 78.72 | 79.32 | 72.1 | 83.56 | 77.61 | 79.34 | 71.91 | 83.5 |
|  | Mean | 53.4 | 62.72 | 51.35 | 66.81 | 40.75 | 56.58 | 44.75 | 61.31 | 31.98 | 53.54 | 40.72 | 58.36 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.06 | 0.05 | 0.05 | 0.05 | 0.06 | 0.05 | 0.05 | 0.05 | 0.06 | 0.05 |
|  | 50% | 0.07 | 0.06 | 0.07 | 0.05 | 0.10 | 0.07 | 0.09 | 0.06 | 0.14 | 0.07 | 0.10 | 0.07 |
|  | 95% | 0.21 | 0.15 | 0.21 | 0.13 | 0.56 | 0.22 | 0.30 | 0.17 | 1.89 | 0.28 | 0.39 | 0.19 |
|  | Mean | 0.10 | 0.07 | 0.09 | 0.07 | 0.18 | 0.09 | 0.12 | 0.08 | 0.43 | 0.11 | 0.15 | 0.09 |
| 𝜌DL |  | 99.68% | 99.74% | 99.73% | 100% | 99.64% | 99.69% | 100% | 99.91% | 98.46% | 99.70% | 99.80% | 99.86% |
| 𝜌UL |  | 99.60% | 99.87% | 99.52% | 100% | 99.56% | 99.64% | 99.63% | 99.73% | 96.78% | 99.56% | 99.52% | 99.69% |
| BO |  | 17% | 13.00% | 13.00% | 11.00% | 35% | 22.00% | 22.00% | 17.00% | 55% | 28.00% | 28.00% | 21.00% |
| 𝜆 |  | 0.2 files/s |  |  |  | 0.3 files/s |  |  |  | 0.35 files/s |  |  |  |

Table 2: coexistence results of DL and UL performance for SCS 60KHz of NRU in indoor scenario

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 15.66 | 19.43 | 36.84 | 41.97 | 7.53 | 14.67 | 29 | 34.94 | 3.11 | 10.98 | 24.29 | 30.57 |
|  | 50% | 48.84 | 61.14 | 95.91 | 101.8 | 35.61 | 53.78 | 83.89 | 91.97 | 27.16 | 47.46 | 76 | 87.3 |
|  | 95% | 77.93 | 78.16 | 107.1 | 107.2 | 77.3 | 78 | 107.2 | 107.2 | 75.23 | 77.83 | 107.1 | 107.2 |
|  | Mean | 49.69 | 57.19 | 85.24 | 88.05 | 38.52 | 51.51 | 77.5 | 82.54 | 30.88 | 47.84 | 72.89 | 79.57 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.08 | 0.06 | 0.04 | 0.04 | 0.11 | 0.07 | 0.05 | 0.04 | 0.15 | 0.08 | 0.05 | 0.05 |
|  | 95% | 0.25 | 0.21 | 0.11 | 0.10 | 0.51 | 0.27 | 0.14 | 0.11 | 1.13 | 0.34 | 0.16 | 0.13 |
|  | Mean | 0.11 | 0.09 | 0.05 | 0.05 | 0.18 | 0.11 | 0.06 | 0.06 | 0.32 | 0.12 | 0.07 | 0.06 |
| UL: UPT CDF [Mbps] | 5% | 18.43 | 25.52 | 27.56 | 34.53 | 6.98 | 17.7 | 19.27 | 27.88 | 1.49 | 12.15 | 15.54 | 24.38 |
|  | 50% | 53.63 | 70.6 | 78.99 | 85.97 | 38.48 | 58.61 | 63.04 | 77.35 | 29.2 | 53.89 | 55.19 | 70.51 |
|  | 95% | 79.2 | 79.41 | 93.45 | 93.47 | 78.72 | 79.34 | 93.44 | 93.46 | 77.61 | 79.23 | 93.38 | 93.44 |
|  | Mean | 53.4 | 62 | 71.19 | 75.7 | 40.75 | 55.84 | 62.69 | 69.87 | 31.98 | 52.07 | 57.53 | 66.71 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.07 | 0.06 | 0.05 | 0.05 | 0.10 | 0.07 | 0.06 | 0.05 | 0.14 | 0.07 | 0.07 | 0.06 |
|  | 95% | 0.21 | 0.16 | 0.14 | 0.12 | 0.56 | 0.22 | 0.20 | 0.14 | 1.89 | 0.31 | 0.25 | 0.16 |
|  | Mean | 0.10 | 0.08 | 0.07 | 0.06 | 0.18 | 0.09 | 0.08 | 0.07 | 0.43 | 0.11 | 0.10 | 0.07 |
| 𝜌DL |  | 99.68% | 99.69% | 99.72% | 100% | 99.64% | 99.67% | 99.99% | 99.89% | 98.46% | 99.72% | 99.75% | 99.83% |
| 𝜌UL |  | 99.60% | 99.85% | 99.61% | 100% | 99.56% | 99.61% | 99.74% | 99.76% | 96.78% | 99.55% | 99.63% | 99.73% |
| BO |  | 17% | 14.00% | 11.00% | 10.00% | 35% | 23.00% | 18.00% | 16.00% | 55% | 29.00% | 23.00% | 19.00% |
| 𝜆 |  | 0.2 files/s |  |  |  | 0.3 files/s |  |  |  | 0.35 files/s |  |  |  |

Table 3: coexistence results of DL and UL performance for SCS 30KHz of NRU in outdoor scenario 1

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 4.20 | 12.96 | 20.73 | 23.92 | 0.49 | 4.33 | 8.14 | 16.42 | 0.03 | 1.45 | 3.37 | 11.17 |
|  | 50% | 39.56 | 54.91 | 76.96 | 81.54 | 16.26 | 34.20 | 49.63 | 67.80 | 8.99 | 23.88 | 32.23 | 53.21 |
|  | 95% | 77.24 | 77.81 | 100.75 | 100.92 | 73.04 | 77.18 | 100.36 | 100.77 | 59.11 | 76.74 | 97.34 | 100.43 |
|  | Mean | 41.45 | 51.74 | 70.17 | 72.84 | 23.94 | 38.30 | 53.46 | 64.89 | 16.09 | 29.81 | 40.85 | 56.06 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.07 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.10 | 0.07 | 0.05 | 0.05 | 0.24 | 0.12 | 0.08 | 0.06 | 0.39 | 0.16 | 0.12 | 0.07 |
|  | 95% | 0.92 | 0.31 | 0.19 | 0.17 | 4.18 | 0.89 | 0.49 | 0.24 | 6.95 | 2.02 | 1.11 | 0.36 |
|  | Mean | 0.26 | 0.11 | 0.08 | 0.07 | 0.84 | 0.25 | 0.16 | 0.09 | 1.28 | 0.48 | 0.30 | 0.12 |
| UL: UPT CDF [Mbps] | 5% | 1.92 | 10.41 | 11.31 | 14.94 | 0.01 | 1.90 | 3.41 | 9.72 | 0.00 | 0.15 | 0.49 | 5.49 |
|  | 50% | 32.70 | 46.97 | 49.43 | 57.52 | 11.22 | 28.17 | 24.98 | 43.32 | 5.15 | 18.32 | 14.48 | 32.83 |
|  | 95% | 78.22 | 78.58 | 83.06 | 83.10 | 70.27 | 77.94 | 82.70 | 83.00 | 50.61 | 73.79 | 81.95 | 82.80 |
|  | Mean | 36.38 | 47.21 | 51.18 | 55.42 | 19.69 | 33.23 | 34.36 | 46.58 | 12.29 | 25.22 | 24.48 | 39.09 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 | 0.06 | 0.05 | 0.05 | 0.05 | 0.08 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.12 | 0.09 | 0.08 | 0.07 | 0.32 | 0.14 | 0.16 | 0.09 | 0.60 | 0.21 | 0.26 | 0.12 |
|  | 95% | 1.71 | 0.37 | 0.34 | 0.26 | 8.00 | 1.69 | 1.16 | 0.40 | 8.00 | 4.99 | 4.51 | 0.71 |
|  | Mean | 0.42 | 0.13 | 0.12 | 0.10 | 1.31 | 0.42 | 0.33 | 0.14 | 2.01 | 0.85 | 0.85 | 0.22 |
| 𝜌DL |  | 100.0% | 100.0% | 100.0% | 100.0% | 98.8% | 100.0% | 100.0% | 100.0% | 97.1% | 99.8% | 99.9% | 100.0% |
| 𝜌UL |  | 99.3% | 100.0% | 100.0% | 100.0% | 95.6% | 99.6% | 100.0% | 100.0% | 90.4% | 98.0% | 99.2% | 100.0% |
| BO |  | 16.7% | 7.7% | 6.6% | 5.7% | 42.9% | 20.2% | 16.2% | 9.0% | 58.7% | 33.6% | 29.1% | 13.6% |
| 𝜆 |  | 0.076 files/s |  |  |  | 0.094 files/s |  |  |  | 0.105 files/s |  |  |  |

Table 4: coexistence results of DL and UL performance for SCS 60KHz of NRU in outdoor scenario 1

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 4.20 | 12.61 | 22.04 | 30.90 | 0.49 | 4.72 | 9.26 | 24.78 | 0.03 | 1.42 | 3.10 | 20.17 |
|  | 50% | 39.56 | 56.70 | 83.91 | 94.48 | 16.26 | 38.18 | 54.67 | 84.90 | 8.99 | 25.20 | 35.06 | 76.87 |
|  | 95% | 77.24 | 77.84 | 105.59 | 105.64 | 73.04 | 77.25 | 105.34 | 105.61 | 59.11 | 76.91 | 104.04 | 105.55 |
|  | Mean | 41.45 | 52.15 | 75.14 | 81.40 | 23.94 | 40.32 | 58.77 | 75.56 | 16.09 | 31.30 | 44.96 | 71.21 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.07 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.10 | 0.07 | 0.05 | 0.04 | 0.24 | 0.10 | 0.07 | 0.05 | 0.39 | 0.16 | 0.11 | 0.05 |
|  | 95% | 0.92 | 0.32 | 0.18 | 0.13 | 4.18 | 0.81 | 0.42 | 0.16 | 6.95 | 2.00 | 1.20 | 0.20 |
|  | Mean | 0.26 | 0.11 | 0.07 | 0.06 | 0.84 | 0.23 | 0.14 | 0.07 | 1.28 | 0.48 | 0.30 | 0.08 |
| UL: UPT CDF [Mbps] | 5% | 1.92 | 10.63 | 11.47 | 23.14 | 0.01 | 2.42 | 4.03 | 17.25 | 0.00 | 0.17 | 0.58 | 13.30 |
|  | 50% | 32.70 | 48.27 | 57.85 | 75.59 | 11.22 | 32.12 | 33.45 | 60.47 | 5.15 | 19.61 | 17.77 | 52.43 |
|  | 95% | 78.22 | 78.70 | 93.29 | 93.36 | 70.27 | 78.15 | 93.08 | 93.32 | 50.61 | 76.97 | 91.76 | 93.21 |
|  | Mean | 36.38 | 48.03 | 57.49 | 67.02 | 19.69 | 35.18 | 41.90 | 60.32 | 12.29 | 26.56 | 28.91 | 55.35 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.06 | 0.05 | 0.04 | 0.04 | 0.08 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.12 | 0.08 | 0.07 | 0.05 | 0.32 | 0.12 | 0.12 | 0.07 | 0.60 | 0.20 | 0.22 | 0.08 |
|  | 95% | 1.71 | 0.37 | 0.34 | 0.17 | 8.00 | 1.44 | 0.97 | 0.23 | 8.00 | 4.84 | 3.84 | 0.30 |
|  | Mean | 0.42 | 0.13 | 0.12 | 0.08 | 1.31 | 0.38 | 0.26 | 0.09 | 2.01 | 0.81 | 0.77 | 0.11 |
| 𝜌DL |  | 100.0% | 100.0% | 100.0% | 100.0% | 98.8% | 100.0% | 100.0% | 100.0% | 97.1% | 99.7% | 99.8% | 100.0% |
| 𝜌UL |  | 99.3% | 100.0% | 100.0% | 100.0% | 95.6% | 99.9% | 100.0% | 100.0% | 90.4% | 98.0% | 99.4% | 100.0% |
| BO |  | 16.7% | 7.8% | 6.2% | 4.6% | 42.9% | 18.4% | 13.9% | 6.6% | 58.7% | 32.8% | 27.7% | 8.3% |
| 𝜆 |  | 0.076 files/s |  |  |  | 0.094 files/s |  |  |  | 0.105 files/s |  |  |  |

Table 5: coexistence results of DL and UL performance for SCS 30KHz of NRU in outdoor scenario 2

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 4.47 | 10.13 | 19.73 | 20.55 | 1.04 | 6.07 | 15.33 | 18.88 | 0.01 | 1.37 | 7.13 | 12.13 |
|  | 50% | 37.97 | 48.46 | 69.85 | 72.27 | 20.19 | 39.81 | 58.14 | 65.23 | 7.45 | 22.92 | 39.70 | 52.48 |
|  | 95% | 77.08 | 77.58 | 100.81 | 100.90 | 73.39 | 77.31 | 100.61 | 100.74 | 52.23 | 74.70 | 99.09 | 100.61 |
|  | Mean | 39.48 | 47.24 | 67.49 | 68.70 | 26.65 | 41.34 | 60.95 | 65.03 | 14.14 | 28.92 | 45.84 | 56.38 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.08 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.10 | 0.08 | 0.06 | 0.06 | 0.19 | 0.10 | 0.07 | 0.06 | 0.45 | 0.17 | 0.10 | 0.08 |
|  | 95% | 0.83 | 0.39 | 0.20 | 0.19 | 2.53 | 0.58 | 0.25 | 0.21 | 6.55 | 1.78 | 0.53 | 0.33 |
|  | Mean | 0.23 | 0.14 | 0.08 | 0.08 | 0.56 | 0.18 | 0.10 | 0.09 | 1.34 | 0.46 | 0.18 | 0.12 |
| UL: UPT CDF [Mbps] | 5% | 1.56 | 7.44 | 10.59 | 12.80 | 0.01 | 4.24 | 7.84 | 10.39 | 0.00 | 0.04 | 2.53 | 6.31 |
|  | 50% | 30.16 | 40.38 | 44.85 | 51.01 | 13.65 | 34.49 | 34.88 | 45.15 | 3.43 | 17.91 | 17.94 | 32.51 |
|  | 95% | 77.91 | 78.66 | 83.12 | 83.15 | 69.14 | 78.25 | 82.80 | 83.04 | 42.84 | 71.06 | 81.97 | 82.88 |
|  | Mean | 34.52 | 42.24 | 47.81 | 51.61 | 21.40 | 37.41 | 41.30 | 47.87 | 9.98 | 24.38 | 27.28 | 39.77 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 | 0.06 | 0.05 | 0.05 | 0.05 | 0.09 | 0.06 | 0.05 | 0.05 |
|  | 50% | 0.13 | 0.10 | 0.09 | 0.08 | 0.27 | 0.12 | 0.11 | 0.09 | 0.80 | 0.21 | 0.22 | 0.12 |
|  | 95% | 1.55 | 0.52 | 0.37 | 0.30 | 6.86 | 0.87 | 0.49 | 0.38 | 8.00 | 4.00 | 1.45 | 0.62 |
|  | Mean | 0.41 | 0.18 | 0.13 | 0.12 | 1.11 | 0.25 | 0.18 | 0.14 | 2.31 | 0.79 | 0.42 | 0.20 |
| 𝜌DL |  | 100.0% | 100.0% | 100.0% | 100.0% | 99.8% | 100.0% | 100.0% | 100.0% | 97.7% | 99.8% | 100.0% | 100.0% |
| 𝜌UL |  | 99.5% | 100.0% | 100.0% | 100.0% | 97.0% | 100.0% | 100.0% | 100.0% | 87.5% | 98.3% | 100.0% | 100.0% |
| BO |  | 19.5% | 11.8% | 8.8% | 7.9% | 40.4% | 16.8% | 11.9% | 9.8% | 67.8% | 35.6% | 23.3% | 14.8% |
| 𝜆 |  | 0.094 files/s |  |  |  | 0.105 files/s |  |  |  | 0.12 files/s |  |  |  |

Table 6: coexistence results of DL and UL performance for SCS 60KHz of NRU in outdoor scenario 2

| Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| DL: UPT CDF [Mbps] | 5% | 4.47 | 10.28 | 20.71 | 29.24 | 1.04 | 6.41 | 14.31 | 25.15 | 0.01 | 1.16 | 6.07 | 20.71 |
|  | 50% | 37.97 | 50.53 | 74.86 | 84.57 | 20.19 | 42.18 | 61.34 | 80.51 | 7.45 | 24.38 | 40.62 | 72.12 |
|  | 95% | 77.08 | 77.58 | 105.55 | 105.62 | 73.39 | 77.39 | 105.44 | 105.63 | 52.23 | 75.87 | 104.67 | 105.57 |
|  | Mean | 39.48 | 48.37 | 71.78 | 77.69 | 26.65 | 42.30 | 64.13 | 74.94 | 14.14 | 29.81 | 47.79 | 70.27 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.05 | 0.05 | 0.04 | 0.04 | 0.08 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.10 | 0.08 | 0.05 | 0.05 | 0.19 | 0.09 | 0.06 | 0.05 | 0.45 | 0.16 | 0.10 | 0.06 |
|  | 95% | 0.83 | 0.39 | 0.19 | 0.14 | 2.53 | 0.57 | 0.27 | 0.16 | 6.55 | 1.93 | 0.58 | 0.19 |
|  | Mean | 0.23 | 0.13 | 0.08 | 0.06 | 0.56 | 0.18 | 0.10 | 0.07 | 1.34 | 0.48 | 0.19 | 0.08 |
| UL: UPT CDF [Mbps] | 5% | 1.56 | 8.36 | 11.18 | 20.18 | 0.01 | 4.25 | 7.88 | 17.83 | 0.00 | 0.04 | 2.51 | 13.54 |
|  | 50% | 30.16 | 43.23 | 54.00 | 69.32 | 13.65 | 35.40 | 40.73 | 63.13 | 3.43 | 18.01 | 20.83 | 55.04 |
|  | 95% | 77.91 | 78.61 | 93.27 | 93.35 | 69.14 | 78.30 | 93.21 | 93.31 | 42.84 | 72.81 | 91.74 | 93.26 |
|  | Mean | 34.52 | 43.85 | 55.63 | 64.67 | 21.40 | 38.19 | 47.88 | 61.69 | 9.98 | 25.23 | 31.32 | 56.60 |
| UL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.04 | 0.04 | 0.06 | 0.05 | 0.04 | 0.04 | 0.09 | 0.05 | 0.04 | 0.04 |
|  | 50% | 0.13 | 0.09 | 0.07 | 0.06 | 0.27 | 0.11 | 0.10 | 0.06 | 0.80 | 0.20 | 0.19 | 0.07 |
|  | 95% | 1.55 | 0.46 | 0.36 | 0.19 | 6.86 | 0.89 | 0.50 | 0.22 | 8.00 | 4.38 | 1.40 | 0.29 |
|  | Mean | 0.41 | 0.16 | 0.12 | 0.08 | 1.11 | 0.25 | 0.16 | 0.09 | 2.31 | 0.81 | 0.39 | 0.11 |
| 𝜌DL |  | 100.0% | 100.0% | 100.0% | 100.0% | 99.8% | 100.0% | 100.0% | 100.0% | 97.7% | 99.6% | 100.0% | 100.0% |
| 𝜌UL |  | 99.5% | 100.0% | 100.0% | 100.0% | 97.0% | 100.0% | 100.0% | 100.0% | 87.5% | 98.2% | 99.9% | 100.0% |
| BO |  | 19.5% | 11.0% | 8.1% | 6.1% | 40.4% | 16.7% | 11.3% | 7.3% | 67.8% | 35.6% | 22.6% | 9.4% |
| 𝜆 |  | 0.094 files/s |  |  |  | 0.105 files/s |  |  |  | 0.12 files/s |  |  |  |