3GPP TSG RAN WG1 Meeting #95                                            R1-1812969

Spokane, USA, Nov. 12th– 16th, 2018

Agenda Item:		    7.2.1.4

Source:			Samsung

Title:		   Performance evaluation for NoMA

Document for:		Discussion and Decision

# Introduction

In RAN1 #92 meeting, the evaluation assumptions for link-level evaluation have been agreed [1]. In RAN1 #92bis meeting, evaluation assumptions for system-level evaluation were discussed and some of the link-level parameters were also clarified [2]. In RAN1 #93 meeting, the evaluation assumptions as well as evaluation metrics for LLS and SLS are further discussed [3]. In RAN1 #94 meeting, the template for collecting LLS results and also SLS results were discussed [4]. In RAN1 #94bis meeting, LLS results were discussed and summarized [5]. In this contribution, more link-level evaluation results and system-level evaluation results for IGMA are shown.

# Updated LLS evaluation results for IGMA

The LMMSE detector combined with hybrid IC is introduced in [6]. In this section, the LLS evaluation results are shown.

The mMTC scenario with large TBS is considered. Especially case 4 with 60 bytes TBS and 8 UEs, and case 5 with 75 bytes TBS and 6 UEs are evaluated. Fig. 1 shows the evaluation results for these two cases. Note that for LMMSE detector, hybrid-IC is considered.

(a) Case 4 with 8 UEs                           (b) Case 5 with 6 UEs

Fig. 1. Evaluation results for LMMSE detector

From Fig. 1, we can observe that by using LMMSE detector with hybrid-IC can achieve better performance with higher detection complexity, especially for high SNR region. Hybrid-IC can help reduce the detection complexity with lower average iteration number.

Based on above evaluation results, we have following observation.

Observation 1: In high channel coding rate case, LMMSE detector can achieve better performance with higher detection complexity compared to ESE detector.

# SLS evaluation results for NoMA

In this section, SLS evaluation is discussed and preliminary SLS evaluation results are demonstrated to show the performance gain of IGMA.

## 3.1 Validation results for PHY abstraction

PHY abstraction is one of important aspects. It brings the link-level evaluation results into system-level evaluation and the accuracy of PHY abstraction will also impact the performance of SLS. One potential PHY abstraction method which has been captured in TR 38.802 is listed in Appendix. In this subsection, validation results, including mMTC scenario and eMBB scenario will be demonstrated to show the effectiveness of this method.

### mMTC scenario

For mMTC scenario, we segment each higher layer packet in to TB with 40 bytes. As a result, the validation results for 40 bytes TBS are shown. The TDL-C channel is considered. Both ideal channel estimation and realistic channel estimation are applied.

The validation results with ideal channel estimation for mMTC scenario are shown in Fig. 2.

(a) 1UE                                        (b) 4UE

(c) 6UE

Fig. 2. Validation results ideal channel estimation for mMTC scenario.

The validation results with realistic channel estimation are shown in Fig. 3.

(a) 1UE                                        (b) 6UE

(c) 8UE

Fig. 3. Validation results ideal channel estimation for mMTC scenario.

As can be observed from Fig. 2 and 3, by choosing appropriate β, the BLER results obtained by L2S mapping method are quit aligned with the evaluated BLER curve, for both small UE number and large UE number, and for both ideal channel estimation and realistic channel estimation, which demonstrates the effectiveness of this PHY abstraction method.

### eMBB scenario

For eMBB scenario, we consider TB with 80 bytes and TDL-A channel is considered. Realistic channel estimation is applied to validation results.

In Fig. X3, the validation results for eMBB scenario are shown.

(a) 4UE                                        (b) 8UE

(c) 12UE

Fig. 4. Validation results for eMBB scenario.

From Fig. 4, we can see that for eMBB case, with proper β value, the results obtained by L2S mapping are aligned with evaluation results, even for different UE numbers.

Based on the above validation results, we have following observations.

Observation 2: The PHY abstraction in Appendix can produce accurate L2S mapping results.

## 3.2 Preliminary SLS evaluation results

In this sub-section, the preliminary SLS results for mMTC and eMBB scenario are shown. The evaluation assumptions are listed in Appendix.

For mMTC, as agreed, the traffic model defined in TR 45.820 is used. The higher layer protocol overhead of 29 bytes is considered. For packet segmentation, 5 bytes overhead is considered for each TB. Apparently, if the TB number is large, it will come with more overall overhead. Considering that for 20 bytes TB, the overhead is quite large and more TBs will usually decrease the SINR. Meanwhile, given for a certain data bits, for 40 bytes TB, the overhead introduced by 5 bytes overhead is more acceptable compared that for 20 bytes TB as the number of 40-byte TBs is relatively decreased. As a result, in the SLS evaluation, one packet is segmented into TBs with 40 bytes.

Random activation is considered for the grant free transmission, one RB is allocated for each UE in baseline evaluation, while all 6 RBs are occupied by each UE in NoMA simulation. In addition, MA signatures as well as DMRS are semi-static configured.

HARQ retransmission with random back-off is considered and the maximum number of retransmission is set to 8.

In Fig. 5, the higher layer packet drop rate (PDR) vs. packet arrival rate (PAR) is shown.

Fig. 5. System-level evaluation results for mMTC scenario

As can be observed by Fig. 5, for IGMA, the PDR for realistic channel estimation and ideal channel estimation is close. This is due to that from link-level evaluation, the performance gap for ideal channel estimation and realistic channel estimation is not large. The gap between IGMA and OFDM is large, which means that more UE s can be supported by using IGMA. Specifically, IGMA performs 2~3 times better than OFDM, even for realistic channel estimation.

For eMBB scenario, random activation is also considered for the grant free transmission, 2 RBs are allocated for each UE in baseline evaluation, while all 12 RBs are occupied by each UE in NoMA simulation. In addition, MA signatures as well as DMRS are semi-static configured. Same HARQ retransmission scheme with mMTC scenario is considered for eMBB scenario.

The SLS results for eMBB scenario are shown in Fig. 6.

Fig. 6. System-level evaluation results for eMBB scenario

For eMBB scenario, IGMA can still achieve more than 2 times gain in terms of PDR vs. PAR.

Based on above SLS evaluation results, we have following observation.

Observation 3: IGMA can achieve 2~3 times gain compared with OFDM in terms of PDR vs. PAR.

# Conclusion

In this contribution, LLS evaluation results for IGMA are shown, including ideal cases on various scenarios, and non-ideal cases, including power imbalance, timing offset, as well as frequency offset. Based on above evaluation results and analysis, we have following observations and proposals.

Observation 1: LMMSE detector can achieve better performance compared to ESE detector with higher detection complexity.

Observation 2: The PHY abstraction in Appendix can produce accurate L2S mapping results.

Observation 3: IGMA can achieve 2~3 times gain compared with OFDM in terms of PDR vs. PAR.

# References

Chairman’s note, 3GPP TSG RAN WG1 #92.

Chairman’s note, 3GPP TSG RAN WG1 #92bis.

Chairman’s note, 3GPP TSG RAN WG1 #93.

Chairman’s note, 3GPP TSG RAN WG1 #94.

Chairman’s note, 3GPP TSG RAN WG1 #94bis.

R1-1812967, “Receivers for NoMA”, Samsung, 3GPP TSG RAN WG1 #95.

S. H. Moon et al. “Link performance estimation techniques for MIMO-OFDM systems with maximum likelihood receiver,” IEEE Trans. Wireless Commun., vol. 11, no. 5, May 2012, pp. 1808-1816.

# Appendix

## A.1 Link-level evaluation assumptions

The evaluation assumptions for LLS are shown in Table 1 as follows.

Table 1. LLS evaluation assumptions

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz | 4 GHz, 700 MHz as optional |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point | CP-OFDM as starting point |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 | SCS = 15 kHz #OS = 14 |
| Allocated bandwidth | 6 as the starting point | 12 for 60 kHz SCS and 24 for 30kHz SCS as the starting point | 12 as the starting point |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |
| Target BLER for one transmission | 10% | 0.10% | 10% |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4 Rx or 8 Rx for 4 GHz 8 Rx as optional |  |  |
| UE antenna configuration | 1 Tx |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |
| Max number of HARQ transmission | 1 as starting point. | 1 as starting point. More values, 2 for URLLC can be used. | 1 as starting point. |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |
| Distribution of avg. SNR | Both equal and unequal | Equal | Both equal and unequal |
| Timing offset | 0 as starting point. For grant-free without perfect TA (asynchronous), timing offset is within [0, y] as starting point, where y has two values at least for the purpose of evaluation: • Case 1: CP/[2] < y <= CP+rms_DS, with detailed value FFS • Case 2: 2*CP>=y > CP, with detailed value FFS |  |  |
| Frequency error | 0 as starting point. Also evaluate uniform distribution between -70 and 70 Hz for 700MHz carrier frequency, and uniform distribution between [-140] and [140] Hz for 4GHz carrier frequency. |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |

## A.2 System-level evaluation assumptions

| Parameters | mMTC | URLLC | eMBB | Further specified values |
| --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |
| Inter-BS distance | 1732m | 200m for 4GHz 500m for 700MHz | 200m |  |
| Carrier frequency | 700MHz | 4GHz or 700MHz | 4GHz |  |
| Simulation bandwidth | 6 PRBs | 12 PRBs | 12 PRBs | Clarify the simulation bandwidth in the SLS assumptions is the bandwidth for uplink transmission. |
| Number of UEs per cell | 20 UEs per sector |  |  |  |
| Channel model | UMa in TR 38.901; The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz. |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |
| BS antenna configurations | 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value |  |  |  |
| BS antenna height | 25m |  |  |  |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |
| UE antenna configuration | 1Tx |  |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |  |
| UE antenna gain | 0dBi |  |  |  |
| UE distribution | For mMTC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell For URLLC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell; Note: Other option(s) not precluded, e.g., 500m ISD, 80% of users are outdoors (3km/h), 20% of users are indoor (3km/h). For eMBB: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell |  |  |  |
| UE power control | Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC. |  |  |  |
| HARQ/repetition | HARQ combining with random back-off. The maximum number for transmissions is 8. |  |  |  |
| Channel estimation | Ideal & Realistic |  |  |  |
| BS receiver | MMSE-IRC |  |  |  |

## A.3 PHY abstraction for ESE

The effective SNR mapping (ESM) PHY abstraction is applied for ESE with soft interference cancelation. Generally, for ESM, the effective SNR is calculated as

SNReff=f-1(1Nn=0N-1f(SINRn))

where N is the symbol block size, SINRn is the SINR for the n-th sub-carrier, SNReff is the effective SNR for the entire block and function f(∙) is an invertible function. Based on this effective SNR, the corresponding BLER can be obtained based on SNR-BLER mapping table under AWGN channel for specific MCS.

Unfortunately, it is hard to get the SINR in above equation for non-orthogonal multiple access schemes. For some non-orthogonal multiple access schemes, advanced receivers with iterative detection and decoding are applied to mitigate the multi-user interference. It is hard to model those kinds of receivers since the detection and decoding are combined together, while for most existing receiver modeling methods, only detection is considered. To facilitate the PHY abstraction and avoid receiver modeling, an approximation approach is applied and is summarized as follows:

Step 1: Calculate the upper bound post-processing SINR.

For non-orthogonal multiple access, the optimal performance can be achieved if the signals from multiple UEs can be separated completely. In this sense, the post-processing SINR of PIC detector is regarded as upper bound. If per-RE power of transmitted signal is normalized to 1, the post-processing SINR after PIC detection for the n-th sub-carrier of the k-th UE is expressed as

SINRk,nPIC=hk,n2σ2+In

where hk,n denotes the frequency domain channel coefficient vector of the n-th sub-carrier of the k-th UE, σ2 denotes the noise power and In denotes the power of inter-cell interference on the n-th sub-carrier.

Step 2: Approximate the real post-processing SINR based the upper bound.

Although by using advanced receiver, such as chip-by-chip MAP detector, multi-user interference can be mitigated or even eliminated, there still will be some performance degradation, especially when the number of serviced UEs is large. A scaling factor β is used to emulate this performance degradation. Denote CPICk,n=log2(1+SINRk,nPIC) as the capacity for PIC detector and for non-orthogonal multiple access, the achievable capacity is a scaled version which is expressed as follows

CNoMAk,n=log2(1+ SINRk,n)=βCPICk,n

where SINRk,n denotes the approximated SINR for n-th sub-carrier of the k-th UE and based on this scaled capacity, SINRk,n can be calculated as

SINRk,n=1+SINRk,nPICβ-1

The parameter β can describe the capacity loss due to the superposition of multiple UEs and should be optimized by off-line link level simulations for different number of UEs under different cases.

Step 3: Calculate the effective SNR.

The approximated SINR for n-th sub-carrier of the k-th UE obtained in step 2 is used for the mapping of effective SNR. Several methods can be applied and the received-bit information rate (RBIR) [4] for SNR mapping is used due to its simplicity. The effective SNR is expressed as

SNReff,k=RBIR-11NnRBIRSINRk,n,Q

where SNReff,k denotes the effective SNR for the k-th UE and Q denotes the modulation order. The function RBIRSNR,Q denotes the RBIR metric given SNR and modulation order Q and RBIR-1(∙) is its inverse function given RBIR metric to find corresponding SNR. The RBIR metric function is pre-calculated off-line and stored as a look-up table. The Table 25 in [4] is re-used in our SLS evaluations.

Step 4: Obtain BLER according to the SNR-BLER mapping.

After getting the effective SNR for the k-th UE, the corresponding BLER is obtained according to the SNR-BLER mapping relationship which is pre-calculated for given MCS under AWGN channel.

The only parameter that should be optimized is the scaling factor β and the optimization can be completed by solving a minimum mean square error problem, as discussed in [7].