3GPP TSG RAN WG1 Meeting #95			R1-1813331

Spokane, USA, November 12th – 16th, 2018

Source:	NTT DOCOMO, INC.

Title:	Updated evaluation results of reliability for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

Reliability is the successful probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. The minimum requirement for reliabilty is 1-10-5 success probability of transmitting a layer 2 PDU of 32 bytes within 1 ms in channel quality of coverage edge [1].

Methodolgy of reliability evaluation is showed as follow [2].

Methodology of reliability evaluation:

- Step 1:  Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values.

- Step 2:	. Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value

- Step 3:	Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of. SINR taking into account retransmission

- Step 4:	The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

Methodology of reliability evaluation:

- Step 1:  Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values.

- Step 2:	. Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value

- Step 3:	Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of. SINR taking into account retransmission

- Step 4:	The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

At the RAN1#93 meeting, the details of reliability evaluation methodology and parameters were agreed [3]. Based on it, we present the initial evaluation results of reliability of UL at the RAN1#94 meeting. We further update the UL reliability results by utilizing the MCS specified in R15 for URLLC at RAN1 #94b.   At this meeting, we will provide our initial evaluation results of reliability of URLLC DL.

- System level evaluation results of pre-processing SINR

In this section, we provide the pre-processing SINR on Urban Macro URLLC ConfigA and ConfigB.

In Figure 1 and 2, the CDF curves for UL pre-processing SINR on Urban Macro URLLC Config A and B scenario are provided. To align the bandwidth between system level and link level simulation for PUSCH transmission, in which different modulation and coding schemes lead different bandwidth occupancy, we provide the UL pre-processing SINRs considering bandwidth of 30/45 RBs. The evaluation assumptions could be found in Annex 1.

Figure 1． CDF curve for UL pre-processing SINR on Urban Macro URLLC ConfigA

Figure 2． CDF curves for UL pre-processing SINR on Urban Macro URLLC ConfigB

In Figure 3, the CDF curves for DL pre-processing SINR on Urban Macro URLLC ConfigA and ConfigB are provided. The evaluation assumption could be found in Annex 1.

Figure 3． CDF curves for DL pre-processing SINR on Urban Macro URLLC

Based on the above figures, the SINR values of 5% percentile of CDF of different scenarios are shown in the following table.

TABLE 1

| Test Environment | UL/DL | Configuration |  | 5% CDF SINR(dB) |
| --- | --- | --- | --- | --- |
| Urban Macro URLLC | UL | ConfigA | RB=30 | -3.5 |
|  |  | ConfigA | RB=45 | -4.4 |
|  |  | ConfigB | RB=30 | -1.6 |
|  |  | ConfigB | RB=45 | -1.7 |
|  | DL | ConfigA |  | -2.4 |
|  |  | ConfigB |  | -2.1 |

Observation 1: The 5% percentile SINR is -4.4dB to -3.5dB for UL of Urban Macro-URLLC Config A, and -1.7 dB to -1.6dB for UL of Urban Macro-URLLC Config B.

Observation 2: The 5% percentile SINR is -2.4dB for DL of Urban Macro-URLLC Config A, and -2.1 dB for DL of Urban Macro-URLLC Config B.

- Link level evaluation results

In this section, we provide the link level results of PDCCH、PUSCH and PDSCH. The assumptions for LLS of PDCCH 、PUSCH and PDSCH could be found in Annex 2. Payload size of 40bit, excluding 24bit CRC, is assumed for PDCCH.  Different aggregation levels for PDCCH are assumed. Packet size of 32 bytes for PUSCH and PDSCH was simulated considering mini-slot configurations with 4 symbol and 7 symbol per slot with different code rates.

3.1 URLLC config.A

For URLLC config.A, the BLER performance for PDCCH, PUSCH and PDSCH as a function of the received SINR is shown in Figure 4, 5 and 6, respectively.

Figure 4. BLER of PDCCH to received SINR

Figure 5. BLER of PUSCH to received SINR

Figure 6. BLER of PDSCH to received SINR

3.2 URLLC config.B

For URLLC config.B, the BLER performance for PDCCH, PUSCH and PDSCH as a function of the received SINR is shown in Figure 7, 8 and 9, respectively.

Figure 7. PDCCH average BLER as a function of received SINR

Figure 8. PUSCH average BLER as a function of received SINR

Figure 9. PDSCH average BLER as a function of received SINR

- Performance of UL reliability

4.1 Methodology of UL reliability calculation

For UL transmission, reliability and latency performance based on grant free mechanism is calculated. We utilize Alternative 1 in [3] to derive reliability under a given SINR, i.e. each transmission is independent and soft combining is not considered. Table 2 and 3 shows the descriptions of terminologies for calculation of successful probability and latency, respectively.

Table 2 Successful probability of PDCCH/PUSCH transmission

| Successful Probability | Description |
| --- | --- |
| pPDCCH | Successful probability of PDCCH transmission |
| pPUSCH | Successful probability of PUSCH transmission |

Table 3 Transmission and processing time for PDCCH/PUSCH

| Time | Description |
| --- | --- |
| tPDCCH | PDCCH transmission time |
| tPUSCH | PUSCH transmission time |
| TgNB | gNB processing time for PUSCH |
| TUE | UE prepare time for PUSCH |

Then, we can describe the total reliability as:

We can describe the total latency as:

Where M is repetition times of initial PUSCH transmission, N is repetition times of PUSCH retransmission and K-1 is the number of re-transmission times triggered by DCI. The gNB/UE L1 processing time for UE capability #2 under 30kHz SCS is 5.5 symbol [4].

4.2  Performacne of UL reliability

When we calculate the reliability performance, one transmission attempt with M PUSCH transmission scheme is considered. In this case, K=1, HARQ re-transmission is not needed.

4.2.1 Performacne of URLLC config.A

The reliability evaluation results of config A are shown in Table 4.

Table 4 Relibility evalution results for Config.A

| OS | MCS | pPUSCH | M | PUL | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- |
| OS = 4 | 4 | >0.99999 | 1 | >0.99999 | Yes |
| OS = 7 | 4 | >0.999995 | 1 | >0.999995 | Yes |
| OS = 7 | 2 | >0.999998 | 1 | >0.999998 | Yes |

4.2.2 Performance of URLLC config.B

The reliability evaluation results of config B are shown in Table5.

Table 5. Relibility evalution results for Config.B

| OS | MCS | pPUSCH | M | PUL | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- |
| OS = 4 | 4 | 0.99979 | 2 | 0.9999999559 | Yes |
| OS = 7 | 4 | 0.999875 | 2 | 0.999999984375 | Yes |
| OS = 7 | 2 | >0.999995 | 1 | >0.999995 | Yes |

Observation 3:  IMT-2020 reliability requirements for Configuration A (4 GHz) UL can be met with both 7-OS mini-slot and 4-OS mini-slot configuration.

Observation 4：IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with both 4-OS mini-slot and 7-OS mini-slot configuration.

- Performance of DL reliability

5.1 Methodology of DL reliability calculation

For DL transmission, reliability and latency performance based on mini-slot aggregation mechanism is calculated. We utilize Alternative 1 in [3] to derive reliability under a given SINR, i.e. each transmission is independent and soft combining is not considered. Table 6 and 7 shows the descriptions of terminologies for calculation of successful probability and latency, respectively.

Table 6 Successful probability of PDCCH/PDSCH/PUCCH transmission

| Successful Probability | Description |
| --- | --- |
| pPDCCH | Successful probability of PDCCH transmission |
| pPDSCH | Successful probability of PDSCH transmission |
| pPUCCH-NACK | Successful probability of PUCCH NACK detection |
| pPUCCH-DTX | Successful probability of PUCCH DTX detection |

Table 7 Transmission and processing time for PDCCH/PDSCH/PUCCH

| Time | Description |
| --- | --- |
| tPDCCH | PDCCH transmission time |
| tPDSCH | PDSCH transmission time |
| tPUCCH | PUCCH NACK transmission time |
| Tp | Processing time for PDSCH |

Then, we can describe the total reliability of DL as:

We can describe the total latency as:

where K is the total number of transmission attempts allowed in 1ms; M is the number of PDSCH transmissions in one transmission attempt; N is the number of PUCCH NACK transmissions in one transmission attempt.

5.2 Performacne of DL reliability

When we calculate the reliability performance, one transmission attempt with 1 PDCCH and M PDSCH transmission scheme is considered. In this case, K=1, HARQ re-transmission is not needed. Therefore, only pPDCCH and pPDSCH    are needed. With the assumption 2-symbol and AL=16, successful probability of PDCCH transmission is pPDCCH =1 for both config.A and config.B.

5.2.1 Performacne of URLLC config.A

The DL reliability evaluation results of config A are shown in Table 8.

Table 8 Relibility evalution results for Config.A

| OS | MCS | pPDCCH | pPDSCH | M | PDL | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- |
| OS = 7 | 2 | 1 | 0.999973 | 2 | 0.99999999 | Yes |
| OS = 7 | 3 | 1 | 0.999838 | 2 | 0.999999974 | Yes |
| OS = 7 | 4 | 1 | 0.999222 | 2 | 0.999999394 | Yes |

5.2.2 Performance of URLLC config.B

The DL reliability evaluation results of config B are shown in Table 9.

Table 9 Relibility evalution results for Config.B

| OS | MCS | pPDCCH | pPDSCH | M | PDL | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- |
| OS = 7 | 2 | 1 | 0.99999026 | 1 | 0.99999026 | Yes |
| OS = 7 | 3 | 1 | 0.99990278 | 2 | 0.999999991 | Yes |
| OS = 7 | 4 | 1 | 0.99948392 | 2 | 0.999999734 | Yes |

Observation 5:  IMT-2020 reliability requirements for Configuration A (4 GHz) DL can be met with 7-OS mini-slot configuration.

Observation 6：IMT-2020 reliability requirements for Configuration B (700 MHz) DL can be met with 7-OS mini-slot configuration.

- Summary

In this contribution, the evaluation results of reliability are provided. The following observation are made.

Observation 1: The 5% percentile SINR is -4.4dB to -3.5dB for UL of Urban Macro-URLLC Config A, and -1.7 dB to -1.6dB for UL of Urban Macro-URLLC Config B.

Observation 2: The 5% percentile SINR is -2.4dB for DL of Urban Macro-URLLC Config A, and -2.1 dB for DL of Urban Macro-URLLC Config B.

Observation 3:  IMT-2020 reliability requirements for Configuration A (4 GHz) UL can be met with both 7-OS mini-slot and 4-OS mini-slot configuration.

Observation 4：IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with both 4-OS mini-slot and 7-OS mini-slot configuration.

Observation 5:  IMT-2020 reliability requirements for Configuration A (4 GHz) DL can be met with 7-OS mini-slot configuration.

Observation 6：IMT-2020 reliability requirements for Configuration B (700 MHz) DL can be met with 7-OS mini-slot configuration.

References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, June 2017.

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Huawei, May 2018.

- 3GPP TS 38.214, “NR; Physical layer procedures for data”, V15.2.0

Annex 1: Assumptions of system level simulations

| Urban Macro - URLLC | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 16 Tx/Rx = (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU=(Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 2TXRU=(Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°, 90° polarization | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°, 90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 2MHz/4MHz/7Mhz | 2MHz/4Mhz/7Mhz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Channel model B | Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [95°] in LCS | [99°] in LCS |
| Handover margin (dB) | 0 | 0 |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula) from port 0 | Based on RSRP (formula) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |

Annex 2: Assumptions of link level simulations

Aussumptions of PDCCH

| Parameters | Value |
| --- | --- |
| DCI payload (excluding 24bits CRC) | 40bits |
| System bandwidth | 20MHz |
| Carrier Frequency | 4GHz, 700MHz |
| Number of symbols for CORESET | 1,2 |
| CORESET BW (contiguous PRB allocation) | 20MHz |
| Subcarrier spacing | 30KHz |
| Aggregation level | 4, 8, 16. |
| Transmission type | Interleaved (interleaver row =6) |
| REG bundling size | 6 |
| Resource mapping | Distributed transmission |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel estimation | Realistic(MMSE) |
| Channel model | TDL-C (delay spread: 300ns,UE speed: 3 km/h) |
| Number of BS antennas | 2Tx |
| Number of UE antennas | 4Rx for 4GHz, 2Rx for 700MHz |

Aussumptions of PUSCH/PDSCH

| Parameters |  | Value |
| --- | --- | --- |
| Payload (excluding 24bits CRC) |  | 256bits |
| System bandwidth |  | 40MHz |
| Carrier Frequency |  | 4GHz, 700MHz |
| Modulation and Code rate |  | MCS4 ,MCS3,MCS2 for PDSCH MCS4 ,MCS2 for PUSCH |
| Subcarrier spacing |  | 30KHz |
| Waveform |  | CP-OFDM |
| Number of symbol for PDSCH |  | 7(2xDMRS+5xData), |
| Number of symbol for PUSCH |  | 4(1xDMRS+3xData), 7(2xDMRS+5xData), |
| DMRS configuration |  | Front-loaded DMRS (2/7 for OS7) |
| Channel coding |  | LDPC ( BP for decoding) |
| Channel estimation |  | Realistic(2D-MMSE) |
| Channel model |  | TDL-C (delay spread: 300ns,UE speed: 3km/h) |
| Number of BS antennas |  | For DL：4Tx for config A, 2Tx for config B For UL：4Rx for config A, 2Rx for config B |
| Number of UE antennas |  | For DL: 2Rx For UL: 1Tx |
| Transmission scheme |  | rank 1 with precoder cycling for PDSCH |
| Number of PRB allocated to UE for PDSCH | OS=7,MCS2 | 45 |
|  | OS=7,MCS3 | 35 |
|  | OS=7,MCS4 | 28 |
| Number of PRB allocated to UE for PUSCH | OS=4,MCS4 | 45 |
|  | OS=7,MCS4 | 30 |
|  | OS=7,MCS2 | 45 |