3GPP TSG RAN WG1 Meeting #95	R1-1813612

Spokane, USA, November 12-16, 2018

Source:	Ericsson

Title:	Intra-site vs. inter-site clustering for NC-JT and DPS in Dense Urban scenario

Agenda Item:	7.2.8.6

Document for:	Discussion and Decision

# Introduction

In [1], we have presented some initial simulation results for NC-JT under dense urban scenario based on the assumptions agreed in RAN1#94bis, where the coordination set consists of 3 sectors within each site, i.e. with ‘intra-site’ clustering.  In this contribution, we present some additional results when the coordination set consists of 3 cells from three adjacent sites, one cell from each site, i.e., ‘inter-site’ clustering.

# NC-JT with Inter-site Clustering

## Simulation Assumptions

In the inter-site clustering simulation, the coordination set includes 3 cells from three adjacent sites, one cell from each site. The 3 cells form a clover leave pattern as shown in Figure 1. Each UE is attached to one cluster (or a coordination set).  Up to two cells are selected dynamically in NC-JT for transmitting data to a UE at the same time.  Again, ideal backhaul is assumed. In addition to the agreed 4 and 16 Tx ports per TRP in the evaluation assumptions, the case of 2 Tx ports with 8x1 column antenna was also simulated for comparison. The simulated schemes include:

- NC-JT: Dynamic selection of one or two cells (i.e., dynamic switching between single-TRP and NC-JT) for a UE within the associated cluster, one codeword per TRP

- DPS: Dynamic selection of a cell for a UE within the associated cluster.

- single TRP: Independent scheduling per cell

SU-MIMO is used in all the simulations. The maximum rank per TRP for all schemes is 4 for 4 and 16 Tx ports and 2 for 2 Tx ports. Other simulation assumptions can be found in the Appendix.

One inter-site cluster

3-sector site

Figure 1.  Inter-site clustering.

## Simulation Results

The simulation results are shown in Table 1 to Table 4 below.  Note that DPS is already supported in NR Rel-15, naturally DPS should also be considered as the baseline in evaluating NC-JT for NR Rel-16. Therefore, the numbers in red colour in the tables mean that they are worse than either single TRP or DPS and the numbers in green colour indicate that they are better than DPS and single TRP.

For all three cases with 2, 4, and 16 Tx ports, inter-site clustering consistently performs worse for both NC-JT and DPS, from low load to higher loads. The reason for that is because in case of inter-site clustering, only small number of cell edge UEs having similar pathlosses to the two gNBs can benefit from NC-JT or DPS. While in the intra-site clustering case, there are more UEs having similar pathlosses to the gNBs and thus more UEs could be scheduled with NC-JT.

In addition, for 4 and 16 Tx ports DPS consistently performs better than NC-JT in both intra- and inter-clustering, even at 10% of resource utilization (RU). For 2 Tx ports, NC-JT only provides better mean throughput than DPS at RU below 20%, mainly due to the fact that ranks 3 and 4 can be scheduled to a UE with NC-JT. The benefit of NC-JT disappears at RUs above 20%.

Table 1:  4 ports results under dense urban at 4GHz

| Cell edge UE throughput gain |  |  |  |  |  | Mean UE throughput gain |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RU (single TRP) | single TRP | Intra-site clustering |  | Inter-site clustering |  |  | RU (single TRP) | single TRP |  | Intra-site clustering |  | Inter-site clustering |  |
|  |  | DPS | NC-JT | DPS | NC-JT |  |  |  | DPS |  | NC-JT | DPS | NC-JT |
| 10% | 0% | 8% | 2% | 6% | 0% |  | 10% | 0% | 1% |  | -1% | 0% | -2% |
| 20% | 0% | 11% | 5% | 6% | -4% |  | 20% | 0% | 1% |  | -2% | -2% | -6% |
| 40% | 0% | 16% | 5% | -4% | -13% |  | 40% | 0% | -1% |  | -5% | -6% | -11% |
| 50% | 0% | 15% | 9% | -8% | N/A |  | 50% | 0% | 0% |  | -3% | -10% | N/A |

Table 2: 16 ports results under dense urban at 4GHz.

| Cell edge UE throughput gain |  |  |  |  |  |  |  |  |  |  |  | Mean UE throughput gain |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RU (single TRP) | single TRP |  | Intra-site clustering |  |  |  | Inter-site clustering |  |  |  | RU (single TRP) |  | single TRP |  | Intra-site clustering |  |  |  |  | Inter-site clustering |  |  |
|  |  |  | DPS |  | NC-JT |  | DPS |  | NC-JT |  |  |  |  |  | DPS |  |  | NC-JT |  | DPS |  | NC-JT |
| 10% |  | 0% |  | 17% |  | 17% |  | 12% |  | 12% |  | 10% |  | 0% |  | 2% | 2% |  | 1% |  | 1% |  |
| 20% |  | 0% |  | 22% |  | 19% |  | 16% |  | 12% |  | 20% |  | 0% |  | 4% | 3% |  | 1% |  | -1% |  |
| 40% | 0% |  | 32% |  | 26% |  | 17% |  | 10% |  | 40% |  | 0% |  | 7% |  |  | 6% |  | 0% |  | -1% |
| 50% |  | 0% |  | 39% |  | 35% |  | 17% |  | 15% |  | 50% |  | 0% |  | 10% | 9% |  | 1% |  | -2% |  |

Table 4:  2 ports results under dense urban at 4GHz.

| Cell edge UE throughput gain |  |  |  |  |  | Mean UE throughput gain |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RU (single TRP) | single TRP | Intra-site clustering |  | Inter-site clustering |  | RU (single TRP) | single TRP | Intra-site clustering |  | Inter-site clustering |  |
|  |  | DPS | NC-JT | DPS | NC-JT |  |  | DPS | NC-JT | DPS | NC-JT |
| 10% | 0% | 10% | 1% | 8% | -4% | 10% | 0% | 1% | 10% | 1% | 5% |
| 20% | 0% | 6% | -7% | 3% | -13% | 20% | 0% | 0% | 0% | -1% | -6% |
| 40% | 0% | -7% | -20% | -26% | -39% | 40% | 0% | -7% | -12% | -15% | -22% |
| 50% | 0% | -6% | -17% | -30% | -42% | 50% | 0% | -10% | -14% | -21% | -28% |

For both Rel.16 NC-JT and Rel.15 DPS, intra-site coordination performs better than inter-site coordination in dense urban scenario.

# Conclusion

In this contribution, we have presented simulation results comparing inter-site vs. intra-site clustering for NC-JT and DPS for dense urban scenario. We made the following observation:

Observation 1	For both Rel.16 NC-JT and Rel.15 DPS, intra-site coordination performs better than inter-site coordination in dense urban scenario.

# References

R1-1813271 “On multi-TRP and multi-panel”, Ericsson, RAN1#95.

# Simulation Assumptions

| Parameters | Dense urban (Macro Only) |
| --- | --- |
| Carrier frequency | 4GHz |
| Channel model | TR38.901 |
| TP antenna configuration | 2 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,1,2,1,1,1,1) 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) 16 ports: (M, N, P, Mg, Ng Mp, Np) = (8,4,2,1,1,2,4) (dH, dV) = (0.5, 0.8)λ |
| UE antenna configuration | 4Rx Port (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Coordination assumptions | - 3 cells from the same site (intra-site clustering) - 3 cells from 3 adjacent sites, one cell from each site (inter-site clustering) |
| Traffic model | ftp model 1, 500kB packet size |
| Cell layout | 19 sites with 57 homogeneous cells |