3GPP TSG-RAN WG1 Meeting #95	Draft CR R1-1813617

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of RLM-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 02.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | To clarify the selection rules for RLM-RS that were agreed in RAN1#94bis |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that shortest search space periodicity of a CORESET with active TCI state for PDCCH is used first as criteria for selecting the RLM-RS. As second criteria, if more than one CORESETs are associated with same SS periodicity, the CORESETs are selected in order of the CORESET ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential ambiguity how the specification text regarding the RLM-RS selection for L=4 case would remain, resulting different understanding which DL RS are used for RLM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5	Radio link monitoring

<omitted text>

-    For , the UE selects the  RS provided for active TCI states for PDCCH receptions in order starting from CORESETs associated with the search space sets with the lowest monitoring periodicity. If more than one selected CORESETs are associated with search spaces sets having same monitoring periodicity, the UE selects the CORESETs in order starting from the CORESET with highest index as described in Subclause 10.1.