3GPP TSG RAN WG1 #96                                                                                     R1-1901923

Athens, Greece, 25th February – 1st March, 2019

Source:	OPPO

Title:	Enhancements to initial access procedure for NR-U

Agenda Item:	7.2.2.2.2

Document for:	Discussion and Decision

Introduction

During RAN1#AH 1901, the enhancements to initial access procedure and initial access signals and channels for NR-U were discussed. Agreements on initial access signals and channels for NR-U were made as follows [1]:

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

In this contribution, the enhancements to initial access procedure in NR-U are discussed.

Discussion

SS/PBCH block transmission procedure

During study item duration, the definition of DRS transmission window was agreed. At most [Y] candidate SSB positions is considered beneficial in a DRS transmission window. Y is the maximum number of candidate SSB position, for e.g., Y = [64]. The maximum number of transmitted SSBs is [X] within DRS transmission window, X <= 8. That is, at most 8 SSB are transmitted on 8 SSB positions belonging to Y candidate SSB positions. If gNB gets the channel access, it is beneficial to transmit all the transmitted SSB within one COT as much as possible. Since the maximum number of transmitted SSB is 8, it is preferred that the positions of all the actual transmitted SSB are restricted within 8 continuous candidate SSB positions. Then the legacy 8 bits bitmap can be reused for actual transmitted SSB position indication.

Proposal 1: The transmitted SSBs are restricted within 8 continuous candidate SSB positions in the DRS transmission window.

The duration of DRS transmission window is FFS. The DRS is mainly used for discovery and measurement. In LAA, the DMTC occasion duration is 6ms. In NR-U, a similar time window will be defined. In NR, the SS/PBCH block burst set are transmitted within 5ms for all the numerologies. If a common DRS transmission windows duration is defined for different numerologies, the possible maximum number of candidate SSB position are different with different numerologies. Currently the SSB transmission window are within 5ms. In order to provide enough candidate SSB positions for possible LBT failure, the number of candidate SSB positions should be as much as possible in the DRS transmission window. Therefore, for different numerologies, the number of candidate SSB positions should be different. A unified Y for different numerologies is not beneficial to maximize the candidate SSB positions in the DRS transmission window. The current SSB position in a half frame defined in NR is shown in the following figure.

Figure 1: Normal SSB pattern in a half frame in NR

It was agreed in last meeting that UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers. Higher layers can configure 15KHz or 30KHz SCS for SS/PBCH block.  For the same DRS transmission window, the number of candidate SSB positions can be different for 15KHz and 30KHz SCS for SS/PBCH block.

Proposal 2: It is allowed the number of candidate SSB positions within the DRS transmission window can be different for different numerologies.

For initial access UE, frame synchronization should be achieved based on the detected SS/PBCH blocks. In NR, the SSB index and half frame indicator carried in SSB can be used for this purpose. In NR-U, due to possible LBT failure, the actual position of transmitted SSB may be in the new candidate SSB positions other than the normal positions defined in NR. For the SSBs in these new positions, legacy SSB index determined by L will be not enough for frame synchronization of UE. As discussed during study item, extended SSB indexes can be an efficient way for the additional transmission instances.

Observation 1: Extended SSB indexes can be an efficient way for frame synchronization acquisition with the extended candidate SSB positions.

The QCL assumption from the detected SSB should be derived through the SSB. For a specific actual transmitted SSB, it should convey the QCL information for UE to combine the SSBs with same QCL assumption. In Rel-15, the QCL assumption is derived from SSB index. In NR-U, The SSB index is extended for synchronization of UE. If the extended SSB indexes are also used to derive QCL assumption by UE, the actual transmitted SSB with a specific QCL assumption should be restricted to the time instance corresponding to the extended SSB indexes. It will introduce restriction to the start of TXOP for actual SSB transmission. Or, with earlier start of TXOP, gNB has to wait for the corresponding time instance to transmit the actual SSB with a specific QCL assumption. It is proposed to consider this issue. Flexible start of SSB transmission with any QCL assumption is preferred.

Observation 2: If QCL assumption is derived by the extended SSB indexes, the time instance of transmitted SSB with a specific QCL assumption is fixed to the ones derived by the extended SSB indexes.

Whenever gNB wants to initial a downlink transmission, it should first start channel access procedure. A TxOP can start after LBT succeeds. Before MCOT has passed, gNB should stop downlink transmission and wait for the next LBT success. Since the start of TxOP is not predictable due to LBT result, the end of TxOP can be locate on any transmission instances. For the DRS transmission, if the whole SSB burst set cannot be transmitted within a single TxOP, how to handle the DRS transmission in this case should be considered. Another case is that the start of a TxOP is too late for a whole SSB burst set to transmit within the DRS window. Although the end of TxOP is later than the end of DRS transmission window, the TxOP can still not cover the whole SSB burst set transmission instances. The two cases are illustrated in the following figures.

Case 1: TxOP cannot cover the whole SSB burst set transmission

Case 2: Part of SSBs in a SSB burst set cannot be transmission before the end of SSB transmission window

Figure 2: Two cases for SSB burst set transmission

For the above two cases, the incomplete SSB burst set transmission is caused by some specific TxOP position compared to the DRS transmission window. How to handle these cases should be studied. For example, in these cases, incomplete SSB burst set transmission is allowed in NR-U. Or gNB should find other TxOP for the whole SSB burst set transmission, instead of incomplete SSB burst set transmission.

Proposal 3: It should be clarified how to handle the cases of incomplete SSB burst set transmission due to specific TxOP position.

As discussed above, the start of SSB burst set transmission can locate on all the possible start position in the DRS transmission window. It bring challenges to the indication of actual transmitted SSB in a SSB burst set. In NR, the bitmap of actual transmitted SSB is semi statically indicated to UE, since the SSB can be transmitted in the nominal SSB positions. However, in NR-U, the SSB burst set transmission in a DRS transmission window is dynamically based on LBT results. The current indication of actual transmitted SSB should be enhanced in NR-U to timely inform UEs for rate matching.

Proposal 4: The indication of actual transmitted SSB is enhanced in NR-U to address uncertain SSB transmission positions in a DRS transmission window.

RACH procedure

1）Msg1

Both contention-based RACH and contention-free RACH should be supported for NR-U at least for standalone scenario. RACH procedure can be triggered by initial access, RRC connection re-establishment procedure, handover, DL or UL data arrival but UL is non-synchronized, request for OSI, beam failure recovery, etc., which may require a dense PRACH resource configuration. The PRACH resource configuration is indicated by system information and should be reserved by idle UE for initial access procedure. However, since the DL transmission burst is opportunistic due to LBT, frequently appeared PRACH resources may cause frequently interruption of a DL transmission burst, which would lead to inefficient transmission in unlicensed carriers.

Two PRACH configurations with different periodicities can be considered to enhance PRACH configuration in NR-U. As shown in Figure 3, PRACH configuration 1 with longer periodicity indicated by system information can be used for idle UE for initial access, PRACH configuration 2 with shorter periodicity configured by RRC and indicated by dynamical signaling can be used for connected UE performing RACH procedure. For example, activation command and de-activation command can be introduced to dynamically indicate the activated time window, within which PRACH resource configured by PRACH configuration 2 can be used for preamble transmission.

Figure 3 PRACH resource configuration for NR-U

Proposal 5: Two PRACH configurations with different periodicities can be considered to enhance PRACH configuration in NR-U.

2）Msg2

For a RACH procedure, a UE needs to monitor a corresponded Msg2 within a preconfigured RAR window after transmitting a preamble. If the UE does not detect the PDCCH with an associated RA-RNTI within the window, it needs to retransmit the preamble and wait for a RAR again. In unlicensed bands, a gNB needs to perform LBT procedures before replying a RAR. If the gNB cannot obtain an available channel within the RAR window due to LBT failures, the gNB will not reply the RAR and the UE will not detect any PDCCH scrambled by the associated RA-RNTI.

To handle this issue, in RAN1#94 meeting, it has been agreed to extend the maximum RAR window size to support more occasions for the RAR transmission. However, such a longer RAR window also increases the RAR monitoring occasions for a UE. If a gNB responded a RAR to a UE and the UE missed the response, or if a gNB did not prepare to respond a UE when contentions existed, the fixed long RAR window may introduce unnecessary power consumptions and delay the msg1 retransmissions. Thus, it seems extending the maximum RAR window size is a good option to introduce more RAR transmission occasions, but using a fixed long RAR window is not a good solution considering the power consumption and the access delay for a RACH procedure. How to design the RAR window and the RACH procedure in NR-U needs to be further considered. For example, a flexible RAR window can be introduced.

Observation 3: A fixed long RAR window may introduce unnecessary power consumptions and delay the msg1 retransmissions.

For potential RACH resource enhancements in NR-U, multiple PRACH transmissions before Msg2 reception in RAR window was identified in last meeting. The number of allowed transmission is pre-defined or indicated. The resource of the multiple PRACH transmissions should be configured in RMSI or RRC signaling. These PRACH resource are grouped for multiple PRACH transmission. For gNB side, if more than one preamble from the PRACH resource group are received by gNB before RAR window termination, gNB has to respond to each detected preamble, since it has no idea whether these preamble are from same or different UE. For UE side, if multiple RARs for the PRACH resource group are received, it should determine which responded preamble is used for power control of Msg3. Otherwise, gNB can indicate which preamble is used for power control of Msg3. After determining the preamble, the total power ramp-up from the first to the last random access preamble could be determined and used to compute the power of PUSCH for Msg3 transmission.

Proposal 6: For multiple PRACH transmissions before Msg2 reception, UE should determine the ramp-up power of preamble for the power control of Msg3.

Mobility procedure

In RAN1#95 meeting, the following agreements on RLM/RRM were made [2].

Agreement:

- For RLM, the following recommendations are considered beneficial for further design in the WI:

  - Identifying a set of RLM-RS, e.g., DRS, SS/PBCH blocks, CSI-RS

    - Transmission of the RS in a COT may be subject to LBT

  - Identifying which set(s) of RLM-RS are used for in-sync and out-of-sync evaluations

    - For example, determining which RLM-RS within or outside the DMTC for RLM can be utilized for in-sync and out-of-sync evaluations

  - Potential definition of a metric, e.g., Rel-15 out-of-sync indication or new metric, to accurately identify instances of unsuccessful detection of RLM-RS. Whether/how to report such a metric to higher layers is to be further studied.

Agreement:

It is considered beneficial for the time-domain measurement windows for RRM measurements and RLM to be different.

For RLM, the transmission of RS for RLM is subject to LBT. The RLM-RS may be within or outside the DMTC. The actual transmitted RLM-RS should be identified for in-sync and out-of-sync evaluations. Rel-15 RLM metric or a new defined metric can be used for potential metric for RLM in NR-U.

In the DMTC, multiple candidate DRS transmission positions will be defined, like SSB candidate positions. If SSB is configured as RLM-RS, there exists uncertainty of the success transmission of the SSB for measurement due to potentially LBT failures before the NR-U gNB send those SSBs. In our view, if UE can accurately identify the instances of unsuccessful or successful RLM-RS transmission, only the RLM-RS successfully transmitted by a gNB can be included in the measurement samples. In this case, Rel-15 RLM metric can be reused as much as possible. UE should determine whether a RLM-RS instance is used for IS/OOS evaluation. This can be done by two options.

- Explicit signaling

For explicit signaling, UE is informed the successful transmitted RLM-RS for in-sync and out-of-sync evaluations. Figure 4 shows an example of measurement in NR-U with opportunistic transmission. When DL LBT succeeds, it is informed to UE by gNB. Within the following COT of gNB, the configured RLM-RS for measurement is detected and measured by UE as effective measurement results.

Figure 4 Measurement in NR-U with opportunistic transmission

- Implicit determination

For implicit determination, UE can determine the validity of a RLM-RS instance based on the reception of other channels or signals. For example, if UE successfully decodes a PDCCH just before and/or after a RLM-RS instance, UE can determine the RLM-RS instance is valid.

With the information of valid RLM-RS instance explicitly or implicitly, Rel-15 RLM metric can be used for potential metric for RLM in NR-U.

Proposal 7: The validity of RLM-RS instance is determined before it is used for in-sync and out-of-sync evaluations.

- New metric for RLM

In RAN1 #95 meeting, it has been agreed that a new metric can be potentially defined to identify instances of unsuccessful detection of RLM-RS. This will be another way to handle the false-alarm issue which is triggered by the LBT failure.

In LTE and NR Rel-15, there are only two metrics for RLM. One stands for a better channel state and is used to evaluate the IS indication to higher layer, another stands for a worse channel state and is used to evaluate the OOS indication to higher layer. In NR-U, the RLM-RS may not be transmitted due to LBT failure. For this reason, if the two metrics are reused, some OOS indication may be triggered by the LBT failure. In such a case, the RLF and RRC reestablishment may be triggered when the channel state is still good enough. However, with a new metric, an UE can differentiate the cases whether the channel is good(IS), the channel is not good(OOS) and the channel state is unknown(unknown state). With these information, more radio link states can be differentiated, and it is helpful to reduce the false-alarm in RLF.

Proposal 8: New metric for RLM shall be introduced to reduce the false-alarm in RLM/RLF.

Furthermore, consider the new metric for RLM, some related issues shall be discussed as well, such as the relationship between different metrics, how to handle the unknown state, the generation of cell level indications based on beam level evaluations and how to estimate RLF in new assumptions.

Paging and system information delivery

In [3], the WI scope includes paging and system information enhancement.

Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

System information: specify required NR modifications to enhance acquisition of system information in NR-U including increased transmission opportunities in time domain for SI transmission.

The two aspects are mainly RAN2 issues. However, they may have impacts on RAN1 specifications. In physical layer, the PDCCH monitoring for paging and system information during initial access procedure may be enhanced with increased transmission opportunities. RAN1 related enhancement on paging and system information delivery can be taken into accounts.

Observation 4: The enhancements to paging and system information may need modifications to RAN1 specification.

Conclusion

In this contribution, enhancements to initial access procedure in NR-U are discussed. The following are observed and proposed.

Proposal 1: The transmitted SSBs are restricted within 8 continuous candidate SSB positions in the DRS transmission window.

Proposal 2: It is allowed the number of candidate SSB positions within the DRS transmission window can be different for different numerologies.

Proposal 3: It should be clarified how to handle the cases of incomplete SSB burst set transmission due to specific TxOP position.

Proposal 4: The indication of actual transmitted SSB is enhanced in NR-U to address uncertain SSB transmission positions in a DRS transmission window.

Proposal 5: Two PRACH configurations with different periodicities can be considered to enhance PRACH configuration in NR-U.

Proposal 6: For multiple PRACH transmissions before Msg2 reception, UE should determine the ramp-up power of preamble for the power control of Msg3.

Proposal 7: The validity of RLM-RS instance is determined before it is used for in-sync and out-of-sync evaluations.

Proposal 8: New metric for RLM shall be introduced to reduce the false-alarm in RLM/RLF.

Observation 1: Extended SSB indexes can be an efficient way for frame synchronization acquisition with the extended candidate SSB positions.

Observation 2: If QCL assumption is derived by the extended SSB indexes, the time instance of transmitted SSB with a specific QCL assumption is fixed to the ones derived by the extended SSB indexes.

Observation 3: A fixed long RAR window may introduce unnecessary power consumptions and delay the msg1 retransmissions.

Observation 4: The enhancements to paging and system information may need modifications to RAN1 specification.

References

- Draft minutes report RAN1#AH 1901 v010

- Draft minutes report RAN1#95 v010

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm