3GPP TSG RAN WG1 Meeting #96	R1-1902230

# Athens, Greece, Febuary 25-March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on LBRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS38.212 v15.4.0 for determining TBS_LBRM, the maximum number of layers for PDSCH and PUSCH is assumed based RRC configurations or UE capabilities. However, there is ambiguity before the UE reports its capabilities and there can be a mis-understanding between the gNB and the UE for the maximum number of layers for PDSCH/PUSCH in determining TBS_LBRMThen, the UE cannot decode PDSCH and cannot receive any RRC configuration messages. Another reason is for UL |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before UE capability is reported, the maximum number of layers in determining TBS_LBRM is assumed to be 1 for PDSCH and PUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot receive PDSCH after RACH procedure and therefore the UE cannot receive any RRC configuration messages. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: If the network implements this change and the UE does not, the UE may or may not receive PDSCH. If the UE implements this change and the network does not, the UE may or may not receive PDSCH. |  |  |  |  |  |  |  |  |

5.4.2	Rate matching for LDPC code

The rate matching for LDPC code is defined per coded block and consists of bit selection and bit interleaving. The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as .

5.4.2.1	Bit selection

The bit sequence after encoding  from Subclause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Subclause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Subclause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Subclause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	elseif both the higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH are provided, X is given by the maximum of maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH

-	otherwise, X is given by the provided higher layer parameter between maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	 elseif the higher layer parameter maxNumberMIMO-LayersPDSCH is provided, X is given by that parameter

-	otherwise X is 1

-	maximum modulation order configured for the serving cell, if configured by higher layers; otherwise a maximum modulation order  is assumed for DL-SCH ;

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial bandwidth part if there is no other bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Subclause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured BWPs of a carrier |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

< Unchanged parts are omitted >