3GPP TSG RAN WG1 #96                                            R1-1902256

Athens, Greece, 25th February – 1st March, 2019

Agenda item:	7.2.2.1.3

Source:	Samsung

Title:	Uplink signal and channel design for NR-U

Document for:	Discussion and Decision

# Introduction

A work item on NR-based access to unlicensed spectrum was agreed in RAN #82 meeting [1]. UL signals and channels design is one of objectives of the work item. In RAN1 Ad-Hoc meeting 1901 [2], the following agreements and conclusions regarding UL interlace design for NR-U was made:

Agreement: For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth: a. 15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace b. 30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH Working assumption: - For a given SCS, the following interlace design is supported at least for PUSCH: - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth - Point A is the reference for the interlace definition - For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths - FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz - FFS: Whether and how partial interlace allocation is supported

In this contribution, we provide our views on remaining details of PUSCH, PUCCH and SRS design.

# General UL design

2.1 Sub-PRB interlacing for 60KHz

PRB-based block-interlace design has been identified as beneficial for 15 and 30 kHz SCS, and further study PRB or sub-PRB for 60kHz SCS.

On one hand, sub-PRB provides larger multiplexing capacity and transmission power efficiency for the small resource allocation case, because of larger number of interlaces M. However, the benefit vanishes if a UE requires more than one interlace. On the other hand, channel estimation performance degrades due to less number of DMRS samples per PRB, and inter-cell interference increases due to degraded cross-correlation of shorter DMRS sequence. Furthermore, sub-PRB interlace complicates the whole system design and requires great standard effort.

Proposal 1: Not support sub-PRB based design for all UL signals/channels.

2.2 Interlace structure for wideband operation with single serving cell

In NR-U, the carrier bandwidth as well as BWP bandwidth can be larger than 20MHz and is an integer multiple of 20MHz. In the last RAN1 meeting, ‘common’ interlace was agreed as working assumption. ‘Common’ interlace is defined with reference point A. For a given SCS, the interlace spacing is the same regardless of carrier BW. Obviously, ‘common’ interlace is similar to common PRB design in NR phase-1. The center of subcarrier 0 of resource block 0 of ‘common’ interlace 0 for subcarrier spacing configuration  coincides with point A. Similar to physical PRB in NR phase-1, ‘Physical’ interlaces should be defined within a BWP, e.g., the resource block 0 of ‘physical’ interlace 0 coincides with physical PRB 0 in the BWP. Moreover, the interlace allocation should also fit sub-band LBT within the BWP, e.g., how to indicate PRBs of one ‘physical’ interlace on parts of one active BWP where CCA is successful at UE (if option 3 for UL wideband operation is supported), and how to deal with PRBs falling into the guard band of sub-band (if RAN4 deems necessary to reserve some RBs for protection).

Proposal 2: Physical interlace allocation should be defined within a BWP. FFS the impact of sub-band LBT on interlace allocation within a BWP.

# PUSCH Design

In last meeting, discussion on the partial starting slot in a UL transmission burst was performed, but no consensus was drawn yet. To support multiple UL starting positions, two options were discussed.

-	Option 1: PUSCH(s) as in Rel-15 NR

-	Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome.

For Option 1, gNB can schedule multiple type-B PUSCHs to occupy a full UL slot, e.g., 7 PUSCHs with 2 symbol duration. UE transmits the corresponding PUSCHs in the remaining symbols from the nearest candidate starting position after successful LBT. The main drawback is DMRS overhead, especially for DFT-s-OFDM waveform for which data and DMRS is TDMed, and also UL grant or DFI overhead carrying implicit/explicit HARQ-ACK for each 2-symbol PUSCH.

For Option 2, similar as FeLAA, UE can firstly prepare the PUSCH according to the UL grant, e.g., PUSCH starting position and duration, and transmit partial PUSCH starting from the nearest candidate starting symbol after LBT by puncturing either from the start of slot or the end of slot. The data punctured due to failure of LBT could be recovered by CBG-based retransmission as in NR phase-1. HARQ-ACKs of these punctured CBGs are not counted towards CWS adaptation decision.

For option 2, one key point is how to aid gNB to identify the actual starting point, and how to reduce the impact on channel estimation. In FeLAA, there is one DMRS per slot. By detecting DMRS in each slot respectively, eNB can identify whether the first slot is transmitted or not. The channel per slot is estimated by the DMRS in each slot. In NR, the first DMRS is located in the first scheduled symbol(s) for PUSCH mapping type B, and it is in the 3rd or 4th symbol in a slot for PUSCH mapping type A. If the candidate starting position can be after the first DRMS symbol, then, DMRS is punctured which does not only degrade channel estimation (or even no CE result if there is no additional DMRS) but also complex the starting position detection at gNB side. Therefore, DMRS should be movable to ensure it is always located in the first full symbol of transmitted PUSCH for channel estimation and starting position detection. To reduce the complexity of regenerating DMRS sequence according to LBT result, removing symbol index from DMRS sequence initialization can be considered, though interference randomization in partial starting slot would be slightly degraded.

Proposal 3: Support flexible starting position for PUSCH by puncturing from the start or the end of the slot, with always placing DMRS in the first full symbol of transmitted PUSCH.

# PUCCH

NR phase 1 supports both short PUCCH (PUCCH format 0/2) and long PUCCH (PUCCH format 1/3/4). Considering the similar scenarios requiring different PUCCH format on licensed carriers, e.g., to support different UL coverage or to cope with different UCI payload, and also for NR-U specific scenarios, such as to utilize the remaining symbols or slots within gNB’s shared COT, at least more than one PUCCH format should be supported.

RAN1 has identified legacy PUCCH format 2 and 3 with legacy contiguous waveform are beneficial for some scenarios. To meet the minimum temporal allowance of 2 MHz, Y=12/6/3 PRBs for 15/30/60 kHz SCS should be allocated. In NR phase-1, UE determines the actual number of PRBs X for the PUCCH transmission to be the minimum (the number of PRBs configured by PUCCH resource, the minimum number of PRBs resulting to a coding rate no smaller than configured code rate for UCI). If the same mechanism applies for PUCCH format 2 and 3, the actual number of PRBs for PUCCH transmission X would be smaller than Y, although the configured number of PRBs per PUCCH resource is larger than Y. Therefore, the determination of actual number of PRBs for PUCCH transmission should be modified to X’ = maximum (minimum(the number of PRBs configured by PUCCH resource, the minimum number of PRBs resulting to a coding rate no smaller than configured code rate for UCI), Y) to set the lower bound. UE performs UCI rate matching according to X’ PRBs. Alternatively, UE maps X/2 PRBs on each edge of configured PUCCH resource frequency resource for better PSD efficiency.

Proposal 4: For legacy PUCCH format 2 and 3 with contiguous frequency resource allocation, the actual number of PRBs for PUCCH transmission should be determined by max (min(the number of PRBs configured by PUCCH resource, the minimum number of PRBs resulting to a coding rate no smaller than configured code rate for UCI), the minimum number of PRBs occupying at least 2MHz).

RAN1 also identified the interlace-based waveform can be beneficial in some scenarios. For the sake of efficient multiplexing between different UL channels, the common design for these UL channels would be desirable.

The following common aspects have been identified as important to consider in the detailed design of interlace-based PUCCH format:

-	Flexible number of OFDM symbols

-	Flexible UCI payload

-	Coding of UCI payload

-	Number of supported PUCCH formats

-	Support for user multiplexing of both UCI payload and DMRS on an interlace

-	Multiplexing method of UCI payload and DMRS

-	PUCCH waveform

-	Performance

Similar as Rel-15 PUCCH design, both short and long PUCCH are needed for different coverage requirement. Correspondingly, for coverage or power limited UE, DFT-s-OFDM waveform is preferred, as its low PAPR/CM property leads to high PA efficiency and also in turn results in good cell coverage and extended UE battery life. In NR-U, the PAPR/CM performance of DFT-s-OFDM degrades due to non-adjacent interlace-based resource allocation, yet its PAPR/CM is still better than CP-OFDM. Therefore, short PUCCH with CP-OFDM waveform and long PUCCH with DFT-s-OFDM should be supported for NR-U. For short PUCCH with CP-OFDM, DMRS and UCI can be multiplexed in FDM manner as legacy PUCCH format 2, and multiplexed in TDM manner as legacy PUCCH format 3/4 for long PUCCH with DFT-s-OFDM.

For UCI payload, similar range of UCI is expected for NR-U, e.g., from 1 bit for SR to hundreds of bits for HARQ-ACK of multiple CCs and slots as well as CSI report. In practical, the scenario for 1~ 2 bit UCI payload in unlicensed band would be much rare than in licensed band, as it would be more desirable to transmit all necessary UCI information once UE accesses the channel after LBT. Considering the limited usage of such small UCI payload, it is not worth dedicatedly designing PUCCH format for only a small UCI payload. Instead, PUCCH formats for medium UCI payload can also support the small UCI payload case with marginal standard effort. The same coding method for 1 or 2 bit CSI in NR phase-1 of padding zero until 3 bits can be reused for NR-U.

Proposal 5: NR-U supports one short PUCCH with CP-OFDM (enhanced PUCCH format 2) and one long PUCCH with DFT-s-OFDM (enhanced PUCCH format 4) based on block interlace waveform for a variable UCI payload from 1 to thousands bits.

Since the minimum resource of enhanced PUCCH formats is one interlace, the mechanism to increase the multiplexing capacity per interlace to achieve comparable capacity with legacy NR PUCCH with only 1 PRB should be introduced. The multiplexing factor is configurable to accommodate variable UCI payload and different channel fading environment.

For enhanced PUCCH format 2, as shown in Figure 1, within one interlace, 2 or 4 UEs can be multiplexed by allocating a length-2/4 OCC in frequency for UCI and different cyclic shift or cyclic shift + time-domain length-2 OCC for DMRS. The number of multiplexed UEs depends on the channel character of frequency-selective and time-varying fading.

Figure 1(a) 1-symbol enhanced PUCCH format 2             Figure 1(b) 2-symbol enhanced PUCCH format 2

For enhanced PUCCH format 4, within one interlace, up to L UEs can be multiplexed by length-L pre-DFT OCC for UCI and different cyclic shift for DMRS. As well understood in NR phase-1, length-L pre-DFT spread is equivalent to allocating UEs on different combs on every Lth RE, and fortunately. To preserve the orthogonality within one interlace, UCI information bit should be repeated on every (12*N/L)th samples, wherein the total number of REs per interlace is 12*N. Theoretically, L can be as large as 12*N, as the orthogonality between UEs does not degrade by selective channel fading. Orthogonal DMRS resource is the bottleneck of multiplexing capacity that is sensitive to frequency selective fading. The extreme aggressive case would be using all of 12 CSs within a PRB, as shown in Figure 2. Time-domain spread is not desirable as DMRS symbols are not adjacent.

Figure 2

Proposal 6: To support comparative multiplexing capacity with Rel-15 PUCCH,

- For enhanced PUCCH format 2, support frequency domain spread for UCI and frequency+time domain spread for DMRS.

- For enhanced PUCCH format 4, support pre-DFT spread over the whole interlace for UCI and frequency domain spread for DMRS.

In case the same UCI information is repeated in one OFDM symbol, e.g., with frequency-domain spread in enhanced PUCCH format 2, PAPR/CM would increase. One simpler solution is to perform PRB-specific randomization such as switching OCC sequence across PRBs.

Proposal 7: PRB-specific randomization can be supported to reduce PAPR/CM.

# SRS

SRS waveform should also meet OCB requirement. Both NR phase-1 and LTE eLAA supports comb-based wideband SRS. In NR-U, reusing the legacy comb-based wideband SRS over at least 20MHz sub-band meets OCB requirement, and also provides good SRS detection performance and sufficient SRS multiplexing capacity (up to 4 combs and 12 CSs). Wideband SRS can be multiplexed with other UL channels in a TDM manner. There is no need to introduce another new SRS waveform.

Proposal 8: NR-U supports legacy comb-based waveform for SRS.

In NR-U, the UL/DL configuration on unlicensed band can be very dynamically changed according to the channel access result. The semi-statically determined SRS transmission occasion may fall into a DL slot/symbol, and gNB may not timely utilize the available resources to facilitate SRS transmission, e.g., at the end of DL burst. The efficiency of P-SRS and SP-SRS would be seriously degraded. If P-SRS and SP-SRS are to be supported in NR-U, the transmission is permitted only if the transmission occasion is within UL slot/symbols indicated by SFI or other COT indication.

For AP-SRS, the SRS transmission occasion is determined by the slot containing DCI triggering SRS and the offset configured by SRS resource. Typically, it would be desirable to transmit SRS along with PUSCH or PUCCH to avoid additional LBT. Semi-statically configured slot offset (up to 4 SRS resource per SUL/non-SUL) may not be flexible enough to always support contiguous SRS and PUSCH/PUCCH transmission. It would be desirable to introduce additional offset to bundle SRS and PUSCH/PUCCH transmission.

Proposal 9: If P-SRS or SP-SRS is configured, UE performs the transmission in the corresponding SRS occasion only if the occasion is within UL slot/symbols indicated by SFI or other COT indication. If AP-SRS is configured, additional timing offset to bundle SRS and PUSCH/PUCCH in contiguous symbols can be supported.

In addition, the restriction of SRS location at the end of slot or after the PUSCH is undesirable for resource utilization. For example, flexible SRS location can fill the gap between the end of DL transmission and HARQ-ACK feedback, and support contiguous PSUCH/PUCCH and SRS transmission.

Proposal 10: NR-U supports configurable SRS location other than last 6 symbols of the slot.

# Conclusions

The proposals made in this contribution are summarized below:

Proposal 1: Not support sub-PRB based design for all UL signals/channels.

Proposal 2: Physical interlace allocation should be defined within a BWP. FFS the impact of sub-band LBT on interlace allocation within a BWP.

Proposal 3: Support flexible starting position for PUSCH by puncturing from the start or the end of the slot, with always placing DMRS in the first full symbol of transmitted PUSCH.

Proposal 4: For legacy PUCCH format 2 and 3 with contiguous frequency resource allocation, the actual number of PRBs for PUCCH transmission should be determined by max (min (the number of PRBs configured by PUCCH resource, the minimum number of PRBs resulting to a coding rate no smaller than configured code rate for UCI), the minimum number of PRBs occupying at least 2MHz).

Proposal 5: NR-U supports one short PUCCH with CP-OFDM (enhanced PUCCH format 2) and one long PUCCH with DFT-s-OFDM (enhanced PUCCH format 4) based on block interlace waveform for a variable UCI payload from 1 to thousands bits.

Proposal 6: To support comparative multiplexing capacity with Rel-15 PUCCH,

- For enhanced PUCCH format 2, support frequency domain spread for UCI and frequency+time domain spread for DMRS.

- For enhanced PUCCH format 4, support pre-DFT spread over the whole interlace for UCI and frequency domain spread for DMRS.

Proposal 7: PRB-specific randomization can be supported to reduce PAPR/CM.

Proposal 8: NR-U supports legacy comb-based waveform for SRS.

Proposal 9: If P-SRS or SP-SRS is configured, UE performs the transmission in the corresponding SRS occasion only if the occasion is within UL slot/symbols indicated by SFI or other COT indication. If AP-SRS is configured, additional timing offset to bundle SRS and PUSCH/PUCCH in contiguous symbols can be supported.

Proposal 10: NR-U supports configurable SRS location other than last 6 symbols of the slot.

# References

[1] RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc.

[2] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting Ad-Hoc 1901, Taipei, Taiwan, China, January 21st – 25th, 2019.