3GPP TSG RAN WG1 Meeting #96	 R1-1903063

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.3

Source:	Huawei, HiSilicon

Title:	Further evaluation of coexistence in 6GHz

Document for:	Discussion and decision

Introduction

In RAN1 #95 meeting [1] and RAN #82 meeting [2], the following agreements for 6GHz band channel access mechanism were made:

Agreement:

- For 6GHz band (e.g. US 5925 – 7125 MHz, or European 5925 – 6425 MHz, or parts thereof), since both NR-U and Wi-Fi are new systems to be deployed in these bands (band definitions depending on regulations), a technology neutral fairness criterion is required for the coexistence evaluation. Though it is not in the scope of this study to define a fairness criterion with other RATs, a fairness criterion for coexistence has been discussed but no conclusions were reached. For the 6 GHz coexistence evaluation in this study, the assumption of same CCA-ED threshold across technologies is applied as the technology neutral channel access mechanism.

- In the 6GHz band, the channel access mechanism for NR-U will use, at least, energy detection as part of the coexistence mechanism for enabling coexistence amongst RATs including at least NR-U, [LTE-LAA], and Wi-Fi. Extensions are to be discussed in line with the framework on channel access as captured in the TR 38.889, Section 7.2.1.2 (i.e., WiFi 11a/11ax preamble, existing NR signal with potential enhancements, existing NR channel with potential enhancements) and, if agreed, the corresponding 3GPP specification impact, if any, should be addressed.

In RAN1 Ad Hoc meeting 1901[3], the following agreements for preamble transmission were made

Agreement:

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

Based on the above agreements, in this document we provide the coexistence results between NRU and 802.11ax in 6GHz for different traffic load in both indoor and outdoor scenarios assuming different coexistence mechanisms. It is a resubmission of R1-1901333.

Coexistence performance for DL+UL NR-U and Wi-Fi (11ax) in indoor/outdoor scenario in 6GHz

During the RAN1#92[4], some agreements were made that for sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (e.g. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations. In this section, we provide the results in both indoor and outdoor assuming 802.11ax and NRU assuming the different coexistence mechanisms. The packet arrival rate is selected such that the mean buffer occupancy (BO) of the reference WiFi - WiFi scenario with PD=-82dbm, ED=-62dbm can reach low/medium/high traffic load as an agreement in TR38.889. The assumed coexistence mechanisms are following.

Scheme 1: 11ax PD=ED=-82dBm; NRU ED=-82dBm

Scheme 2: 11ax PD=ED=-72dBm; NRU ED=-72dBm

Scheme 3: 11ax PD=ED=-62dBm; NRU ED=-62dBm

Scheme 4: 11ax PD=-82dBm, ED=-62dBm; NRU PD=-82dBm, ED=-62dBm

Scheme 5: 11ax PD=-82dBm, ED=-62dBm; NRU ED=-72dBm (existing scheme as 5GHz)

Scheme 6: 11ax PD=-82dBm, ED=-62dBm; NRU ED=-62dBm

Scheme 7: 11ax PD=-82dBm, ED=-72dBm; NRU ED=-72dBm

Scheme 8: 11ax PD=-82dBm, ED=-62dBm; NRU PD=-82dBm, ED=-62dBm (with 11a preamble)

Scheme 9: 11ax PD=-82dBm, ED=-72dBm; NRU PD=-82dBm, ED=-72dBm (with 11a preamble)

It can be observed that performance of both 802.11ax and NRU can be improved compared with existing mechanism adopted in 5GHz (scheme 5) when common PD/ED threshold higher than -82 dBm (scheme 2, 3) is adopted. More spatial reuse gain can be achieved by increasing the common ED threshold. The performance also increases with increasing ED threshold in case of single RAT coexistence.

In scheme 4, we assume NRU has its own preamble and uses the same PD level to detect another NRU preamble as Wi-Fi is using to detect another Wi-Fi preamble. The UPT degrades due to lack of spatial reuse in NRU only scenario.

In scheme 8 and scheme 9, we assume NRU has the 11a preamble before the NRU burst which allows 11ax to use the PD threshold to detect the NRU burst, meanwhile NRU can also detect the 11ax burst because there is 11a-like preamble before 11ax burst. In the case both NRU and 11ax are using the same PD threshold of -82dbm to detect the each other signals. We observed that the 11ax performance cannot be improved by introducing 11a preamble before NRU burst (scheme 8 and 9) in coexisting scenarios compared with the existing mechanism defined for 5GHz (scheme 5).

Observation 1: The UPT of NRU will degrade when lower PD than ED is adopted in NRU due to lack of spatial reuse when PD is effective.

Observation 2: UPT of both 802.11ax and NRU will increase while using higher common PD/ED threshold.

Observation 3: The 11ax cannot benefit from introducing 11a preamble before NR-U transmission in coexisting scenario.

Figure 1. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor scenario in low load

Figure 2. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor sceanrio in low load

Figure 3. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor scenario in medium load

Figure 4. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor sceanrio in medium load

Figure 5. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor scenario in high load

Figure 6. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in indoor sceanrio in high load

Figure 7. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in low load

Figure 8. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in low load

Figure 9. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in medium load

Figure 10. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in medium load

Figure 11. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in  high load

Figure 12. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 1 in high load

Figure 13. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in low load

Figure 14. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in low load

Figure 15. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in medium load

Figure 16. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in medium load

Figure 17. Average DL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in high load

Figure 18. Average UL UPT(Mbps) performance for NR-U/Wi-Fi coexistence in outdoor scenario 2 in high load

Conclusion

In this contribution, we made the following observation and proposals from the results of indoor/outdoor coexistence evaluation in 6GHz for NR-U and 802.11ax:

Observation 1: The UPT of NRU will degrade when lower PD than ED is adopted in NRU due to lack of spatial reuse when PD is effective.

Observation 2: UPT of both 802.11ax and NRU will increase while using higher common PD/ED threshold.

Observation 3: The 11ax cannot benefit from introducing 11a preamble before NR-U transmission in coexisting scenario.

- References

Chairman’s notes for RAN1 #95 meeting

Chairman’s notes for RAN#82 meeting

Chairman’s notes for RAN1 AdHoc 1901

Chairman’s notes for RAN1 #92 meeting

- Appendix A

Table A: Summary of simulation assumptions for indoor Sub-7GHz

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30KHz and 60KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

- Appendix B

Table B: Summary of simulation assumptions for outdoor Sub-7GHz

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Micro layer: Random drop(all micro BSs are outdoor) 1 micro BSs per operator per macro BS |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30kHz and 60kHz |
| Macro ISD | 200 m (opt 1) and 300 m (opt 2) |
| Maximum distance inter-operator | 30 m (opt 1) |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback and OLLA |
| Scheduling | Proportional fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

- Appendix C

Table c: Summary of simulation assumptions for 802.11ac/ax

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| MCS |  |  | 802.11ac/ax MCS with 256QAM |
| Carrier Frequency |  |  | 5GHz |
| Carrier Channel Bandwidth |  |  | 20MHz |
| Number of carriers |  |  | 1 |
| Number of users per operator |  |  | Exactly 5 per gNB per 20MHz |
| MCOT |  |  | 4 ms |
| Frame aggregation |  |  | A-MPDU |
| MPDU size |  |  | 1500B MSDU + 14 B header |
| Guard interval |  | 11ac | 0.8us |
|  |  | 11ax | 0.8us |
| Channel Model |  |  | NR UMi street canyon |
| AP Tx Power |  |  | 23dBm |
| STA Tx Power |  |  | 18dBm |
| MAC | Coordination |  | DCF |
|  | SIFS/DIFS |  | 16us/43us |
|  | Detection |  | Energy detection & preamble detection |
|  | RTS/CTS |  | No |
|  | Contention |  | EDCA |
| CCA-PD |  |  | -82dbm for 11ac |
| CCA-ED |  |  | -62dbm for 11ac |
| ACK Modeled |  |  | Immediate ACK |
| OFDM symbol length |  |  | 4 us for 11ac; 13.6us for 11ax |